# Appendix 1: Quick Jumper Setting Reference

### JP1: Clear CMOS memory jumper

Use this 3-pin jumper to clear all the current data stored in the CMOS memory.

| 1 2 3 JP1 Function | Jumper Cap |
| --- | --- |
| Normal operation | Short pins 1-2 |
| Clear CMOS | Short pins 2-3 |

### JP2: Keyboard power on jumper

Use this 3-pin jumper to enable keyboard power on with hot keys or password.

| 1 2 3 JP2 Function | Jumper Cap |
| --- | --- |
| Enable keyboard power on | Short pins 1-2 |
| Disable keyboard power on | Short pins 2-3 |

### JP3, JP6, JP7, JP11: Select Celeron or Joshua Processor for Socket-370

Use these 3-pin jumper sets to select the processor type you are using in the PGA370 processor socket.

| Function JP3 JP6 JP7 JP11 | Jumper Cap |
| --- | --- |
| Intel Celeron 1 2 3 | Short pins 1-2 |
| Cyrix Joshua | Short pins 2-3 |

### JP4: Detect Chassis Open jumper

Use this 2-pin jumper to enable chassis open detection.

| 1 2 JP4 Function | Jumper Cap |
| --- | --- |
| Disable Chassis Open | Open pins 1-2 |
| Enable Chassis Open | Short pins 2-3 |

### JP5: Supend-to-RAM jumper

Use this 3-pin jumper to enable the Suspend-to-RAM function.

| Function JP5 | Jumper Cap |
| --- | --- |
| Enable Suspend-to-RAM 1 2 3 | Short pins 1-2 |
| Disable Suspend-to-RAM | Short pins 2-3 |

### JP8: Set System Bus Frequency to 133 MHz

Use this 3-pin jumper to set the system bus frequency. In the normal setting, the system automatically selects the correct frequency according to the kind of processor installed. In the Force 133 MHz setting, the system uses a 133 MHz system bus even if the processor is designed to operate with a 66 or 100 MHz bus.

| Function JP8 | Jumper Cap |
| --- | --- |
| Normal operation 1 2 3 | Short pins 1-2 |
| Force 133 MHz | Short pins 2-3 |

### JP9: Set System Bus Frequency to 100 MHz

Use this 3-pin jumper to set the system bus frequency. In the normal setting, the system automatically selects the correct frequency according to the kind of processor installed. In the Force 100 MHz setting, the system uses a 100 MHz system bus even if the processor is designed to operate with a 66 or 133 MHz bus.

| Function JP9 | Jumper Cap |
| --- | --- |
| Normal operation 1 2 3 | Short pins 1-2 |
| Force 100 MHz | Short pins 2-3 |

### JP10: Automatic (BIOS) or Manual configuration

Use this 2-pin jumper to automatically (BIOS) or manually set the CPU core voltage and system bus multiplier ratio. When set to manual configuration, use the VID and BF jumpers to define proper configuration. It is recommended that you set this jumper to automatic configuration.

| 1 2 JP10 Function | Jumper Cap |
| --- | --- |
| Automatic configuration | Short pins 1-2 |
| Manual configuration | Open pins 1-2 |

### JP12: Flash BIOS jumper

Use this 3-pin jumper to enable or disable Flash BIOS protection. If enabled, the existing BIOS cannot be flashed with another version.

| Function JP12 | Jumper Cap |
| --- | --- |
| Enable Flash BIOS 1 2 3 | Short pins 1-2 |
| Disable Flash BIOS | Short pins 2-3 |

VID1

VID5

1   2   3

VID: Set CPU core voltage jumpers

Use this 3 x 5-pin jumper set to manually set the CPU core voltage. See later in this chapter for information on the core voltage setting required for the processor that you have installed.

| Volt. | Short Pin Settings |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VID1 | VID2 | VID3 | VID4 | VID5 |
| Auto* | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |
| 2.05V | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 |
| 2.00V | 1-2 | 2-3 | 2-3 | 2-3 | 2-3 |
| 1.95V | 2-3 | 1-2 | 2-3 | 2-3 | 2-3 |
| 1.90V | 1-2 | 1-2 | 2-3 | 2-3 | 2-3 |
| 1.85V | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 |
| 1.80V | 1-2 | 2-3 | 1-2 | 2-3 | 2-3 |
| 1.75V | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 |
| 1.70V | 1-2 | 1-2 | 1-2 | 2-3 | 2-3 |
| 1.65V | 2-3 | 2-3 | 2-3 | 1-2 | 2-3 |
| 1.60V | 1-2 | 2-3 | 2-3 | 1-2 | 2-3 |
| 1.55V | 2-3 | 1-2 | 2-3 | 1-2 | 2-3 |
| 1.50V | 1-2 | 1-2 | 2-3 | 1-2 | 2-3 |
| 1.45V | 2-3 | 2-3 | 1-2 | 1-2 | 2-3 |
| 1.40V | 1-2 | 2-3 | 1-2 | 1-2 | 2-3 |
| 1.35V | 2-3 | 1-2 | 1-2 | 1-2 | 2-3 |
| 1.30V | 1-2 | 1-2 | 1-2 | 1-2 | 2-3 |
| 3.5V | 2-3 | 2-3 | 2-3 | 2-3 | 1-2 |
| 3.4V | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 |
| 3.3V | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 |
| 3.2V | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 |
| 3.1V | 2-3 | 2-3 | 1-2 | 2-3 | 1-2 |
| 3.0V | 1-2 | 2-3 | 1-2 | 2-3 | 1-2 |
| 2.9V | 2-3 | 1-2 | 1-2 | 2-3 | 1-2 |
| 2.8V | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 |
| 2.7V | 2-3 | 2-3 | 2-3 | 1-2 | 1-2 |
| 2.6V | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 |
| 2.5V | 2-3 | 1-2 | 2-3 | 1-2 | 1-2 |
| 2.4V | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 |
| 2.3V | 2-3 | 2-3 | 1-2 | 1-2 | 1-2 |
| 2.2V | 1-2 | 2-3 | 1-2 | 1-2 | 1-2 |
| 2.1V | 2-3 | 1-2 | 1-2 | 1-2 | 1-2 |

*Auto: When all 1-2 pins are shorted, the core voltage will automatically be determined.

BF4

BF4

1   2   3

BF: Set system bus multiplier ratio jumpers

Use this 3 x 4-pin jumper set to manually set the system bus multiplier ratio. See later in this chapter for information on the system bus multiplier ratio setting required for the processor that you have installed.

| Ratio | Short Pin Settings |  |  |  |
| --- | --- | --- | --- | --- |
|  | BF1 | BF2 | BF3 | BF4 |
| Auto* | 1-2 | 1-2 | 1-2 | 1-2 |
| 2 | 2-3 | 2-3 | 2-3 | 2-3 |
| 4 | 2-3 | 1-2 | 2-3 | 2-3 |
| 3 | 2-3 | 2-3 | 1-2 | 2-3 |
| 5 | 2-3 | 1-2 | 1-2 | 2-3 |
| 2.5 | 2-3 | 2-3 | 2-3 | 1-2 |
| 4.5 | 2-3 | 1-2 | 2-3 | 1-2 |
| 3.5 | 2-3 | 2-3 | 1-2 | 1-2 |
| 5.5 | 2-3 | 1-2 | 1-2 | 1-2 |
| 6 | 1-2 | 2-3 | 2-3 | 2-3 |
| 8 | 1-2 | 1-2 | 2-3 | 2-3 |
| 7 | 1-2 | 2-3 | 1-2 | 2-3 |
| Res. | 1-2 | 1-2 | 1-2 | 2-3 |
| 6.5 | 1-2 | 2-3 | 2-3 | 1-2 |
| 1.5 | 1-2 | 1-2 | 2-3 | 2-3 |
| 7.5 | 1-2 | 2-3 | 1-2 | 1-2 |
| 2 | 1-2 | 1-2 | 1-2 | 1-2 |

*Auto: When all 1-2 pins are shorted, the system bus multiplier ratio will automatically be determined.

### PANEL1: Panel connectors for switches and indicators

Use the panel connector to implement the switches and indicators on your system case.

| Function | Pins |
| --- | --- |
| Speaker | +1, 3, 5, 7 |
| 2 1 22 21 Speaker Power LED KeyLock HDD LED Reset SW Suspend SW Power SW Green LED PANEL1 Power Indicator | +2, +4, 6 |
| Keylock | 8, 10 |
| Green Indicator | 13, 14 |
| Hard Disk Indicator | +15, 16 |
| Reset Switch | 17, 18 |
| Suspend Switch | 19, 20 |
| Power Switch | 21, 22 |