# Jumpers and Connectors

## Setting the Jumpers

The table below summarizes the function and jumper settings of each jumper on the SI56P AVIO.  You can refer to the next section ( Graphic descriptions of Jumper Settings) for the correct methods to set jumpers.

| Function |  | Jumper Settings |
| --- | --- | --- |
| CPU TypeH | Pentium 75MHz (50MHz Host Clock) | JP15 short 1-2 JP22 open JP24 open |
|  | Pentium 90MHz (60MHz Host Clock) | JP15 short 3-4 JP22 open JP24 open |
|  | Pentium 100MHz (66MHz Host Clock) | JP15 short 1-2, 3-4 JP22 open JP24 open |
|  | Pentium 120MHz (60MHz Host Clock) | JP15 short 3-4 JP22 open JP24 short |
|  | Pentium 133MHz (66MHz Host Clock) | JP15 short 1-2, 3-4 JP22 open JP24 short |
|  | Pentium 150MHz (60MHz Host Clock) | JP15 short 3-4 JP22 short JP24 short |
|  | Pentium 166MHz (66MHz Host Clock) | JP15 short 1-2, 3-4 JP22 short JP24 short |
|  | Cyrix 6x86-P120+ (50MHz Host Clock) | JP15 short 1-2 JP22 open JP24 short |
|  | Cyrix 6x86-P150+ (60MHz Host Clock) | JP15 short 3-4 JP22 open JP24 short |
|  | Cyrix 6x86-P166+ (66MHz Host Clock) | JP15 short 1-2, 3-4 JP22 open JP24 short |

Continued .....

| Function |  |  | Jumper Settings |
| --- | --- | --- | --- |
| CPU Voltage |  | +3.3V (Standard) | JP16 short 1-2 |
|  |  | +3.385V (VR) | JP16 short 3-4 |
|  |  | +3.525V (VRE) | JP16 short 5-6 |
| External Cache Memory Size | 256KB (default) |  | JP 6 short 1-2 JP 7 short 1-2 |
|  | 512KB |  | JP 6 short 2-3 JP 7 short 1-2 |
|  | 1MB |  | JP 6 short 2-3 JP 7 short 2-3 |
| Mixed/ Pure Cache Setting | Pure |  | JP1 short 1-2 JP2 short 1-2 |
| for Standard SRAM only | Mixed (default) |  | JP1 short 2-3 JP2 short 2-3 |
| Super I/O Setting | Enable (default) |  | JP11 open |
|  | Disable |  | JP11 short |
| DRQ/DACK of Parllel Port For | DRQ1/DACK1 (default) |  | JP12 short 1-2 JP13 short 1-2 |
| ECP Mode | DRQ3/DACK3 |  | JP12 short 2-3 JP13 short 2-3 |
| On-Board VGA | Enable (default) |  | JP27 short |
|  | Disable |  | JP27 open |
| Clear CMOS | Clear |  | JP26 short |
|  | Normal (default) |  | JP26 open |
| BIOS Voltage | Programming Flash ROM With +5 Voltage (default) |  | JP25 short 1-2 |
|  | Programming Flash ROM with +12 Voltage |  | JP25 short 2-3 |

Table 3-1.  Jumper Settings

|  | The table below presents the detailed jumper settings for different CPU clock. For example, if Pentium 100mhz CPU is installed, you should set Host Clock as 66MHz and CPU Core Clock as Host Clock *1.5. |
| --- | --- |

| Function |  | Jumper Settings |
| --- | --- | --- |
| Host Clock | 50MHz | JP15 short 1-2 |
|  | 60MHz | JP15 short 3-4 |
|  | 66MHz (default) | JP15 short 1-2, 3-4 |
| CPU Core Clock | Host Clock * 1.5 | JP22 open JP24 open |
|  | Host Clock * 2 | JP22 open JP24 short |
|  | Host Clock * 2.5 | JP22 short JP24 short |
|  | Host Clock * 3 | JP22 short JP24 open |

## Graphic Descriptions of Jumper Settings

### CPU Type

- Pentium 75MHz (50MHz Host CLK)

- Pentium 90MHz (60MHz Host CLK)

- Pentium 100MHz (66MHz Host CLK)CPU

- Pentium 120MHz (60MHz Host CLK)

- Pentium 133MHz (66MHz Host CLK)

- Pentium 150MHz (60MHz Host CLK)

- Pentium 166MHz (66MHz Host CLK)

- Cyrix 6X86-P120+ (50MHz Host CLK)

- Cyrix 6X86-P150+ (60MHz Host CLK)

- Cyrix 6X86-P166+ (66MHz Host CLK)

### CPU Voltage

### External Cache Memory Size

- 256KB (default)

Pinelined Burst SRAM (This Pipelined Burst SRAM is only supported 256KB 	on this motherboard.)

Or

Standard SRAM

- 512KB

Standard SRAM

- 1MB

Standard SRAM

## Connectors

The following table lists all connectors located on the SI56P AVIO.  They are used to connecting with some peripheral devices to enhance the operating performance of the system.  Please refer to the mainboard layout figure on the next page for the positions of all the connectors.

| Connector | Function |
| --- | --- |
| J1 | 5-pin PS/2 Mouse Connector (Optional) |
| J2 | Power Connector |
| J3 | PS/2 Keyboard Connector (Optional) |
| J4 | AT Keyboard Connector |
| J5 | Feature Connector |
| J6 | VGA Connector |
| J7 | PS/2 Mouse Connector (Optional) |
| J8 | FDD Connector |
| J9 | HD Active LED Connector |
| J10 | COM2 Connector |
| J11 | COM1 Connector |
| J12 | Primary IDE Connector |
| J13 | Printer Connector |
| J14 | Secondary IDE Connector |
| J15 | Battery Connector |
| J16 |  |
| J26 | +12V Fan Power Connector |
| JP8 | IR Connector (Optional) |

Table 3-2.   Connectors

## Board Layout

Figure 3-1.   SI56P AVIO Mainboard Layout

This Page Intentionally Left Blank.