Chapter 1

Overview

Based on the new highly-integrated VIA 580VPX, the mainboard combines blistering PentiumÒ processor performance with support for intelligent diagnostic and power management features to provide a powerful and versatile platform.

With its switching voltage regulator, the VA-502 runs a complete range of Intel PentiumÒ processors, including the Intel Pentium processor with MMXÔ technology, as well as the AMD-K5Ô and Cyrix/IBM 6x86Ô, and is easily upgradable to the Cyrix/IBM MXÔ and the AMD-K6Ô. For added power and performance, the VA-502 takes up to 512KB Pipeline Burst Level II cache and up to 512MB DRAM via four 72-pin SIMM sockets and two 168-pin DIMM sockets which accept high-speed EDO, and lightning-fast SDRAM memory types.

The VA-502 comes with a full set of I/O features conveniently integrated on the rear I/O panel, including two USB connectors. The board also has an integrated PCI Bus Master Enhanced IDE controller with support for the new Ultra DMA/33 protocol, which doubles ATA-2 Hard Disk Drive data transfer rates to 33MB/s while maintaining full backwards compatibility with existing PIO Mode 3, PIO Mode 4 and DMA Mode 2 devices.

Chapter 1 of this manual gives you a brief overview of the VA-502 mainboard, including its main components and features. Chapter 2 contains advice on how to upgrade and install key components on the mainboard, while Chapter 3 provides detailed information about the board’s BIOS settings. For the most up-to-date information about your mainboard and the latest FAQs and BIOS updates, visit FIC Online at www.fic.com.tw.

# Package Checklist

Please check that your package contains all the items listed below. If you discover any item is damaged or missing, please contact your vendor.

- The VA-502 mainboard

- This user manual

- One IDE HDD cable

- One floppy disk drive cable

- One printer and COM1 cable

- One COM2 cable

- Software utility

- One USB riser card (optional)

## The VA-502 Mainboard

# Main Features

The VA-502 mainboard comes with the following high-performance features:

- Easy Installation

- Award BIOS with support for Plug and Play, auto detection of Hard Drive

- and IDE features, and MS Windows 95 compatible.

- Flexible Processor Support

Onboard 321-pin ZIF socket and switching voltage regulator support         |||||||||complete range of leading-edge processors:

Intel PentiumÒ P55C with MMXÔ technology 166/200/233 MHz processors.

Intel PentiumÒ P54C/P54CS 90/100/120/133/150/166/200 MHz processors.

|||||||||AMD-K6Ô-166 (166 MHz) / K6-200 (200 MHz) / K6-233 (233 MHz)    |||||||| processors.

|||||||||AMD-K5Ô- PR90 (90 MHz) / K5-PR100 (100 MHz) / K5-PR120 (90     |||||||||MHz) / K5-PR133 (100 MHz) / K5-PR150 (105 MHz) / K5-PR166 (116 |||||||||MHz) / K5-PR200 (133 MHz) processors.

||||||||Cyrix 6x86MXÔ- PR166 (150 MHz) / 6x86MX-PR200 (166 MHz) / 6x86- |||||||||MX-PR233 (200 MHz) / 6x86MX-PR266 (233 MHz) processors. Please |||||||||read page 23 for details.

|| || | Cyrix 6x86Ô- PR133+ (110 MHz) / 6x86-PR150+ (120 MHz) / 6x86-    |||||||||PR166+ (133 MHz) / 6x86-PR200+ (150 MHz) processors.

|||||||||IBM 6x86MXÔ- PR166 (150 MHz) / 6x86MX-PR200 (166 MHz) / 6x86- |||||||||MX-PR233 (200 MHz) / 6x86MX-PR266 (233 MHz) processors. Please |||||||||read page 25 for details.

||||||||IBM 6x86Ô- PR133+ (110 MHz) / 6x86-PR150+ (120 MHz) / 6x86- |||||||||PR166+ (133 MHz) processors. |

|    || || | |

- Leading Edge Chipset

- VIA 580VPX chipset, including a CPU interface controller, advanced

- cache controller, integrated DRAM controller, synchronous ISA bus

- controller, PCI local bus interface, integrated power management unit.

- Ultra-fast Level II Cache

Supports onboard 256KB/512KB synchronous PBSRAM direct-mapped writ||write-back cache memory.

- Versatile Main Memory Support

- Accepts up to 512MB RAM in two banks using four SIMMs of 8, 16, 32,

- 64, 128MB with support for FPM and EDO DRAM and two DIMMs of 8,

- 16, 32, 64MB with support for SDRAM and EDO DRAM.

- ISA & PCI Expansion Slots

- Three 16-bit ISA and four 32-bit PCI expansion slots provide all the room

- you need to install a full range of add-on cards.

- USB Support

- Two USB ports on an optional riser card allow convenient, high-speed Plug |||||||||and Play connections to the growing number of USB compliant external |||||||||peripheral devices on the market.

- Enhanced PCI Bus Master IDE Controller

- Integrated Enhanced PCI local bus IDE controller with two connectors

- supports up to four IDE devices such as Hard Disk, CD-ROM or Tape

Backup drives via two channels for high speed data throughput. This

controller supports PIO Modes 3 and 4, and DMA Mode 2 for optimized

system performance.

- Super Multi I/O

- Integrated SMC FDC 37C669/UMC UM8669F chipset features two ||||||||16550A UART compatible serial ports, one EPP/ECP capable parallel port, ||||||||one IR port, and one Floppy Disk Drive connector.

# Advanced  Features

This mainboard comes equipped with the most advanced new features that not only optimize the performance of the latest processors but also enhance the manageability, power management capabilities, and user-friendliness of your system.  This section provides detailed information on these features, and how they are implemented on the mainboard.

- Optimized Intel MMXÔ Performance

The mainboard utilizes the advanced features of the VIA 580VPX to optimize the unrivaled performance of the Intel PentiumÒ processor with MMXÔ technology, allowing you to enjoy a richer video, audio, digital imaging and communications experience from the latest generation of multimedia software. To provide you with additional flexibility, the mainboard also supports other leading-edge processors featuring Intel’s MMXÔ technology, including the AMD-K6Ô processor.

- Lightning-Fast SDRAM Performance

The mainboard supports the new generation of lightning-fast SDRAM (Synchronous Dynamic Random Access Memory) via its two onboard 168-pin DIMM sockets. SDRAM delivers an added boost to overall system performance by increasing the CPU-to-memory data transfer rate to 528MB/sec compared to 264MB/sec for conventional EDO DRAM. SDRAM performance on the VA-502 is further boosted by the board’s integrated I2C controller, which optimizes the memory timing settings.

- Blistering Ultra DMA/33 HDD Performance

With its integrated Enhanced PCI Bus Master IDE controller that supports the new Ultra DMA/33 protocol, this mainboard doubles HDD data transfer rates to 33MB/sec, compared  to 16MB/sec for conventional PIO Mode 3, PIO Mode 4, and DMA Mode 2 devices. By reducing the CPU’s workload and increasing CPU utilization, Ultra DMA/33 significantly improves system performance when running applications under WindowsÒ 95 and WindowsÒ NT environments. The Ultra DMA/33 protocol is completely backward compatible with conventional ATA-2 HDD devices; so the mainboard also supports existing PIO Mode 3, PIO Mode 4 and DMA Mode 2 devices using the same cable.

With the integrated Enhanced PCI Bus Master IDE controller you can connect up to four Enhanced IDE peripheral devices to your system. All devices are categorized in the same way that IDE hard disks were configured in the past, with one device set as the master device and the other as the slave device. We recommend that Hard Disk Drives use the primary IDE connector and that CD-ROM Drives utilize the secondary IDE connector for optimum system performance.

## Infrared (IR) Connections

This mainboard features support for highly-sophisticated SIR technology, which allows bi-directional and cordless data transactions with other IrDA compliant computers and peripheral devices using infrared as a medium. This transmission is carried out in either Full Duplex Mode or Half Duplex Mode. The former allows simultaneous data transmission and reception, while the latter disables the reception when transmission occurs.

The I/O chipset on this mainboard features a SIR interface that is fully compliant with the IrDA standard. An IrDA device can be installed via a 9-pin D-type connector in the rear panel of the computer which is linked by a cable to the onboard IrDA pinhead.

The serial port COM2 on this mainboard is designed to be a SIR compliant port. If you wish to install the SIR connection feature, you need to adjust the BIOS option for high-speed performance.

## Universal Serial Bus (USB) Functionality

This mainboard features integrated support for state-of-the-art USB technology, which provides high-speed and easy-to-use Plug & Play connections to the future generation of external peripherals, such as keyboards, mouse, monitors, game devices, scanners, printers, and fax/modems.

USB overcomes conventional I/O bottlenecks by combining the I/O ports into a single dual-channel connector. For optimum ease of use and flexibility, USB not only allows the automatic detection and configuration of peripherals after installation, but also enables the simultaneous connection.

This mainboard features an optional USB riser card (see the photo below) with bracket that can be installed in one of the I/O expansion slots on the rear panel of the system, as shown in the illustration below. It provides fast and convenient Plug and Play peripheral connections outside your computer, allowing you take full advantage of the universal functionality and flexibility of USB technology.