## Contents:

Chapter 1

### Specification.......................................................................2

### Chapter 2 Hardware Configuration

### 1. Layout Placement ..............................................................3

### 2. Jumper Setting ...................................................................4

3. CPU Quick Index ..............................................................6

4. ALS120 Introduction……………………………………7

Chapter 3 BIOS Setup

Section 1. Main menu .........................................................…9

- Section 2.Standard CMOS Setup ......................................…11

##### Section 3. BIOS Features Setup .....................…...................13

- Section 4. Chipset Setup/Integrated Peripherals Features Setup…..16

- Section 5. Power Management Setup..................…...............22

- Section 6. PnP/ PCI Configuration Setup..............….............26

Chapter 1

1. Specification

＠Chipset : INTEL 82443 BX AGP set.

＠CPU : INTEL PENTIUMⅡCPU at 233MHz~450MHz (SLOT1).

*Host bus frequency: 66MHz / 100MHz.

＠Four 168 pin DIMM sockets to support (8 , 16 , 32 , 64 , 128 or 256MB) 3.3V EDO DRAM or SDRAM.

＠Switching Voltage Regulator.

＠Optional GL518SM : with Digital Temperature Sensor , Thermal Watchdog ,

Voltage Monitors, Fan Speed Monitors & ON / OFF Control.

＠Supports one AGP slot, three ISA slots and three PCI slots.

*Accelerated Graphics Port Supports AGP 66MHz / 133MHz (2X) .

＠Dual Master IDE connectors support up to four IDE devices.

＠Dual channel Ultra DMA / 33 Bus Master IDE ports (Support up to 4 IDE devices and LS-120MB Floppy Drive ) Support:

*PIO Mode 3&4.

*DMA Mode 2.

*Ultra DMA / 33.

＠Supports USB Connector.

＠Supports PS/2 keyboard and mouse connector.

＠One FDC port supports two devices up to 2.88MB.

＠Supports IrDA TX / RX header.

＠Supports ACPI.

＠When you set up your clock at 100MHz, Please use RAM Module : SDRAM under –8 or include –8 (80NS).

@Highly-integrated plug and play audio subsystem for IDE CD-ROM.

@Subsystem control logic.

@	Built-in 3D Sound Effect Processor.

@	Digital to analog and analog to digital conversions.

@	Mixer functions.

@	Built-in ALSFM Synthesizer.

@	Enhanced Game/MIDI port support.

@ALS120 fully supports the ISA Plug and Play standard. For non-ISA Plug and Play compatible systems, a software utility “ISA Configuration Utility” (ICU) is available that sets the IRQ and DMA addresses.

### Chapter 2 Hardware Configuration

### 1. Layout Placement

2. Jumper Setting

#### J12: CPU Voltage Frequency Ratio Jumper Setting

| Ratio Jumper | X3.0 | X3.5 | X4.0 | X4.5 | X5.0 | X5.5 | X6.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J12 | 1 2 | 1 2 | 1 2 | 1 2 | 1 2 | 1 2 | 1 2 |

#### J24: ROM size              J25: Flash ROM Type Voltage   J22: CMOS Clear Function Select

| Jumper | 2M | 1M |  | Jumper | +12V | +5V |  | Jumper | Normal | Clear CMOS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J24 | 1 | 1 |  | J25 | 1 | 1 |  | J22 | 1 | 1 |

#### JP1: CPU Clock Generator Jumper Setting

| Jumper | 66MHz | 100MHz or Auto |
| --- | --- | --- |
| JP1 |  |  |

#### JP404、JP405 : Sound  Line Out / Speaker Setting

|  | LINE OUT | SPK OUT |
| --- | --- | --- |
| JP404 | 1 3 5 | 1 3 5 |
| JP405 | OFF | 1 2 |

J403: MIC Mono/Stereo

|  | ##### J403 |
| --- | --- |
| ##### Mono |  |
| ##### Stereo |  |

J404、J405: Sound  Disable/ Enable

|  | ### SOUND CHIPS DISABLE | ### SOUND CHIPS ENABLE |
| --- | --- | --- |
| ##### J404 |  |  |
| ##### J405 |  |  |

#### J25: Other Jumper Setting

+

+

SMI_LED

+

IR

29

1

PS_ON

SPK +

P_LED

RST

H_LED

SMI_SW

#### Other Jumper Setting

| JP6 | CHA FAN |
| --- | --- |
| JP7: SB-LINK PCI Sound Card connector | SB-LINK connector can support Creative-compatible PCI sound card. If you have a Creative-compatible PCI sound card installed, it is necessary to link the card to SB-LINK connector for compatibility issue under DOS environment. |
| J11 | Up: PS/2 Mouse port Down: Keyboard port |
| J13 | ATX Power Connector |
| J14 | CPU FAN |
| J16: EISCA Cooler | External Thermal Sensor & Temperature Controller. Note: EISCA (Enhanced Intelligent System Cooler Architecture) |
| J17 | COM1 |
| J18 | COM2 |
| J19 | LPT Connector |
| J20 | FDD Connector |
| J21: LAN_WAKEUP | By using a network card that supports this feature and a network management software (such as ADM), you can wake up a system through a local area network. |
| U26 | AGP Slot |
| U27 | USB Connector |
| JP401 | CD AUDIO IN（MITSUMI） |
| JP402 | CD AUDIO IN（IDE1） |

3. CPU Quick Index

| CPU TYPE | PRINTING | MHz | MUL / BUS | J12 | JP1 |
| --- | --- | --- | --- | --- | --- |
| Pentium® II 233 MHz | 233MHz | 233 | X3.5 / 66 | 1-2&7-8 | CLOSE |
| Pentium® II 266 MHz | 266MHz | 266 | X4.0 / 66 | 3-4&5-6&7-8 | CLOSE |
| Pentium® II 300 MHz | 300MHz | 300 | X4.5 / 66 | 3-4&7-8 | CLOSE |
| Pentium® II 333 MHz | 333MHz | 333 | X5.0 / 66 | 5-6&7-8 | CLOSE |
| Pentium® II 350 MHz | 350MHz | 350 | X3.5 / 100 | 1-2&7-8 | OPEN |
| Pentium® II 400 MHz | 400MHz | 400 | X4.0 / 100 | 3-4&5-6&7-8 | OPEN |
| Pentium® II 450 MHz | 450MHz | 450 | X4.5 / 100 | 3-4 & 7-8 | OPEN |
| Pentium® !!! 450 MHz | 450MHz | 450 | X4.5 / 100 | 3-4 & 7-8 | OPEN |
| Pentium® !!! 500 MHz | 500 MHz | 500 | X5.0 / 100 | 5-6 & 7-8 | OPEN |

| CPU TYPE | PRINTING | MHz | MUL / BUS | J12 | JP1 |
| --- | --- | --- | --- | --- | --- |
| Pentium® Celeron 266 MHz | 266MHz | 266 | X4 / 66 | 3-4&5-6&7-8 | CLOSE |
| Pentium® Celeron 300 MHz | 300MHz | 300 | X4.5 / 66 | 3-4&7-8 | CLOSE |
| Pentium® Celeron 300A MHz | 300MHz | 300 | X4.5 / 66 | 3-4&7-8 | CLOSE |
| Pentium® Celeron 333 MHz | 333MHz | 333 | X5/ 66 | 5-6&7-8 | CLOSE |
| Pentium® Celeron 366 MHz | 366MHz | 366 | X5.5 / 66 | 7-8 | CLOSE |
| Pentium® Celeron 400 MHz | 400MHz | 400 | X6 / 66 | 1-2&3-4&5-6 | CLOSE |

4. ALS120 Introduction

The ALS120 Integrated Audio Subsystem is a single chip designed for plug and play specification which minimizes support and installation issues. This highly-integrated plug and play audio subsystem for IDE CD-ROM interface incorporates the following functions:

• 	Subsystem control logic.

•	Built-in 3D Sound Effect Processor.

• 	Digital to analog and analog to digital conversions.

• 	Mixer functions.

• 	Built-in ALSFM Synthesizer .

• 	Enhanced Game/MIDI port support.

• 	The ALS120 fully supports the ISA Plug and Play standard. For non-ISA Plug and Play compatible systems, a software utility “ISA Configuration Utility” (ICU) is available that sets the IRQ and DMA addresses. Sound card implementations designed around the ALS120 are completely jumper and switch free.

Input/Output Connections

Phonejacks, connectors, and headers are used to connect other devices to the sound cards. A phonejack is a single hole receptacle for a phoneplug and headers and connectors are multi-hole receptacles for multi-pin plugs. The Mainpeaker Out, Line Out, Line In, and Microphone In connections are phonejacks, the Game/MIDI Port is a D-shell 15-pin connector, and the Wavetable, CD-ROM audio, CD-ROM data/external controller connections are headers of various sizes.

Speaker Out - The Speaker Out phonejack provides the built-in power amplifier outputs for the left and right stereo channels.  The power amplifier has a maximum output power of two watts per channel with four ohm speakers.

Line In - The Line In phonejack is used to attach monaural or stereo devices such as a cassette, Digital Audio Tape, or Minidisc players for playback, mixing, or recording.

Mic In - The Microphone In phonejack is used to attach a monaural microphone for live audio input for playback, mixing, or recording.

Game/MIDI Port - The Game/MIDI Port connector is used to attach a joystick for game interaction or to attach an external MIDI device for playback, mixing, or recording.

ATAPI IDE/Sony CD-ROM Audio - The ATAPI IDE/Sony CD-ROM Audio connector is used to connect the audio cable from either an ATAPI IDE or Sony CD-ROM drive for playback, mixing, and recording.

Mitsumi CD-ROM Audio - The Mitsumi CD-ROM Audio connector is used to connect the audio cable from a Mitsumi CD-ROM drive for playback, mixing, or recording.  Only one of the two CD-ROM audio connectors may be used at a time.