INSTALLING THE MP SPECIFICATION HAL

The MP specification is a MultiProcessor Specification proposed by Intel for an industry standard multi-processor platform. To install and run the MP specification HAL with Windows NT**, you need a MP specification compliant system. You can install the MP specification HAL either during regular Windows NT installation or later, when you have a running NT system.

This version of the HAL will run on MPS Version 1.1 compliant system with Windows NT 3.1 and Updates 1 and 2.

1.	Installing the MP specification HAL during Windows NT Installation:

To install the MP specification HAL during regular Windows NT installation, you need the Windows NT OS installation disk and CD provided by Microsoft and the MP specification HAL Installation Disk. To install the MP specification HAL, you need  to do the following:

·	Insert the Windows NT installation disk in the drive and the NT  installation CD in the CD-ROM. Power up your computer.

·	Press <ENTER> to continue Windows NT installation when the sign-on  welcome message        comes up. The install program will scan for SCSI adapters first. Choose the ones suitable for your system.

·	Choose "Custom Setup" (by pressing "C").

The install program then checks your system and comes up with a list of system resources.

·	Choose "Computer" in this resource list.

You get a list of available options for your computer.

·	Choose "Other" in this list.

The install program asks you to insert the manufacturer provided install disk. Insert the MP specification HAL Installation Disk and press <ENTER>

·	Use the regular Windows NT installation process from here on.

The Windows NT install program will now copy all other files needed for Windows NT from the installation CD provided by Microsoft. It will automatically copy the multiprocessor version of the Windows NT kernel that is needed to run the MP specification HAL.

2.	Installing the MP specification HAL on an existing Windows NT System:

You can also install the MP specification HAL on a MP specification compliant system after regular NT installation. You may be currently running either the uniprocessor version or the multiprocessor version of NT.

a) To install the MP specification HAL on an existing uniprocessor NT installation:

The MP specification HAL requires the multiprocessor version of the NT kernel provided by Microsoft on the NT installation CD. You need to copy the multiprocessor NT kernel to your systems directory as follows:

·	Open a DOS command shell and go to your NT systems directory.

cd %SystemRoot%\System32

·	Save your current uniprocessor kernel file "ntoskrnl.exe" to another file.

copy ntoskrnl.exe ntkrnlup.exe

·	Save the current HAL you are running

copy hal.dll halsaved.dll

·	Expand/Copy the MP kernel from the Windows NT installation CD.

For example, if your NT installation CD is mapped as drive e:, you need to type the following from the DOS command shell:

expand e:\i386\ntkrnlmp.ex_ ntkrnlmp.exe

copy ntkrnlmp.exe ntoskrnl.exe

·	Copy the MP specification HAL "mpshal11.dll" from the MP specification HAL installation disk:

copy a:\mpshal11.dll .

copy mpshal11.dll hal.dll

·	Reboot your computer

b) To install the MP specification HAL on an existing multiprocessor NT installation:

If you are currently running the multiprocessor version of the Windows NT kernel, you only need to copy the MP specification HAL in the NT systems directory.

·	Open a DOS command shell and go to your NT systems directory.

cd %SystemRoot%\System32

·	Save the current HAL you are running

copy hal.dll halsaved.dll

·	Copy the MP specification HAL "mpshal11.dll" from the MP specification HAL Installation disk:

copy a:\mpshal11.dll .

copy mpshal11.dll hal.dll

·	Reboot your computer

____________

** Windows NT is a trademark of Microsoft Corporation.