ENGINEERING RELEASE NOTE

S3 Savage4 NT 4.00 Driver

Version 8.15.03

June 11, 1999

# Purpose

Fix bugs from previous release. It runs on rev B, C and D.

# Products Supported

Savage4/4+ rev B, rev C, revD

# Software Requirements

Microsoft NT4.00 (service pack 4 is recommended).

The driver includes DirectX support at the level provided by NT4.00 service pack 3 (sometimes called DirectX 2.5).

Resolution, color depth, and refresh rate support is dependent on the BIOS used with the driver.  The driver will specifically remove support for any color depth other than 8,16, or 32, however.

# Applications/Utility Components

There are no applications/utility components included with this release.  However, the S3SWITCH and S3GAMMA utilities should be installed separately.

# Installation Instructions

Use the standard method for installing NT 4.0 drivers: install from Settings page of ControlPanel/DisplayProperties component. The installation disk for this release should include the following files:

- VERSION.TXT

- README.TXT

- S3SAVAG4.INF

- S3MINI.SYS

- S3SAVAGE.DLL

- S3MICD.DLL

## Bug Fixes2

# NT4.00_8.15.03

- Performance driver.

# NT4.00_8.15.02

- Fix CR27506 - FIFO noise showed while WinBench 99 running for NEC.

# NT4.00_8.15.01

- Support Savage4 Rev D. Enable hardware cursor on dual CPU system for Rev D.  No CR78 adjust for Rev D.

# NT4.00_8.14.04

- Fix CR26818 - Mouse cursor disappear when move over the running AVI file area.

- Fix CR27506 - FIFO noise showed while WinBench 99 running.

# NT4.00_8.14.03

- Fix the DCT 1.50 GDI stress hang up.

- Fix the Ancient Pathways screen saver hang up.

- Fix CR27367 - ICD-Screen Saver: System crashes with OpenGL screen savers at 16-bit and 32-bit color depth.

# NT4.00_8.14.02

- Go back to use hardware cursor, Use CR78=0 to fix the bits mismatch.

# NT4.00_8.14.01

# ICD

- Fix CR26449. If you find any more problems with GLDemo, be sure to test on at least two systems and exactly report the steps necessary to reproduce the problem.

# NT4.00_8.13.02

- Fix the extra line for Japanese photoretouch in 16bpp and 32bpp modes and 256 color mode hang problem.

# NT4.00_8.13.01

# ICD

- Bugfixes from 8.12.03 didn’t work. New attempt to fix CR25964, CR26447, CR26448.

- CR26449 is not yet fixed. Please note the test recommendations at the end of the file.

- Reserve the last 4K frame buffer for BIOS use.

- Fix the GetCurrentScanLine for LCD.

- Fix the line draw problem for SGRAM

# NT4.00_8.12.03

# ICD

- Fix CR25964, CR26449, CR26447, CR26448.

# NT4.00_8.12.02

- Fix the hang  for mode change from any mode to 16x12x8 if panel is on and then turn off.

# NT4.00_8.12.01

- CR25962 - 2D- MODE CHANGE: If S3Switch2 utility is installed, sys hangs if mode change from10x7x16 to 16x12x8.

# NT4.00_8.11.15

- Fix the ICD hang up for WHQL on some systems.

# NT4.00_8.11.14

- Fix the hang up for WHQL on some systems.

# NT4.00_8.11.13

- Fix the bits mismatch for dynamic hardware/software cursor.

# NT4.00_8.11.12

- Use Dynamic hardware/software cursor.

- Adjust FIFO parameter for mode 1600x1200x32@60.

# NT4.00_8.11.11

- Use software cursor.

- CR26468 - WHQL 3D TEST (DUAL CPU ONLY): 8.11.10, Dr. Watson conform.exe error, aborting test after 2 hrs. & multipe errors

- CR26469 - HCT LAB- WHQL: on COMPAQ/Gainward board & NEC boards, fails to display 8x6 & above modes if Multi-system data-switch is used (splitter box).

# NT4.00_8.11.10

- Use software cursor and turn off stream for dual cpu system.

# NT4.00_8.11.09

- CR25875 - 2D (MPEG w/ SPEEDY on DUAL CPU): 8.11.04->08, system hard hangs after 30 minutes to 2 hrs, less than 1 minute w/ Dirloop.

- CR25954 - DDRAW- DUAL CPU (DCTv100): HCT Short or BLT test from Manual/DDraw causes a GPF hang, IRQL_NOT_LESS_OR_EQUAL.

- CR25952 - 2D- DFP- (AMOVIEw/TASK MANAGER): 8.11.06->08, FIFO noise occurs at several modes, CRT is OK.

- CR25951 - 2D- 8.11.06->08, If S3Switch2 util is installed, cursor is mapped & restricted to a 1/2 inch area along top of screen at 10x7x16bpp@60Hz

- CR25876 - 2D- DFP only (FLIPCUBE): 8.11.04->06, blue line noise occurs at 16 & 32bpp modes.

- CR25962 - 2D- MODE CHANGE(DUAL CPU): 8.11.06->08, If S3Switch2 utility is installed, sys hangs if mode change from10x7x16 to 16x12x8.

- CR26066 - ICD- Half-Life: No textures on weapons, hands, characters.

- CR26067 - ICD- Half-Life: Slow performance overall (less than one frame per second)

# NT4.00_8.11.08

- CR25504 - ACCESSORIES/ ANALOG CLOCK: 8.11.07, second hand is not moving properly, it only moves at 15 second intervals.

- CR25756 - CPQ 8.11.03: Switch DOS full screen to window while enable 3D Maze (OpenGL) screen saver in 16bpp or 32bpp modes cause "Dr. Watson" error message.

- CR25757 - CPQ 8.11.03: Play Foxbear or 2D Flipcube while enable 3D Pipes (OpenGL) screen saver cause "Dr. Watson" error message. (reference: CR25544).

- CR25860 - 2D- Run WinBench99 cause system random hang. (Still exist in 8.11.05->07).

- CR25953 - 2D- DUAL CPU PII 300LX(MPEG/AVI): stretching clip causes system to hang at several modes, no stretching required on DUAL PII 450BX sys.

- CR25975 - DOCUMENTATION - VERSION TEXT: 8.11.07 and all previous, Build date indicates 1998, it should be 99.

- CR26054 - 2D- Speedy:8.11.07, Face doesn't draw after on-the- fly mode change.

- CR26063 - ICD- SCREENSAVER: 8.11.07, 3D text object in small view box becomes blocky if Flying objects saver is activated.

- CR26138 - WHQL:- WIN2K Build 1988 on DUAL CPU SYS: 8.11.07, Stop error, KMODE_EXCEPTION_NOT_HANDLED 1E(1D, 03, 00, 54) at boot

- CR26168 - 2D-  WHQLWinBench 98 Video Test Section: 8.11.07, Cinepack video corruption at full screen.

# NT4.00_8.11.07

- CR25917 - open MPEG clip or window on desktop, will casue wrap around problem on right edge of desktop.

- NO CR - HCT low resolution fail

- NO CR - Winbench 99 hang

- ICD:

- CR25756 DOS full screen 3D maze (16 bit) appears to work completely. Because 3D maze is software rendering in 32 bit mode, there is a much larger window of opportunity for the display mode change to wreak havoc with the frame buffer virtual address and cause a Dr. Watson.

- CR25757 Foxbear/Flipcube is improved. Dr. Watson errors are now significantly less frequent and 3d pipes screen saver doesn't always recover properly when foxbear/flipcube is terminated. (i.e. strange colors, double images, and bleed through into an occluding window).

- CR25544 System doesn't hang. Rare Dr. watson, and 3D pipes may not properly recover. (i.e. strange colors, double images, and bleed through into an occluding window).

- Dlist conformance test failure

- Large texture in QuakeIII (map test_fog2)

- Texture flashing in QuakeII 8x6x32

- Fix clipping of flat shaded triangles.

# NT4.00_8.11.05

- CR25422 - AMOVIE/DESKTOP: 8.00.07->8.11.03 S3&CPQ, desktop shifts to the left about 1/8", as soon as a clip is opened, goes back when closed.

- CR25423 - DFP&CRT, TASK MANAGER: 8.00.07->8.11.03CPQ, moving/resizing Performance window causes FIFO noise at modes 10x7x16@60Hz.

- CR25876 - 2D- DFP only (FLIPCUBE): 8.11.04->06, blue line noise occurs at 16 & 32bpp modes

- CR25477 - Xing DVD Player: system hangs when playing DVD.

- ICD:

- CR25281 - OpenGL: WHQL: DCT OpenGL test hangs system

- CR25530 - PIPES SAVER or GLPERF: 8.10.02, objects from applet samples are copied onto an overlaying window at 16/32bpp modes.

- CR25576 - CPQ 8.11.02 : run Screen Saver 3D Pipes (OpenGL), appears 'black rectangle blocks' .

- Some fixes for mode switch problems.

- Antialiased lines in 8.11.05 were broken, should be fixed with this release.

# NT4.00_8.11.04

- CR25794: DFP: HW cursor doesn’t line-up with pointer handles (hotspot)at 640x480 &800x600

- CR25795: DFP: Video clips are offset & desktop is not centered.

- CR25797: DFP: 2D Flipcube or Foxbear hangs system at 8x6x16/32, 10x7x32

- CR25845: DFP: Winbench99: system hangs when direct Draw/Animate color Depth, 8 bit tests is ran.

- CR24998: Video play hangs dual CPU system

- CR25001: SPEEDY on DUAL CPU Systems Only: 8.00.02->06, Launch Speedy, press F2 twice, System hard hangs (mode tested 10x7x16bpp @85Hz).

- CR25007: WINTACH BenchMark Test on DUAL CPU SYS: 8.00.02->06, System hard hangs when test is launched, all modes.

- CR25249: WB98/99 CPU speed423MHx at Ddraw/Fill 32bpp

- CR25420: clipping a moveable clip offscreen right will cause garbage at left edge of desktop.

- CR25422: desktop shifts to the left about 1/8”, as soon as a clip is openedd, goes back when closed.

- CR25423: DFP: FIFO noise in Performance window.

- CR25509: DFP: Clipping a video offscreen right will cause a banding noise about 1/8” width

- CR25538: AMOVIE clips: moving app offscreen right will cause pixel drops along left edge of desktop

- CR25581: Can’t play overlay in interlaced modes.

- CR25581: Corruption on  3D Pipe openGL screen saver in interlaced modes.

- CR25757: CPQ 8.11.03: Play Foxbear or 2D Flipcube while enable 3D Pipes (OpenGL) screen saver cause "Dr. Watson" error message. (reference: CR25544).

- ICD:

- CR25281 - OpenGL: WHQL: DCT OpenGL test hangs system.

- CR25282 - OpenGL: Quake2: Crosshair and console test is large and blurry. (Alpha Blending issue)

- CR25289 - 3D Labs Demo X29: OpenGL: The plane is missing. Some flashing

- CR25292 - MARS ROVER  (IBM Test Case OGL1): 8.00.06->8.11.03, gradient sky texture looks choppy (jagged) not smooth, 16 bpp only, 32bpp is OK.

- CR25401 - Sin: 640x480 (in-game): Palletized textures broken. When 8-bit textures are active then all textures and overlay are rainbow colored.

- CR25530 - PIPES SAVER or GLPERF: 8.10.02, objects from applet samples are copied onto an overlaying window at 16/32bpp modes.

- CR25753 - CPQ 8.11.03: 3D Maze (OpenGL) screen saver cause "Dr. Watson" error message in interlace mode 1024x768x32bpp @43.

- CR25754 - CPQ 8.11.03: Draw 3D Maze (OpenGL) screen saver out of box in interlace modes.

# NT4.00_8.11.03

- CR25558: Run SPEEDY in any mode cause system hang up.

- Part of CR25577: There are some corruption when run Foxbear in interlace modes.

- CR25579: Draw pixel on edge of window when move it cover text font (pixel drop).

- CR25580: Screen Saver shows only black screen.

- CR25581: Can't play OVERFLAY in 16bpp and 32bpp interlace modes  (butterfly image incorrect in 8bpp interlace modes).

- CR25554: one mode is missing in the mode table of readme file.

- CR25528: Chip Type in DISPLAY PROPERTIES needs to be updated to Savae4.

- ICD:

- CR25539 - Viewperf6.1: True Color mode: Black overlay with repeating 1 1/2  inch rectangles which  the background shows through

- CR25503 - Opengl: Viewperf: PROCDRS hangs in loading phase of test.

- CR25285 - Indy 3D: OpenGL application: Click INDY 3D official and the system will hard lock.

- CR25542 - 3D Screen Saver: Blank (Black backlit) screen when "Screen Saver" tab is active and the screen saver activates after set "Wait Time"

- CR25580 - 8.11.02 : Screen Saver shows only black screen

# NT4.00_8.11.01

- Rev B support

# NT4.00_8.10.02

- CR25381 –  Extra horizontal lines in Winbench 98.

- CR25384 –  Random black lines in Corel Draw 6.0.

- CR25284 – 3D Studio Max: Will not boot up (OpenGL must be chosen as API).

- CR25502 – Opengl: Viewperf: DX-04 Dr Watson error in last phase of test.

# NT4.00_8.10.01

- CR25281 – DCT OpenGL test hang.

- CR25283 – Viewperf hang.

- CR25286 – Glaze: preset 2 hang.

# NT4.00_8.00.07

- CR24936 - Control Panel(mouse properties):8.00.02->06, phantom white vertical lines appear next to mouse graphics.

- CR25071 - 8.00.05:There are corruption on the left side of window when move window in 16bpp or 32bpp modes.

- CR25126 - Microsoft Office97 APPS:8.00.05->06, clipping a moveable app-window offscreen right will cause garbage at left edge of desktop.

- CR25221 - 8.00.06:Run intro to NT test in mode 1280x1024x8@85 cause whole screen corruption.

- CR25222 - Task manager(Perf. Tab) w/40K Bios:8.00.06, moving/resizing window cause FIFO noise at modes 1024x768x32bpp @130 Hz

- CR24828 - 8.00.02. There are some FIFO noise when run Winbench99.

- CR25228 - MS Office 97 App w/Display Box:8.00.06, overlapping display box over an office app menu bars causes pixel drop alone left side in 16 and 32 bpp modes.

- CR25002 – COREL DRAW 6.0(IBM Test Case SCEN2):  8.00.02->06, Resizing Tutorial box corner over Rulers will cause portions of box to be copied onto the ruler.  Application bug.

# NT4.00_8.00.06

- Test ICD.  Suggested tests include:  OpenGL Conformance tests, Quake, Quake2, 3d Studio Max, ViewPerf, GLPerf, Tspeed3, standard NT-provided OpenGL screensavers.

# NT4.00_8.00.03

- Run WinBench99.

- Known Limitations or Issues

- Supported number of simultaneously running OpenGL contexts is limited to 8.

- Trilinear MIPmap filtering doesn’t work in Glaze v3.0.

## Test Recommendations

ICD

The multiple pipes screensaver issue is known and will be fixed at some later time. Please try to reproduce the problems regarding all screensavers on NEC boards.