SiS IDE Driver Installation For Windows NT

=============================================

^NOTICE

If your Microsoft NT is installed from floppy, please start

SiS IDE Driver installation from step (01).

If your Microsoft NT is installed from CD-ROM, please start

SiS IDE Driver installation from step (05).

01) From the "Main" Group in "Program Manager", Start "^Control Panel".

02) From the "Control Panel", choose "^Devices".

03) Use the "Start_up" button to change the "^Scsidisk" device's startup

setting from "disable" to "boot".

04) Use the "Start_up" button to change the "^Atdisk" device's startup

setting from "boot" to "disable".

-----------------------------------------------------------------------------

05) From the Options menu in "^Windows NT setup", choose

"^Add/Remove SCSI Adapters".

06) In the SCSI Adapter Setup dialog box, choose the "^Add" button

07) In the "^Adapter:" list dialog box, choose

"^Other (Requires a disk from a hardware manufacturer)"

08) Next, you will see the "^Insert Diskette" dialog box,

insert the SiS driver disk into Drive A and type

"A:NT35" into dialog box.

09) Next, In "Select OEM Option" dialog box, choose

"^SiS 551x Series PCI MASTER IDE Miniport Driver" and click "OK".

10) In the "^Select SCSI Adapter Option" dialog box, click on the

"^Install" button in the dialog box.

11) When the "^SCSI Adapter Setup" dialog box reappear,

check whether "^IDE CD-ROM [ATAPI 1.2] / Dual_channel PCI IDE" is listed.

If yes, select it with mouse and click "^Remove" button.

12) Installation is successful if the "^SCSI Adapter Setup" dialog box

reappear, and "^SiS 551x Series PCI MASTER IDE Miniport Driver" will be

listed as an installed driver.

13) Reboot your system.