SiS Linux LAN driver for SiS630/SiS630E/SiS630S/730S

Date:11/01/00

ver. 1.07.04a

-------------------------------------------------------------------------------------------

Currently, you can find SiS900 LAN support in Linux kernel 2.2.15 or

later version. That's for SiS630 and SiS900 only.

For SiS630E/SiS630S/730S LAN support in Linux system, SiS provides update driver for you to update your Linux kernel to support SiS630E/SiS630S/SiS730S. It's backward compatible with SiS630 and SiS900.

Here is the example based on kernel 2.3.xx to update Linux LAN driver.

1. Download the latest kernel distribution from

ftp://ftp.tw.kernel.org/pub/linux/kernel/v2.3/

2. You have to be superuser to be able to update driver kernel.

Use "su" to become a super user.

$su

3. make new directory to /usr/src/linux2351

#mkdir linux2351

move the compressed tared file to /usr/src/linux2351

#mv linux-2.3.51.tar.gz /usr/src/linux2351

(or #mv linux-2.3.51.tar.bz2 /usr/src/linux2351)

4. uncompress and untar the file by

#cd /usr/src/linux2351

#gzip -d linux-2.3.51.tar.gz

(or #bzip2 -d linux-2.3.51.tar.bz2)

#tar -xovf linux-2.3.51.tar

5. After unzip is completed, there will be a directory

/usr/src/linux2351/linux created.

Make sure unzip is successful, and then change directory to

/usr/src/linux2351

#ls -al

drwxr-xr-x   6 root     root         1024 Jan 24 13:38 .

drwxr-xr-x  22 root     root         1024 Nov 25 14:08 ..

drwxr-xr-x  15 root     root         1024 Jan 24 17:57 linux

6. You can find the sis900.c and sis900.h files in

/usr/src/linux2351/linux/drivers/net.

Replace sis900.c and sis900.h with the update files

#cp sis900.c /usr/src/linux2351/linux/drivers/net

#cp sis900.h /usr/src/linux2351/linux/drivers/net

7. Start to re-compile the Linux Kernel.

#cd /usr/src/linux2351/linux

If you are using X-window enviroment you can configure the kernel by

#make xconfig

or if you are using text console

#make menuconfig

8. There are many options you can choose, but the one that is

related to our SiS900 is in "Network device support".

First select "Ethernet" and select "SiS900(7016)" to "y".

Save your kernel configuration by selecting "Save and Exit"

in the main menu.

9. Recompile the kernel by

#make bzlilo

After the compilation finished there will be one file in

/usr/src/linux2351/linux

#ls -al

-rw-r--r--   1 root     root       298358 Jan 25 11:49 System.map

and another file in /usr/src/linux2351/linux/arch/i386/boot

#ls -al

-rw-r--r--    1 root     root       887678  Jan   3 02:09 bzImage

If you are using RedHat move these two file to /boot

# mv /usr/src/linux2351/linux/System.map /boot/System.map

# mv /usr/src/linux2351/linux/arch/i386/boot/bzImage /boot/vmlinuz

10.#cd /etc

Tell the linux kernel loader (LILO) about the new kernel

#modify this file /etc/lilo.conf

(example : #pico lilo.conf

......

image=/boot/vmlinuz

label=linux

root=dev/hda5

initrd=/boot/initrd-2.2.12-2.0.img

read only

image=/boot/vmlinuz-2.2.14-2.0

lable=old

....... )

#lilo

Now you can reboot your system

#reboot

After the system is rebooted, login and check if the driver is working

properly with dmesg You can see SiS900 information.

11.#netconf

enter setting in the Network configurator/Basic host information/

IP address : xxx.xxx.xxx.xxx

netmark : xxx.xxx.xxx.xxx

net device : eth0

kernel module : sis900