Frequently Asked Questions

01. The screen would be vibrating while recording program through software.

1. Please turn on the DMA mode from system and HDD.

1.1 Please enter control panel, then enter device manager

1.2 Select disk device (same as picture left)

1.3 Please select the DMA after enter the default of disk device (same as picture right)

1.4 Reset up the computer

1.1 Please enter the device manager of the control panel

1.2 Please enter the device manager, then select the Direct memory access controller

1.3.Select Settings, Reserve DMA buffer, to 64 Kbytes reserved

1.4. Reset up the computer.

2. Please update to VIA 4 in 1 service pack for VIA chipset main board.

3. Please update Microsoft DirectX Driver

02. No sound release while record playing back:

1.1 Please be sure the Line out has been connected with Line in of the sound card

1.2 Please check if the Line-in of the Record Property of volume control has been enable.

1.3 If it is unable record the sound, please use the MIC-IN of the sound card

0.3 The reference step of upgrading picture quality

1.1 Please update Microsoft DirectX Driver.

1.2 Please update the VGA Card Driver

1.3 Please update the Driver of your main board (Ex: VIA chipset, or Intel chipset driver and utilities.)

※DirectX download:

http://www.microsoft.com/directx/

※VIA Chipset download:

http://www.via.com.tw/jsp/en/dr/driver.jsp

※Intel Chipset download:

http://support.intel.com/support/chipsets/driver.htm

Including driver and Utility

04. Error message appear “initial h/w fail” while opening the application:

1.1That is because of the reopening of the application.  Please shut down the computer normally, and then execute the application after restart the computer.

1.2 The system did not detect the USB device or the USB device did not put in correctly.  Please re-put USB device or restart the computer.

1.3The old driver has the compatible problem with the new driver please clean the old driver completely.

1.4 Please refer to the “The reference step of upgrading picture quality”

05. After setting up the application, no picture appear after the first channel scanning

Please check if the TV system is correct while selecting.

1.Reset PC-TV FM Application

2.Execute PC-TV FM once and then select the nation

06. The application is in blue picture while opening

1.1 Please be sure that the driver of VGA card is correct or please update the driver of VGA card to the latest version.

  - Please update DirectX Driver.

07. The remote control has no function

Please check your sensor is connected with right location and the power of the battery in the remote control.

08. How to shut down by remote control?

Step 1. Press the “ Power” key on the remoter to quit the PC-TV application.

Step 2. Press the “Reset” key and there will comes out the “Shut Down Message” dialog box showing the message as below Press “Power “ to shut down your computer or any other key to cancel’

Step 3. Press the “ Power” key again to shut down your PC power

09. The voice burst out while recording play back:

1.1 Please install DirectX 8.0

1.2 Please update sound card driver

10. The TV picture is in black while recording:

1.1 Please be sure the VGA card supporting Overlay or update the driver of VGA card

1.2 Please update DirectX driver

11. The TV capture card could not be installed correctly and the message "unknow device" would come out in the Windows:

Please check if the TV capture card is put in the PCI slot correctly or try another one PCI slot.

12. What the actual step to install with USB TV BOX?

Please install USB TV Box Application, and the system would ask to restart the computer.  After entering windows, please input the USB device on the USB port. The USB TV BOX device will be automatically detected by the computer

13. Why the system shut down while using application to record the program

Please check if your system is under suspend mode or not. It is impossible to over 4GB if the file be record under FAT32

14. What operation system would be supported by TV series?

MS WINDOWS98SE/ME/Win2000

15. The file size limitation of single file:

1.1	Windows 98 FAT16: 2GB

1.2	Windows 98 FAT32: 4GB

1.3	Windows 2000 NTFS: No limitation