# IB3

# 使  用  手  冊

內建 3D-AGP 顯示 / 3D-音效

Socket 370 奔騰處理器主機板

# NO. G03-IB3R1C

### 發佈日期2001年02月

商標:

*  Pentium 和MMX都是Intel公司的註冊商標。其他商標及名稱皆屬其所屬公司所有。

* 包含在此文件之規格及資料僅為使用資訊的提供，任何修改將不另行通知，並且不應

視為廠商的承諾。

目  錄

使用者需知	ii

手冊版本資訊	1

散熱解決方案	1

第一章  主機板簡介

1-1	主機板特性	2

1-2	規格	3

1-3	主機板配置圖	4

1-4	晶片設計圖及跳線設定	5

第二章  硬體安裝

2-1	硬體安裝步驟	6

2-2	檢查主機板的跳線設置	6

2-3	安裝CPU	8

2-3-1   常用術語	8

2-3-2   設定 CPU 前端匯流排頻率以及記憶體的時脈	10

2-3-3   CPU 的安裝	10

2-3-4   超頻	11

2-4	安裝記憶體	12

2-5	擴充卡	12

2-5-1   擴充卡安裝程序	12

2-5-2   設定擴充卡的 IRQ	13

2-5-3   主機板的中斷列表	13

2-6	連接埠，接頭	14

第三章  BIOS 介紹

3-1	進入 SETUP	21

3-2	線上說明	22

3-3	主目錄	22

3-4	CMOS 的標準設定	24

3-5	BIOS 特性的進階設定	25

3-6	晶片組參數的進階設定	27

3-7	周邊配備設定	29

3-7-1   內建之 IDE 裝置的功能設定	29

3-7-2   內建之 SIO 功能設定	30

3-7-3   內建裝置之功能設定	31

3-8	電源管理的設定	32

3-8-1   重新載入計時器事件	34

3-9	PNP/PCI 組態設定	35

3-10	系統環境狀態監控之設定	36

3-11	頻率及電壓控制	37

3-12	載入原廠預設值/最佳化之設定	37

3-13	設定監督者/使用者密碼	38

第四章  驅動程式及附贈軟體的安裝

支援WINDOWS 95/98/98SE/NT4.0/2000 的MAGIC INSTALL	39

4-1 	INF	安裝INTEL 810E 晶片組系統驅動程式	40

4-2	IDE 	安裝 ULTRA ATA 33/66 驅動程式	41

4-3 	VGA	安裝內建式VGA之驅動程式	42

A. WINDOWS 95/98/98SE/NT4.0	42

B. WINDOWS 2000	42

4-4 	PC-HEALTH    安裝硬體監控程式 IIII	44

4-5 	安裝AC97音效卡驅動程式和EDITING/PLAYBACK程式	44

4-6 	PC-CILLIN     安裝 PC-CILLIN 2000 防病毒程式	45

4-7 	如何使用 ALSRACK EDITING&PLAYBACK 播放程式	47

4-8 	如何使用PC-HEALTH	48

4-9 	如何關閉內建式音效卡	48

4-10  	如何更新 BIOS	48

# 使用者需知

本手冊的版權屬於其製造廠商。其中的任何部分（包括所描述之產品和軟體）都不允許在未經其製造廠商書面授權的情況下以任何形式或者採取任何方法複製、傳播或翻譯成任何語言。

本手冊包含了使用本主機板所必須的所有資訊，並且我們確保本手冊能完全滿足使用者的需求，如有任何改變或修正將不另行通知。廠商提供本手冊是不帶任何方式的擔保，而且將不對一切直接的、間接的、特殊的、偶然的或是因此而產生的損害（包括利潤損失，商業損失，使用數據時的損失，商業中斷等等）負責。

本手冊所使用的產品名稱及公司名稱可能不是其註冊商標或其註冊版權。僅用於說明或解釋之作用，並無意侵犯其所有者的權益。

手冊版本資訊

版本            	版本記錄       		日期

1.0			First Release	  	  2001年02月

項目表

- 主機板

- IDE/Floppy 排線

- 主機板應用程式光碟片

□	USB Port 排線 (選購性配備)

- COM1/COM2排線

- Audio (音效) / 搖桿排線

- LPT/PS2 滑鼠排線

- 使用手冊

Intel 中央處理器的散熱解決方案 – 風扇

由於科技的日新月異，中央處理器 (CPU) 亦持續往更快速、更高的效能發展。因此在建置電腦系統時，散熱的處理變得越來越重要了，一個適當的散熱環境，是讓系統更加穩定及長期操作時的關鍵。提供適當散熱環境的最終目的，則在於維持中央處理器之溫度，能低於電腦機殼之最大特定溫度。

一個好的風扇，除了要有較高的轉速外，適當的散熱片面積亦是相當重要的因素。它可透過其表面之散熱片區域的範圍，集中來自中央處理器的高熱，並透過附加的風扇讓熱氣流傳導出去。除此之外，散熱膏亦能有效的將高熱由中央處理器傳輸到散熱片。為了達到散熱傳導的最佳效果，Intel建議您使用散熱膏，並以固定夾將風扇附加在處理器上。

當您為系統選擇適當的風扇時，請參考以下網址中Intel所推薦與Intel處理器一起使用之風扇。

有關Intel Pentium® !!! 處理器之散熱片及風扇銷售廠商，請至以下網址:

http://developer.intel.com/design/Pentiumiii/components/index.htm

有關Intel® Celeron™ 處理器之散熱片及風扇銷售廠商，請至以下網址:

http://developer.intel.com/design/celeron/components/index.htm

## 第一章

IB3 主機板簡介

# 1-1	主機板特性

IB3是為使用 Intel 新一代 Pentium 處理器而設計，採用FC-PGA370封裝設計，可支援PentiumIII/Celeron FC-PGA/PPGA 類型之CPU，且記憶體可擴充至256MB (或 512MB 如果該記憶體採用 128Mb 封裝技術)。

## 這片主機板除了使用最新的 DDM (Dynamic Display Memory；動態顯示記憶體使用模式) 技術來增加系統顯示效能外，還有許多其他功能可以符合系統未來之需求。另外，它內建的 3D 圖形加速器，更可以讓使用者不需額外的VGA卡即可擁有強大的顯示功能。

它的另一個特色則是採用了自動設定以及不需跳線的方式，讓使用者在安裝不同 CPU 時可以不用更改任何跳線設定。同時，使用者亦可在 BIOS SETUP 中的 “Miscellaneous Control” 更改 CPU 前端匯流排頻率 (CPU Host Clock) 和 CPU倍頻，進而達到超頻的目的。

該主機板使用了 Intel 810E的晶片組，其133MHZ前端匯流排頻率及133MHZ記憶體介面提供了一條通往133MHZ處理器和PC133 SDRAM的升級途徑。同時，它還提供了 ULTRA ATA 66 介面以支援 ATA-66 的硬式磁碟機，全面提高系統性能。

IB3 同時內建有整合型AC’97 2.1 CODEC，能與Sound Blaster ProÒ完全相容，給你帶來最佳音效品質及相容性。另外，它還內含1 個USB介面，可連結 2 個 USB 裝置 (經由加裝 2 個選購性的連結頭)，足以迎合未來對USB的需求。而且本主機板含有內建的硬體監控功能，可監控並保護你的電腦 。

另外，如果你使用的是 ATX 電源供應器，它還可以提供 RTC Alarm、Ring-In Wake Up、Wake On LAN、鍵盤/滑鼠喚醒、電源恢復後自動啟動等之省電管理的功能。

本主機板除了提供有高階的性能，還能同時滿足未來規範的需要，無疑是您購買主機板的最佳選擇。

# 1-2	 規格

| 規 格 | 說 明 |
| --- | --- |
| 設計尺寸 | - Baby AT 主機板架構，4層板，尺寸：22 x 17 cm |
| 晶片組 | - INTEL 810E 晶片 |
| 時脈產生器 | - Winbond 83194AR-We - 支援 66/100/133MHz 系統匯流排頻率 (CPU 匯流排頻率) - BIOS 可支援 CPU 匯流排頻率微調 |
| CPU插座 | - 奔騰 III 5001GHz 處理器 (FC-PGA 封裝) - 賽揚(™) 300800MHz 處理器 (FC-PGA封裝) - 支援66, 100 及 133MHz CPU 匯流排頻率 - 預留對未來 Intel奔騰 III 處理器的支援 - 支援 Cyrix MIII 處理器 |
| 記憶體插座 | - 168-針DIMM插座 x 2 - PC-100/PC-133 SDRAM，可擴充至 512MB - 支援3.3V SDRAM DIMM |
| 擴充插槽 | 3個32位元PCI插槽 |
| 整合型 VGA | - 3D 圖形加速器 - 8MB Frame Buffer DVMT (動態顯示記憶體使用技術) - 3D Hyper Pipelined 技術 - 3D Graphics Visual /Texturing 加強技術 - 1600 x 1200 x 8-bit Color 在 85Hz 更新率的最高解析度 - H/W Motion Compensation Assistance 的 MPEG2 軟體解碼 - 每秒30個畫面的 DVD 撥放功能 |
| 整合型 IDE | - 2個支援ULTRA DMA 33/66 的 Bus Master IDE埠，可支援如 Tape Backup, CD-ROM 、ZIP、LS-120 等加強型 IDE 裝置 |
| 音效 | - 整合型AC’97數位式音效控制器 - 內建 AC’97 Audio CODEC - 包含音效卡驅動程式及應用程式 |
| BIOS | - Award 2Mb Flash ROM |
| Multi I/O | - PS/2滑鼠和 PS/2 鍵盤介面 - 1個軟碟驅動器 - 1個並列埠 - 2個串列埠 - 2個USB 接頭 - 音效介面 (輸入、輸出、麥克風及搖桿介面) |

# 1-3  主機板配置圖

# Figure 1-1

1-4	晶片設計圖及跳線設定

## 跳線

| 跳線 | 名稱 | 說明 |  | 頁碼 |
| --- | --- | --- | --- | --- |
| JP3, JP4, JP5, JP6 | CPU及SDRAM時脈頻率設定 | Auto 66Mhz 100Mhz 133Mhz | JP3:ON, JP4:ON, JP5:1-2, JP6:1-2 JP3:Off, JP4:Off, JP5:2-3, JP6:2-3 JP3:Off, JP4:Off, JP5:1-2, JP6:2-3 JP3:Off, JP4:Off, JP5:1-2, JP6:1-2 | p.6 |
| JP2 | V I/O 電壓 | 1-2 : V I/O 設成3.3V (預設值) 2-3 : V I/O 設成3.45V |  | p.7 |
| JP8 | 使用鍵盤開機功能設定 (K.B. 電壓) | 1-2 : 啟動此功能 (只有使用ATX的電源 供應器方有此功能) 2-3 : 關閉此功能 |  | p.7 |
| JP10 | 清除鍵盤開機密碼設定 | 1-2 : 預設值；2-3 : 清除密碼 |  | p.7 |
| JP11 | 清除CMOS | 1-2 Default for R.T.C. ; 2-3 Reset R.T.C. |  | p.8 |

## 連接器

| 連接器 | 名稱 | 說明 | 頁碼 |
| --- | --- | --- | --- |
| ATPOW | AT電源介面 | 12-Pin Block | p.14 |
| ATXPOW | ATX電源介面 | 20-Pin Block | p.14 |
| ATKB | 鍵盤連結頭 | 5-Pin Female | p.14 |
| MS_PIN | PS2 滑鼠連結頭 | 6-Pin Block | p.15 |
| LPT | 並列埠介面 | 26-Pin Block | p.15 |
| COM1, COM2 | COMA, COMB通信接頭 | 10-Pin Block | p.15 |
| FDD | 軟碟介面 | 34-Pin Block | p.16 |
| IDE1 | 第一個 IDE 介面 | 40-Pin Block | p.16 |
| IDE2 | 第二個 IDE 介面 | 40-Pin Block | p.16 |
| IDELED | IDE 運轉指示燈 | 2-Pin Connector | p.17 |
| FPC | 機殼面板連結介面 | 16-Pin Block | p.17 |
| IR | IR 紅外線介面 | 5-Pin Block | p.18 |
| USB | USB Port 介面 | 10-Pin Block | p.18 |
| FAN1, FAN2 | 風扇電源接頭 | 1-2 12V Power Connector 2-3 ACPI FAN power control Connector | p.18 |
| PWR_BTN | ATX電源開關 | 2-pin Connector | p.18 |
| VGA | VGA 顯示卡連結頭 | 16-pin Block | p.19 |
| Audio | 音效介面 | 10-pin Connector | p.19 |
| Game | 搖桿介面 | 16-pin Connector | p.19 |
| CDIN | CD 音效輸入介面 | 4-pin Block | p.20 |
| WOL | 遠程網路啟動介面 | 3-pin Block | p.20 |

## 擴充插槽

| 插座 / 插槽 | 名稱 | 說明 |
| --- | --- | --- |
| DIMM1, DIMM2 | DIMM 記憶體擴充插槽 | 168-Pin DIMM SDRAM Module Expansion Socket |
| Zip Socket370 | CPU插槽 | Celeron PPGA / Coppermine FC-PGA CPU Socket |
| PCI1, PCI2, PCI3 | PCI 擴充插槽 | 32-bit PCI Local Bus Expansion Slots |

## 第二章

硬體安裝

# 2-1	硬體安裝步驟

在使用你的電腦之前，你必須完成下列步驟：

- 檢查主機板設定

- 安裝CPU

- 安裝記憶體

- 安裝擴充卡

- 連接排線，面板電線及電源

- 設定BIOS參數

- 安裝軟體驅動程式及應用程式

# 2-2	檢查主機板的跳線設置

- CPU及SDRAM 時脈頻率設定 (3-pin) : JP3, JP4, JP5, JP6

主機板的 CPU 及 SDRAM 時脈頻率可透過JP3、JP4、JP5、JP6跳線調整如下表：

|  | CPU/SDRAM (MHz) | JP3 | JP4 | JP5 | JP6 |
| --- | --- | --- | --- | --- | --- |
|  | Auto | ON | ON | 1-2 | 1-2 (預設值) |
|  | 66/100 | OFF | OFF | 2-3 | 2-3 |
|  | 100/100 | OFF | OFF | 1-2 | 2-3 |
|  | 133/100 | OFF | OFF | 1-2 | 1-2 |

Pentium III 以及Celeron 370 腳座CPU 一覽表

| Celeron |  |  |  | Pentium III |  |  |  | Pentium III |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Speed | Bus | Ratio |  | Speed | Bus | Ratio |  | Speed | Bus | Ratio |
| 300/66 | 66MHz | 4.5x |  | 400/100 | 100MHz | 4x |  | 750/100 | 100MHz | 7.5x |
| 333/66 | 66MHz | 5.0x |  | 450/100 | 100MHz | 4.5x |  | 800EB/133 | 133MHz | 6.0x |
| 366/66 | 66MHz | 5.5x |  | 500/100 | 100MHz | 5.0x |  | 800/100 | 100MHz | 8.0x |
| 400/66 | 66MHz | 6.0x |  | 500E/100 | 100MHz | 5.0x |  | 850/100 | 100MHz | 8.5x |
| 433/66 | 66MHz | 6.5x |  | 533B/133 | 133MHz | 4.0x |  | 866/133 | 133MHz | 6.5x |
| 466/66 | 66MHz | 7.0x |  | 533EB/133 |  |  |  | 933/133 | 133MHz | 7.0x |
| 500/66 | 66MHz | 7.5x |  | 550/100 | 100MHz | 5.5x |  | 1.0 GHz/133 | 133MHz | 7.5x |
| 533/66 | 66MHz | 8.0x |  | 550E/100 | 100MHz | 5.5x |  |  |  |  |
| 566/66 | 66MHz | 8.5x |  | 600E/100 | 100MHz | 6.0x |  |  |  |  |
| 600/66 | 66MHz | 9.0x |  | 600B/133 | 133MHz | 4.5x |  |  |  |  |
| 633/66 | 66MHz | 9.5x |  | 600EB/133 |  |  |  |  |  |  |
| 667/66 | 66MHz | 10.0x |  | 650/100 | 100MHz | 6.5x |  |  |  |  |
| 700/66 | 66MHz | 10.5x |  | 667/133 | 133MHz | 5.0x |  |  |  |  |
| 766/66 | 66MHz | 11.5x |  | 700/100 | 100MHz | 7.0x |  |  |  |  |
| 800/100 | 100MHz | 8x |  | 733/133 | 133MHz | 5.5x |  |  |  |  |

- 因為 CPU製造商已將CPU的倍頻鎖定，所以你不須再設定CPU的倍頻。此表僅供參考使用。

你也可以在 Miscellaneous Control 選項下的“Host Clock” 設定CPU 前端匯流排的頻率 (請參閱 11 頁)。

- V I/O 電壓設定 (3-pin) : JP2

- 鍵盤開機功能設定 (3-pin) : JP8

如果你想使用鍵盤來開機 (按下< >)，請將 JP8 設為 enable (使用該功能)，或disable (取消該功能)。該功能需要一個至少能供應 300mA 電源給+5VSB 引腳的ATX 電源供應器。初始值設為 disable。

- 清除鍵盤開機密碼設定 (3-pin) : JP10

你可以在BIOS  SETUP中設定鍵盤開機密碼以確保電腦安全。如果忘記所設定之密碼，則可透過 BIOS 或 JP10清除鍵盤開機密碼。

- 清除 CMOS RAM (3-pin) : JP11

主機板必須使用一個電池將主機板的配置資料保存在CMOS RAM裡，再透過跳帽將 JP11的1-2腳短路來存儲CMOS數據。

注意！	當系統斷電時，你可以將 2-3腳短路來清除CMOS數據。然後再放回短接 1-2腳。在系統通電時，切勿清除CMOS，亦不可突然拔掉電源線以避免導致主機板損壞。

# 2-3	安裝CPU

# 2-3-1  常用術語：

晶片組 (Chipset 亦稱 core logic) – 2 個或以上的積體電路所組成。用於控制系統處理器、隨機存取記憶體(RAM)、輸出/輸入裝置、擴充卡等等之介面。

處理器插槽 / 插座 (Processor slot/socket) – 主機板上可供中央處理器 (CPU)  嵌入的插槽或插座。

擴充插槽 (Slot 有AGP, AMR, PCI, ISA, RAM) – 可讓各式擴充卡或記憶體嵌入的插槽。目前有AGP插槽、AMR插槽、PCI插槽、ISA插槽以及供記憶體使用的RAM插槽等等。

AGP 擴充槽 (Accelerated Graphics Port) – 一種供顯示卡使用的高速介面插槽。目前有 1X (66MHz)、2X (133MHz)、4X (266MHz) 等模式。

PCI 擴充槽 (Peripheral Component Interconnect) – 一種供顯示卡、音效卡、網路卡、數據機等裝置使用之高速介面插槽；其執行頻率為 33MHz。

ISA 擴充槽 (Industry Standard Architecture) – 一種供舊式音效卡或數據機等裝置使用之低速介面插槽；其執行頻率約為 8MHz。

串列埠 (Serial Port) – 一種供滑鼠及外接式數據機使用之低速介面連接埠。

並列埠 (Parallel Port) – 一種供印表機使用之低速介面連接埠。

PS/2 – 一種供滑鼠及鍵盤使用之低速介面連接埠。

USB (Universal Serial Bus) – 一種供滑鼠、鍵盤、掃描器、數位照相機使用之中等速度介面連接埠。

音效裝置 (Sound) – 音效卡或整合於主機板上的音效介面。一般說來，該裝置含有喇叭接頭、麥克風、搖桿控制介面以及 MIDI 音效裝置。

區域性網路 (LAN；Local Area Network) – 用於連接區域性網路的介面。

基本輸出/輸入系統 (BIOS；Basic Input/Output System) – 用於系統的啟動和制定不同裝置彼此間之關係的邏輯程式。

驅動程式(Driver) – 用於定義該裝置之特性，以便其它裝置或軟體使用。

中央處理器(Processor 亦稱 CPU) – 一種用於個人電腦運算之主要的晶片。

前端匯流排頻率 (Front Side Bus Frequency)：經由主機板上之時脈產生器所產生的一種工作頻率。可供 CPU、DRAM、PCI 匯流排使用。

CPU 倍頻 (Bus Frequency Ratio)： 用於搭配前端匯流排頻率，以計算 CPU 工作頻率。目前大部分之 CPU 的倍頻在出廠時即被鎖死，故大部分的主機板已無此設定。

CPU 內部頻率 (CPU Internal Frequency)：CPU 的內部頻率，亦是 CPU 實際的工作頻率。此一頻率是由前端匯流排頻率 (Front Side Bus Frequency) 乘以CPU 倍頻(Bus Frequency Ratio) 所計算出來。

CPU L2 快取記憶體 (CPU L2 Cache)：一種位於 CPU 內部的快取記憶體。一般說來，Pentium !!! CPU 的內部有 256K 或更高，而 Celeron CPU 則為 128K。

關於Pentium及Celeron 370-腳座CPU

Pentium iii FC-PGA

如右圖所示，你可在 “PENTIUM III”  CPU 的表面看到如下的代碼：

RB 80526 P2 866 256

RB	:  FC–PGA packing

P2	:  P2–133MHz front side bus frequency

PY–100MHz front side bus frequency

866	:  CPU internal frequency, where here is

866MHz

256	:  the size of L2 cache, where here is 256K

Celeron FC–PGA

如右圖所示，你可在 “Celeron” CPU 的表面看到如下的代碼：

566/128/66/1.5V

566	:  CPU internal frequency, where here is 566MHz

128	:  the size of L2 cache, where here is 128K

66	:  front side bus frequency, where here is 66MHz

1.5V	:  the voltage for the CPU

# 2-3-2  設定CPU 前端匯流排頻率以及記憶體的時脈

使用JP3, JP4, JP5, JP6跳帽來設定前端總線頻率以及SDRAM頻率，如下表所示：

| CPU/SDRAM (MHz) | JP3 | JP4 | JP5 | JP6 |
| --- | --- | --- | --- | --- |
| Auto | ON | ON | 1-2 | 1-2 (預設值) |
| 66/100 | OFF | OFF | 2-3 | 2-3 |
| 100/100 | OFF | OFF | 1-2 | 2-3 |
| 133/100 | OFF | OFF | 1-2 | 1-2 |

例如：使用前端匯流排頻率為133MHz 的奔騰 866CPU 及 PC133 SDRAM，可將JP3和JP4 設成ON，JP5 和JP6 設成1-2。這會使CPU以 133MHz 外頻，記憶體頻率為100MHz 的模式工作。

有經驗且要超頻的使用者請參考 2-3-4 超頻部分的內容

# 2-3-3  安裝CPU

此主機板提供了一個 370 ZIF SOCKET 插座。安裝在主機板上的 CPU 必須裝有風扇以防止 CPU 過熱。 如果你尚未購買風扇，請在安裝系統前請購買一個合適的風扇。

警告 !	請確保處理器之散熱片的表面有充足的空氣流通，且CPU冷卻風扇工作正常。否則將使處理器和主機板因過熱而造成損壞。如果需要的話，你可以另外安裝輔助風扇。

安裝 CPU 前，先請關閉你的系統再移除外殼。找到 ZIF 插槽並先從插槽一側拉起拉桿使之向上成90度。將CPU從如下圖所示的正確方位插入。有凹口的一角應該朝向拉桿的末端。因為CPU四個角中有兩角缺了一個引腳，因此會適合於如圖所示的方位。

當你將CPU插入ZIF插槽時，不要使用太大的力量，插入後只要輕輕把拉桿沿正確方向按下即可。

# 2-3-4  超頻

警告 !	本部分內容僅供有經驗的主機板安裝者參考。超頻將導致系統喪失穩定性，甚至將縮短處理器的使用壽命。

調整過 JP3、JP4、JP5、JP6 的跳帽後，你可以在BIOS CMOS SETUP UTILITY選擇超頻。進入CMOS SETUP UTILITY之後再選擇 “Frequency/ Voltage control” 則你將看到如下所示的螢幕。

你可選擇你所需要的設定：

| CPU/SDRAM (MHz) | JP3 | JP4 | JP5 | JP6 |
| --- | --- | --- | --- | --- |
| Auto | ON | ON | 1-2 | 1-2 (預設值) |
| 66/100 | OFF | OFF | 2-3 | 2-3 |
| 100/100 | OFF | OFF | 1-2 | 2-3 |
| 133/100 | OFF | OFF | 1-2 | 1-2 |

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Miscellaneous Control |  |
| --- | --- |
| Auto Detect DIMM/PCI Clk Enabled Spread Spectrum Disabled CPU Host/SDRAM Clock [Default] CPU Clock Ratio X 3 CPU Host/SDRAM Clock. Default [ ] 133/100 MHz..... [ ] 138/103 MHz..... [ ] 140/105 MHz..... [ ] 150/112 MHz..... [ ]. :Move ENTER:Accept ESC:Abort. | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

經由選按 PageDown/PageUp 方向鍵，你可以更改 CPU Host / SDRAM 匯流排的頻率：

當你將 CPU 頻率設定在 66MHz 時，你可以選擇66/100, 75/112, 83/124的頻率。

當你將 CPU 頻率設定在100MHz，你可選擇95/95, 100/100, 117/117, 124/124的頻率。

當你將 CPU 頻率設定在133MHz  時，你可以選擇133/100, 138/103, 140/105, 150/112的頻率。

注意!	雖然我們的工程師曾經進行幾小時的超頻試驗，但我們不建議使用者進

行超頻。

# 2-4	安裝記憶體

此主機板提供有二個168-針 Dual Inline Memory Module (DIMM) 插槽，可使記憶體從最小的 32MB 擴充至最大的 512MB記憶體。

有效記憶體配置

| DIMM 1 | DIMM 2 | System can be Accept or Not |
| --- | --- | --- |
| 168-pin DIMM |  | Accept |
| 168-pin DIMM | 168-pin DIMM | Accept |
|  | 168-pin DIMM | Accept |

一般來說，將記憶體安裝到主機板上是非常容易的，你可以參考圖2-4安裝記憶體的簡圖。

Figure 2-4

注意！	當你將DIMM記憶體完全插入DIMM插槽時，請將兩端的白色護耳緊緊地卡好，使其恰好卡住兩端的凹口。

# 2-5	擴充卡

警告!	當添加、移除擴充卡，或其他系統組件時務必請關掉電源，以避免對主機板和擴充卡造成損害。

# 2-5-1  擴充卡安裝程序

- 仔細閱讀擴充卡所附之文件，將所有相關之必要的軟、硬體設定好，比如跳線。

- 移除電腦外殼，並將你想要安裝之插槽處的金屬支架拆除。

- 將該擴充卡插入並穩固地壓下去。

- 鎖上螺絲。

- 將系統機殼放回原位。

- 如果有必要，請在 BIOS內設定其參數。

- 安裝擴充卡所須的相關驅動程式。

# 2-5-2  設定擴充卡的IRQ

某些擴充卡需要指定 IRQ 方可使用。一般來說，每一個 IRQ 的埠口位址祇能單獨地指定給某一個裝置使用。在標準設計中，有16個IRQ是可用的，但其中的大部分都已被系統使用中。

IRQ 的基本中斷分配表

| IRQ | Priority | Standard function |
| --- | --- | --- |
| 0 | 1 | System Timer |
| 1 | 2 | Keyboard Controller |
| 2 | N/A | Programmable Interrupt |
| 3 * | 11 | Communications Port (COM2) |
| 4 * | 12 | Communications Port (COM1) |
| 5 * | 13 | Sound Card (sometimes LPT2) |
| 6 | 14 | Floppy Disk Controller |
| 7 * | 15 | Printer Port (LPT1) |
| 8 | 3 | System CMOS/Real Time Clock |
| 9 * | 4 | ACPI Mode when enabled |
| 10 * | 5 | IRQ Holder for PCI Steering |
| 11 * | 6 | IRQ Holder for PCI Steering |
| 12 * | 7 | PS/2 Compatible Mouse Port |
| 13 | 8 | Numeric Data Processor |
| 14 * | 9 | Primary IDE Channel |
| 15 * | 10 | Secondary IDE Channel |

* 上述的 IRQ 通常可供 ISA 或 PCI 介面裝置使用。

# 2-5-3  主機板的中斷列表

主機板共用的中斷指令如下表所示：

|  | INT A | INT B | INT C | INT D |
| --- | --- | --- | --- | --- |
| Slot 1 |  |  |  |  |
| Slot 2 |  |  |  |  |
| Slot 3 |  |  |  |  |
| Onboard VGA |  |  |  |  |
| Onboard USB |  |  |  |  |
| AC97/MC97 |  |  |  |  |

注意 !	如果你在共用的插槽上使用PCI卡，請確認驅動程式可支援”Shared IRQ”或者該卡不需要分配任何IRQ。否則兩個PCI組之間將產生衝突，進而使得整個系統不穩定而且 PCI 卡將不可使用。

# 2-6	連接埠，接頭

- AT 電源介面 (12-pin block) : ATPOW

此為 AT 電源供應器的介面，其 12-pin 的定義如下表。大部分的電源供應器提供有

(P8 and P9) 兩個由六條電線所組成的插頭。請如下圖將兩個接頭的黑色電線對準中間，再插至主機板的接頭。

- ATX 電源介面 (20-pin block) : ATXPOW

此為 ATX 電源供應器的介面，其 20-pin 的定義如下表。ATX 電源供應器電源經由個人電腦面板上一個 2-pin 的開關控制。

* 	我們建議你在關機後如需重新開機，請在四秒鍾後再重新啟動系統，以保護你的系統。

- 鍵盤連結頭 (5-pin female) : ATKB

此連結頭可用來連接 IBM 相容之標準 101 鍵的鍵盤。

- PS/2 滑鼠介面 (5-pin block) :  MS_PIN

如果你要使用這個介面，則需另外購買一個 PS/2 滑鼠的轉接頭，以便連接到電腦機殼上。

5.   並列埠介面 (26-pin Block) : LPT

請將附在主機板的並列埠排線連接到主機板上，再將有鐵片的那一端接到機殼上即可使用該項功能。詳細資料請參閱第三章的 “INTEGRATED PERIPHERALS SETUP”。

注意 :	串列式印表機必需連接至串列埠。你可經由 BIOS 選取相對應的 IRQ 來啟動並列埠。

6.   串列埠介面 (Two 10-pin blocks) : COM1, COM2

COM1 和 COM2 為此一主機板的串列埠介面，使用者可經由連接隨機附送的串列埠排線，連結成二個串列埠接頭。該串列埠可經由 BIOS 設定為 disable 或 enable。詳細資料請參閱第3-7-3章的 “Onboard Serial Port”。

7.   軟碟介面 (34-pin block) : FDD

該介面經由一條 34-pin 排線與軟碟連接。一般來說，排線有紅邊的方向與 Pin 1 相應，所以在裝置排線時應將紅邊對應軟碟介面的 Pin 1 方向。

8.   第一個 IDE 介面 (40-pin block) : IDE1

該介面經由一條 40-pin 排線與硬碟連接，同樣地，也是紅邊對介面Pin 1，本產品所附的 ATA-66 排線可用於連接 ATA-66 硬碟。

- 第二個 IDE 介面 (40-pin block) : IDE2

該介面為另一個 IDE 裝置介面，同樣可經由排線連接兩個 IDE 裝置。

- 每個連接埠能連接兩個硬碟。第一個 HDD 相當於 “Master”， 第二個 HDD 相當於 “Slave”。

- 為了性能的考慮，我們強烈建議請不要將CD-ROM或DVD-ROM驅動器與硬碟安裝在同一個 IDE 通道上。否則，此通道上的系統性能將會降低。

  - IDE 運轉指示燈: IDELED

將硬碟運轉指示燈連接到電腦機殼的接頭。

  - 面板指示燈連結開關：

- A.  Turbo LED 開關 : TURBO LED

主機板加速開關的預設值為"開啟" 狀態。當系統電源開啟時，加速燈會一直亮著。你也可以將電腦機殼的電源 LED 連接到這裡，表示電源開或關的顯示。

B.  Reset 開關 : RESET

這個 2-pin 接頭可連接電腦機殼上「reset」的電源線，以達到不關閉系統電源的情況下重啟電腦的目的。

C.  鍵盤鎖開關 & 電源 LED開關 : KEYLOCK & POWER LED

KEYLOCK 接頭可連接電腦機殼上「key」的電源線，以鎖住鍵盤，如此可達到資料保護的作用。而 POWER LED 則可讓你將電腦機殼上的 Power LED 線連到此一開關，當系統電源開啟時，Power LED 的燈就會亮起來。

D.  喇叭連線開關 : SPKR

這個 4-pin 接頭可連接電腦機殼上「speaker」的開關，以供機殼上的喇叭使用。

  - IR 紅外線介面 : IR

該介面支援可選購的紅外線無線傳輸以及接收組件。你必須在 BIOS setup 中設定其參數以使用此功能。

  - USB 擴充埠介面 : USB

此接頭是用來連接附加的 USB介面插頭。透過外加一條可選購的USB排線，即可

使用附於面板上的兩個額外USB插頭。

  - 風扇電源接頭 : FAN1, FAN2

這些介面支援 350mA (4.2 瓦)或以下的冷卻風扇，根據風扇生產廠商的不同，電

線和插座也會不同。紅線應當是陽極，而黑線則是接地。將風扇接頭插到主機板

時，應考慮連接頭的極性。

  - ATX 電源按鈕 : PWR_BTN

- 這個 2-pin 接頭可連接電腦機殼上的電源開關，供電腦啟動或關閉使用。

Selections

- One touch Power ON/OFF

  - VGA 介面 : VGA

此一連結埠介面可用於連結顯示器裝置。

  - 音效介面 : Audio

音效介面有輸出、輸入、麥克風三個介面。

輸出：		音效輸出至喇叭

輸入：		音效輸入至音效晶片

麥克風：	由麥克風輸入

  - 搖桿介面 : GAME

- 這個介面可同時支援兩支搖桿。

  - CD 音效輸入介面 : CDIN

  - 遠程網路啟動介面：WOL

※ Wake On LAN 功能需要電源供應器在提供 5VSB 時有大於 750mA 的電流才可啟動。

## 第三章

BIOS介紹

BIOS是一段儲存在快讀寫式記憶體 (FLASH ROM) 之基本輸出、入控制程式。該程式是主機板與操作系統間的一架橋樑。電腦啟動時，會先由 BIOS 程式進行控制 。 首先執行一個稱為 POST（開機自我檢測）的自我測試，它會偵測所有硬體設備，並確認同步硬體參數。當完成所有檢測時，它才將系統的控制權移交給操作系統（OS）。由於BIOS是硬體與軟體聯繫的唯一通道，所以是系統穩定性的關鍵因素，進而確保系統性能可達到最佳狀態。

如圖 3-1 所示，在BIOS設定程式主目錄中，可看到一些選項。我們將在本章的後面逐步解釋這些選項，首先讓我們先看看你將在此用到之功能鍵的簡單描述：

- 按<Esc> 鍵，可退出 BIOS 設定程式。

- 按  (向上，向下，向左，向右) 鍵，可在主目錄中選擇你想確認或修改的

選項。

- 當你想要對選項進行參數設定時請按 Page Up/Page Down 或+/–鍵。

- 當完成對參數的設定後，請按<F10>鍵，儲存修改的參數並退出BIOS設定程

式，同時電腦也會自動重新開機。

# 3-1	進入  Setup

在啟動電源開關並且按住<Del>就可以馬上進入Setup 程式。如果你來不及在POST過程中按下<Del>鍵順利進入CMOS SETUP，那麼可以透過把電源關掉，然後再打開電源開關，或者是直接按下電腦機殼上的 “RESET” 按鈕重啟動系統，還是同時按下<Ctrl>, <Alt> 和<Delete> 鍵來以重新啟動電腦，並再按 Del 鍵試一次。如果沒能在正確時間內按下以上所有的鍵，或者系統重新啟動失敗，此時在螢幕上會顯示錯誤訊息如下：

Press <F1> to continue, <Ctrl-Alt-Esc> or <Del> to enter Setup

你可按 <F1> 鍵繼續，或按 <Ctrl-Alt-Esc> 組合鍵重新啟動電腦，還是按<Del>

鍵，進入 BIOS 設定程式。

# 3-2	線上說明

主目錄

所選取之設定功能的說明，會以反白方式顯示在螢幕底部。

狀態頁安裝目錄/選項頁安裝目錄

按 F1 鍵，則會彈出一個線上說明的小視窗，該視窗描述了該選項中可用之指令以及可能的選擇。再按<Esc>鍵，則可退出該線上說明視窗。

# 3-3	主目錄

一旦你進入Award BIOS CMOS Setup，主目錄（圖3-1）會顯示在螢幕中。你可以從14項設定功能選項和兩項退出選項中加以選擇。請使用方向鍵在各選項之間進行選擇，再按<Enter>鍵接受或是進入子目錄。

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software |  |
| --- | --- |
| Standard CMOS Features Advanced BIOS Features Advanced Chipset Features Integrated Peripherals Power Management Setup PnP/PCI Configurations PC Health Status | Miscellaneous Control Load optimized Defaults Load Standard Defaults Set Supervisor Password Set User Password Save & Exit Setup Exit Without Saving |
| Esc : Quit ↑↓→ ← : Select Item F10 : Save & Exit Setup |  |
| Time, Date, Hard Disk Type… |  |

Figure 3-1

Standard CMOS Features

CMOS 的標準設定。

Advanced BIOS Features

BIOS 特性的進階設定。

Advanced Chipset Features

晶片組參數的進階設定，透過更改其設定之參數，可提高系統性能。

Integrated Peripherals

周邊配備設定。

Power Management Setup

電源管理的設定。

PnP/PCI configurations

PnP（即插即用）與 PCI 匯流排的組態設定。

PC Health Status

該項目顯示系統狀態，如 CPU 溫度、風扇轉速等等。

Miscellaneous Control

該項目可讓你指定其它相關控制的設定。

Load Optimized Defaults

載入最佳化設定。

Load Standard Defaults

載入原廠的預設值。

Set Supervisor/User Password

設定監督者/使用者密碼。

Save & Exit Setup

儲存 CMOS 的設定，然後退出 Setup 程式。

Exit Without Saving

放棄 CMOS 所有的修改，然後退出 Setup 程式。

# 3-4	 Standard CMOS Features

The items in Standard CMOS Setup Menu are divided into several categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Standard CMOS Features |  |
| --- | --- |
| Date (mm:dd:yy) Tue, Jan, 16 2001 Time (hh:mm:ss) 20 : 12 : 36 IDE Primary Master Press Enter None IDE Primary Slave Press Enter None IDE Secondary Master Press Enter None IDE Secondary Slave Press Enter None Drive A 1.44M, 3.25 in. Drive B None Video EGA/VGA Halt On All Errors Base Memory 640K Extended Memory 64512K Total Memory 65536K | Item Help |
|  | Menu Level > Change the day, moth, year and century |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Date

The date format is <day><month><date><year>.

Day	  Day of the week, from Sun to Sat, determined by BIOS. Read-only.

Month	  The month from Jan. through Dec.

Date	  The date from 1 to 31 can be keyed by numeric function keys.

Year	  The year depends on the year of the BIOS.

Time

The time format is <hour><minute><second>.

Primary Master/Primary Slave

Secondary Master/Secondary Slave

Press PgUp/<+>or PgDn/<–>to select Manual, None, Auto type.  Note that the specifications of your drive must match with the drive table.  The hard disk will not work properly if you enter improper information for this category.  If your hard disk drive type is not matched or listed, you can use Manual to define your own drive type manually.

If you select Manual, related information is asked to be entered to the following items.  Enter the information directly from the keyboard.  This information should be provided in the documentation from your hard disk vendor or the system manufacturer.

If the controller of HDD interface is SCSI, the selection shall be “None”.

If the controller of HDD interface is CD-ROM, the selection shall be “None”

Access Mode	The settings are Auto Normal, Large, and LBA.

Cylinder		number of cylinders

Head			number of heads

Precomp		write precomp

Landing Zone	landing zone

Sector		number of sectors

# 3-5	 Advanced BIOS Features

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Advanced BIOS Features |  |
| --- | --- |
| Anti-Virus Protection Disabled CPU Internal Cache Enabled External Cache Enabled CPU L2 Cache ECC Checking Disabled Processor Number Feature Enabled Quick Power On Self Test Enabled First Boot Device Floppy Second Boot Device HDD-0 Third Boot Device LS120 Fourth Boot Device Enabled Swap Floppy Drive Disabled Boot Up Floppy Seek Enabled Boot Up NumLock Status On Gate A20 Option Fast Typematic Rate Setting Disabled x Typematic Rate (Chars/Sec) 6 x Typematic Delay (Msec) 250 Security Option Setup OS Select For DRAM > 64MB Non-OS2 HDD S.M.A.R.T. Capability Disabled Report No FDD For WIN95 No | Item Help |
|  | Menu Level > Allows you to choose the VIRUS warning feature for IDE Hard disk boot sector protection. If this function is enabled and someone attempt to write data into this area, BIOS will show a warning message on screen and alarm beep |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Anti-Virus Protection

Allow you to choose the VIRUS Warning feature for IDE Hard Disk boot sector protection.  If this function is enabled and someone attempt to write data into this area, BIOS will show a warning message on screen and alarm beep.

Disabled (default)	No warning message to appear when anything attempts to access the boot sector or hard disk partition table.

Enabled	Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector of hard disk partition table.

CPU Internal Cache

The default value is Enabled.

Enabled (default)	Enable cache

Disabled		Disable cache

Note:	  The internal cache is built in the processor.

External Cache

Choose Enabled or Disabled.  This option enables the Level 2 cache memory.

CPU L2 Cache ECC Checking

Choose Enabled or Disabled.  This option enables the Level 2 cache memory ECC (error check correction).

Processor Number Feature

This option is for Pentium III processor.  During Enabled, this will check the CPU Serial number.  Disabled this option if you don’t want the system to know the Serial number.

Quick Power On Self-Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If this is set to Enabled.  BIOS will shorten or skip some check items during POST.

Enabled (default)	Enable quick POST

Disabled		Normal POST

First/Second/Third/Fourth Boot Device

The BIOS attempts to load the operating system from the devices in the sequence selected in these items.  The settings are Floppy, LS/ZIP, HDD-0/HDD-1/HDD-3, SCSI, CDROM, LAN and Disabled.

Swap Floppy Drive

Switches the floppy disk drives between being designated as A and B.  Default is Disabled.

Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks.  360K type is 40 tracks while 760K, 1.2M and 1.44M are all 80 tracks.

Boot Up NumLock Status

The default value is On.

On (default)	  Keypad is numeric keys.

Off		  Keypad is arrow keys.

Gate A20 Option

Normal		  The A20 signal is controlled by keyboard controller or chipset hardware.

Fast (default)	  The A20 signal is controlled by port 92 or chipset specific method.

Typematic Rate Setting

Keystrokes repeat at a rate determined by the keyboard controller. When enabled, the typematic rate and typematic delay can be selected.  The settings are: Enabled/Disabled.

Typematic Rate (Chars/Sec)

Sets the number of times a second to repeat a keystroke when you hold the key down.  The settings are: 6, 8, 10, 12, 15, 20, 24, and 30.

Typematic Delay (Msec)

Sets the delay time after the key is held down before is begins to repeat the keystroke.  The settings are 250, 500, 750, and 1000.

Security Option

This category allows you to limit access to the system and Setup, or just to Setup.

System	The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt.

Setup (default)	The system will boot, but access to Setup will be denied if the correct password is not entered prompt.

OS Select For DRAM > 64MB

Allows OS2 to be used with >64MB or DRAM. Settings are Non-OS/2 (default) and OS2.  Set to OS/2 if using more than 64MB and running OS/2.

Report No FDD For Win 95

Whether report no FDD for Win 95 or not.  The settings are: Yes, No.

# 3-6	 Advanced Chipset Features

The Advanced Chipset Features Setup option is used to change the values of the chipset registers.  These registers control most of the system options in the computer.

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Advanced Chipset Features |  |
| --- | --- |
| SDRAM CAS Latency Time 3 SDRAM Cycle Time Tras/Trc 6/8 SDRAM RAS-to-CAS Delay 3 SDRAM RAS Precharge Time 3 Special Buffer Strength Enabled System BIOS Cacheable Enabled Video BIOS Cacheable Enabled Memory Hole at 15M-16M Disabled CPU Latency Timer Disabled Delayed Transaction Disabled On-Chip Video Window Size 64MB | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Note:  Change these settings only if you are familiar with the chipset.

SDRAM CAS Latency Time

When synchronous DRAM is installed, the number of clock cycles of CAS latency depends on the DRAM timing.  The settings are: 2 and 3.

SDRAM Cycle Time Tras/Trc

Select the number of SCLKs for an access cycle.  The settings are: 5/7 and 6/8.

SDRAM RAS-to-CAS Delay

This field let’s you insert a timing delay between the CAS and RAS strobe signals, used when DRAM is written to, read from, or refreshed.  Fast gives faster performance; and Slow gives more stable performance.  This field applies only when synchronous DRAM is installed in the system.  The settings are: 2 and 3.

SDRAM RAS Precharge Time

If an insufficient number of cycles is allowed for the RAS to accumulate its charge before DRAM refresh, the refresh may be incomplete and the DRAM may fail to retain date.  Fast gives faster performance; and Slow gives more stable performance.  This field applies only when synchronous DRAM is installed in the system.  The settings are: 2 and 3.

System BIOS Cacheable

Selecting Enabled allows caching of the system BIOS ROM at F0000h-FFFFFh, resulting in better system performance.  However, if any program writes to this memory area, a system error may result.  The settings are: Enabled and Disabled.

Video BIOS Cacheable

Select Enabled allows caching of the video BIOS, resulting in better system performance.  However, if any program writes to this memory area, a system error may result. The settings are: Enabled and Disabled.

Memory Hole At 15M-16M

You can reserve this area of system memory for ISA adapter ROM.  When this area is reserved, it cannot be cached.  The user information of peripherals that need to use this area of system memory usually discusses their memory requirements. The settings are: Enabled and Disabled.

CPU Latency Timer

During Enabled, A deferrable CPU cycle will only be Deferred after it has been in a Snoop Stall for 31 clocks and another ADS# has arrived.  During Disabled, A deferrable CPU cycle will be deferred immediately after the GMCH receives another ADS#.

Delayed Transaction

The chipset has an embedded 32-bit posted write buffer to support delay transactions cycles.  Select Enabled to support compliance with PCI specification version 2.1.  The settings are: Enabled and Disabled.

On-Chip Video Window Size

This option enabled/disabled the on-chip video windows size for VGA driver use.  The settings are: enabled, Disabled.

# 3-7	 Integrated Peripherals

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Integrated Peripherals |  |
| --- | --- |
| > OnBoard IDE Function Press Enter > OnBoard PCI DEVICE Press Enter > Winbond SuperIO Device Press Enter Init Display First PCI Slot | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

OnBoard IDE Function

Please refer to section 3-7-1

OnBoard PCI Device

Please refer to section 3-7-2

Winbond SuperIO Device

Please refer to section 3-7-3

Init Display First

This item allows you to decide to activate whether PCI Slot or AGP VGA first.  The settings are: PCI Slot, AGP Slot.

3-7-1   OnBoard IDE Function

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software OnBoard IDE Function |  |
| --- | --- |
| On-Chip Primary PCI IDE Enabled On-Chip Secondary PCI IDE Enabled IDE Primary Master PIO Auto IDE Primary Slave PIO Auto IDE Secondary Master PIO Auto IDE Secondary Slave PIO Auto IDE Primary Master UDMA Auto IDE Primary Slave UDMA Auto IDE Secondary Master UDMA Auto IDE Secondary Slave UDMA Auto IDE HDD Block Mode Enabled | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

On-Chip IDE Primary/Secondary PCI IDE

The integrated peripheral controller contains an IDE interface with support for two IDE channels.  Select Enabled to activate each channel separately.  The settings are: Enabled and Disabled.

IDE Primary/Secondary Master/Slave PIO

The four IDE PIO (Programmed Input/Output) fields let you set a PIO mode (0-4) for each of the four IDE devices that the onboard IDE interface supports.  Modes 0 through 4 provide successively increased performance. In Auto mode, the system automatically determines the best mode for each device.  The settings are: Auto, Mode 0, Mode 1, Mode 2, Mode 3, Mode 4.

IDE Primary/Secondary Master/Slave UDMA

Ultra DMA/33 implementation is possible only if your IDE hard drive supports it and the operating environment includes a DMA driver (Windows 95 OSR2 or a third-party IDE bus master driver).  If your hard drive and your system software both support Ultra DMA/33 and Ultra DMA/66, select Auto to enable BIOS support.  The settings are: Auto, Disabled.

IDE HDD Block Mode

Block mode is also called block transfer, multiple commands, or multiple sector read/write.  If your IDE hard drive supports block mode (most new drives do), select Enabled for automatic detection of the optimal number of block read/writes per sector the drive can support.  The settings are: Enabled, Disabled.

3-7-2   OnBoard PCI DEVICE

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software OnBoard PCI DEVICE |  |
| --- | --- |
| USB Controller Enabled USB Keyboard Support Disabled ** AC97 Codec is ALC100/200 ** AC97 Audio Auto Game Port Address 201 Midi Port Address 330 Midi Port IRQ 10 | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

USB Controller

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have USB peripherals.  The settings are: Enabled, Disabled.

USB Keyboard Support

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have a USB keyboard.  The settings are: Enabled, Disabled.

AC97 Audio

This item allows you to decide to enable/disable the OnBoard AC 97’ Codec chip to support Audio function.  The settings are: Auto, Disabled.

Game Port Address/Midi Port Address

This will determine which Address the Game Port/Midi Port will use

3-7-3   Winbond SuperIO Device

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Winbond SuperIO Device |  |
| --- | --- |
| POWER ON Function BUTTON ONLY KB Power ON Password Enter HOT Key Power ON Ctrl-F1 Oboard FDC Controller Enable Onboard Serial Port 1 3F8/IRQ4 Onboard Serial Port 2 2F8/IRQ3 UART Mode Select Normal RxD , TxD Active Hi,Lo IR Transmission Delay Enabled UR2 Duplex Mode Half Use IR Pins IR-Rx2Tx2 Onboard Parallel Port 378/IRQ7 Parallel Mode SPP EPP Mode Select EPP1.7 ECP Mode Use DMA 3 PWRON After PWR-Fail Off | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Onboard FDC Controller

Select Enabled if your system has a floppy disk controller (FDD) installed on the system board and you wish to use it.  If you install add-on FDC or the system has no floppy drive, select Disabled in this field.  The settings are: Enabled and Disabled.

Onboard Serial Port 1/Port 2

Select an address and corresponding interrupt for the first and the second serial ports.  The settings are: 3F8/IRQ4, 2E8/IRQ3, 3E8/IRQ4, 2F8/IRQ3, Disabled, Auto.

UART Mode Select

This item allows you to determine which InfraRed(IR) function of the onboard I/O chip, this functions uses.

Onboard Parallel Port

There is a built-in parallel port on the on-board Super I/O chipset that Provides Standard, ECP, and EPP features. It has the following option:

Disabled

(3BCH/IRQ7)/	Line Printer port 0

(278H/IRQ5)/ 	Line Printer port 2

(378H/IRQ7) 	Line Printer port 1

Onboard Parallel Mode

SPP 	: Standard Parallel Port

EPP 	: Enhanced Parallel Port

ECP	: Extended Capability Port

SPP/EPP/ECP/ECP+EPP

To operate the onboard parallel port as Standard Parallel Port only, choose “SPP.”  To operate the onboard parallel port in the EPP modes simultaneously, choose “EPP.”  By choosing “ECP”, the onboard parallel port will operate in ECP mode only.  Choosing “ECP+EPP” will allow the onboard parallel port to support both the ECP and EPP modes simultaneously.  The ECP mode has to use the DMA channel, so choose the onboard parallel port with the ECP feature.  After selecting it, the following message will appear: “ECP Mode Use DMA” at this time, the user can choose between DMA channels 3 to 1.  The onboard parallel port is EPP Spec. compliant, so after the user chooses the onboard parallel port with the EPP function, the following message will be displayed on the screen: “EPP Mode Select.”  At this time either EPP 1.7 spec. or EPP 1.9 spec. can be chosen.

PWRON After PWR-Fail

# This option will determine how the system will power on after a power failure

# 3-8	 Power Management Setup

The Power Management Setup allows you to configure your system to most effectively save energy saving while operating in a manner consistent with your own style of computer use.

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Power Management Setup |  |
| --- | --- |
| ACPI Function Enabled Power Management User Define Video off Method V/H SYNC+Blank Video Off In Suspend Yes Suspend Type Stop Grant MODEM Use IRQ 3 Suspend Mode Disabled HDD Power Down Disabled Soft-off by PWR-BTTN Instant-Off Wake-Up by PCI Card Disabled Power On by Ring Disabled CPU Thermal-Throttling 50.0% Resume by Alarm Disabled x Date (of Month) Alarm 0 x Time (hh:mm:ss) Alarm 0 : 0 : 0 > Reload Timer Events Press Enter | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

ACPI Function

This item allows you to Enabled/Disabled the Advanced Configuration and Power Management (ACPI).  The settings are Enabled and Disabled.

Power Management

This category allows you to select the type (or degree) of power saving which is directly related to the following modes:

User Define(default)	Allows you to set each mode individually. When not disabled, each of the ranges is from 1 min. to 1hr. except for HDD Power Down that ranges from 1 min. to 15 min. and disable.

Min Saving	Minimum power management. Doze Mode 1H, Suspend Mode=1 hr., Power Down=15 min.

Max Saving	Maximum power management. Doze Mode 1H, Suspend Mode=1 min., Power Down=1 min.

HDD Power Down

When enabled and after setting time of system inactivity, the hard disk drive will be powered down while all the other devices remain active.

The settings are: 1/2/3/4/5/6/7/8/9/10/11/12/13/14/15 Min and Disabled.

Suspend Mode

When enabled and after the set time of system inactivity, all devices except the CPU will be shut off.

The settings are: l/2/4/8/12/20/30/40 Min, 1 Hour, and Disabled.

Video Off Option

This determines the manner in which the monitor is blanked.  The choice are Suspend  off, All Modes  Off, and Always On.

Video Off Method

This determines the manner in which the monitor is blanked.

DPMS (default)	Initial display power management signaling.

Blank Screen	This option only writes blanks to the video buffer.

V/H SYNC+Blank	This selection will cause the system to turn off the vertical and horizontal synchronization ports and write blanks to the video buffer.

Modem Use IRQ

This determines the IRQ in which the MODEM can use.

The settings are: 3, 4, 5, 7, 9, 10, 11, NA.

Soft-off by PWR-BTTN

Pressing the power button for more than 4 seconds forces the system to enter the Soft-Off state.  The settings are: Delay 4 Sec, Instant-Off.

Wake Up by PCI card

This will enable the system to wake up to PCI LAN Card.

The settings are: Enabled and Disabled.

Power On by Ring

During Disabled, the system will ignore any incoming call from the modem. During Enabled, the system will boot up if there’s an incoming call from the modem.

CPU Thermal-Throttling

Select the CPU THRM-Throttling rate.  The settings are: 12.5%, 25.0%, 37.5%, 50.0%, 62.5%, 75.0%, 87.5%.

Resume by Alarm

This function is for setting date and time for your computer to boot up.  During Disabled, you cannot use this function.  During Enabled, choose the Date and Time Alarm:

Date(of month) Alarm

You can choose which month the system will boot up.  Set to 0, to boot every day.

Time(hh:mm:ss) Alarm

You can choose what hour, minute and second the system will boot up.

Note:	If you have change the setting, you must let the system boot up until it goes to the operating system, before this function will work.

Reload Timer Events

Please refer to section 3-8-1

3-8-1   Reload Timer Events

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Reload Timer Events |  |
| --- | --- |
| Primary IDE 0 Disabled Primary IDE 1 Disabled Secondary IDE 0 Disabled Secondary IDE 1 Disabled FDD,COM,LPT Port Disabled PCI PIRQ[A-D]# Disabled | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Reload Timer Events

Reload Global Timer events are I/O events whose occurrence can prevent the system from entering a power saving mode or can awaken the system from such a mode. In effect, the system remains alert for anything that occurs to a device which is configured as Enabled, even when the system is in a power down mode.

Primary IDE 0

Primary IDE 1

Secondary IDE 0

Secondary IDE 1

FDD, COM, LPT Port

PCI PIRQ[A-D] #

3-9	 PnP/PCI Configuration Setup

This section describes configuring the PCI bus system. PCI, or Personal Computer Interconnect, is a system which allows I/O devices to operate at speeds nearing the speed the CPU itself uses when communicating with its own special components. This section covers some very technical items and it is strongly recommended that only experienced users should make any changes to the default settings.

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software PnP/PCI Configurations |  |
| --- | --- |
| Reset Configuration Data Disabled Resources Controlled By Manual x IRQ Resources Press Enter PCI/VGA Palette Snoop Disabled | Item Help |
|  | Menu Level > Default is Disabled. Select Enabled to reset Extended System Configuration Data ESCD) when you exit Setup if you have installed a new add-on and the system reconfiguration has caused such a serious conflict that the OS cannot boot |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Resource Controlled By

The Award Plug and Play BIOS has the capacity to automatically configure all of the boot and Plug and Play compatible devices.  However, this capability means absolutely nothing unless you are using a Plug and Play operating system such as Windows95/98.  If you set this field to “manual” choose specific resources by going into each of the sub menu that follows this field (a sub menu is preceded by a “>”).

The settings are: Auto (ESCD), Manual.

IRQ Resources

When resources are controlled manually, assign each system interrupt a type, depending on the type of device using the interrupt.

PCI/VGA Palette Snoop

Leave this field at Disabled.  The settings are Enabled, Disabled.

# 3-10  PC Health Status

This section shows the Status of you CPU, Fan, Warning for overall system status.  This is only available if there is Hardware Monitor onboard.

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software PC Health Status |  |
| --- | --- |
| Show PC Health in Post Enabled CPU Warning Temperatyre Disabled Warning_Beep Enabled Shutdown Temperature Disabled Current System Temp. 25。C/77。F Current CPU Temperature 49。C/120。F Current FAN1 Speed. 4172 RPM Current FAN2 Speed. 4358 RPM Vcore 2.0 V Vtt(V) 1.55 V Vcc3.3 3.36 V +5V 4.94 V +12V 11.97 V -12V -12.11 V -5V - 4.94 V VBAT(V) 3.31 V 5VSB(V) 5.34 V | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Show PC Health in Post

During Enabled, it displays information list below.  The choice is either Enabled or Disabled

Current CPU Temperature/Current System Temp/Current FAN1, FAN2 Speed/Vcore/ Vtt/Vcc3.3/+5V/+12V/-12V/-5V (V)

This will show the CPU/FAN/System voltage chart and FAN Speed.

Shutdown Temperature

This option is for setting the Shutdown temperature level for the processor.  When the processor reaches the temperature you set, this will shutdown the system.

# 3-11  Miscellaneous Control

This section is for setting CPU Frequency Control.

| CMOS Setup Utility – Copyright(C) 1984-2001 Award Software Miscellaneous Control |  |
| --- | --- |
| Auto Detect DIMM/PCI Clk Enabled Spread Spectrum Disabled CPU Host/SDRAM Clock Default CPU Clock Ratio X 3 | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Auto Detect DIMM/PCI Clk

This item allows you to enable/disable auto detect DIMM/PCI Clock.

The settings are: Enabled, Disabled.

Spread Spectrum

This item allows you to set the Spread Spectrum.

CPU/SDRAM Clock

This item allows you to select the CPU/SDRAM Clock, refer to Page 11.

CPU Clock Ratio

This item allows you to select the CPU ratio.

# 3-12  Load Standard/Optimized Defaults

Load Standard Defaults

When you press <Enter> on this item, you get confirmation dialog box with a message similar to:

Load Standard Defaults (Y/N)? N

Pressing <Y> loads the BIOS default values for the most stable, minimal-performance system operations.

Load Optimized Defaults

When you press <Enter> on this item, you get a confirmation dialog box with a message similar to:

Load Optimized Defaults (Y/N)? N

Pressing <Y> loads the default values that are factory settings for optimal performance system operations.

# 3-13  Set Supervisor/User Password

You can set either supervisor or user password, or both of them.  The differences are:

Supervisor password: 	Can enter and change the options of the setup menus.

User password:	Can only enter but do not have the right to change the options of the setup menus.  When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD:

Type the password, up to eight characters in length, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the selection and not enter a password.

To disable a password, just press <Enter> when you are prompted to enter the password.  A message will confirm that the password will be disabled.  Once the password is disabled, the system will boot and you can enter Setup freely.

PASSWORD DISABLED.

When a password has been enabled, you will be prompted to enter it every time you try to enter Setup.  This prevents an unauthorized person from changing any part of your system configuration.

Additionally, when a password is enabled, you can also require the BIOS to request a password every time your system is rebooted.  This would prevent unauthorized use of your computer.

You determine when the password is required within the BIOS Features Setup Menu and its Security option.  If the Security option is set to “System”, the password will be required both at boot and at entry to Setup.  If set to “Setup”, prompting only occurs when trying to enter Setup.

## 第四章

驅動程式和附贈軟體的安裝

在主機板的包裝內含有一片 MAGIC INSTALL 光碟片。這張光碟片包含主機板所需的所有驅動程式和一些免費的應用軟體、工具軟體。並且，這光碟片也包含一個自動安裝程式的軟體，它能導引你進行驅動程式的安裝，以及何種驅動程式需要安裝，從而簡化安裝步驟，該安裝程式軟體就是本公司自行開發完成的MAGIC INSTALL。

# 支援 WINDOWS 95/98/98SE/NT4.0/2000 的MAGIC INSTALL

把光碟片插入光碟機，然後將出現MAGIC INSTALL 主畫面。如果沒有出現此畫面，請按下“我的電腦”然後選擇光碟機，或者在 ”開始” 的目錄選擇“執行”﹐輸入“Ｘ:\SETUP.EXE” (假設你的光碟機路徑是 Ｘ：)

在MAGIC INSTALL 主畫面有8項選擇

1. INF	安裝 INTEL 810E晶片組驅動程式

2. IDE	安裝ULTRA ATA33 or ULTRA ATA 66 驅動程式

3. VGA	安裝內建式 VGA顯示器之驅動程式

4. PC-HEALTH	安裝硬體監控設備SMART GUARDIAN 軟體

5. SOUND	安裝AC’97 音效卡驅動程式以及editing/playback的程式

6. PC-CILLIN	安裝PC-CILLIN98 防病毒程式

7. BROWSE CD	瀏覽CD內容

8. EXIT	退出MAGIC INSTALL目錄

每項選擇說明如下﹕

4-1  INF		安裝 INTEL 810E晶片組系統驅動程式

安裝完操作系統(WINDOWS 95/98/ 98SE)之後，在“裝置管理員” （開始/設定/控制台/系統/裝置管理員）將會出現UNKNOWN DEVICE 的訊息，你可按如下步驟安裝 INF 驅動程式：

| - 在MAGIC INSTALL 畫面單擊 INF | - 當晶片驅動軟體安裝畫面出現，單擊 NEXT |
| --- | --- |

| - 顯示此驅動程式支援的主機板列表時， 單擊 NEXT | - 如果你希望重新啟動系統，單擊 Finish |
| --- | --- |

注意: 	MAGIC INSTALL 能自動尋找X:\INTEL810\INF\INFINST.EXE 的程式。

此驅動程式可支援 WINDOWS 95/98/98SE/2000  ( NT4.0不需要)

4-2  IDE		安裝 Intel ULTRA ATA 33/66 Storage 驅動程式

| - 在MAGIC INSTALL 介面單擊 IDE | - 當出現INTEL Ultra ATA Storage Wizard 時，單擊 NEXT |
| --- | --- |

| - 在版權說明出現後，單擊NEXT | - 單擊NEXT 或 單擊BROWSE 以改變路徑 |
| --- | --- |

| - 如果你希望重新啟動系統，單擊 Finish | - 你可以在“增加、刪除程式” 中刪除此驅動程式 |
| --- | --- |

注意: 	此驅動程式的路徑為 X:\INTEL810\Ide\SETUP.EXE

此驅動程式支援 WINDOWS 95/98/98SE/NT4.0/2000

4-3  VGA		安裝內建式 VGA 驅動程式

- 使用WINDOWS 95/98/98SE/NT4.0

| - 在MAGIC INSTALL 介面單擊 VGA | - 當 INTEL 81X Family Chipset Graphics Driver 軟體出現後，單擊 NEXT |
| --- | --- |

| - 在版權說明出現後，單擊NEXT | - 如果你希望重新啟動系統，單擊 Finish |
| --- | --- |

注意: 	此驅動程式的路徑

WIN95 的路徑為 X:\INTEL810\VGA\WIN95\SETUP.EXE

WIN98 and WIN98SE 的路徑為 X:\INTEL810\VGA\WIN9X\SETUP.EXE

NT4.0的路徑為 X:\INTEL810\VGA\NT40\SETUP.EXE

B. 使用 WINDOWS 2000

在Windows 2000 中沒有提供自動檢測功能。請按照如下步驟安裝 VGA 驅動程式：

| - 在 WINDOWS 2000 中， 雙擊“我的電腦/控制台/系統/裝置管理員 | - 單擊 “其他設備/顯示卡” (VGA COMPATIBLE) |
| --- | --- |

| - 單擊“驅動程式/更新驅動程式” | - 出現更新驅動程式精靈時，單擊“下一步” 選擇“顯示所有驅動程式列表，我要自己選擇適合的驅動程式”，單擊“下一步” |
| --- | --- |

| - 在硬體類型中，選擇“顯示卡”，單擊 “下一步” | - 選擇從磁碟機安裝 |
| --- | --- |

| - 當“磁碟機安裝” 視窗出現時，選擇路徑X:\INTEL810\VGA\WIN2000, 單擊“下一步” | - 從 DRIVER LIST 選取 INTEL® 82810E GRAPHICS CONTROLLER，再選按 NEXT。安裝完成後，選按NEXT，再選按FINISH，系統即會自動重新開機 |
| --- | --- |

4-4  PC-HEALTH	安裝硬體監控程式 PC HEALTH MONITOR IIII

| - 在MAGIC INSTALL 畫面單擊 PC -HEALTH | - 當 PC HEALTH MONITOR IIII 安裝精靈出現時單擊“OK” |
| --- | --- |

| - 單擊安裝圖示即可開始安裝軟體 | - 安裝完成後，單擊“OK” |
| --- | --- |

注意：MAGIC INSTALL 會自動偵測 X:\INTEL810\HEALTH\SETUP.EXE 的路徑

此驅動程式支援 WINDOWS 95/98/98SE

4-5   安裝AC97音效卡驅動程式和editing/playback程式

| - 在MAGIC INSTALL 畫面單擊 SOUND | - 然後自動檢測操作系統語言，單擊“NEXT”，即可開始安裝驅動程式，再單擊 “NEXT”，即會出現版權資訊 |
| --- | --- |

| 3. 當系統詢問 “ Remove old device driver”，選按 OK，再按 GO | 4. 選按 ”Finish”，然後重新啟動 Windows |
| --- | --- |

| 5. Windows 重新啟動後，即可自動偵測到供Advance AC97 驅動裝置使用的Intel® 82801AA controller |  |
| --- | --- |

注意:	 此驅動程式的路徑

Windows 95 對應到 X:\CODEC\ALC200\WIN95\SETUP.EXE

Windows 98/98SE 對應到 X:\CODEC\ALC200\WIN98\SETUP.EXE

Windows NT 4.0 對應到 X:\CODEC\ALC200\WINNT\SETUP.EXE

Windows 2000 對應到 X:\CODEC\ALC200\WIN2000\SETUP.EXE

4-6   PC-CILLIN	安裝PC-CILLIN 2000防病毒程式

| - 在MAGIC INSTALL 畫面單擊PC-CILLIN | - 當 ”PC-CILIN 2000 SETUP” 視窗出現後，單擊“NEXT”，即可出現版權說明，單擊“YES”，軟體就開始檢查硬碟 |
| --- | --- |

| - 單擊“NEXT” 或選“BROWSE” 改變安 裝路徑 | - 選按 “NEXT” 並選取所有的網際網路保護 |
| --- | --- |

| 如果你有Proxy Server，輸入你的設定，再選按 “OK” | - 選按 “NEXT” 開始複製檔案及安裝軟體 |
| --- | --- |

| - 如果你想製造一片緊急修復磁碟片，請插空白磁碟片插入1.44MB軟碟機 | - 單擊“Finish”，重新啟動系統 |
| --- | --- |

| 輸入你的名字和 E-mail 位址，以便註冊PC-cillin 2000，或選按 “Cancel” 日後再註冊 | - PC-cillin 2000 安裝完成後，我們建議你選取 ”update” 選項來下載最新的防毒碼，並將它設定成 “Auto refresh” 的模式 |
| --- | --- |

4-7   如何使用ALSRACK EDITING & PLAYBACK 播放程式

| - 單擊 “開始/程式/AVANCE MEDIA PLAYER/ALSRACK”，然後ALSRACK會開始執行 | - 此程式可播放CD片，而且效果可與HI-FI 立體聲系統相媲美，也能播放 *.WAV、 *.MID等檔案 |
| --- | --- |

| - 這是一個音效環境模擬器，提供強大的仿真能力，包括歌劇院、啤酒屋、運動場的音場模擬 |  |
| --- | --- |

# KARAOK 則有高低音調的調整，可以分別進行低音部分之高或低8度的調整，以及高音部分之高或低8度的調整。

# 注意：改變選項之後，聲音效果也會隨之改變，使用者必須重新設定全部選項以取得原來的聲音效果。

# 4-8   如何使用PC-HEALTH

| - 選按 Program  PC HEALTH MONITOR IIII 即可呼叫 PC HEALTH MONITOR IIII 公用程式。你亦可經由在Control Panel 中的Add/Remove 程式的圖示移除此程式 | - 當你開始執行 PC HEALTH MONITOR IIII ，即可看到系統電壓、風扇轉速、CPU/系統溫度。因為這軟體是及時間控軟體，所以上述的值會因環境不同而改變。如果該值超過設定值，則會出現警告訊號和警告聲音 |
| --- | --- |

# 4-9   如何關閉內建式音效卡

進入BIOS SETUP程式，選擇 INTEGRATE PERIPHERALS，選擇 ON-CHIP DEVICE FUNCTION，選擇AC97 AUDIO，按PAGE DOWN 鍵選擇 Disable，即可關閉主機板上的音效裝置。

# 4-10   如何更新 BIOS

第一步.	準備一張啟動磁片 (你可以在 “開始/執行”中輸入“SYS A:”，單擊 “確定”)。

第二步. 	將工具軟體複製到啟動磁片，可以將光碟片的X:\FLASH\AWDFLASH.EXE 複製到磁碟片，或從我們的網頁下載。

第三步. 	從我們的網頁下載最新的IB3 BIOS，並複製到啟動磁片。

第四步.	插入啟動磁片到A磁碟槽，啟動系統，看到 “A：” 的提示後，輸入

“Awdflash A:\IB3xxx.BIN /SN/PY/CC/R” 指令，IB3xxx.BIN 代表最新版本的BIOS，它類似IB3A03.BIN 或 IB3B02.BIN。

SN 	表示  不保存現在的BIOS 數據

PY  	表示  更新BIOS 數據

CC  	表示  清除 CMOS 數據

R	    	表示  重新啟動系統

第五步.	按 ENTER鍵，等BIOS 被重新更新後，系統即自動重新啟動。