Flash utility usage:

FP /N=OEM_Name[,HotlineNo]  BIOSFileName

Note:

- The maximum length of OEM_Name is 16.

- The OEM_Name is case-sensitive.

- The system manufacturer name read from DMI BIOS interface is OEM_Name padded with blanks (ASCII 20h) and the length of OEM_Name+blanks is 16.

- If you need space character for OEM_Name and HotlineNo,

Replace it with a special character “^”.

- After flash the BIOS, turn power off and turn it on, then you can see the OEM_Name by using the DEBUG command as following:

DEBUG

-D F000:1C00

- Some valid examples:

a. The hotline number for “Test Computer” company is

123 4567 890. Then the usage for FP is as following:

FP /N=Test^Computer,123^4567^890 BIOS.BIN

b. The hotline number for “MyComputer” company is

123-4567-890. Then the usage for FP is as following:

FP /N=MyComputer,123-4567-890 BIOS.BIN