Flash utility usage :

FP /N=OEMName[,HotlineNo]  BIOSFileName

Note :

- The maxium length of OEMName is 16.

- The OEMName is casesensitive.

- The system manufacturer name read from DMI BIOS interface is OEMName padded with blanks(ASCII 20h) and the length of OEMName+blanks is 16.

- If you need space character for OEMName and HotlineNo,

Replace it with a special character “^”.

- After flash the BIOS, turn power off and turn it on, then you can see the OEMName by using the DEBUG command as following :

DEBUG

-D F000:1C00

- Some valid examples:

a.The hotline number for “Test Computer” company is

123 4567 890. Then the usage for FP is as following :

FP /N=Test^Computer,123^4567^890 BIOS.BIN

b.The hotline number for “MyComputer” company is

123-4567-890. Then the usage for FP is as following :

FP /N=MyComputer,123-4567-890 BIOS.BIN

c.To disable OEMName and HotlineNo : (FP v1.41 or later)

FP /N=KAPOK,KAPOK  BIOS.BIN