-- version 04.13  -------------------------------------------------------------------

Release version.

-- version 04.13 T2  -------------------------------------------------------------------

1. Fixed system unstable under WinOS with 100 MHz frequency CPU.

-- version 04.13 T1  -------------------------------------------------------------------

1. Suppot KME330 CDRW, Fixed system still of the POST.

-- version 04.12  -------------------------------------------------------------------

Release version.

-- version 04.12 T2 -------------------------------------------------------------------

1. Adjust 694x Reg7B, try to fix Win2000 easy hang.

-- version 04.12 T1 -------------------------------------------------------------------

1. Adjust throttle point for 1G CPU.

(60∘C off, 65∘C cpu speed down to 87.5%-93.75%)

-- version 04.11 -------------------------------------------------------------------

Release version.

-- version 04.11 T1 --------------------------------------------------------------------

1. Fixed can’t turn on fan to hight speed under Win2000.

45∘C	Fan Low Speed

55∘C	Fan Hight Speed

-- version 04.11 T1 --------------------------------------------------------------------

1. Reduce power on warning message when CMOS data invalid.

2. Fixed FDD/FIR function abnormal after resume from S2D under DOS mode..

-- version 04.10 --------------------------------------------------------------------

1. Fixed can’t pass COM1 external loopback test.

Release version base on 0410 T9.

-- version 04.10 T9 --------------------------------------------------------------------

1. Fixed 2nd HDD can’t recognize.

-- version 04.10 T8 --------------------------------------------------------------------

1. Update VGA BIOS to fixed can’t install Portugal, Polish… version Win98se and WinME.

-- version 04.10 T7 --------------------------------------------------------------------

1. Fixed no display when OS restarting then close and open LID switch.

2. Disable CPU serial number.

3. Fixed press Fn+F11, Fn+F12 cause noise during audio device active.

-- version 04.10 T6 --------------------------------------------------------------------

1. Fixed sometimes battery capacity not correct.

2. Fixed sometimes CMOS checksum error.

3. Fixed system hang on POST when detecting CDROM/HDD.

4. Modify ComB mode “IrDA” instead of “FIR”.

6. Show correct speed for 633,667,850 CPU.

[Throttle - Only for 1GHz or above CPU]

58∘C	Throttle Off

62∘C	87.5% - 93.75% CPU performance

72∘C	75% - 81.25% CPU performance

82∘C	68.75% - 75% CPU performance

-- version 04.10 T5 --------------------------------------------------------------------

1. Modify battery low condition

9000ma battery	1. H/W pin action low and  remaining percentage<=7%

2. H/W pin no action low but remaining percentage<=5%

7000ma/7500ma battery	1. H/W pin action low and remaining percentage<=8%

2. H/W pin no action low but remaining percentage<=6%

2. When charger full action and Gas Gauge >= 90% then report remaining percentage=100%

to ACPI OS. (In legacy mode always report physical Gas Gauge value).

This version base on 0409 modify.

-- version 04.10 T3 --------------------------------------------------------------------

1. Try to fix floppy test fail issue. (Adjust ISA bus clock to PCI clock / 5)

-- version 04.10 T2 --------------------------------------------------------------------

1. Try to fix floppy test fail issue. (Adjust ISA bus clock to PCI clock / 6)

-- version 04.09 --------------------------------------------------------------------

1. Fixed DVD ROM down speed issue.

2. When max. error occur then reset 7500ma battery FCC to 6750ma.

-- version 04.08 --------------------------------------------------------------------

1. Revert APM function for M/E battery test tool.

-- version 04.07 --------------------------------------------------------------------

Correct BIOS for M/P.

1. Fixed “keyboard error” or “parity check 2” after flash BIOS.

-- version 04.06 --------------------------------------------------------------------

The BIOS for M/P use.

1. Fixed can’t boot from CDROM after HDD boot failed.

2. Support 7000ma battery for max. error solution.

-- version 04.06 T3 --------------------------------------------------------------------

1. Try to fix sometimes floppy device not found under Win2000.

-- version 04.06 T2 --------------------------------------------------------------------

1. When battery max. error occur then reset battery to default value.

-- version 04.06 T1 --------------------------------------------------------------------

1. Fixed MKE CD R/W and ZIP device can’t detect after do BIOS setup.

-- version 04.05 --------------------------------------------------------------------

1. Reinstall thermal controller for ACPI OS.

2. Modify battery low condition for Legacy mode.

-- version 04.05 T2  --------------------------------------------------------------------

1. Try to fix system hang up during run WinReboot.

-- version 04.05 T1  --------------------------------------------------------------------

1. Try to fix system hang up during run WinReboot.

-- version 04.04  --------------------------------------------------------------------

1. Enabled IcWorks clock spectrum for EMI.

-- version 04.03  --------------------------------------------------------------------

1. Solve system hang up issue, during run Windows reboot program over night.

2. Change “Spdif” to “SPDIF” in setup menu.

-- version 04.03 T1 --------------------------------------------------------------------

1. Update VGA BIOS to fixed Panasonic panel display reverse during mode change.

-- version 04.02 --------------------------------------------------------------------

The BIOS for PVT (H/W D03) only..

1. Initialize correct data to cardbus controller for H/W D03 video-in function..

-- version 04.01 --------------------------------------------------------------------

1. Fixed smetimes press any key can’t wake up from S1.

-- version 03.08 T2 --------------------------------------------------------------------

1. Change battery EDVI from 10v to 9.9v.

2. Fixed sometimes system auto shutdown.

-- version 03.08 T1 --------------------------------------------------------------------

1. Fixed insert/remove R2 card cause system hang after resume from S4 under Win2000.(Bug#95)

2. Try to fix system hang up, When running the “WinBoot” over night.(Bug#4)

3. Improve 3Dmark2000 task abnormal close.(Bug#114)

-- version 03.07 --------------------------------------------------------------------

For Windows logo test only.

1. No S/W battery low low shutdown protection.

2. No MDC ring-in waku-up function.

3. EC periodic send event to system for battery status change monitor.

-- version 03.06 --------------------------------------------------------------------

1. Try to fix system hang up, When running the WinBoot over night.

2. Shutdown the system, when battery percentage less 1%.

-- version 03.06 T2 --------------------------------------------------------------------

1. Fixed LID switch function abnormal cause system no display.

2. Fixed no display after resume from MDC ring.

-- version 03.06 T1 --------------------------------------------------------------------

1. Fixed LID switch function abnormal cause system auto power on.

conjugation H/W rework

2. Disable CKE to fix system power failed when install IBM 64MB SDRAM & 933 MHz CPU.

-- version 03.05 --------------------------------------------------------------------

1. Fixed SMC IrCC show yellow mark after resume from S4BIOS.

2. Fixed can’t boot from ZIP device.

3. Adjust battery low percentage from 12% to 8% (base on H/W pin).

Adjust battery low percentage from 7% to 6% (base on S/W control).

4. Update VGA BIOS try to fix panel display reverse.

5. Update VGA BIOS try to fix panel background was flicker when shutdown dialog box appears.

-- version 03.05 T2 --------------------------------------------------------------------

1. Cancel 0305T1BIOS Adjust battery low percentage from 12% to 10% (base on H/W pin).

It same as 0304; 12%

2. Cancel update 0304 VGA BIOS

It same as 0303 VGA BIOS (2000/08/14).

-- version 03.05 T1 --------------------------------------------------------------------

1. Adjust battery low percentage from 12% to 10% (base on H/W pin).

-- version 03.04 --------------------------------------------------------------------

1. Fixed high frequency noise.

2. Update VGA BIOS to fixed panel display reverse.

-- version 03.03 --------------------------------------------------------------------

1. Fixed MDC can’t dialing after resume from standby mode.

Limitation: After resume you need disconnect first then dialing.

2. Fixed sometimes battery remaining capacity not correct. (Bug#10,12)

3. Try to fix system will auto reset/shutdown when plug in/out AC adapter. (Bug#11)

4. Fixed do battery low hibernate then plug AC adapter and wakeup system, It still has battery low beep sound. (Bug#13)

5. Fixed Anykey/LidSwitch event cause system auto power on. (Bug#20,22)

-- version 03.02 --------------------------------------------------------------------

1. Fixed Pcmcia R2 card function abnormal under Win2000.

2. Fixed press any key cause system auto power on under DOS mode.

NOTE: The BIOS need  conjugation H/W rework, 1394 route to INTD.

-- version 03.01 --------------------------------------------------------------------

1. Fixed first time can’t enter standby mode under Win98.

2. Check H/W pin to determine battery remaining percentage.

3. Fixed system easy hang under Win2000 and run 3Dmark 2000.

4. Fixed only attached to CRT then remov CRT cause display can’t switch to LCD.

NOTE:The BIOS for H/W D02 only.

-- version 02.06 --------------------------------------------------------------------

1. Fixed cardbus card function abnormal after restart from Win98/Win2000.

2. Prevent system enter S2D mode under WinNT4.0(non APM/ACPI OS).

-- version 02.06t4 --------------------------------------------------------------------

1. Fixed high frequency noice.(Bug#77)

2. Fixed system can’t resume from standby or montior off.(Bug#58,68)

conjugation VGA driver version M631CD31(Win98/Win2000)

3. Implement Cardbus function for NT4.(Bug#95)

-- version 02.06t3 --------------------------------------------------------------------

1. Update CPU micro code.

2. Fixed the FDD still can access when set user password.(Bug#91)

3. Report battery detail information for ACPI OS.(Bug#92)

-- version 02.06t2 --------------------------------------------------------------------

1. Fixed battery low continue beep when system enter standby mode.

2. Implement APM function 530Ah(get power status).(Bug#88)

3. Report fan device under ACPI OS.

4. Fixed system auto shutdown during resume from S4BIOS without AC.

-- version 02.06t1 --------------------------------------------------------------------

1. Implement battery police feature.

conjugation D/D  - D01a

H/W - U10 pin 10 翹腳 & pull high

2. Fixed can't wake-up on LAN.

-- version 02.05 --------------------------------------------------------------------

1. Fixed speaker test fail under legacy mode. (Bug#23,28)

2. Fixed can't pass AmiDiag6 COM3 port test. (Bug#25)

3. Remove Fn+F3(LCD expand) function.

-- version 02.05t4 --------------------------------------------------------------------

1. Try to fix system hang/reset issue.

2. Disable clock spread spectrum.

3. Fixed Fn+F10(brightness adjust) no function. (Bug#55)

-- version 02.05t3 --------------------------------------------------------------------

1. Reduce access EC RAM on SCI cycle try to fix system random reset under ACPI OS.

2. Adjust fan active point. 25.C->fan low speed, 35.C->fan high speed.

-- version 02.05t2 --------------------------------------------------------------------

1. Disable EC polling event under Win98 ACPI mode.

2. Adjust fan active point. 30.C->fan low speed, 40.C->fan high speed.

-- version 02.04t2 --------------------------------------------------------------------

1. Fixed AmiDiag6.0 PnP function 42 test fail. (Bug#24,26)

2. Fixed HCT fail on unreport I/O.

3. Modify 694xReg54, 694xReg6D (VIA suggest)

-- version 02.04t1 --------------------------------------------------------------------

1. Adjust DRAM drive strength to maximum.

2. Modify host bridge some register (VIA suggest).

3. Change VGA BIOS to support Hitachi panel. (Bug#2)

4. Add. ID "KAPOK:8500V-000" for ME test utility identify

-- version 02.03 --------------------------------------------------------------------

1. Update VGA BIOS to support Video-IN function.

2. Fixed system can't auto power off after do S4 under Win2000.

3. Fixed audio resource conflict at I/O 0x220, 0x388.

-- version 02.02 --------------------------------------------------------------------

1. The BIOS for 8500V H/W version D01.

2. Fixed can't turn on LCD.

-- version 02.01+ --------------------------------------------------------------------

The BIOS for 8500V H/W version D01.

-- version P08t6 --------------------------------------------------------------------

1. Fixed system can't pass "ACPI Power Button Override Test".

2. Implement HotKey function under ACPI OS.

3. Fixed repeat beep from PC speaker.

-- version P08t5 --------------------------------------------------------------------

1. Auto detect MDC module.

2. Fixed system sometimes can't shutdown.

3. Fixed CMOS checksum error after do RTC wakeup.

4. Fixed IR function abnormal.

-- version P08t4 --------------------------------------------------------------------

1. Enable clock spread spectrum with ICS9248-39.

2. Fixed system hang during install Win2000.

-- version P08t3 --------------------------------------------------------------------

1. Remove 4 speaker support.

2. Enable MC97 codec for MDC modem.

3. Fixed PS2 mouse can't use.

-- version P08t2 --------------------------------------------------------------------

1. Fixed keyboard can't use after resume from POS by anykey wakeup.

2. Fixed battery information not correct under ACPI Win98.

3. Implement power off function for power button.

(conjugation H/W rework)

4. Fixed system sometimes can't power on.

(conjugation H/W rework)

5. Modify thermal rule.

39.C below	: Fan Off

40.C - 55.C	: Fan Normal Speed

56.C above	: Fan Fast Speed

-- version P08t1 --------------------------------------------------------------------

Support remote(CPU Die) and local(Thermal Sensor) temperature polling.

-- version P07 --------------------------------------------------------------------

1. Fixed about unreport I/O in Win98 when test with HCT.

2. Fixed internal keyboard matrixs not correct.

3. Adjust DRAM frequency to match CPU frequency.

4. Show correct message for 733MHz CPU.