P5MVP3

MAIN BOARD

User’s Guide

### Trademarks

Microsoft MS-DOS, WindowsTM, Windows95 、 Windows98 and Windows2000 are registered trademarks of Microsoft Corporation. Intel, PentiumII, PentiumIII and CeleronTM are registered trademarks of Intel Corporation. Award is a registered trademark of Award Software, Inc. Other trademarks and registered trademarks of products appearing in this manual are the properties of their respective holders.

Rev:1.6

Date : June, 2000

0

Read me first

- The “LOAD SETUPD EFAULTS” function loads the system default

data directly from ROM and initializes the associated hardware

properly . This function is Necessary When you accept this mainboard ,

or the system CMOS data will corrupt .

LOAD SETUP DEFAULT

- KBPO(Key Board Power On) Function : There is a basic requirement that

the “+5V SB”power of the ATX power supply must be>=0.1A(100Ma).

Package Checklist

Please check your package which should include all items listed below.

If you find any item damaged or missed , please contact your supplier.

- One mainboard

- One manual

- One IDE ribbon cable

- One floppy ribbon cable

- One Parallel port ribbon cable

- One Serial port ribbon cable(COM1,COM2)

- One PS/2 mouse cable(Optional)

- One AGP driver&Ultra_DMA IDE driver CD

1

Contents

page

## Chapter 1-Introduction………………………………………………………3

1-1 Notice………………………………………………………………4

## Chapter 2-Hardware Design…………………………………………………5

2-1 Mainboard Layout…………………………………………………7

2-2 Jumpers/connectors settings……………………………………….8

2-3 System Memory Configuration………………………………… ..18

2-4 ATX Power ON/OFF Control……………………………………..19

2-5 Extemal Modem Ring-in Power ON and Keyboard Power

### ON Functions………………………………………………… …..20

Chapter 3-Award BIOS Setup………………………………………………20

3-1 Standard CMOS Setup…………………………………………….21

3-2 BIOS Features Setup………………………………………………22

3-3 Chipset Features Setup…………………………………………….27

3-4 Power Management Setup…………………………………………30

3-5 PNP/PCI Configuration……………………………………………34

3-6 Load Setup Detectios………………………………………………36

3-7 Integrated Periphera………………………………………………..36

3-8 Change Supervisor or User Password……………………………...40

3-9 IDE HDD Auto Detection………………………………………….41

3-10 HDD LOWLEVEL FORMAT…………………………………....44

3-11 Save & Exit Setup………………………………………………...44

3-12 Exit Without Saving……………………………………………...44

2

#### Chaper 1 Introduction

This mainboard is a high performance system hardware based on Intel

Pentium processor and is equipped with an AGP slot , three PCI slots,

Three standard ISA slots , Super Multi-I/O controller and dual port

PCI -IDE connectors for the future expansion .The hardware dimension is

220mm x 230mm with MVP3 dual Design and a four-layer-design

technology .

Specification

- VIA Apllo MVP3 Design AGP/PCIset chipset

- Intel Pentium Processor , Pentium Processor with MMX

technology , AMD K5/K6/K6-2/K6-3 , Cyrix 6x86L/6x86MX &

idt C6 operating at 120~500MHZ with 321 ZIF socket 7 provides

scalability to accept faster Processors in the future .

- Supports up to 640 MegaBytes of memory (168-Pin DIMM

SOCKET x2 & 72 -Pin SIMM SOCKET x2) .

- Supports 512KB(Pipelined Burst SRAM)L2 Cache .

- Supports three 16 bit ISA slots , three 32 bit PCI slots and AGP

lot and  Provides two independent high performance PCI IDE

interfaces capable of supporting PIO Mode 3/4 and

Ultra-DMA33 devices .

- Supports a floppy port , a parallel port (EPP , ECP port) , two serial

ports(16550 Fast UART compatible) , 1USB Connector , a PS/2 style

mouse connector and an AT style keyboard connector .

3

- Supports Award Plug & Play BIOS .

- Supports CPU Hardware sleep , APM (Advanced Power Management)

and ACPI (Advanced Configuration Power Interface) .

- Supports an ATX power supply connector for a Remote On/Off , a phone-

Ring power On and a Keyboard power On Function .

- Supports Switching Regulator for CPU power supply .

- Support hardware monitor function .

1-1 Notice :

- This mother board adept Chips Design

MVP3(VT82C598MVP) .

P5MVP3 difference :

- P5MVP3 (VT82C598MVP) North Bridge Support

60MHZ~100MHZ : Bus clock (Support 100MHZ)

二. When you adjust 100MHZ on P5MVP3

Suggest you use DIMM module . Because

The DIMM and SIMM is mixed. If The SIMM timing

Is not the same with DIMM, maybe Can,t match with DIMM.

4

## Chapter 2 Hardware design

## 2-1 Mainboard Layout

### This mainboard is designed with VIA ApolloMVP3 AGP/PCIset chipset which is developed by VIA Corporation to fully support Pentium Processor PCI/ISA system . By providing a massive increase in the bandwidth available between the video card and the processor (66MHZ) , the unique feature of AGP supported by VIA Apollo MVP3 chipset improves the speed of rendering and texturing for 3D graphics. The chipset also provides an integrated IDE controller with two high Performance IDE interfaces for up to four IDE devices (hard devices , CD-ROM device , etc) .

### The SMSC37C669 Super I/O controller provides the standard PC I/O function: one floppy interfacd , two 16 Byte FIFO serial ports and one EPP/ECP capable parallel port . This mainboard layout is shown in the next page for user,s reference . care must be taken when inserting memory modules, CPUs or even plugging PCI card into associated slots to avoid damaging any circuits or sockets on board. A cooling fan is strongly recommended when installing Pentium/Pentium MMX/K5/K6/K6-2/K6-3/6x86/ 6x86L/ 6x86MX/C6 processor due to possible overheat .

This mainboard supports a minimum of 8MB and a maximum of 512MBof system Memory while Onboard 512KB cache to increase system performance .

5

This mainboard supports standard Fast Psge,EDO (Extended Data Out or Hyper Page Mode) or synchronous DRAM . This mainboard provides two 168-pin DIMM & two 72-pin SIMM sites for memory expansion.The sockets support 1Mx64(8MB),2Mx64(16MB), 4Mx64(32MB), and 8Mx64(64MB) single-sided or double-sided memory modules . The memory timing requires 70 ns Fast page devices

or 60ns EDO DRAM . (DRAM Modules be parity [x36] or non-parity [x32] .

### This mainboard supports two Onboard PCI IDE connectors , and automatically detects IDE harddisk type by BIOS utility automatic .This mainboard supports A ward Plug & Play BIOS for the ISA and PCI cards . The BIOS can be located in Flash EPROM which can replace BIOS code easily if neces-sary

6

P5MVP3 Layout

Figure 2-1

7

#### 2-2 Jumpers/Connectors Settings

This section describes some of the connectors on the mainboard .

## 一.  PRN1-Parallel Port

This mainboard provides a 2 x 13-pin parallel port connector .

## 二.  COM1/COM2-Serial Port Connectors

This mainboard provides two 2 x 5-pin serial port connectors .

COM1 and COM2 .

三.  FDC1-Floppy Drive Connector

This mainboard has a 2 x 17-pin floppy drive connector .

四.  IDE1/IDE2-Primary/Secondary IDE Connectors

This mainboard has a 32-bit Enhanced PCI IDE Controller that

Provides two connectors, IDE0 (primary) and IDE1 (secondary) .

五.  Jumper Setting :

JP1: 1-2  (5V For ISSI TAG RAM)

2-3  (3.3V For UMC TAG RAM)

JP2: 1-2  (Reserved)

  - For 512k cache

JP3: 1-2    Interleave Burst

  - linear Burst

8

P5~JP11: clock gernerator Jumper

1.DRAM clock adjust

| DRAM Speed | JP5 | JP9 | ←Fixed as 66MHZ ←SDRAM clock = CPU Bus clock |
| --- | --- | --- | --- |
| Run AGP clock 66MHZ | 2-3 | 1-2 |  |
| Run CPU clock 66/75/83/95/100MHZ | 1-2 | 2-3 |  |

2. The system Bus clock Jumper setting

| CPU Bus | ##### JP6 | JP7 | JP8 | JP10 | JP11 | ← only For MVP3 ← (VT82C598MVP) |
| --- | --- | --- | --- | --- | --- | --- |
| 60MHZ | 2-3 | 2-3 | 2-3 | 2-3 |  |  |
| 66MHZ | 1-2 | 2-3 | 2-3 | 2-3 |  |  |
| 75MHZ | 2-3 | 1-2 | 2-3 | 1-2 | 1-2 |  |
| 83MHZ | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 |  |
| 95MHZ | 2-3 | 1-2 | 1-2 | 1-2 | 2-3 |  |
| 100MHZ | 1-2 | 1-2 | 1-2 | 1-2 | 2-3 |  |

- When you adjust CPU Bus clock to 100MHZ

The SDRAM  has to can support 100MHZ timming

###### JP12:CPU Fan connector

| ##### Pin | Description |
| --- | --- |
| ##### 1 2 | Ground +12v |

9

10

JP13:AT/ATX power Select

###### JP14:Clear CMOS Data

Clear the CMOS memory by shorting this jumper momentarily;

then remove the cap to retain new settings.

JP16~JP18:CPU Speed selectors

| Multiplier | JP16 | JP17 | JP18 |  |
| --- | --- | --- | --- | --- |
| 1.5x/3.5x | OFF | OFF | OFF |  |
| 2.0x/6.0x | ON | OFF | OFF |  |
| 2.5x | ON | ON | OFF |  |
| 3.0x | OFF | ON | OFF |  |
| 4.0x | ON | OFF | ON |  |
| 4.5x | ON | ON | ON |  |
| 5.0x | OFF | ON | ON |  |
| 5.5x | OFF | OFF | ON |  |

| Pentium/MMX/idt-C6 AMD K5/K6/K6-2K6-3 | Multiplier | Bus clock | Cyrix/IBM MII |
| --- | --- | --- | --- |
| 133MHZ | 2 x | 66MHZ | 6x86L/MX-PR166 |
| 150MHZ |  | 75MHZ | 6x86L/MX-PR200 |
| 150MHZ | 2.5 x | 60MHZ | 6x86MX-PR166 |
| 166MHZ |  | 66MHZ | 6x86MX-PR200 |
| 188MHZ |  | 75MHZ | 6x86MX-PR233 |
| 210MHZ |  | 83MHZ | 6x86MX-PR266 |
| 250MHZ |  | 100MHZ | MII-366GP |
| 180MHZ | 3 x | 60MHZ |  |
| 200MHZ |  | 66MHZ | 6x86MX-PR233 |
| 225MHZ |  | 75MHZ | 6x86-PR266 |
| 250MHZ |  | 83MHZ | MII-333GP |
| 300MHZ |  | 100MHZ |  |
| 233MHZ | 3.5 x | 66MHZ | MII-300GP |
| 262MHZ |  | 75MHZ | MII-333GP |
| 290MHZ |  | 83MHZ |  |
| 333MHZ |  | 95MHZ |  |
| 350MHZ |  | 100MHZ |  |
| 240MHZ | 4 x | 60MHZ |  |
| 266MHZ |  | 66MHZ |  |
| 300MHZ |  | 75MHZ |  |
| 333MHZ |  | 83MHZ |  |
| 380MHZ |  | 95MHZ |  |
| 400MHZ |  | 100MHZ |  |
| 300MHZ | 4.5x | 66MHZ |  |
| 450MHZ |  | 100MHZ |  |
| 333MHZ | 5x | 66MHZ |  |
| 475MHZ |  | 95MHZ |  |
| 500MHZ |  | 100MHZ |  |
| 366MHZ | 5.5x | 66MHZ |  |
| 550MHZ |  | 100MHZ |  |
| 600MHZ | 6X | 100MHZ |  |

11

12

### JP20:Flash ROM Voltage Select

JP19:CPU Vcore voltage selection : For Pentium Processor

with MMX technology AMD K6 and cyrix 6x86L/6x86MX

JP19(Vcore voltage setting)

| Vcore voltage | 1-2 | 3-4 | 5-6 | 7-8 |
| --- | --- | --- | --- | --- |
| 2.0V | OFF | OFF | OFF | OFF |
| 2.1V | ON | OFF | OFF | OFF |
| 2.2V | OFF | ON | OFF | OFF |
| 2.3V | ON | ON | OFF | OFF |
| 2.4V | OFF | OFF | ON | OFF |
| 2.5V | ON | OFF | ON | OFF |
| 2.6V | OFF | ON | ON | OFF |
| 2.7V | ON | ON | ON | OFF |
| 2.8V | OFF | OFF | OFF | ON |
| 2.9V | ON | OFF | OFF | ON |
| 3.0V | OFF | ON | OFF | ON |
| 3.1V | ON | ON | OFF | ON |
| 3.2V | OFF | OFF | ON | ON |
| 3.3V | ON | OFF | ON | ON |
| 3.4V | OFF | ON | ON | ON |
| 3.5V | ON | ON | ON | ON |

| CPU Type | Vcore |
| --- | --- |
| Pentium(P54C),6x86,K5 | 3.52V |
| K6-233(or higher) | 3.2V |
| K6-166/200,6x86MX(M2) | 2.9V |
| MMX (P55C),6x86L | 2.8V |
| K6-2 450~500,K6-3 400~500 | 2.4V |
| K6-PR/3D,266/300~400 6X86MX(MZ) | 2.2V |

NOTE: 1. Refer to the table above to choose the correct voltage

For the CPU everytime that you install a CPU , and ,

make sure that your JP19 is matched with the CPU

voltage , otherwise will damage the CPU or make the

system unstable .

2. When the new CPU is announced and is not listed on

this manual , please refer to the above table , select the

correct voltage setting for it .

J1:ATX Style Power Connector

The ATX power supply a single 20-pin connector .

| Pin | Description | Pin | Description |
| --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 | 3.3V 3.3V Ground +5V Ground +5V Ground Power OK 5VSB +12V | 11 12 13 14 15 16 17 18 19 20 13 | 3.3V -12V Ground PS-ON Ground Ground Ground -5V +5V +5V |

Software Power-Off

Follow the steps below to use the “Software Power-Off Control”

function in Windows 95 with ATX power supply .

1.Click the START button on the Windows 95 task bar .

2.Select Shut Down The Computer to turn off the computer .

The message “It is now safe to turn off your computer .”will

not be shown when using this function .

J7(13,15)(PWR ON)-Power Button & Suspend Switch Connector

When the system is turned off , push the power button to turn the

system back on .

When the system is on , push the power button rapidly to switch the

system to the Suspend mode , and , by pushing and holding the

button for more than 4 seconds to turn the system completely off .

When the system is in the Suspend mode , push the power button

rapidly to turn the system on .

J2:AT Power – Power Supply Connector

This power supply connector has two sets of six-wire connectors .

Plug the dual connectors onto the board and make sure that the

black leads are in the center .

Note:   Before connecting the power supply , make sure it is not

connected to the power source .

14

J3:Keyboard Connector

A 5-pin female DIN keyboard connector is located at the upper

Right corner of the mainboard . Plug the keyboard jack directly

into this connector .

J4: PS/2 Mouse Connector

Attach mouse cable to this 5-pin connector .

J5:IR→Infrared Port Module Connector

The system board provides a 4-pin infrared connector-J5 as an optional module for wireless transmitting and receiving .

15

16

J6:USB Connector

Attach the USB cable to provide connection to USB devices .

J7-Pin Description

J7(2,4,6,8)(SPK)-Speaker Connector

| Pin | Description |
| --- | --- |
| 2 4 6 8 | +5V Ground NC Data out |

The speaker connector is a 4-pin connector for connecting the

system and the speaker .

J7(1,3,5,7,9) (Key-Lock)- Keylock & power LED Connector

| Pin | Description |
| --- | --- |
| 1 3 5 7 9 | LED Output Ground Ground Keylock Ground |

J7(12,14) (HD-LED)-HDD LED Connector

| Pin | Description |
| --- | --- |
| 14(+) 12(- ) | +5V Active Low |

Attach the cable of hard disk drive LED to this connector . The

LED lights when an HDD is active .

J7(18,20) (RST)-Reset Switch Connector

Attach the reset push button cable to this connector

| Setting | Description |
| --- | --- |
| Open Close | Normal Mode Reset System |

The system board has a 2-pin connector for rebooting your

Computer without having to tun off your power switch . This

Prolongs the life of the system,s power supply .

17

18

2-3  System Memory Configuration

This mainboard supports different type of settings for the system memory .

The following figures and provides all possible memory combinations .

This mainboard supports 2 Kinds of powerful and flexible SDRAM frequency

selections . These can be synchronous with CPU bus clock or fixed as 66MHZ.

By implementing the VCS (Virtual Clock Synchronization)technology , this

mainboard refers to the use of delay-lcok-loop(DLL) to enable synchronous

and pseudo-synchronous operation of the processor and DRAM , AGP and PCI

buses . The JP5,JP9 allows user to set the SDRAM Frequency

between 66/100MHZ .

SDRAM Clock select

| DRAM Speed | JP5 | JP9 | ←Fixed as 66MHZ ←SDRAM clock =CPU Bus clock |
| --- | --- | --- | --- |
| Run AGP Clock 66MHZ | 2-3 | 1-2 |  |
| Run CPU clock 66/75/83/100 | 1-2 | 2-3 |  |

Jumper,s position:

Notice:When you adjust 100MHZ . Please only use DIMM module

(Recommanded), Because the DIMM and SIMM both be installed

The SIMM timing is not the same maybe can,t match with DIMM.

JP5(2-3),JP9(1-2):

Pseudo-synchronous Status(Fixed as 66MHZ)

A more stable and compatible operation condition for non-100MHZ based

SDRAM when you are using 100MHZ based CPU . This setting is suitable

for those users who are like to remain the usage of current SDRAM module .

JP5(1-2),JP9(2-3),JP10(1-2),JP11(2-3):

Synchronous Status(SDRAM

Clock = CPU  Bus Clock Increasing the bus speeds from the traditional

66MHZ to 100MHZ  greatly improves system performance because the

speed at which data traveling between the CPU and memory is increased

by 50% . However , there is one thing  you should bear in mind .

Please make sure you are using 125MHZ(-8) based or above SDRAM

module.

2-4 ATX Power ON/OFF Control

This mainboard equips an ATX power connector which is a single 20-PIN input

device for an ATX power supply . An ATX power supply provides a build-in

remote Power ON/OFF function . To implement the function , a momentary

switch which is nomally open should be connected to the position J7 (PIN 13,

15) as the system ,s power ON/OFF button . Note that an AT power supply

does not offer this function .

Based on the ATX power connector , this mainboard has been designed to

support both ACPI and Soft-OFF functions . According to the definition

of ACPI , a Suspend mode will be enabled while you push the J7(system

power ON/OFF) button less than 4 seconds . Nevertheless , the system will

be turned off by pressing for more than 4 seconds . Regarding the

19

Soft-OFF(coming from this mainboard circuit controller) , it is another way

to turn off your system . Your system can be shut down automatically

by an operation system such as Windows 95 .

2-5 External Mode Ring-in Power ON Functions

On The basts of bounded functions in I/O Chipset , the serial ports are

able to Support the Extemal Modem Ring-in Power On function.

Once user connect the Extemal modem to COM1 or COM2, this

mainbard mainboard allows user to turn On their system through the

remote and host,s dial-up control.

Chapter 3 AWARD BIOS SETUP

Award,s ROM BIOS provides a built-in setup program which allows

user to modify the basic system configurations and hardware parameters .

The modified data will be stored in a battery-backed CMOS RAM so that

data will be retained even when the power is turned off .In general ,

the information saved in the CMOS RAM stays unchanged unless

there is a configuration change in the system , such as a hard drive

replacement or a new device installation .

If this does happen , you will need to reconfigure your configuration parameter .

To Enter Setup Program

Power on the computer and press <Del> Key immediately . This will bring

you into BIOS CMOS SETUP UTILITY .

20

Figure3-1 CMOS SETUP UTILITY

The menu displays all major selection items . Select the item you

need to reconfigure .The selection is made by moving cursor (press

any direction key ) to the item and press the ‘Enter’ key . An on-line

help message is displayed at the bottom of the screen as the cursor is

moving to various items which provides a better understand-ing of each

function . When a selection is made , the menu of selected item will appear

so the user can modify the associated configuration parameters .

3-1 STANARD COMS SETUP

Choose “STANDARD COMS SETUP” in the CMOS SETUP

UTILITY Menu(Fig3-1) The STANDARD CMOS SETUP Allows

user to configure system setting such as the Current date and time , type

of hard disk installed , floppy type , and display type . Memory

Size is auto-detected by the BIOS and displayed for your reference . When

21

a field is Highlighted(use direction keys to move cursor and <Enter> key

to select),the entries in theField will be changed by pressing <pgDn>

or<PgUp>keys or user can enter new data directly From the keyboard .

Figure3-2 STANDARD CMOS SETUP

NOTE:If the Primary Master/Slave and the Secondary Master/Slave are set as

“Auto”, the hard disk size and model will be auto-detected .

NOTE:The “Halt On:”field is to determine when to halt the system by the

BIOS if an error occurrs.

3-2 BIOS FESTURES SETUP

By selecting the “BIOS FEATURES SETUP” option in the CMOS

SETUP UTILITY Menu , user can change system related parameters

in the displayed in the displayed menu .This menu shows all of the

manufacturer,s default values of this mainboard . Again , user Can move

the cursor by pressing direction keys and <PgDn>or<PgUp>keys to modify

the Parameters . Pressing [F1] key to display help message of the selected item .

22

Figure3-3 BIOS FEATURES SETUP

Note: The Security Option contians “setup” and “system” .The “setup”

indicates that the password setting is for CMOS only while the “system “

indicates the pass- word setting is for both CMOS and system boot up .

- Virus Warning :This category flashes on the screen . During and after

the system boots up.

Any attempt to write to the boot sector or partition table of the hard disk drive

will halt the system and an error message will appear . You should then run

an anti-virus program to locate the virus program to locate the virus . Keep in

mind that this feature protects only the boot sector , not the entire hard

drive . Default value is Disabled .

Enabled : Activates automatically when the system boots up causing a warning

23

Message to appear when any attempt to access the boot sector or hard

disk partition table .

Disabled : No warning message to appear when any attempt to access the

boot sector or Hard disk partition table .

- CPU Internal Cache/External Cache : These two categories speed up

Memory access . However , it depends on CPU/chipset design .The default

value is Enabled . If your CPU is without Internal Cache then this item

“CPU Internal Cache” will not be shown .

Enabled : Enable cache .

Disabled : Disable cache .

- Quick Power On Self Test : This category speeds up Power On

Self Test(POST) after you power on the computer . If it is set to Enable ,

BIOS will shorten or skip some checking items during POST.

Enabled : Enable quick POST.

Disabled: Normal post.

- Boot Sequenec : This category determines which drive is searched first

for the O/S(Operating System) . The default value is A,C .

A,C : The system will search for floppy disk drive first then hard disk drive .

C,A : The system will search for hard disk drive first then floppy disk drive .

Swap Floppy Drive : This will swap your physical drive letters A & B if you

are using two floppy disks . The default value is Disabled .

Enabled : Floppy A & B will be swapped under the O/S .

Disabled : Floppy A & B will be not swapped .

- Boot Up Floppy Seek : During Power-On-Self-Test(POST),BIOS

will determine if the

24

Installed floppy drive is 40 or 80 tracks . Only 360K type is 40 tracks while

760K , 1.2M and 1.44M are all 80 tracks . The default value is Enabled .

Enabled : BIOS searches for floppy disk drive to determine if it is 40

or 80 tracks . Note that BIOS can not tell from 720K ,

1.2M or 1.44M drive type As they are all 80 tracks.

Disabled :BIOS will not search for the type of floppy disk drive by track

number .Note that there will not be any warning message if

the drive installed is 360K .

- Boot Up NumLock Status : The default value is On .

On :Keypad is number keys .

Off : Keypad is arrow keys .

- Boot UP System Speed : Select default system speed . The system

will run at the Selected speed after the

system boots .

High : Set the speed to high .

Low :Set the speed to low .

- Gate A20 Option : This refers to the way the system addresses

memory above 1MB (extended memory). The default value is Fast .

Normal : The A20 signal is controlled by keyboard controller or chipset

hardware.

Fast   : The a20 signal is controlled by Port 92 or chipset specific method .

- Typematic Rate Setting :This determines the typematicr rate .

Enabled :Enable typematic rate and typematic delay programming .

Disabled : Disable typematic rate and typematic delay programming .

The system  BIOS will use defualt value of 2 items

and the default is controlled by the Keyboard .

25

- Typematec Rate(Chars/Sec):

6 : 6 characters per second .        8: 8 characters per second.

10:10 characters per second .       12:12 characters per second.

15:15 characters per second .       20:20 characters per second.

24:24 characters per second.        30:30 characters per second.

- Typematic Delay(Msec) : This determines the time between the

first and second

Character displayed , when holding a key .

250 :250msec.

500:500 msec.

750:750 msec.

1000:1000 msec.

- Security Option :This category allows you to limit access to the system and

Setup, or just or Setup. The default value is Setup .

System : The system will not boot and the access to Setup will be denied if the

correct password is not entered at the prompt .

Setup :  The system will boot , but the access to Setup will be denide if the

correct password is not entered at the prompt .

- PCI/BGA Palette Snoop :This filed controls the ability of a primary

PCI VGA Conntroller to share a common palette (when a snoop write

cycles) with an ISA Video card . The default value is Disabled .

Enabled : If an ISA card connects to a PCI VGA card via the

VESA connector and the ISA card connects to VGA monitor and

uses the RAMDAC of PCI card , the PCI/VGA Palette Snoop is

enabled .

26

Disabled : Disable the VGA card Palette snoop function .

- Video BIOS Shadow :It determines whether video BIOS will be

copied to RAM . However , it is optional from chipset design . Video

Shadow will increase the video Speed.

Enabled :    Video shadow is enabled .

Disabled:    Video shadow is disabled .

- C8000 - CBFFF Shadow :

CC000 – CFFFF Shadow :

D0000 – D3FFF Shadow :

D4000 – D7FFF Shadow :

D8000 – DBFFF Shadow :

DC000 – DFFFF Shadow :

These categories determine whether optional ROM will be copied

to RAM by 16K byte or 32Kbyte per unit and the size depends

on the chipset .

Enabled : Optional shadow is enabled.

Disabled : Optional shadow is disabled .

3-3 CHIPSET FEATURES SETUP

Choose the “CHIPSET FEATURES SETUP” in the CMOS

SETUP UTILITY Menu to display the following menu .

Figure3-4 CHIPSET FEATURES SETUP

27

Note : When you insert slower memery modules in the system and

set a faster timing , maybe the system will hang up.

- DRAM Timing :The default value is 60ns.

60ns : 2 (faster) Burst Wait State , for 60~70ns Fast Page

Mode/EDO DRAM.

70ns: 3 (slower) Burst Wait Slate , for 70ns page Mode/EDO

DRAM.

- SDRAM Cycle length: The default value is 2.

2:2 HCLKS.

3:3 HCLKS.

- DRAM Read Pipeline :The default value is Enabled.

Disabled :Normal Setting.

Enabled :This field enableds the pipelining of DRAM read cycle.

- Cache RD+CPU Wt Pipeline: The default value is Enabled.

Disabled :Normal Setting.

Enabled :This field enableds the pipelining of Cache reads and

CPU writes cycle.

- Cache Timing :The default value is fastest.

Fast :Cache burst mode timing =3 1 1 1 2 1 1 1.

Fastest : Cache burst mode timing=3 1 1 1 1 1 1 1.

- Video BIOS Cacheable :The default value is Enabled.

Enabled :Enabled the Video BIOS Cacheable to speed up the

VGA Performance.

Disabled :Normal Setting.

28

- System BIOS Cacheable : The default value is Eisabled .

Enabled :Allow caching of the system BIOS ROM at

F0000h-FFFFFh.

Disabled :Normal Setting.

- Memory Hole at 15M-16M :The default value is Disabled.

Disabled :Normal Setting.

Enabled :This field enableds the main memory(15~16MB) remap to

ISA BUS.

- AGP Aperture Size :The amount of the system memory that the AGP

card Is allowed to share .The options available are4M,8M,16M, 32M,

64M, 128M,256M.The default value is 64M.

- Onchip USB:Enabled

USB Keyboard Support : Enabled

Enabled :Enabled USB Keyboard.

- Auto Detect DIMM/PCI CLK : use the default setting.

- System/CPU Warning Temp : The default value 60℃/140℉.

Disabled :Normal Setting

Enabled : The options available is 50℃/122℉~70℃/158℉.

- Current CPU Temp : This is the current temperature of the CPU.

3-4 POWER MANAGEMENT SETUP

Choose the “POWER MANAGEMENT SETUP” in the CMOS

SETUP UTILITY to display the following screen . This menu allows

the user to modify the power management parameters and IRQ signals .

In general , these parameters should not be changed unless it is

absolutely necessary.

Figure3-5 POWER MANAGEMENT SETUP

Again , users can move the cursor by pressing direction keys to the

field needed to be Modified and press <PgDn>or<PgUp>to alter

item selection. You can only changeThe content of  Doze Mode,

Standby Mode, and Suspend Mode when the PowerManagement is

set to ‘User Define’.

3-4-1 The Description of the Power Management

- Power Management mode selection:

Disabled :The system operates in NORMAL conditions

(Non-GREEN) And the Power Management

function is disabled.

Max.saving :This mode will maximize the power saving capability.

29

30

Min.saving :This mode will minimize the power saving capability.

User define :Allow user to define time-out parameters to control

power saving mode .Refer to item B shown below.

- Time-out parameters:

HDD Standby

HDD Standby timer can be set from 1 to 15 mimute(s).

System Doze

The “System Doze” mode timer starts to count when there is no

“PM events” occurred. The valid time-out setting is from 1 mimute

up to 1hour.

System Suspend

This function works only when the Pentium Procssor is installed .

The timer starts to count when “System Standby” mode timer is timed

out and no “PM Events” occurred. Valid range is from 1 minute

up to 1hour.

3-4-2 Description of the Green Functions

This mainboard supports HDD power Down, Doze and Suspend

power saving Functions . The detailed description of these functions

is provided in the next page.

- PM Controlby APM

If Advanced Power Management(APM) is installed on your

system , selecting Yes gives better power savings.

- Video Off Method

Determines the manner in which the monitor is blanked.

V/HSYNC+Blank:System turns off vertical and horizontal

31

synchronization ports and writes blanks to the video buffer.

DPMS Support:Select this option if your monitor supports

the Display Power Management Signaling (DPMS) standard of

the Video Electronics Standards Association(VESA).Use the softw

are supplied for your video subsystem to select video power

Management values.

Blank Screen:System only writes blanks to the video buffer.

- Video off Option

Determines whin to activate the video off feature for monitor

power management.

The settings are Video off after Suspend/standby/Doze/(N/A).

- Soft-off by PWRBTN

This field is for the soft-off function setting. When the board

utilizes an ATX Power supply , two types of settings are

offered: Delay 4 Sec. And instant-off. When the setting is

Delay 4 Sec, users can power off the system by pressing

POWER-ON button(J7) for 4 seconds. However, if users press

POWER-ON Button for less than 4 seconds, the system will

enter the Suspend Mode. When Tee setting is instant-Off, users

first press on POWER-ON button will power off the system, but

the second press will power on the system.

HDD Standby Mode

When system stops reading or wiriting HDD, the timer starts to count .

The system Will cut off the HDD power when timer runs out of time .

The system will not resume Operation until either a read from or a

32

write to HDD command is executed again.

Doze Mode

The system hardware will drop down CPU clock from nomal working

speed When Doze mode time-out occurs.

Suspend Mode

When the system suspend timer times out , the system will enter the

suspend mode And the chipset will stop CPU clock immediately .The

power consumption in Suspend Mode is lower than in standby mode.

The screen is also blanked out.

PMEvents:

AWARD BIOS defines 7 PM Events in the power management mode

(Doze & Suspend). The user can initialize any PM Events to be “Enable”

or “Disable”. When the system detects all of the enabled ebents do not

habe any activity, It will start the system Doze timer first if the

“Power Management” is not “Disabled”. Once the system Doze timer

is timed out , it will process doze power saving procedure by starting

the system suspend timer. When the suspend timer times out , all of the

CPU clock will stop by dropping system clock down tozero and remains

this way until any one of the “Enabled “event occrrs.

33

34

3-5 PNP / PCI CONFIGURATION

The PNP /PCI configuration program is for the user to modify the

PCI/ISA IRQ signals When various PCI/ISA cards are inserted in the

PCI or ISA slots.

WARNING :Any misplacing IRQ could cause system cant,t pick

out the rescouces.

Figure3-6PCICONFIGURATIONSETUP

Resource Controlled By:The default value is Manual.

Manual: The field defines that the PNP Card,s resource is controlled

by manual. You can setup whether IRQ-X or DMA-X is assigned to

PCI/ISA PNP Or legacy ISA Cards.

Auto: If your ISA card and PCI card are all PNP card. Set this field to

“Auto” The BIOS will assign the interrupt resource automatically.

- Reset Configuration Data:The default value is Disabled.

Disabled: Normal Setting

Enabled:If you plug some Legacuy cards in the system and record into

ESCD (Extended System Configuration Data). You can set this

field to be Enabled and to clear ESCD at one time, when

some Legacy cards are Removed.

- PCI IDE IRQ Map To:The default value is PCI/AUTO.

When you have true PCI card(s) plugged into the system , you

will not need to change any thing here in the SETUP

program. However, if you do not Know whether you are using

a true PCI card, please refer to your PCI Card user,s manual

for the details.

When you have a Legacy card (described in section 2-5) to plug

Into the system ,a proper setting is extremely important or it may

cause The system hung up. The diagram shown below tells you

how the Rotating Priority Mechanism is designed.

Figure3-7The Combination of PCIINT#lines

35

36

3-6 LOAD SETUP DEFAULTS

The “LOAD SETUP DEFAULTS” function loads the system default

data directly from ROM and initializes the associated hardware properly.

This function will be necessary only when the system CMOS data is corrupted.

Figure 3-8 LOAD SETUP DEFAULT

3-7 INTEGRATED PERIPHIERALS

Note: If you don,t use the Onboard IDE connector, but use On-card

(PCI or ISA card)IDE connector. You have to set Onboard Primary

PCI IDE, Disabled and Onboard Secondary PCI IDE: Disabled

From CHIPSETF EATURESS ETUP UTILIIY.

The Onboard PCI IDE cable should be equal to or less than

18 inches(45cm).IDE HDD Block Mode: The default value is Enabled.

Enabled: Enabled IDE HDD Block Mode. The HDD transfer rate is better

than Disable.

Disabled: Disable IDE HDD Block Mode.

- PCI Slot IDE 2nd Channel: The default value is Enabled.

Enabled :Enable secondary IDE port and BIOS will assign IRQ15 for this

port.

Disabled :Disable secondary IDE port and IRQ15 is available for other

device.

- Onboard Primary PCI IDE: The default value is Enabled.

Enabled :Enable Onboard 1 st channel IDE port.

Disabled: Disable Onboard 1 st channel IDE port .When use On-card

(PCI or ISA Card) IDE connector.

- Onboard Secondary PCI IDE: The default value is Enabled.

Enabled:Enable Onboard 2nd channel IDE port.

Disabled :Disable Onboard 2nd channel IDE port When use On-card

(PCI or ISA Card) IDE connector.

- IDE Primary Master PIO:The default value is Auto.

Auto    :BIOS will automatically detect the Onboard Primary Master

PCI IDE HDD Accessing mode.

Mode0~4:Manually set the IDE Accessing mode.

- IDE Primary Slave PIO:The default value is Auto.

Auto     :BIOS will automatically detect the Onboard Primary Slave PCI

37

IDE HDD Accessing mode.

Mode0~4 : Manually set the IDE Accessing mode.

- IDE Secondary Master PIO :The default value is Auto.

Auto   : BIOS will automatically detect the Onboard Secondary Master

PCI IDE HDD Accessing mode.

Mode0~4:Manually set the IDE Accessing mode.

- IDE Secondary Slave PIO :The default value is Auto.

Auto   : BIOS will automatically detect the Onboard Secondary Slave

PCI IDE HDD Accessing mode.

Mode0~4:Manually set the IDE Accessing mode.

- Onboard FDCController :The default value is Enabled.

Enabled : Enable the Onboard floppy drive interface controller.

Disabled :Disable the Onboard floppy drive interface controller,

When using On-card ISA FDC,s controller.

- Onboard UART1:This field allows the user to sellect the serial port.

The default Value is 3F8H/IRQ4.

COM1:Enable Onboard Serial port1 and address is 3F8H/IRQ4

COM2:Enable Onboard Serial port1 and address is 2F8H/IRQ3

COM3: Enable Onboard Serial port1 and address is 3F8H/IRQ4

COM4: Enable Onboard Serial port1 and address is 2F8H/IRQ3

Disabled:Disable Onboard Serial port 1.

- Onboard UART2:This field allows the user to sellect the serial port.

The default Value is 2F8H/IRQ3.

COM1:Enable Onboard Serial port2 and address is 3F8H/IRQ4

COM2:Enable Onboard Serial port2 and address is 2F8H/IRQ3

38

COM3: Enable Onboard Serial port2 and address is 3F8H/IRQ4

COM4: Enable Onboard Serial port2 and address is 2F8H/IRQ3

Disabled:Disable Onboard Serial port 2

- Onboard UART 2 Mode: The default value is standard .This field allows

the User toSelect the COM2 port that can support a serial Infrared Interface.

Standard:Support a Support a Serial Infrared Interface IrDA.

HPSIR:Support a HP Serial Infrared Interface format.

ASKIR:Support a Sharp Serial Infrared Interface format.

- Onboard Parallel port:This field allows the user to sellect the LP port.

The default Value is 378H/IRQ7.

378H     :Enable Onboard LPT port and address is 378H and IRQ7

278H     : Enable Onboard LPT port and address is 278H and IRQ5

3BCH    : Enable Onboard LPT port and address is 3BCH and IRQ7

Disabled  :Disable Onboard LPT port.

NOTE:Parallel Port address is 378H/3BCH that selects the rounting of IRQ7 for LPT1.

Parallel Port address is 278H that selects the rounting of IRQ5 for

LPT1.

- Parallel port Mode :This field allows the user to sellect the parallel port mode.

The default value is ECP+EPP.

Normal :Standard mode, IBM PC/AT Compatible bidirectional parallel port.

EPP :Enhanced Parallel Port mode.

ECP :Extended Capability Port mode.

EPP+ECP :ECP Mode&EPP Mode.

ECP Mode USE DMA:This field allows the user to sellect DMA1 or DMA3 for

39

The ECP mode .The default value is DMA3.

DMA1      :The filed selects the rounting of DMA1 for the ECP mode.

DMA3      :The filed selects the rounting or DMA3 for the ECP mode.

3-8 CHANGE SUPERVISOR or USERPASSWORD

To change the password, choose the “SUPERVISOR PASSWORD or

USER PASS- WORD” option from the CMOS SETUP UTILITY

menu and press[Enter].

NOTE:Either “Setup”or “System” must be selected in the “Security Option”

of the BIOS FEATURES SETUP menu(Refer to Figure3-3 for the details).

- If CMOS is corrupted or the option is not used, a default password stored int

he ROM Will be used. The screen will display the following message.

Enter Password:

Press the [Enter]key to continue after proper password is given.

- If CMOS is corrupted or the option was used earlier and the use wish to

change Default password, the SETUP UTILITY will display message and

ask for a Confirmation.

Confirm Password:

- After pressing the [Enter] key (ROM password if the option was not used )

or current Password (user-defined password), the user can change the

password and store new One in CMOS RAM. A maximum of 8characters

can be entered.

40

3-9 IDE HDD AUTO DETECTION

The “IDE HDD AUTO DETECTION” utility is a very useful tool especially

when you Do not know which kind of hard disk type you are using . You

can use this utility to detect  the correct disk type installed in the system

automatically . But now you can set HARD DISK TYPE to Auto

in the STANDARD CMOS SETUP. You do not need the”IDE HDD

AUTO DETECTION”utility.The BIOS will Auto-detect the hard disk

size and model on Display during POST.

Figure3-9 IDE HDD AUTODETECTION

NOTE:HDD Modes

The Award BIOS supports 3 HDD modes:NORMAL,LBA and LARGE

NORMAL mode.

Generic access mode that is neither the BIOS nor the IDE controller will

make transformations during accessing.

The maximum nunbers of cylinders,head&sectors for NORMAL mode are

1024,16 and 63.

no Cyclinder  (1024)

x  no.Head      ( 16  )

x  no.Sector     ( 63  )

x  no.per sector  (512  )

528 Megabytes

If an user sets the HDD to NORMAL mode. The maximum accessible HDD

Size Will be 528 Megabytes even though its physical size may be greater than

that!

LBA(Logical Block Addressing) mode:This is a new HDD accessing method

to Overcome the 528 Megabyte bottleneck.

The number of cylinders,heads and sectors shown in the setup may not be

the nunber Physically contained in the HDD.

During the HDD accessing ,the IDE controller will transform the logical address

Described by sector, head and cylinder into its own physical address inside

the HDD.

The maximun HDD size supported by LBA mode is 8.4 Gigabytes which is

Obtained by the following formula:

no Cyclinder  (1024)

x    no.Head      ( 255  )

x    no.Sector     ( 63  )

x    bytes per secttor  (512  )

8.4Gigabytes

41

42

LARGE mode:This is an extended HDD access mode supported by

Award Software. Some IDE HDDs contain more than 1024 cylinders without

LBA support( in some Cases,user does not want LBA).The Award BIOS

provides another alternative to Support these kinds of LARGE mode:

CYLS     HEADS   SECTOR   MODE

1120        16        59        NORMAL

560        32        59        LARGE

BIOS tricks DOS(or other OS) that the number of cylinders is less than 1024

by dividing it by 2.At the same time,the number of heads is multiplied

by 2.A reverse transformation process will be made inside INT12h in order

to access the right HDD address

Maximum HDD size:

no Cyclinder  (1024)

x    no.Head      ( 32  )

x    no.Sector     ( 63  )

x    bytes per secttor  (512  )

1 Gigabytes

Note:

To support LBA or LARGE mode of HDDs , there must be some

softwares involved. All Softwares are located in the Award HDD

Service Routine(1NT 13h). It may fail to accessA HDD with LBA

(LARGE)mode selected if you are running under on Operating System

Which replaces the whole 1NT13H.UNIX operating system do not support

either LBA Or LARGE and must utilize the Standard mode.UNIX can

support drives larger than 528MB.

43

3-10 HDD LOWLEVEL FORMAT

Interleave

Select the interleave number of the hard disk drive that you wish to

perform a low Level format on. You may select from 1to 8.

Check the  documentation that Came with drive for the correct

interleave number, or select 0 for automatic Detection.

Auto scan bad track This allows the utility to scan first then format by

each track .Start Press<Y>to start low level format

3-11 SAVE&EXIT SETUP

The “SAVE&EXIT SETUP” option will bring you back to boot up

procedure with all the changes you just recorded in the CMOS RAM.

3-12 EXIT WITHOUT SAVING

The “EXIT WITHOUT SAVING”option will bring you back to normal

boot up procedure without saving any data into CMOS RAM. All old data

in the CMOS will not be destroyed.

44