SECTION 1.

PRODUCT INFORMATION

Thanks for purchasing Magic 586 socket-7 mainboard.

This user’s manual contains all the information and features that show you how to use the Magic 586 mainboard. Please take a moment to familiarize yourself with the design and organization of this manual.

  - Manual Features

This manual is divided into the following four sections:

Section 1: Product Information

A brief overview of what comes in the mainboard package, the mainboard layout and the specification it appears.

Section 2: Hardware Installation

Tell you the usage of the mainboard jumpers and the connectors.

Section 3: CMOS Setup Utility

A summary of the mainboard CMOS (BIOS) Setting.

Section 4: BIOS/Software Utility

Introduction of some useful mainboard BIOS/Software utility.

1-2    Package Check List

This mainboard package contains the following items. Please inspect the package contents and confirm that everything is there. If anything is missing or damaged, call your vendor for instructions before operating.

The package includes:

- One Magic 586 Mainboard

- One Floppy Interface Cable

- One IDE Interface Cable

- Audio Cable

- One Motherboard Resource CD

- One User’s Manual

  - Mainboard Specifications

| ##### Form Factor | - Baby AT form factor , 22 cm x 22.5 cm |
| --- | --- |
| CPU | - Supports Pentium / Socket-7, external bus speed up to 100MHZ |
| System Memory | - DIMM 168-pin x 3 , supports SDRAM up to 768MB - Supports 64M-bit DRAM / SDRAM technology - 512K/1MB L2 cache |
| Chipset | - VIA MVP3 VT82C598MVP + MVP4 VT82C686A chipset for socket-7 CPU |
| Expansion Slots | - 1 x AGP bus / 2 x ISA bus / 3 x PCI bus with bus master mode - 1 x AMR connector for AC97 modem card |
| Serial Port | - Two serial ports UART 16C550 compatible - Supports HPSIR or ASKIR infrared interface |
| Parallel Port | - One parallel port supports : SPP (standard parallel port), EPP(enhanced parallel port) and ECP(extended capabilities port) |
| Floppy Interface | - Supports drives inches/format as: - 3.5 inches—720KB/1.44MB/2.88MB - 5.25 inches—360KB/1.2MB |
| IDE Interface | - Dual PCI IDE interface support up to 4 x IDE HDD or CDROM - Supports PIO mode 4 , DMA mode 2 & Ultra DMA 33 |
| USB Interface (optional) | - Two USB ports supported - USB legacy keyboard function supported |
| PS/2 Mouse | - PS/2 mouse supported by connector onboard |
| Keyboard | - AT compatible keyboard - MINI-DIN PS/2 keyboard connector (optional) |
| RTC and Battery | - RTC build in chipset - Lithium (CR-2032) battery |
| Wake Up Function | - Modem ring wake up / LAN wake up / RTC alarm wake up |
| Synchronous Switching Regulator | - High efficient synchronous switching regulator for CPU core voltage - Supports over-voltage / over-current protection function |
| Power connector | - Supports AT(12-pin) power connector - Supports ATX(20-pin) power connector |
| BIOS | - Award BIOS - Year 2000 Compliance - PCI 2.1/ PNP BIOS v1.0a / APM v1.2 / DMI v2.0 compliance - Supports ACPI (Advanced Configuration and Power Interface) and OS Directed Power Management - Anti-Virus protection supported - Floppy drive swapping function supported - Supports SOFT power |
| Sound | - AC97 audio & game / MIDI ports |
| LED Indicator Connectors | - System power LED connector - HDD activity LED connector - System Suspend LED connector |

1-4 Mainboard Layout

KB

GAME

PORT

PS2 MS

AT

Power

COM1

CD

USB

Printer

ATX Power

AGP FAN

FDD

ARM

COM2

I

S

A

1

I

S

A

2

P

C

I

1

P

C

I

2

P

C

I

3

A

G

P

B I OS

IR

VIA

598MVP

D

I

M

M1

D

I

M

M2

D

I

M

M3

VIA

686A

JP2

JP10 ~ JP13

JP3 ~ JP9

CPU FAN

JP14

PANEL

Wake on LAN

Socket 7

CPU FAN

IDE1

JP1

IDE2

Jumpers

| - JP1 | L2 Cache Burst Type |
| --- | --- |
| - JP2 - JP3, JP4 - JP5, JP6, JP7, JP8, JP9 - JP10, JP11, JP12, JP13 - JP14 | CPU Clock Ratio SDRAM Clock Select CPU Bus Clock CPU Vcore Voltage Clear CMOS |

Expansion Sockets

| - Socket 7 - DIMM 1 - DIMM 2 - DIMM 3 | CPU Socket Support 168-pin DIMM Memory Support 168-pin DIMM Memory Support 168-pin DIMM Memory |
| --- | --- |

Expansion Slots

| - ISA Slot 1 & Slot 2 | 16-bit ISA Bus Expansion Slot |
| --- | --- |
| - PCI Slot 1 to Slot 3 - ARM | 32-bit PCI Bus Expansion Slot ARM Expansion Slot for AC97 Devices |

Connectors

| - AT KB | AT Keyboard Connector |
| --- | --- |
| - PS/2 Mouse | PS/2 Mouse Connector |
| - COM1/COM2 | Serial Port 1 / Serial Port 2 |
| - PRINTER | Printer (Parallel) Port Connector |
| - AT POWER - ATX POWER | AT Power Connector ATX Power Connector |
| - AGPFAN - CPUFAN - CHAFAN | AGP Fan Connector CPU Fan Connector Chassis Fan Connector |
| - Floppy | Floppy Drive Connector |
| - Primary IDE | Primary IDE Connector |
| - Secondary IDE | Secondary IDE Connector |
| - GAME PORT - CD | Game/Audio Connector Audio CD-IN Connector |
| - IR - USB - Wake on LAN - Panel: | Infrared Port Connector Universal Serial Bus Port 1 and Port 2 LAN Wake Up Connector |
| - PWR LED | Baby-AT Power LED Connector |
| - KBLCK | Keyboard Lock Switch Connector |
| - SLP | Suspend Switch Connector |
| - SPEAKER | Chassis Speaker Connector |
| - HDD LED | HDD LED Connector |
| - RESET - GRN LED | Reset Switch Connector Green LED Connector |