SECTION 2.

HARDWARE INSTALLATION

This section gives you a step-by-step procedure on how to install your system.  Follow each section accordingly.

# 2-1	Jumper Settings

Please refer the following figures for the locations of the jumpers on the  mainboard.

CMOS Clear Setting

##### JP14          CMOS

Clear CMOS

Default

(Normal)

To clear CMOS, please follow the steps below:

1. Power off  the system and unplug the chassis AC power cord.

2. Short JP14 at pin 2-3 for few seconds.

3. Set JP14 back to its Normal position at pin 1-2.

4. Plug the AC power cord to the chassis.

5. Power on the system and load the BIOS setup default.

2-1.2   CPU Type Setting

This mainboard supports socket-7 Pentium series CPU. Install your CPU type with the following jumper settings.

- CPU Voltage:

JP10, JP11, JP12, JP13 are used to select the CPU Voltage.

1

2

3

(Default)

JP10

JP11

JP12

JP13

| CPU Voltage | JP10 | JP11 | JP12 | JP13 | CPU Type |
| --- | --- | --- | --- | --- | --- |
| 3.5V | 2-3 | 1-2 | 2-3 | 2-3 | AMD-K5 |
| 3.4V | 1-2 | 1-2 | 2-3 | 2-3 | INTEL P54C |
| 3.3V | 2-3 | 2-3 | 2-3 | 2-3 | IDT C6 |
| 3.2V | 1-2 | 2-3 | 2-3 | 2-3 | AMD-K6 233 |
| 3.1V | 2-3 | 1-2 | 1-2 | 2-3 |  |
| 3.0V | 1-2 | 1-2 | 1-2 | 2-3 |  |
| 2.9V | 2-3 | 2-3 | 1-2 | 2-3 | AMD K6-166/200 / Cyrix M2 |
| 2.8V | 1-2 | 2-3 | 1-2 | 2-3 | Intel P55C / Cyrix 6x86L |
| 2.7V | 2-3 | 1-2 | 2-3 | 1-2 |  |
| 2.6V | 1-2 | 1-2 | 2-3 | 1-2 |  |
| 2.5V | 2-3 | 2-3 | 2-3 | 1-2 |  |
| 2.4V | 1-2 | 2-3 | 2-3 | 1-2 | AMD-K6-III |
| 2.3V | 2-3 | 1-2 | 1-2 | 1-2 |  |
| 2.2V | 1-2 | 1-2 | 1-2 | 1-2 | AMD-K6-II |
| 2.1V | 2-3 | 2-3 | 1-2 | 1-2 |  |
| 2.0V | 1-2 | 2-3 | 1-2 | 1-2 |  |

- CPU Clock Ratio:

JP2 is used to select the CPU clock ratio.

###### 2    1

JP2

###### 6    5

(Default)

| Ratio | JP2 JP2 JP3 |
| --- | --- |
| 1.5x / 3.5x | ALL OPEN 2-3 1-2 |
| 2.0x | 5-6 |
| 2.5x / 1.75x | 3-4, 5-6 |
| 3.0x | 3-4 |
| 4.0x | 1-2, 5-6 |
| 4.5x | 1-2, 3-4, 5-6 |
| 5.0x | 1-2, 3-4 1-2 2-3 |
| 5.5x | 1-2 2-3 2-3 |

- CPU Clock Frequency:

CPU Clock Frequency:

JP5, JP6, JP7, JP8, JP9 are used to select the CPU clock frequency.

1

2

3

JP5

JP6

JP7

JP8

JP9

(Default)

| CPU Clock Frequency | PCI Clock Frequency | AGP Clock Frequency | JP5 | JP6 | JP7 | JP8 | JP9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100.2MHz | 33.30MHz | 66.6MHz | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 |
| 95.25MHz | 31.75MHz | 63.5MHz | 2-3 | 1-2 | 2-3 | 2-3 | 2-3 |
| 83.3MHz | 33.30MHz | 66.6MHz | 2-3 | 2-3 | 1-2 | 2-3 | 1-2 |
| 133.3MHz | 44.30MHz | 88.7MHz | 2-3 | 1-2 | 1-2 | 2-3 | 2-3 |
| 75MHz | 37.50MHz | 75.0MHz | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 |
| 124MHz | 41.30MHz | 82.7MHz | 2-3 | 1-2 | 2-3 | 1-2 | 2-3 |
| 66.8MHz | 33.40MHz | 66.8MHz | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |
| 112MHz | 37.30MHz | 74.7MHz | 2-3 | 1-2 | 1-2 | 1-2 | 2-3 |

- SDRAM Clock Select:

JP3, JP4 are used to select the SDRAM clock.

1

2

3

JP3

JP4

(Default)

| JP3 | JP4 | SDRAM Clock Select |
| --- | --- | --- |
| 1-2 | 1-2 | DRAM Run CPU Clock |
| 2-3 | 2-3 | DRAM Run AGP Clock |

- Burst Type:

JP1 is used to select the CPU Burst Type.

JP1

3   2   1

(Default)

| JP1 | Burst Type |
| --- | --- |
| 1-2 | Interleave Burst |
| 2-3 | Linear Burst |

Below is a reference of different CPU currently supported by this mainboard.

| System Frequency (MHz) | CPU Clock Ratio | CPU Clock Frequency |
| --- | --- | --- |
| 100 (Intel P54C-100; AMD K5-100&133) | 1.5x | 66MHz |
| 116 (AMD K5-166) | 1.75x | 66MHz |
| 133 (Intel P54C-133;Cyrix P166+) | 2x | 66MHz |
| 150 (Cyrix P200&M2-200;C6-150) | 2x | 66MHz |
| 166 (Intel P54C&55C-166;Cyrix M2-200; AMD K6-166) | 2.5x | 66MHz |
| 200 (Intel P54C&55C-200;AMD K6-200;Cyrix M2-233; IDT C6-200) | 3x | 66MHz |
| 225 (IDT C6-225;Cyrix M2-300) | 3x | 75MHz |
| 233 (Intel P55C-233; AMD K6-233;Cyrix M2-300) | 3.5x | 66MHz |
| 263 (Cyrix M2-333) | 3.5x | 75MHz |
| 266 (AMD K6-266;Cyrix M2-333) | 4x | 66MHz |
| 300 (AMD K6-II 300) | 4x | 75MHz |
| 333 | 5x | 66MHz |
| 375 (AMD K6-II 350) | 5x | 75MHz |
| 400 (AMD K6-III 400) | 4x | 100MHz |

# 2-2   Connectors

# 2-2.1   Panel Connector

The Panel Connector is a 22-pin onboard connector. Connect the Power LED, Keyboard Lock Switch, Suspend Switch, Speaker, Green LED, HDD LED, and Reset Switch to the corresponding pins as shown in the below figure.

| - PWR LED | Power LED Connector (3 pins) |
| --- | --- |
| - KBLCK | Keyboard Lock Switch Connector (2 pins) |
| - SLP | Suspend Switch Connector (2 pins) |
| - SPEAKER - GRN LED | Chassis Speaker Connector (4 pins) Green Status LED Connector (3 pins) |
| - HDD LED | HDD LED Connector (4 pins) |
| - RESET | Reset Switch Connector (2 pins) |

### 2-2.2   AT Power Connector

Connect the 12-pin AT power supply cable to this power connector. Make sure the right plug-in direction and the power supply is off before connecting or disconnecting the power cable.

AT Power

### 2-2.3   ATX Power Connector

Connect the 20-pin ATX power supply cable to this power connector. Make sure the right plug-in direction and the power supply is off before connecting or disconnecting the power cable.

ATX Power

### 2-2.4   Fan Connectors

Connect the CPU, Chassis & AGP Fan cables to the 3-pin fan connectors shown below. The fan connectors are marked as CPUFA1, CHAFA1 & AGPFA1 on the mainboard.

FAN CONNECTOR

##### GND

+12V

SENSE

##### AGPFA1

CHAFA1

CPUFA1

### 2-2.5   PS/2 Mouse Connector

Connect the PS/2 mouse to the onboard 6-pin Mini-Din connector marked as PS2 MS.

3  1  2  4

###### PS2 MS

5          6

| Pin 1 2 3 4 5 6 | Description MS DATA NC GND +5V MS CLK NC |
| --- | --- |

### 2-2.6   Keyboard Connector

Connect the AT keyboard to the onboard keyboard connector marked as     AT KB.

###### AT KB

### 2-2.7   Serial Device(COM1/COM2) Connectors

Connect your serial device(s) to the onboard 9-pin serial connectors marked as COM1 and COM2.

###### COM2

1

9

2

10

1

9

2

10

###### COM1

### 2-2.8   Printer Connctor

Connect your local printer to the onboard 25-pin printer connector marked as  LPT.

1

13

14

26

###### LPT

### 2-2.9   Floppy Drive Connector

Connect the floppy drive cable to the onboard 34-pin floppy drive connector marked as FDD.

FDD

### 2-2.10   IDE Hard Disk and CD-ROM Connector

Connect your IDE devices to the onboard 40-pin IDE connectors marked as IDE1 and IDE2.

###### IDE2

IDE1

Primary IDE Cable

IDE1

###### Slave

###### Master

###### IDE2

###### Secondary IDE Cable

###### Slave

###### Master

It is suggested that you connect the IDE devices to your IDE cables as the figure shown above. Each IDE channel, either Primary or Secondary, supports two IDE devices which must be set differently to master mode and slave mode.

(Refer to your hard disk and CD-ROM user’s manual for detailed settings of IDE master and slave mode.)

### 2-2.11   IrDA Connector

Connect your IR device to the onboard IrDA connector marked as IR.

12345

IR

Pin         Description

- +5V

- NC

- IRRX

- GND

- IRTX

2-2.12   USB Device Connector

Connect your USB device(s) to the onboard USB connector marked as USB.

###### USB

| Pin 1 3 5 7 9 | Description V0 D0- D0+ GND GND | Pin 2 4 6 8 10 | Description V1 D1- D1+ GND GND |
| --- | --- | --- | --- |

### 2-2.13   Wake on LAN Connector

This mainboard supports wake up on LAN function. To use this function, you need a Wake on LAN supported network card and software.

1    2    3

Pin         Description

- +5V Standby

- GND

- Signal

Wake on LAN

# 2-3   System Memory Installation

There are 3 pcs 168-pin DIMM (Dual Inline Memory Module) sockets on the mainboard which support SDRAM and EDO DRAM memory.

There are 3 168-pin DIMM sockets (DIMM1, DIMM2 & DIMM3) that allow you to install system memory maximum up to 768MB SDRAM.

    - Type

This mainboard supports SDRAM DIMM and EDO DIMM.

    - Speed

For SDRAM, the memory speed normally marked as: -15, -12, -10, -8, -7.

The meaning is,

-15 = 15ns, and the maximum clock is 66MHz

-12 = 12ns, and the maximum clock is 83MHz

-10 = 10ns, and the maximum clock is 100MHz

-8 = 8ns,   and the maximum clock is 125MHz

-7 = 7ns,   and the maximum clock is 142MHz

For EDO, the access time can be 50ns, 60ns & 70ns.

    - Buffered and Non-buffered

Only the non-buffered DIMM can be used in this mainboard. The difference between buffered and non-buffered DIMM can be identified by the notch position shown below.

    - 2-clock and 4-clock signal

Both 2-clock and 4-clock SDRAM DIMM supported by this mainboard.

    - Parity and Non-parity

This mainboard supports 64 bit Non-parity and 72 bit Parity DIMM modules.

    - Memory Auto detection by BIOS

This mainboard BIOS can automatically detect the DIMM memory size and type, so you do not need to adjust any hardware or software settings.

    - Suggested Memory combination

This mainboard supports the following SDRAM / EDO combination.

| DIMM Location | DIMM Size |
| --- | --- |
| DIMM1 | 8, 16, 32, 64, 128, 256MB |
| DIMM 2 | 8, 16, 32, 64, 128, 256MB |
| DIMM 3 | 8, 16, 32, 64, 128, 256MB |

Total Memory Size = DIMM1 + DIMM2 + DIMM3

# 2-4    Game/Audio Connector

Connect the audio cable to the onboard Game/Audio connector marked as GAME PORT.  The onboard CD-IN connector marked as CD is for CD-ROM audio.

CD

GAME PORT