SECTION 4.

SOFTWARE UTILITY

  - BIOS Flash/Upgrade Procedures

To upgrade your mainboard BIOS, please follow the steps below:

1.  Get the newest version BIOS to fit your mainboard model from your                  mainboard distributor.

! Boot up your system without loading config.sys and autoexec.bat !

2.  For Win95/98 system, press F8 before Win95/98 boot-up, and select                "Safe mode command prompt only".

For Non-Win95/98 system, boot-up the system into DOS prompt with                a bootable floppy disk.

3.  Run A:\> or C:\>bxa120.exe

(Assuming model is BXA, execution file is bxa120.exe, and your floppy              drive is drive A and hard disk drive is drive C)

4.  After loading the new BIOS code, the utility will prompt you to save                   original BIOS code into your HDD or floppy. Please press "Y" to store               it as "BIOS.OLD".

5.  After the old BIOS has been successfully saved, press "Y" to upgrade              BIOS.

!!! DO NOT TURN OFF THE POWER DURING THE FLASHING                            PROCESS.!!!

6.  After the flashing process, reboot the system by turn off the power.

7.  Press "DEL" key to enter BIOS setup during POST. Reload the "BIOS              SETUP DEFAULT" and reconfigure the other items as your previous                setting.

8.  Then save and exit.

4-2    VIA Chipset Driver Utility

This mainboard has five sets of VIA chipset drivers as below. All drivers can be found under the directory \IDE\VIA in the bundled CD title. Please follow the installation instruction in the bundled CD to install all these necessary drivers.

Drivers Overview:

- VIA Bus Master IDE Driver

- Microsoft Windows 95/Windows 98

- Microsoft Windows NT

- VIA AGP Driver

- Microsoft Windows 95/Windows 98

- VIA IRQ Routing Driver

- Microsoft Windows 95/Windows 98

For all VIA chipsets using VT82C586A or VT82C586B south-bridge chipsets.

- VIA USB Filter Driver

This driver adds support for more USB devices to the VT82C586B and VT82C596

- PCI Bridge Patch

This patch allows Win95/98 to recognize the VIA chipset in the system device manager.

4-3    VIA Audio Driver Utility

This mainboard has three sets of audio drivers for different operation system. All drivers can be found under the directory \AUDIO\VIA in the bundle CD title.

Drivers Overview:

- Microsoft Windows DOS

- Microsoft Windows 95

- Microsoft Windows 98