MICRONICS COMPUTERS, INC.

COMPATIBILITY TEST CENTER

BIOS TRACKING REPORT

| BIOS Release Date: | January 9, 1998 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BIOS Version: | 04 |  |  |  |  |
| BIOS File Name: | TWX-V04.BIN | BIOS Checksum: | 012CE018 | File Date: | 12/26/97 |
|  |  | File Size: | 131,072 | File Time: | 9:22AM |
| Flash Utility Version | AWDFLASH.EXE | Version: | v.5.36 | File Date: | 11/25/97 |
|  |  | File Size: | 16,564 | File Time: | 7:31 PM |
| System Board | TWISTER ATX | FAB | 1B | Revision |  |
| System Board P/N | 08-00309 |  |  |  |  |
| Chipset Version | Intel 430TX, | Intel PIIX 4, | SMC FDC37C665IR |  |  |
| Chipset Version P/N |  |  |  |  |  |
| Sign-On Message | Twister-04 |  |  |  |  |

| Limitation | 1) Do not support Samsung 128MB SDRAM w/SPD, Fujitsu 32 MB SDRAM w/SPD, Hyundai 32 MB SDRAM w/SPD. |
| --- | --- |
|  | 2) Support NEC 64 MB SDRAM w/SPD, Toshiba 32, 16 MB SDRAM w/SPD and 64, 128 MB EDO DRAM, Hyundai 16, 32 MB SDRAM w/SPD, Micron 16MB SDRAM w/SPD, Toshiba 16MB SDRAM w/o SPD. |

| BIOS Feature | 1) Add support to more memory type. |
| --- | --- |
|  | 2) Fix compatibility issue with AHA2940 SCSI controller and SRAM memory. |