Special Note: 
The BIOS and DEVICE DRIVERS contained here are exclusively for Micro-Star products only. Micro-Star assumes no responsibility for any damages resulting from improper use or lack of technical expertise.

WARNING!!!!!
DON'T FLASH FROM A FLOPPY DISK!!!!
DON'T FLASH WHEN YOUR SYSTEM IS RUNNING FINE!!!!
DON'T FLASH WHEN YOUR SYSTEM ISN'T RUNNING STABLE!!!!
DON'T FLASH IF YOU DON'T KNOW WHAT YOU ARE DOING!!!!

Award BIOS Flash Procedure

(Special Thanks to Forum Moderator Bas for his valuable contribution)
1. Check your BIOS for BIOS FLASH DISABLED and ENABLE it, some boards have this function.

For Example:

K7N420 Pro (MS-6373) – BIOS setup/Integrated Peripherals/BIOS Protect/Disabled

2. Know Your model number

a. Open your case and look for the MS-XXXX number on the motherboard between the PCI slots, also check the version number that is written there as well. (some motherboards are named like K7T Turbo, make sure you have the right version number)

b. Boot up your system & take a look at the top left screen during POST, you should see something like “W6373MS v2.1 122101” This means the board is MS-6373 with BIOS version 2.1

3. Download the BIOS that matches your motherboard AND version number (some motherboards are named like K7T Turbo, make sure you have the right version number). BIOS can be found according to the CPU type of your board like if your CPU is Intel P4 478 type then look here

http://www.msi.com.tw/support/bios/socket_478.php

if you cannot find the BIOS that matches your board, try here http://www.msi.com.tw/support/bios/more.php

4. Extract the BIOS-archive that you have downloaded to c:\test

5. Boot your system from a Win98 or WinME boot-floppy.

How to make boot floppy in case your don’t have it ready:

For Win9X, You can type [C:\ format a:/s] from the DOS prompt.
For WinMe, You can make a boot floppy from control panel--> add/remove program-->make boot floppy. Remove autoexec.bat & config.sys file if there's any.
For Win2000 & WinXP, there's no way to make boot floppy, so you have to either use Win9X or WinME boot floppy.

7. When you get the A:\ prompt, type the following sequence:
C: <enter>
cd\test <enter>
C:\test> awdfl783m BIOS file

(to save or not to save old BIOS is the user’s decision)

8. Reboot your system when prompted

P.S. If your system doesn't boot after you flashed, please refer to the BIOS recovery method as described http://www.msi.com.tw/support/bios/boot.htm

# AMI BIOS Flash Procedure

(Special Thanks to Forum Moderator Bas for his valuable contribution)
1. Check your BIOS for BIOS FLASH DISABLED and ENABLE it, some boards have this function.

For Example:

K7T266 Pro (MS-6380) –BIOS setup/chipset setup/BIOS protection/disabled

2. Know Your model number

a. Open your case and look for the MS-XXXX number on the motherboard between the PCI slots, also check the version number that is written there as well. (some motherboards are named like K7T Turbo, make sure you have the right version number)

b. Boot up your system & take a look at the top left screen during POST, you should see something like “A6380MS v1.7 122101” This means the board is MS-6380 with BIOS version 1.7

3. Download the BIOS that matches your motherboard AND version number (some motherboards are named like K7T Turbo, make sure you have the right version number). BIOS can be found according to the CPU type of your board like if your CPU is Intel P4 478 type then look here

http://www.msi.com.tw/support/bios/socket_478.php

if you cannot find the BIOS that matches your board, try here http://www.msi.com.tw/support/bios/more.php

4. Extract the BIOS-archive that you have downloaded to c:\test

5. Boot your system from a Win98 or WinME boot-floppy.

How to make boot floppy in case your don’t have it ready:

For Win9X, You can type [C:\ format a:/s] from the DOS prompt.
For WinMe, You can make a boot floppy from control panel--> add/remove program-->make boot floppy. Remove autoexec.bat & config.sys file if there's any.
For Win2000 & WinXP, there's no way to make boot floppy, so you have to either use Win9X or WinME boot floppy.

7. When you get the A:\ prompt, type the following sequence:
C: <enter>
cd\test <enter>
C:\test> amifl827 BIOS file

(to save or not to save old BIOS is the user’s decision)

8. Reboot your system when prompted

P.S. If your system doesn't boot after you flashed, please refer to the BIOS recovery method as described http://www.msi.com.tw/support/bios/boot.htm

Flashing the BIOS for Users using NTFS file system (Thanks to Forum Moderator Assaf for his valuable contribution)

1.Extract the bios to a FLOPPY (not the startup floppy).

2.While booting the win98/ME floppy you will see a RAMDRIVE.  After boot has

finished it will be accessible as a regular HDD.

At the screen after the floppy has booted there will be a line "The

Diagnostic tools were successfully loaded to drive..."

This drive is the RAMDRIVE and can be treated as any other drive.

3.Replace the floppy and copy the bios file and flash utility to the

RAMDRIVE and then follow the regular flash procedure like from an HDD, only

from the RAMDRIVE.

*To save the bios the user must specify the full path a:\filename or it will

be saved to the RAMDRIVE which will be erased.