Version 1.01

Manual Rev: 1.0

Release Date: October 1997

FCC-B Radio Frequency Interference Statement

This equipment has been tested and found to comply with the limits for a class B digital device, pursuant to part 15 of the FCC rules. These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. Operation of this equipment in a residential area is likely to cause harmful interference, in which case the user will be required to correct the interference at his own expense.

Notice 1

The changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Notice 2

Shielded interface cables and A.C. power cord, if any, must be used in order to comply with the emission limits.

VOIR LA NOTICE D'INSTALLATION AVANT DE RACCORDER AU RESEAU.

Edition

October 1997

Copyright Notice

The material in this document is the intellectual property of MICRO-STAR INTERNATIONAL.  We take every care in the preparation of this document, but no guarantee is given as to the correctness of its contents.  Our products are under continual improvement and we reserve the right to make changes without notice.

Trademarks

All trademarks used in this manual are the properties of their respective owners.

Intel, Pentium, and MMX are registered trademarks of Intel Corporation.

PS/2, OS/2 and AT are registered trademarks of IBM Corporation.

Windows 95 and Windows NT are registered trademarks of Microsoft.

Netware is a registered trademark of Novell.

Award is a registered trademark of Award Software Inc.

AMD is a registered trademark of AMD.

Cyrix is a registered trademark of Cyrix.

Table of Contents

| Chapter 1: Introduction | 1-1 |
| --- | --- |
| Mainboard Features --------------------------------------------------- | 1-2 |
| Kffd Parts of the Mainboard------------------------------------------------ | 1-5 |
| Mainboard Layout----------------------------------------------------- | 1-7 |
| Chapter 2: Hardware Installation | 2-1 |
| Central Processing Unit: CPU--------------------------------------- | 2-1 |
| CPU Installation Procedures------------------------------------ | 2-1 |
| CPU Core Speed Derivation Procedures---------------------- | 2-2 |
| CPU Voltage ------------------------------------------------------ | 2-3 |
| CPU Speed Setting: SW1 --------------------------------------- | 2-4 |
| CPU Fan Power Connectors: JFAN1/JFAN2----------------- | 2-8 |
| Flash ROM Programming Voltage: JP2---------------------------- | 2-9 |
| Flash ROM Boot Block Programming: JP4------------------------ | 2-10 |
| External Battery Connector: JBAT--------------------------------- | 2-11 |
| Power Saving LED Connector: JGL-------------------------------- | 2-12 |
| Creative® EMU8000 Wave Table Connector: J11--------------- | 2-13 |
| Memory Installation--------------------------------------------------- | 2-14 |
| Memory Bank Configuration----------------------------------- | 2-14 |
| Memory Installation Procedures-------------------------------- | 2-15 |
| Memory Population Rules--------------------------------------- | 2-16 |
| Case Connector : JFP------------------------------------------------- | 2-17 |
| Floppy Disk Connector: FDC---------------------------------------- | 2-19 |
| Hard Disk Connectors: IDE1 & IDE2------------------------------ | 2-20 |
| ATX 20-pin Power Connector: JWR12----------------------------- | 2-21 |
| Power On/Off Switch: JRMS1/JRMS2----------------------------- | 2-22 |
| IrDA Infrared Module Connector: JIR1---------------------------- | 2-23 |
| CD Line-in: J15/J16/J17--------------------------------------------- | 2-24 |
| Serial Port Connectors: COM A & COM B----------------------- | 2-25 |
| Parallel Port Connector: LPT---------------------------------------- | 2-26 |
| PS/2® Keyboard Connector: PSKBC------------------------------- | 2-27 |
| PS/2® Mouse Connector: PSMSC----------------------------------- | 2-27 |
| USB Connectors: USB----------------------------------------------- | 2-27 |
| Joystick/Midi Connectors--------------------------------------------- | 2-28 |
| Audio Port Connectors------------------------------------------------ | 2-28 |
| Chapter 3 AWARD® BIOS SETUP | 3-1 |
| Entering Setup --------------------------------------------------------- | 3-2 |
| Getting Help------------------------------------------------------------ | 3-2 |
| The Main Menu ------------------------------------------------------- | 3-3 |
| Standard CMOS Setup------------------------------------------------ | 3-5 |
| BIOS Features Setup -------------------------------------------------- | 3-8 |
| Chipset Features Setup ----------------------------------------------- | 3-13 |
| Power Management Setup ------------------------------------------- | 3-17 |
| PNP/PCI Configuration Setup --------------------------------------- | 3-23 |
| Load BIOS/ Setup Defaults ------------------------------------------ | 3-26 |
| Special Features Setup------------------------------------------------- | 3-27 |
| Integrated Peripherals------------------------------------------------- | 3-29 |
| Supervisor/User Password Setting----------------------------------- | 3-34 |
| IDE HDD Auto Detection-------------------------------------------- | 3-35 |
| Chapter 4 AMI® BIOS User’s Guide | 4-1 |
| Enter BIOS Setup------------------------------------------------------ | 4-2 |
| Standard CMOS Setup------------------------------------------------ | 4-4 |
| Advanced CMOS Setup----------------------------------------------- | 4-5 |
| Advanced Chipset Setup---------------------------------------------- | 4-9 |
| Power Management Setup-------------------------------------------- | 4-12 |
| PCI/Plug and Play Setup---------------------------------------------- | 4-16 |
| Peripheral Setup-------------------------------------------------------- | 4-21 |
| Hardware Monitor Setup--------------------------------------------- | 4-25 |
| Chapter 5 AUDIO INSTALLATION GUIDE | 5-1 |
| Install MS-5153 Audio Drivers-------------------------------------- | 5-2 |
| Install MS-5153 Audio Utility Drivers------------------------------ | 5-4 |
| Install MS-5153 Creative Wavesynth------------------------------- | 5-6 |
| Creative® 3D Stereo Enhancement------------------------------ | 5-7 |