Version 1.21

Intel and Pentium are registered trademarks of Intel Corporation.

AT, PS/2 and OS/2 are registered trademarks of IBM Corporation.

Windows 95 and Windows NT are registered trademarks of Microsoft Corp.

Netware is a registered trademark of Novell.

AMI is a registered trademark of American Megatrends, Inc.

Award is a registered trademark of Award Software, Inc.

AMD is a registered trademark of AMD.

Cyrix is a registered trademark of Cyrix.

Other brand, product names and trademarks are properties of their respective owners.

Manual Rev: 1.21

Release Date: September, 1997

FCC-B Radio Frequency Interference Statement

This equipment has been tested and found to comply with the limits for a class B digital device, pursuant to part 15 of the FCC rules. These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. Operation of this equipment in a residential area is likely to cause harmful interference, in which case the user will be required to correct the interference at his own expense.

Notice 1

The changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Notice 2

Shielded interface cables and A.C. power cord, if any, must be used in order to comply with the emission limits.

VOIR LA NOTICE D'INSTALLATION AVANT DE RACCORDER AU RESEAU.

Edition

September 1997

Copyright Notice

This manual is copyrighted. No guarantee is given as to the correctness of its contents. We reserve the right to make changes without notification.

Trademarks

All trademarks used in this manual are the properties of their respective owners.

Table of Contents

| Chapter 1: Introduction | 1-1 |
| --- | --- |
| System Board Features ----------------------------------------------- | 1-2 |
| Kffd Parts of the Mainboard------------------------------------------------ | 1-4 |
| Mainboard Layout----------------------------------------------------- | 1-5 |
| Chapter 2: Hardware Installation | 2-1 |
| Central Processing Unit: CPU--------------------------------------- | 2-1 |
| CPU Installation Procedures------------------------------------- | 2-1 |
| CPU Voltage Setting : JV2 – JV5------------------------------- | 2-2 |
| CPU Speed and Voltage Setting: SW1 & JV2-JV5 --------- | 2-4 |
| CPU Fan Power Connector: JFAN1/JFAN2/JSFAN/JPFAN | 2-9 |
| PCI and ISA Slots------------------------------------------------------ | 2-10 |
| Memory Installation--------------------------------------------------- | 2-11 |
| Memory Bank Configuration----------------------------------- | 2-11 |
| Memory Installation Procedure--------------------------------- | 2-12 |
| Memory Population Rules--------------------------------------- | 2-15 |
| Case Connector : JFP1------------------------------------------------ | 2-16 |
| Serial Port Connectors: COM A & COM B----------------------- | 2-18 |
| Parallel Port Connector: LPT---------------------------------------- | 2-19 |
| Floppy Disk Connector: FDC--------------------------------------- | 2-20 |
| Hard Disk Connector: IDE1 & IDE2------------------------------- | 2-21 |
| Power Supply Connector: J9----------------------------------------- | 2-22 |
| ATX 20-pin Power Connector: JWR1----------------------------- | 2-23 |
| Remote Power On/Off Switch: JRMS1/JRMS2------------------- | 2-24 |
| Remote Power Connector: JRMC1--------------------------------- | 2-25 |
| ATX Power Supply 3.3V: J10---------------------------------------- | 2-26 |
| Keyboard Connector: ATKBC-------------------------------------- | 2-27 |
| Mouse Connector: JMS1--------------------------------------------- | 2-28 |
| Infrared Module Connector: JIR1----------------------------------- | 2-29 |
| USB Connector: USB------------------------------------------------ | 2-30 |
| Power Saving Switch Connector: JGS1---------------------------- | 2-31 |
| Power Saving LED Connector: JGL1------------------------------ | 2-31 |
| Battery Connector: JBAT1------------------------------------------- | 2-32 |
| Flash ROM Programming Voltage: JP1/JP2----------------------- | 2-33 |
| Chapter 3 AWARD BIOS SETUP | 3-1 |
| Entering Setup --------------------------------------------------------- | 3-2 |
| Getting Help------------------------------------------------------------ | 3-2 |
| The Main Menu ------------------------------------------------------- | 3-3 |
| Standard CMOS Setup------------------------------------------------ | 3-5 |
| BIOS Features Setup -------------------------------------------------- | 3-8 |
| Chipset Features Setup ----------------------------------------------- | 3-14 |
| Power Management Setup ------------------------------------------- | 3-18 |
| PNP/PCI Configuration ---------------------------------------------- | 3-24 |
| Load BIOS/ Setup Defaults ------------------------------------------ | 3-27 |
| Integrated Peripherals------------------------------------------------- | 3-28 |
| Supervisor/User Password Setting----------------------------------- | 3-33 |
| IDE HDD Auto Detection-------------------------------------------- | 3-34 |
| Chapter 4 AMI BIOS USER’s Guide | 4-1 |
| Enter BIOS Setup------------------------------------------------------ | 4-2 |
| Standard CMOS Setup------------------------------------------------ | 4-4 |
| Advanced CMOS Setup----------------------------------------------- | 4-5 |
| Advanced Chipset Setup---------------------------------------------- | 4-9 |
| Power Management Setup-------------------------------------------- | 4-13 |
| PCI/Plug and Play Setup---------------------------------------------- | 4-17 |
| Peripheral Setup-------------------------------------------------------- | 4-21 |
| Hardware Monitor Setup--------------------------------------------- | 4-24 |