##### Version 1.0A

Manual Rev: 1.0A

Release Date: August 1999

FCC-B Radio Frequency Interference Statement

This equipment has been tested and found to comply with the limits for a class B digital device, pursuant to part 15 of the FCC rules. These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. Operation of this equipment in a residential area is likely to cause harmful interference, in which case the user will be required to correct the interference at his own expense.

Notice 1

The changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Notice 2

Shielded interface cables and A.C. power cord, if any, must be used in order to comply with the emission limits.

VOIR LA NOTICE D'INSTALLATION AVANT DE RACCORDER AU RESEAU.

MICRO-STAR       MS-5187

For Home or Office Use

Tested to comply with FCC Standard

Edition

August 1999

Copyright Notice

The material in this document is the intellectual property of MICRO-STAR INTERNATIONAL.  We take every care in the preparation of this document, but no guarantee is given as to the correctness of its contents.  Our products are under continual improvement and we reserve the right to make changes without notice.

Trademarks

All trademarks used in this manual are the property of their respective owners.

Intel and Pentium are registered trademarks of Intel Corporation.

PS/2 and OS/2 are registered trademarks of IBM Corporation.

Windows 95 and Windows NT are registered trademarks of Microsoft.

Netware is a registered trademark of Novell.

Award is a registered trademark of Award Software Inc.

CAUTION:  Danger of explosion if battery is incorrectly replaced.    
Replace only with the same or equivalent type recommended by the manufacturer.

###### Safety Instructions

- Always read the safety instructions carefully.

- Keep this User’s Manual for future reference.

- Keep this equipment away from humidity.

- Lay this equipment on a reliable flat surface before setting it up.

- The openings on the enclosure are for air convection hence protects the equipment from overheating.  DO NOT COVER THE OPENINGS.

- Make sure the voltage of the power source and adjust properly 110/220V before connecting the equipment to the power inlet.

- Place the power cord such a way that people can not step on it.  Do not place anything over the power cord.

- Always Unplug the Power Cord before inserting any add-on card or module.

- All cautions and warnings on the equipment should be noted.

- Never pour any liquid into the opening that could damage or cause electrical shock.

- If any of the following situations arises, get the equipment checked by a service personnel:

- The power cord or plug is damaged

- Liquid has penetrated into the equipment

- The equipment has been exposed to moisture

- The equipment has not work well or you can not get it work according to User’s Manual.

- The equipment has dropped and damaged

- If the equipment has obvious sign of breakage

- DO NOT LEAVE THIS EQUIPMENT IN AN EVIRONMENT UNCONDITIONED, STORAGE TEMPERATURE ABOVE 600 C (1400F), IT MAY DAMAGE THE EQUIPMENT.

Table of Contents

| Chapter 1: Introduction | 1-1 |
| --- | --- |
| Mainboard Features --------------------------------------------------- | 1-2 |
| Mainboard Layout----------------------------------------------------- | 1-5 |
| Chapter 2: Hardware Installation | 2-1 |
| Central Processing Unit: CPU--------------------------------------- | 2-1 |
| CPU Installation Procedures------------------------------------ | 2-1 |
| CPU Core Speed Derivation Procedure----------------------- | 2-2 |
| Processor Voltage Setting-------------------------------------- | 2-3 |
| Fan Power Connectors: CFAN1/SFAN1--------------------- | 2-4 |
| CPU Speed and Voltage Setting: SW1, JV1----------------- | 2-5 |
| Clear CMOS Jumper: JBAT1--------------------------------------- | 2-10 |
| Memory Installation--------------------------------------------------- | 2-11 |
| Memory Bank Configuration----------------------------------- | 2-11 |
| Memory Installation Procedures-------------------------------- | 2-12 |
| Memory Population Rules--------------------------------------- | 2-13 |
| Case Connector : JFP1----------------------------------------------- | 2-14 |
| Floppy Disk Connector: FDD-------------------------------------- | 2-16 |
| Hard Disk Connectors: IDE1 & IDE2------------------------------ | 2-17 |
| Power Supply----------------------------------------------------------- | 2-18 |
| ATX 20-pin Power Connector: JPWR1---------------------- | 2-18 |
| Remote Power On/Off Switch: JRMS1---------------------- | 2-19 |
| IrDA Infrared Module Connector: IR1---------------------------- | 2-20 |
| Serial Port Connector: COM A-------------- ---------------------- | 2-21 |
| VGA DB 15 Pin Connector ----------------------------------------- | 2-22 |
| Parallel Port Connector: LPT1-------------------------------------- | 2-23 |
| Mouse and Keyboard Connector: JKBMS1---------------------- | 2-24 |
| Joystick/Midi Connectors-------------------------------------------- | 2-25 |
| Audio Port Connectors----------------------------------------------- | 2-25 |
| USB Connectors------------------------------------------------------- | 2-26 |
| Power Saving Switch Connector: JGS1----------------------------- | 2-27 |
| Power Saving LED Connector: JGL1------------------------------ | 2-28 |
| Wake-Up on LAN Connector: JWOL1----------------------------- | 2-29 |
| Modem Wake Up Connector: JMDM1---------------------------- | 2-30 |
| Modem-In: MODEM1----------------------------------------------- | 2-31 |
| AUX Line In Connector: JAUX1---------------------------------- | 2-32 |
| CD-In Connector: JCD1--------------------------------------------- | 2-33 |
| AMR (Audio Modem Riser)----------------------------------------- | 2-34 |
| PTI (PanelLink TV-Out Interface)---------------------------------- | 2-35 |
| Power LED connector: JPL1---------------------------------------- | 2-36 |
| Chapter 3 Award® BIOS SETUP | 3-1 |
| Entering Setup---------------------------------------------------------- | 3-2 |
| Getting Help------------------------------------------------------------ | 3-2 |
| The Main Menu------------------------------------------------------- | 3-3 |
| Standard CMOS Setup----------------------------------------------- | 3-5 |
| BIOS Features Setup------------------------------------------------- | 3-8 |
| Chipset Features Setup----------------------------------------------- | 3-13 |
| Power Management Setup------------------------------------------- | 3-17 |
| PnP/PCI Configuration Setup--------------------------------------- | 3-22 |
| Load BIOS/Setup Defaults------------------------------------------- | 3-26 |
| Special Features Setup (optional)----------------------------------- | 3-27 |
| Integrated Peripherals------------------------------------------------ | 3-29 |
| Supervisor/User Password Setting---------------------------------- | 3-34 |
| IDE HDD Auto Detection-------------------------------------------- | 3-35 |
| Chapter 4 VIA Chipset Driver | 4-1 |
| Overview--------------------------------------------------------------- | 4-1 |
| Driver Setup & Usage Procedures for Windows® 98------------- | 4-3 |
| Windows® NT 4.0----------------------------------------------------- | 4-6 |