Chapter 1

Introduction

The PCI P6 NA7 PCI/ISA system board is a high-performance personal computer system board based on the Pentiumä Pro microprocessor.

The system board utilizes the Intelâ 82440FX PCI Chipset which has high integration and provides all system control functions.  In addition there is an on-chipset USB and IDE controller which supports IDE PIO/Bus Master operation mode for IDE HDD/CD-ROM.  The PIO mode Supports Mode 0, 1, 2, 3 and 4 with a transfer rate up to 22 MB per/sec.

1.1 System Board Specifications

CPU:

·	2 ZIF type 8 sockets for Dual Intelâ Pentiumä Pro microprocessors.  Each microprocessor has its own Core/Bus ratio jumper Core/Bus ratios are x2, x3, x4, x5, x2.5, x3.5, x4.5, x5.5.

VRM:

- VRM (Voltage Regulator Module) is on board.  It provides power for the processor.  The adjustment mechanism is controlled by the four weighted input (VID0-VID3) from the processor.

Chip Set:

·	Intelâ 82440FX PCI Chipset.

Clock Generator:

- There are two clocks, 60MHz/66.6MHz. Both meet Pentiumä Pro microprocessor and NATOMA (82440FX) Chipset specifications.

Main Memory:

·	Supports SIX memory banks using six 72-pin SIMM sockets.

·	Up to 768 Mbytes main memory.

- Supports Fast Page (FP) Mode, Extended Data Output (EDO) Mode, and Burst Extended Data Output (BEDO) Mode DRAM.

Slots:

·	Five 32-bit PCI Bus slots and three 16-bit ISA bus slots. One shared slot that can be used as ISA or PCI.

On-Board Peripherals:

·     On-Board peripherals include:

¾1 floppy port supports 2 FDD

¾2 serial ports (ComA + ComB)

¾1 parallel port supports SPP, EPP or ECP mode

¾1 FDD port that supports 2 FDDs with 360K, 720K, 1.2M, 1.44M, and 	2.88Mbytes per second transfer rate.

On-Board IDE:

- An IDE controller on the Intelâ 82440FX PCI Chipset provides IDE HDD/CD-ROM with PIO and Bus Master Operation Mode. The PIO Mode supports modes 0, 1, 2, 3, and 4, with a transfer rate of  22 Mbytes second.

On-Board USB:

- 2 port USB features on the Intelâ 82440FX PCI Chipset.

RTC:

- Non-Volatile RTC with 114 bytes of  CMOS RAM.

Dimensions

·     ATX form factor 30cm(L)X26cm(W); 6 layer PCB

Mounting

·	8 mounting holes

1.2 System Board Layout