C O N D O R

3 8 6

The material in this manual is for information only and is subject to change without notice.

REVISION: 1.0

IBM, IBM PC/XT/AT, PC-DOS, MS-DOS, OS/2, INTEL, WEITEK, PHOENIX ARE THE TRADEMARKS OR REGISTERED TRADEMARKS OF THEIR RESPECTIVE OWNERS.

RADIO FREQUENCY INTERFERENCE STATEMENT

This equipment generates and uses radio frequency energy and if not installed and used properly, that is, in strict accordance with the manufacturer's instructions, may cause interference with radio and television reception.

If this equipment does cause interference to radio or TV reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient the receiving antenna.

*	Relocate the computer away from the receiver.

*	Move the computer away from the receiver.

*	Plug the power cord of computer into a different outlet so that computer and receiver are on different branch circuits.

*	Ensure that card slot covers are in place when no card is installed.

*	Ensure that card mounting screws, attachment connector screws, and ground wires are tightly secured.

*	If peripherals are used with this system, it is suggested to use shielded, grounded cables, with in-line filters if necessary.

If necessary, the user should consult the dealer service representative for additional suggestions.

The manufacturer is not responsible for any radio or TV interferences caused by unauthorized modifications to this equipment.  It is the responsibility of the user to correct such interferences.

Note

1.		Electronic components are sensitive to dust and dirt.  Do inspect and clean the computer system regularly.

2.		Turn off the power whenever you install or remove any connector, memory module and add-on card.  Before turning on the power, make sure that all the connectors, memory modules and add-on cards are secured.

3.		After power is on, please wait for a minute.  The system BIOS are going through a self-test during this period and nothing is shown on the screen.  After the self-test, the system BIOS will initialize the display adaptor and show messages.

4.		The SIMM sockets are fragile device.  Do not force the SIMM modules into the sockets.  It may break the locking latches.

Preface

The manual provides information about the installation and maintenance of OCTEK DOVE-386 motherboard.  In-depth explanations of the functions of this motherboard are provided.  In the appendix, the system BIOS setup is explained.

The content in this manual is only for reference and is intended to provide basic information for the general users.  There are also some technical information for hardware and software engineers.

In this manual, there are 4 chapters.  Chapter 1 contains a brief introduction and specification of OCTEK CONDOR-386 motherboard.  In the Chapter 2, the functions of CONDOR-386 are explained.  It also outlines many advanced features of the CPU and the system architecture.  Chapter 3 explains the installation of coprocessor, DRAM modules and jumpers.  Some technical information are provided in the Chapter 4.

System BIOS and the system setup are described in the Appendix A.  All the setup procedures are explained.

Additional information are given in Appendix B, C for the maintenance purpose.

CONTENT

Chapter 1		INTRODUCTION			1-1

Chapter 2		GENERAL FEATURES		2-1

Specification				2-1

Processor					2-3

Math Coprocessor				2-6

Memory System				2-7

Two-way Page Mode Operation	2-7

System Functions				2-8

Chapter 3		INSTALLING COMPONENTS	3-1

Installing Math Coprocessor		3-1

Installing RAM Modules		3-3

Installing External Battery		3-5

Configuration of Memory		3-6

DRAM Configuration			3-7

Control of System Speed		3-8

Reset CMOS Setup			3-8

Information

System Board Jumper Setting		3-9

System Board Connectors		3-10

Chapter 4		TECHNICAL INFORMATION	4-1

Memory Mapping				4-1

I/O Address Map				4-2

System Timers				4-4

System Interrupts				4-6

Direct Memory Access (DMA)		4-7

Real Time Clock and CMOS		4-8

RAM

CMOS RAM Address Map		4-9

Real Time Clock Information		4-10

System Expansion Bus			4-11

APPENDIX A	SYSTEM BIOS				A-1

Setup Utility Description		A-2

Setup Section				A-9

APPENDIX B	OPERATION & MAINTENANCE	B-1

Static Electricity			B-1

Keeping the System Cool		B-1

Cleaning the "Golden Finger"	B-2

Cleaning the Motherboard		B-2

APPENDIX C	TROUBLESHOOTING		C-1

Main Memory Error			C-1

Improper Setting of Wait State	C-1

APPENDIX D	SYSTEM BOARD LAYOUT		D-1

Chapter 1

Introduction

_______________________________

OCTEK CONDOR-386 consists of 32-bit 80386 microprocessor, a large cache memory and highly integrated chipsets to provide high performance, reliability and compatibility.  OCTEK CONDOR-386 is a perfect choice for CAD/CAM workstation and file server and supports sophisticated 32-bit computing applications and multi-user operating systems.

Aimed at supporting advanced CAD/CAM applications, OCTEK CONDOR-386 supports INTEL 80387.  The 32MB memory is installed on board.

Compatibility and reliability are important issues.  I/O channel is compatible to standard AT bus.  Therefore any AT compatible peripherals may be used on OCTEK CONDOR-386.  On board POWERGOOD generator is essential to ensure the reliability of the system and is well-designed to work with all power supplies.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 2

General Features

_______________________________

SPECIFICATION

Processor :

Intel / AMI 80386DX CPU

with optional 80387DX Math Co-processor

Speed :

Turbo/normal speed

Software/hardware selectable

I/O Slot :

Compatible to standard AT bus

Seven 16-bit slots

Programmable AT bus speed

Memory :

Shadow RAM for System BIOS and

Video BIOS

Page mode two-way interleave with

programmable wait state

SIMM sockets for 256K , 1M or 4M modules

Up to 32M bytes on board

System Support Functions :

8-Channel DMA (Direct Memory Access)

16-level interrupt

3 programmable timers

CMOS RAM for system configuration

Real time clock with battery backup

Other Features :

On board POWERGOOD generation

External battery connector

Hardware turbo switch

PROCESSOR

80386DX is a 32-bit microprocessor with 32-bit external data bus and 32-bit external address bus.  Therefore it processes more data at the same time than 80286 and can access a large memory size which is necessary for 32-bit applications.  To combine the wider bus structure and all the advanced functions on chip, total 275,000 transistors are integrated together.

An advanced pipeline architecture is implemented.  Next four Bytes of instructions are prefetched into the CPU whenever the bus is idle.  The size of the prefetch queue is increased to hold 12 Bytes.   This architecture reduces the overhead of retrieving and decoding of the instructions.

80386DX is aimed to provide advanced facilities to sophisticated software, but still remains compatible with existing software in the market.  It can operate at real mode and protected mode.  In Real mode the 386DX operates as a very fast 8086, but with 32-bit extensions if desired.  Real mode is required primarily to setup the processor for Protected mode operation.  Protected mode provides access to the sophisticated memory management,  paging and privilege capabilities of the processor.  Furthermore, new mechanism allows switching between Real mode and Protected mode at high speed operation.

Hence applications using EMS memory or extended memory can run efficiently although they need software driver to access the memory beyond 1MB.

The Protected mode of 80386 is fully compatible with 80286.  All privilege - level and I/O protection system are supported.  New system control instructions, memory paging, I/O permission bit map are provided to make 80386 ideal for multi-tasking operation systems.

In addition, a virtual 8086 mode is provided.  In this mode, the CPU can be considered by the programs as being divided into several 8086 CPUs and each program has their own CPU and memory space.  Several programs for XT/AT as well as operating systems for 80286 and 80386DX can be executed simultaneously.  Programs are isolated and protected from each other by 80386DX.  Each program can be considered as running at a XT/AT.

Internal memory management unit is complicated, but provides a more flexible addressing scheme for the next generation of operation system.  Multitasking, concurrent operation and manipulating huge data base can be accomplished with excellent performance.  Paging mechanism is employed by 80386DX to allow powerful operating system to implement virtual memory.  Each segment is divided into several pages which are 4K Bytes per page.  Page mechanism is transparent to software and allows software to address up to 64 Terabytes.

Furthermore, the 64KB segment boundary which is an barrier of 8088 and 80286 is removed and the segment length can be increased up to 4GB.  It eliminates the need for the software to manage multiple code and data segments and allows software of other operating system to be transferred to PC more easily.

80386DX includes many new instructions for system control, high level language support and processor control.  These instructions are used in protected mode.  New operation systems and software can make use of these instructions for their advanced features, such as concurrent operation and virtual memory.

MATH COPROCESSOR

The demand for sophisticated, number-crunching scientific and business applications has rapidly increased in recent years.  80386DX features an integer Arithmetic Logic Unit which only handles simple integer operations such as addition and multiplication.  Floating-point operations which are actually utilized by applications must be accomplished through software routines.

To overcome this obstacle, external Math coprocessor is necessary.  The Math coprocessor contains complex hardware and large data registers for floating-point numeric operations.  In OCTEK CONDOR the Math coprocessor is optional and  INTEL 80387 is supported.

80387 is upward object-code compatible from 80287 and 8087, but runs 6 to 11 times faster than 80287 used in AT.  It fully implements the IEEE 754 Binary Floating-point Arithmetic standard with a high precision 80-bits internal architecture.

MEMORY SYSTEM

Two banks of DRAMs can be installed. One bank of DRAM refers to four pieces of SIMM modules.  The maximum memory size is 32MB when using 4MB DRAM for all banks.

The memory system supports page mode.  Successive memory accesses within the same page need not require wait state and thus CPU can run at full speed.

To enhance the system performance, shadow RAM mode is supported.  In shadow RAM mode, system BIOS and video BIOS contained in low speed memory such as EPROM and ROM are copied into DRAM.  Improvement is significant because access to DRAM is much faster than ROM.

TWO-WAY PAGE MODE OPERATION

Two-way page mode DRAM access providing higher performance over conventional DRAM access schemes. It is automatically enabled whenever both memory banks of a pair are populated with same DRAM types. The two-way page mode operation results in no additional wait state during read or write operation.

SYSTEM FUNCTIONS

System functions include :

-  Interrupt

-  DMA

-  Timer

-  Real time clock

-  Clock and ready generation

-  I/O channel control

All system functions are 100% compatible to AT standard.  I/O channel of OCTEK CONDOR 386 is designed to be compatible with standard AT bus.  All the expansion cards conformed to the standard AT bus can be used in OCTEK CONDOR-386 without problem.

Chapter 3

Installing Components

_______________________________

Important Note : Turn off the power before installing or replacing any component.

INSTALLING MATH COPROCESSOR

Math coprocessor INTEL 80387 is PGA devices.  Beside the CPU, there is a 68-pin PGA socket.  To install Math coprocessor, be sure to line up pin 1 of the Math coprocessor with pin 1 of the socket as shown below.  Make sure that the coprocessor is firmly inserted into the socket.

Before installing the Math coprocessor, make sure all the pins are straight.  The pins are very fragile.  Once these pins are bent, the coprocessor may be damaged.

Check whether the system BIOS can find the coprocessor after reset.  The system BIOS will display a list of devices on the motherboard after self-test.  If the coprocessor is installed, it should show the type of coprocessor.

INSTALLING RAM MODULES

OCTEK CONDOR-386 has eight sockets for SIMM modules.  Whenever adding memory modules to the motherboard, install four modules at the same time.  Also make sure that the chips on the modules face toward the memory expansion slot as shown in the next page.

To install a module, the module edge is angled into the socket's contact and then the module is pivoted into position, where the locking latches will secure it.  If the module edge is not completely inserted into the socket, it cannot be pivoted to be in vertical position and should be dragged out and inserted again.  Do not force the module into the SIMM socket.  It will damage the locking latches.

The modules should be locked by the locking latches of the sockets firmly.  Please check carefully before turning on the power.  Otherwise, the system will not work properly.

If the BIOS reports an memory error or parity error, drag out the modules and insert them again.  If the locking latches are damaged, contact your dealer to replace the socket.

INSTALLING EXTERNAL BATTERY

To back up the information stored in CMOS RAM, the optional external battery is needed to provide power after the system is turned off.  The connector (P8) for the battery is located beside the keyboard connector on the rear of the board.  A 3.6V battery is used.  Turn off the power before install the battery.  The location of the connector P8 is shown below.

CONFIGURATION OF MEMORY

The configuration of the memory is very flexible.  It can install 256KB , 1MB or 4MB SIMM modules are acceptable.  There are several combinations of DRAM types you may consider.  So, a basic system can be equipped with fewer memory and the system can be upgraded by installing the extended memory.  The different configurations of memory is illustrated at the next page. There are totally two banks of DRAM.  Two banks (bank 0 and 1) are on the motherboard.

The memory size is detected automatically by system BIOS.  This detection is performed during memory test and the size is indicated after reset.  No jumper is needed to be set for the memory size and DRAM type.

To determine what DRAM speed rating to be used is depended on the system speed and wait state.  The highest performance is accomplished by using zero wait state, but high speed DRAM has to be used.

The wait state setting is applied to two banks of memory.   Therefore make sure to install DRAM modules with the same speed rating, or accommodate the wait state setting to the new DRAM type.

The number of wait state is assigned in the BIOS setup.  Improper setting may cause the system malfunction.  In this case, reset the CMOS setup using JP3. Then reset the system and go through the system setup again.

DRAM CONFIGURATION

|  | Bank 0 SIMM (1-4) | Bank 1 SIMM (5-8) | Total Memory |
| --- | --- | --- | --- |
| 1 | 256K | X | 1M |
| 2 | 256K | 256K | 2M |
| 3 | 1M | X | 4M |
| 4 | 256K | 1M | 5M |
| 5 | 1M | 1M | 8M |
| 6 | 4M | X | 16M |
| 7 | 4M | 4M | 32M |

CONTROL OF SYSTEM SPEED

System speed can be controlled by hardware switch and keyboard.  Connector P3 is connected to the turbo switch of the case.  When the system speed is fast, the turbo LED of the case should be turned on.

To change the speed by keyboard, use '-' and '+' of the numeric keypad.  Press 'Ctrl', 'Alt' and '-' for slow speed and Press  'Ctrl', 'Alt' and '+' for fast speed.

Whenever the system speed is set to slow by turbo switch, it cannot be changed by keyboard, and vice versa.

RESET CMOS SETUP INFORMATION

Sometimes, the improper setting of system setup may make the system malfunction.   In this case, turn off the power and disconnect the external battery.  Then place a jumper on JP3 (2-3) for a while.   The internal CMOS status register will be cleared.  Then remove the jumper and turn on the power.  The BIOS finds the CMOS status register is reset and regards the setup information as invalid.  So it will prompt you to correct the information.  In normal operation, JP3 place in (1-2).

SYSTEM BOARD JUMPER SETTING

There are several options which allow user to select by hardware switches.

Security Selection

| JP2 |  |
| --- | --- |
| 1-2 | enable |
| open | disable * |

Note :  * factory setting

SYSTEM BOARD CONNECTORS

Under typical conditions, these connectors should be connected to the indicators and switches of the system unit.

| Connector | Function |
| --- | --- |
| P1 | Hardware reset connector |
| P2 | Speaker connector |
| P3 | Turbo switch connector |
| P4 | Turbo LED connector |
| P5 | Power LED & Ext-Lock connector |
| P6, P7 | Power supply connector |
| P8 | External battery connector |
| KB1 | Keyboard connector |

Pin assignments of the connectors are illustrated as follows:

P 1 - Hardware Reset Connector

| Pin | Assignment |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

P 2 - Speaker Connector

| Pin | Assignment |
| --- | --- |
| 1 | Data out |
| 2 | +5 Vdc |
| 3 | Ground |
| 4 | +5 Vdc |

P 3 - Turbo Switch Connector

| Pin | Assignment |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

P 4 - Turbo LED Connector

| Pin | Assignment |
| --- | --- |
| 1 | +5 Vdc |
| 2 | LED signal |

P 5 - Power LED & Ext-Lock Connector

| Pin | Assignment |
| --- | --- |
| 1 | +5 Vdc |
| 2 | Key |
| 3 | Ground |
| 4 | Keyboard inhibit |
| 5 | Ground |

P6, P7 - Power Supply Connector

| Pin | Assignment |
| --- | --- |
| 1 | POWERGOOD |
| 2 | +5 Vdc |
| 3 | +12 Vdc |
| 4 | -12 Vdc |
| 5 | Ground |
| 6 | Ground |

| Pin | Assignment |
| --- | --- |
| 1 | Ground |
| 2 | Ground |
| 3 | -5 Vdc |
| 4 | +5 Vdc |
| 5 | +5 Vdc |
| 6 | +5 Vdc |

P 8 - External Battery Connector

| Pin | Assignment |
| --- | --- |
| 1 | + Vdc |
| 2 | not used |
| 3 | Ground |
| 4 | Ground |

KB 1 - Keyboard Connector

| Pin | Assignment |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Spare |
| 4 | Ground |
| 5 | +5 Vdc |

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 4

Technical Information

_______________________________

This section provides technical information about OCTEK CONDOR-386 and is intended for advanced users interested in the basic design and operation of OCTEK CONDOR-386.

MEMORY MAPPING

| Address | Range | Function |
| --- | --- | --- |
| 000000-7FFFFF | 000K-512K | System Board Memory (512K) |
| 080000-09FFFF | 512K-640K | System Board Memory (128K) |
| 0A0000-0BFFFF | 640K-768K | Display Buffer (128K) |
| 0C0000-0DFFFF | 768K-896K | Adaptor ROM / Shadow RAM (128K) |
| 0E0000-0EFFFF | 896K-960K | System ROM / Shadow RAM (64K) |
| 0F0000-0FFFFF | 960K-1024K | System BIOS ROM / Shadow RAM (64K) |
| 100000-7FFFFF | 1024K-8192K | System Memory |
| 800000-FFFFFF | 8192K-16318K | System Memory |

I/O ADDRESS MAP

I/O Address Map on System Board

I/O address hex 000 to 0FF are reserved for the system board I/O.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 000-01F | DMA Controller 1, 8237 |
| 020-03F | Interrupt Controller 1, 8259, Master |
| 040-05F | Timer, 8254 |
| 060-06F | Keyboard Controller |
| 070-07F | Real Time Clock, NMI (non-maskable interrupt) mask |
| 080-09F | DMA Page Register, 74LS612 |
| 0A0-0BF | Interrupt Controller 2, 8259 |
| 0C0-0DF | DMA Controller 2, 8237 |
| 0F0 | Clear Math Coprocessor Busy |
| 0F1 | Reset Math Coprocessor |
| 0F8-0FF | Math Coprocessor Port |

I/O address hex 100 to 3FF are available on the I/O channel.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game I/O |
| 278-27F | Parallel Printer Port 2 |
| 2F8-2FF | Serial Port 2 |
| 300-31F | Prototype Card |
| 360-36F | Reserved |
| 378-37F | Parallel Printer Port 1 |
| 380-38F | SDLC, bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Printer Adapter |
| 3C0-3CF | Reserved |
| 3D0-3DF | Color Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 |

SYSTEM TIMERS

OCTEK CONDOR-386 has three programmable timer/counters controlled by 82C206 and they are defined as channels 0 through 2 :

| Channel 0 | System Timer |
| --- | --- |
| Gate 0 | Tied on |
| Clk in 0 | 1.190 Mhz OSC |
| Clk out 0 | 8259 IRQ 0 |

| Channel 1 | Refresh Request Generator |
| --- | --- |
| Gate 1 | Tied on |
| Clk in 1 | 1.190 Mhz OSC |
| Clk out 1 | Request Refresh Cycle |

| Channel 2 | Tone Generation of Speaker |
| --- | --- |
| Gate 2 | Controlled by bit 0 of port hex 61 PPI bit |
| Clk in 2 | 1.190 Mhz OSC |
| Clk out 2 | Used to drive the speaker |

Note :	Channel 1 is programmed to generate a 15-micro-second period signal.

The 8254 Timer/Counters are treated by system programs as an arrangement of four programmable external I/O ports.  Three are treated as counters and the fourth is a control register for mode programming.

SYSTEM INTERRUPTS

Sixteen levels of system interrupts are provided on OCTEK CONDOR-386. The following shows the interrupt-level assignments in decreasing priority.

Level					Function

Microprocessor NMI	Parity or I/O Channel

Check

Interrupt Controllers

CTLR 1	CTLR 2

IRQ0				Timer Output 0

IRQ1				Keyboard

(Output Buffer Full)

IRQ2				Interrupt from CTLR 2

IRQ8		Real-time Clock Interrupt

IRQ9		Software Redirected to

INT 0AH (IRQ2)

IRQ10	Reserved

IRQ11	Reserved

IRQ12	Reserved

IRQ13	Coprocessor

IRQ14	Fixed Disk Controller

IRQ15	Reserved

IRQ3				Serial Port 2

IRQ4				Serial Port 1

IRQ5				Parallel Port 2

IRQ6				Diskette Controller

IRQ7				Parallel Port 1

┌──────────────────────────────────────┐

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│     ─┐                               │

│      │ ┌─                            │

│      │ │                             │

│      │ │                             │

│      │ │                             │

│      └─┤                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        └─                            │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

└──────────────────────────────────────┘

DIRECT MEMORY ACCESS (DMA)

OCTEK CONDOR-386 supports seven DMA channels.

| Channel | Function |
| --- | --- |
| 0 | Spare (8 bit transfer) |
| 1 | SDLC (8 bit transfer) |
| 2 | Floppy Disk (8 bit transfer) |
| 3 | Spare (8 bit transfer) |
| 4 | Cascade for DMA Controller 1 |
| 5 | Spare (16 bit transfer) |
| 6 | Spare (16 bit transfer) |
| 7 | Spare (16 bit transfer) |

The following shows the addresses for the page register.

| Page Register | I/O Address (HEX) |
| --- | --- |
| DMA Channel 0 | 0087 |
| DMA Channel 1 | 0083 |
| DMA Channel 2 | 0081 |
| DMA Channel 3 | 0082 |
| DMA Channel 5 | 008B |
| DMA Channel 6 | 0089 |
| DMA Channel 7 | 008A |
| Refresh | 008F |

REAL TIME CLOCK AND CMOS RAM

Real time clock and CMOS RAM are contained on board. Real time clock provides the system date and time. CMOS RAM stores system information. Both are backed up by battery and will not lose information after power off.  The following page shows the CMOS RAM Address Map.

CMOS RAM ADDRESS MAP

| Addresses | Description |
| --- | --- |
| 00-0D | * Real-time clock information |
| 0E | * Diagnostic status byte |
| 0F | * Shutdown status byte |
| 10 | Diskette drive type byte - drives A and B |
| 11 | Reserved |
| 12 | Fixed disk type byte - drives C and D |
| 13 | Reserved |
| 14 | Equipment byte |
| 15 | Low base memory byte |
| 16 | High base memory byte |
| 17 | Low expansion memory byte |
| 18 | High expansion memory byte |
| 19-2D | Reserved |
| 2E-2F | 2-byte CMOS checksum |
| 30 | * Low expansion memory byte |
| 31 | * High expansion memory byte |
| 32 | * Date century byte |
| 33 | * Information flags (set during power on) |
| 34-3F | Reserved |

REAL TIME CLOCK INFORMATION

The following table describes real-time clock bytes and specifies their addresses.

| Byte | Function | Address |
| --- | --- | --- |
| 0 | Seconds | 00 |
| 1 | Second alarm | 01 |
| 2 | Minutes | 02 |
| 3 | Minute alarm | 03 |
| 4 | Hours | 04 |
| 5 | Hour alarm | 05 |
| 6 | Day of week | 06 |
| 7 | Date of month | 07 |
| 8 | Month | 08 |
| 9 | Year | 09 |
| 10 | Status Register A | 0A |
| 11 | Status Register B | 0B |
| 12 | Status Register C | 0C |
| 13 | Status Register D | 0D |

SYSTEM EXPANSION BUS

OCTEK CONDOR-386 provides seven 16-bit slots.

The I/O channel supports:

*	I/O address space from hex 100 to hex 3FF

*	Selection of data access (either 8 or 16 bit)

*	24 bit memory addresses (16MB)

*	Interrupts

*	DMA channels

*	Memory refresh signal

The following figure shows the pin numbering for I/O channel connectors JA1 to JA7.

The following figure shows the pin numbering for I/O channel connectors JB1-JB7.

The following tables summarize pin assignments for the I/O channel connectors.

I/O Channel (A-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| A1 | -I/O CH CK | I |
| A2 | SD7 | I/O |
| A3 | SD6 | I/O |
| A4 | SD5 | I/O |
| A5 | SD4 | I/O |
| A6 | SD3 | I/O |
| A7 | SD2 | I/O |
| A8 | SD1 | I/O |
| A9 | SD0 | I/O |
| A10 | -I/O CH RDY | I |
| A11 | AEN | O |
| A12 | SA19 | I/O |
| A13 | SA18 | I/O |
| A14 | SA17 | I/O |
| A15 | SA16 | I/O |
| A16 | SA15 | I/O |
| A17 | SA14 | I/O |
| A18 | SA13 | I/O |
| A19 | SA12 | I/O |
| A20 | SA11 | I/O |
| A21 | SA10 | I/O |
| A22 | SA9 | I/O |
| A23 | SA8 | I/O |
| A24 | SA7 | I/O |
| A25 | SA6 | I/O |
| A26 | SA5 | I/O |
| A27 | SA4 | I/O |
| A28 | SA3 | I/O |
| A29 | SA2 | I/O |
| A30 | SA1 | I/O |
| A31 | SA0 | I/O |

I/O Channel (B-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| B1 | GND | Ground |
| B2 | RESET DRV | I |
| B3 | +5 Vdc | Power |
| B4 | IRQ9 | I |
| B5 | -5 Vdc | Power |
| B6 | DRQ2 | I |
| B7 | -12 Vdc | Power |
| B8 | 0WS | I |
| B9 | +12 Vdc | Power |
| B10 | GND | Ground |
| B11 | -SMEMW | O |
| B12 | -SMEMR | O |
| B13 | -IOW | I/O |
| B14 | -IOR | I/O |
| B15 | -DACK3 | I |
| B16 | DRQ3 | O |
| B17 | -DACK1 | I |
| B18 | DRQ1 | O |
| B19 | -Refresh | I/O |
| B20 | CLK | O |
| B21 | IRQ7 | I |
| B22 | IRQ6 | I |
| B23 | IRQ5 | I |
| B24 | IRQ4 | I |
| B25 | IRQ3 | I |
| B26 | -DACK2 | O |
| B27 | T/C | O |
| B28 | BALE | O |
| B29 | +5 Vdc | Power |
| B30 | OSC | O |
| B31 | GND | Ground |

I/O Channel (C-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| C1 | SBHE | I/O |
| C2 | LA23 | I/O |
| C3 | LA22 | I/O |
| C4 | LA21 | I/O |
| C5 | LA20 | I/O |
| C6 | LA19 | I/O |
| C7 | LA18 | I/O |
| C8 | LA17 | I/O |
| C9 | -MEMR | I/O |
| C10 | -MEMW | I/O |
| C11 | SD8 | I/O |
| C12 | SD9 | I/O |
| C13 | SD10 | I/O |
| C14 | SD11 | I/O |
| C15 | SD12 | I/O |
| C16 | SD13 | I/O |
| C17 | SD14 | I/O |
| C18 | SD15 | I/O |

I/O Channel (D-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| D1 | -MEM CS16 | I |
| D2 | -I/O CS16 | I |
| D3 | IRQ10 | I |
| D4 | IRQ11 | I |
| D5 | IRQ12 | I |
| D6 | IRQ15 | I |
| D7 | IRQ14 | I |
| D8 | -DACK0 | O |
| D9 | DRQ0 | I |
| D10 | -DACK5 | O |
| D11 | DRQ5 | I |
| D12 | -DACK6 | O |
| D13 | DRQ6 | I |
| D14 | -DACK7 | O |
| D15 | DRQ7 | I |
| D16 | +5 Vdc | Power |
| D17 | -MASTER | I |
| D18 | GND | Ground |

THIS PAGE IS INTENTIONALLY LEFT BLANK

Appendix A

System BIOS

_______________________________

The system BIOS provides an interface for the software to control the hardware and is recorded in a ROM (Read Only Memory) chip.  Upon power-up, it will also carry out a thorough diagnostic test to make sure the system is functional.  It will initialize the chipset with setting stored in the CMOS RAM.

Then it proceeds to load the disk operating system and you can start to work with your applications.

In this supplementary, it mainly explains the BIOS Setup Utility, in which you can set up your system to suit your configuration and applications.  In case you have any doubt, consult your dealer.  Improper setting may cause reliability problem or system failure.

SETUP UTILITY DESCRIPTION

A Setup Utility is incorporated into the BIOS which allows the user to change the system configuration, and to select a variety of options.

You may enter the Setup Utility in three ways:

(1)	A configuration change detected, or

(2)	ESC is pressed during cold-boot, or

(3)	CTRL+ALT+ESC is pressed to warm-boot into Setup.

The Setup Utility is an interactive program for the system setup.  You can enter the system configuration, such as clock, hard disk type and floppy disk type. There are many advanced options available to you to improve the system performance.  The settings for these options are preset to a proper value, such that the overall performance is optimal.  In most case, you only need to setup the clock, hard disk type and floppy disk type.

The main screen format of setup utility consists of these four fields:

(1)	Copyright/Version

On the top two lines on the screen,  the BIOS version number is shown.

(2)	Menu Line

A list of setup section names appears on this line, from which a specific section may be selected.  Use 'Left' and 'Right' arrow key to change from one setup section to another and the corresponding Edit-Page (see below) is shown.  Press ENTER key to enter the setup section.

(3)	Edit Page

In the Edit page, setup options are listed.  You can move the cursor to a particular field and change the setting.  Press PgUp key to exit.

(4)	Prompt Line

The Setup-Utility is designed to be usable without the aid of this manual.  In the prompt line, acceptable key-strokes and corresponding explanation are listed.

The main Setup Menu is shown as below when the Setup Utility is invoked.

MR BIOS (tm) Copyright (c) 1991, Microid Research Ver 1.26 Port OPTI335

| CPU Type ......................... 80386DX | Floppy 0 ........................ 1.2M [5¼] |
| --- | --- |
| CPU MHz .................................. 33.4 | Floppy 1 .............................. None |
| Boot Speed ............................... High | Floppy 2 .............................. None |
| Math Unit .................................. n/a | Floppy 3 .............................. None |
|  | Fixed 80 (C:) ................... Type 17 |
| RAM Cache ........................... Enable | Fixed 81 (D:) .................. Type 17 |
| Shadow RAM ........................ Enable | Boot Sequence .................... C: 1st |
| Memory-Base ........................... 640K | Anti-Virus .............................. n/a |
| Memory-Extended ................. 3328K | Security ............................ Disable |
| Memory-System ....................... 128K |  |
| Memory-Total ........................ 4096K | Keyboard ................................ AT |
|  | NumLock ................................ On |
| COM1 .......... 3F8 ¦ LPT1 ........ 378 | Typematic ............................. 30.0 |
| COM2 .......... n/a ¦ LPT2 ........ n/a |  |
| COM3 .......... n/a ¦ LPT3 ........ n/a | Video-Primary ...... V/EGA-Color |
| COM4 .......... n/a ¦ LPT4 ........ n/a | Video-Secondary .................... n/a |

Summary	Clock	Video	Floppy	Fixed	Boot-Seq	Keyboard	More-->

F10 to Record and Exit		Home End Moves Cursor

PROMPT-LINE TEXT

The purpose of this section is to further explain the meanings of the keystroke prompts.  They are somewhat abbreviated due to screen space limitations.

F10 TO RECORD AND EXIT

Press F10 to record the new configuration to CMOS, and terminate the Setup session.  The system will proceed to boot-up.

HOME END (LEFT/RIGHT ARROWS) MOVES CURSOR

The Menu-cursor can be moved respectively to the first entry, last entry, or next leftward/rightward entry.

(ENTER) TO SELECT

When the cursor is in Menu Line, you need to press enter key to enter the Edit Page for a particular setup section.  The cursor will move from the Menu Line into the Edit Page, on the first field.

ESC FOR MENU

When the cursor is currently in the Edit-Page,  press ESC (or PgUp) to return to Menu-Line.

(UP/DOWN/LEFT/RIGHT ARROWS) MOVES CURSOR

The cursor is currently illuminating a field within an Edit-Page.  It may be moved to another field via these cursor keys.

(ENTER) TO EDIT

The cursor is currently illuminating a field within Edit-Page.  This particular field can be entered with numbers or letters.  Press ENTER to enter the editing mode.  The field remains illuminated, and a small blinking underline cursor will appear under the leftmost editable character in that field.  In general, Left-Arrow, Right-Arrow, Space, Backspace, and AlphaNumerics are accepted in edit mode.  ESC will restore the field to its initial state and the blinking underline will disappear.  ENTER will finalize the editing mode and the blinking underline will disappear.  All "edit-mode" keystrokes are prompted.

+-SCROLLS CHOICES

SPACEBAR +- TO CHANGE

SPACEBAR +- SCROLLS CHOICES

The cursor is currently illuminating a field within the Edit-Page which may be changed.  Press SpaceBar or "+" to change to other available options.  The options are rolled through a list in the forward direction.  BackSpace and "-" roll the options in reverse order.

ESOTERIC PROMPTS

A few special-case prompts also exist.  Generally, they specify a range of numbers or a particular set of AlphaNumeric characters that will be accepted in the field.  For example, the CLOCK Time-Of-Day subfield accepts Alphabetic "a" and "p" to indicate am and pm.

The SECURITY utility requires pressing ENTER after selecting a new configuration.  This additional step is not consistent with behaviour of the other utilities, but is necessary so that a new password can be prompted when appropriate, and so the current password is not dismissed if the user simply scroll through available options.

The LOW-LEVEL-FORMAT field column within the FIXED disk configuration utility cannot be accessed until CTRL-F is pressed.  Pressing ESC while the cursor is in that column will move it to a non-Format column on the screen.  While the format is in progress, ESC will immediately terminate the process.

SETUP SECTION

There are currently 13 sections contained in BIOS Setup.  As the cursor is moved across the Menu Line, Page for the respective Setup Section appears in the Edit.  A quick summary of the utilities is given below.

(1)	SUMMARY UTILITY PAGE

In the Summary Section, the basic configurations and characteristic of your system can be viewed here.  Each of the fields are explained below with an example of how they might appear:

CPU

The type and revision number of the CPU is shown.

| CPU 80386DX | indicates a 386DX processor chip |
| --- | --- |

MATH UNIT

Numeric-Coprocessor-Extension type found in the system.

| NPX | Description |
| --- | --- |
| N/A | no coprocessor |

SHADOW RAM

Indicates if any part of the (384K) ROM space is mapped to shadow RAM.

For example, Shadow RAM ..... n/a

Shadow RAM ..... Enable

Shadow RAM ..... Disable

MEMORY-BASE

Indicates the amount of base memory (below 1 Megabyte boundary).  Possible range is 64K to 640K.

MEMORY-EXTENDED

Indicates the amount of extended memory (above 1 Megabyte boundary).  Possible range is 0K to 15360K.

For example, Memory-Extended ...... 1024K

MEMORY-SYSTEM

Indicates the amount of special OEM memory found to be in working order.  Typically, this field will be un-implemented (0K), or will represent the 384K available for shadow RAM or relocation to the Extended Memory pool.

For example, Memory-System ..... 384K

MEMORY-TOTAL

This is simply a sum of the three preceding quantities.

For example, Memory-Total ..... 2048K

COM1, COM2, COM3, COM4

Indicates if RS232 serial port COM1 is present, and its I/O address.

For example, COM1 ..... n/a

COM1 ..... 3F8

COM1 ..... 2F8

LPT1, LPT2, LPT3 and LPT4

Indicates if parallel (printer) port LPT1 is present, and its I/O address.

For example, LPT1 ..... n/a

LPT1 ..... 3BC

LPT1 ..... 378

LPT1 ..... 278

FLOPPY 0, FLOPPY 1, FLOPPY 2 AND FLOPPY 3

Indicates floppy drive type and step rate.  The "type" can be 5¼ 360K, 5¼ 1.2M, 3½ 720K, 3½ 1.4M AND 3¼ 2.8M.  The "step rate" is given as two options : fast or slow required for the read/write head to be moved to an adjacent cylinder.

| Floppy 0 | Description |
| --- | --- |
| None | Card present but no drive |
| 5¼ 1.2M | 5¼ inch |

FIXED 80 (C:), FIXED 81 (D:)

Indicates fixed disk type, step rate encoding and if "Translation Mode" is in effect.  If the "Translation Mode" is enabled for this disk a letter "T" will be shown.  the step rate will be shown if set to non-zero.

| Fixed 80 (C:) | Description |
| --- | --- |
| N/A | Fixed controller card not present |
| None | Drive type "0", no drive present |
| 2 | Drive type "2", no special features |
| 47T | Drive type "47T", Translation Mode |
| 8 {F} | Drive type "8", special Step rate |
| 46T {E} | Drive type "46", Translate+Step rate |

BOOT SEQUENCE

Specifies the selected order in which the disk will be booted.

| Boot-Sequence | Description |
| --- | --- |
| A: 1st | Try A:first, if failure, try C: |
| C: 1st | Try C:first, if failure, try A: |
| Prompt | Screen prompt for drive A: - D: |

ANTI-VIRUS

The Anti-Virus is provided in two options:

enable

disable

SECURITY

The state of password-Security is shown in this field.

For example, Security ..... Enable

Security ..... Disable

KEYBOARD

The keyboard type is shown in this field.

Eg, Keyboard ... AT  PC/AT type keyboard

Keyboard ...	XT  PC/XT type keyboard

NUMLOCK

The programmable powerup NumLock state is shown in this field.

Note: This is only meaningful for AT-type keyboards.

For example, NumLock ..... Off

NumLock ..... On

TYPEMATIC

The programmable powerup "typematic" repeat rate is shown here.

Note: This is only meaningful for AT-type keyboards.

| Typematic | Description |
| --- | --- |
| Default | No rate programmed, speed approx 10 cps |
| 30.0 | 30 cps rate is programmed |

VIDEO-PRIMARY

Indicates the video adaptor which will be in use when the system boots.

| Video-Primary | Description |
| --- | --- |
| None | Special Support, see VIDEO menu |
| Monochrome | B/W card |
| CGA - Snow | CGA, slow access due to "snow" |
| CGA - Fast | CGA, "snow" isn't problem |
| V/EGA-Mono | Advanced Graphics, B/W monitor |
| V/EGA-Color | Advanced Graphics, Color monitor |

VIDEO-SECONDARY

Indicates if a second video card is present in the system.  This will be displayed:

Video-Secondary ..... n/a

Otherwise, refer to VIDEO-PRIMARY above.

(2)	CLOCK CONFIGURATION

The battery backed Real-Time-Clock (RTC) time, date, and daylight savings feature are programmed through this utility.

TIME

The time field is shown in 12 hour format, followed by a time-of-day indicator "a" or "p" (am/pm).  Enter the time directly.  After entering the second, the cursor moves the right most of the field, press `a' for am or`p' for pm.

| Time hh/mm/ss t | Military |
| --- | --- |
| 12:00:00 a | 00:00:00 |
| 09:10:11 p | 21:10:11 |

DATE

The date field is shown in mm/dd/yyyy format.

| Date (mm/dd/yyyy) |  |
| --- | --- |
| 01/23/1990 | January 23, 1990 |

DAYLIGHT SAVINGS

The RTC has a built-in capability to automatically adjust the time on the two daylight savings days of the year.  If enable, on the last Sunday in April, the time increments from 1:59:59 am to 3:00:00 am.  On the last Sunday in October, when the time reaches 1:59:59 am, it is rolled-back to 1:00:00 am.  This feature is only useful in those countries using this scheme, such as United States.  Otherwise, set the field to "Disable".

For example, Daylight Savings ..... Enable

Daylight Savings ..... Disable

(3)	VIDEO CONFIGURATION

The primary video adaptor is declared through this utility.

A "primary" adaptor is defined to be the video card which will be recognized by the operation system.  If there are two video cards present, the other one becomes the "secondary" video adaptor.  The secondary adaptor is placed into a standby state and can only be activated by specialized software.

No jumper setting is required and the BIOS will determine the type of display found in the computer.  In systems with a single video card, the choices are limited to that present card, or "none".  Setting to "none" is useful in certain specialized monitoring/control applications.

When two video cards are present in the system, one must be color, and the other monochrome.  The BIOS will identify these cards, and make both choices available for primary selection.  If one of the cards is V/EGA, the BIOS will automatically detect whether the display card is in color or monochrome mode.  Although V/EGA cards generally require setting dipswitches, the BIOS will override those settings according to the primary adaptor selected via this utility.  Note that some valid dipswitch state must still be set on the V/EGA card so that it may correctly initialize itself.

If a CGA card is found, a second field will appear on the screen.  Its purpose is to allow selection of video access speed.  Some CGA cards will produce screen "snow" when accessed too quickly in 80-column text mode.  Such cards require synchronization with video signals to prevent this undesirable effect.  Most CGA cards do not require this synchronization, and the video access speed will be improved.  Try selecting the fast mode.  If there is no "snow" problem, select fast mode permanently.

(4)	FLOPPY DISK CONFIGURATION

This utility setups the floppy drive subsystem, drives A:, B:, C: and D:.

The BIOS supports the following types of floppy disk drives:

a.   5.25 inch, 360K bytes drive;

b.   5.25 inch, 1.2M bytes drive;

c.   3.5 inch, 720k bytes drive;

d.   3.5 inch, 1.4M bytes drive;

e.   3.5 inch, 2.8M bytes drive;

The "step-rate" parameter controls the speed which the drive head moves from track to track.  Usually the fast rate is selected. In case that your disk drive cannot work with fast rate or your application depends on the timing of the drive, select the slower speed.

(5)	FIXED DISK CONFIGURATION / LOW LEVEL FORMAT

This utility configures the fixed disk subsystem, drives C: and D:

The type of fixed disks may be selected from standard fixed disk table or defined as user-defined type 46 and 47. The drive parameter tables are comprised of these entries:

(1)   Number of Cylinders

(2)   Number of Heads

(3)   Starting Precompensation Cylinder

(4)   Landing Zone Cylinder

(5)   Number of Sectors per Track

Make sure you choose the correct type.  Otherwise, the system will have problem with the fixed disk.  If the type of your fixed disk is not included in the standard table, define it in type 46 and 47. Move the cursor to the "Type" field and select 46 or 47.  Then move the cursor down to the next field, "Cylinders", and press ENTER.  Enter the number of cylinders, and press ENTER when done.  Continue in this fashion, editing the "Heads", "Precomp", "Landing", and "Sector" fields.

Larger capacity fixed disk drives with higher track density may have more  than 1024 cylinders, but the standard BIOS only supports upto 1024 tracks.  There is a translation mode to surpass the 1024 cylinders limitation.  up to 16384 (16K) cylinders can be addressed via the translation mode.  The Translation Mode implementation is designed to be compatible both with programs which exclusively use the BIOS interface, and programs which interpret the fixed disk tables and run the drives directly.  If Translate Mode is not selected, only the first 1024 cylinders will be accessible through the BIOS interface.

To support the translate mode, a Low-level-Format utility is supported by the BIOS.  Press CTRL-F to enter Low-level Format utility.  Enter the range of cylinders to be formatted. The sequence may be increasing or decreasing order.  If the final cylinder is greater than the start cylinder, it is in increasing order.  Otherwise, it is in deceasing order.  Also, the "interleave" is programable.  The default setting is "3".  But most of the fixed disks and controllers nowadays, such as IDE hard disk, support interleave factor of "1".  Consult the manual of hard disk and hard disk controller for detail.

Many advanced fixed disks are already formatted in the factory and low level format should not be done.  Consult your dealer for detail.

Anti-Virus is provided as a user selectable option.  It offers a measure of protection against malicious programs by trapping writes to the main boot sector.  It also traps attempts to low-level-format any region of the fixed disk.

(6)	BOOT SEQUENCE CONFIGURATION

BOOT SEQUENCE

In a typical BIOS implementation, whenever CTRL ALT DEL is pressed or cold-boot, an attempt to boot from drive A: always occurs first.  If the drive is found to be empty, drive C: is booted.  However, hard disk is always used as the boot device in the computer system, and drive A: is only used to transfer files.  So booting directly from drive C: is more convenient and takes less time.

The boot sequence is now selectable.  In addition to the usual "A: first, then C:" sequence, the order may be reversed so that drive C: is accessed first.

To override the boot sequence established in this utility, a special "hot-key" warm-boot sequence CTRL ALT ENTER is available.  The effect is identical to CTRL ALT DEL, except a prompt appears on the screen asking which drive should be booted. For cold-boot, press ENTER during the memory test.  A prompt will be shown to ask which drive is the boot device.

When "Screen-Prompt" is selected, the BIOS will always ask you to choose which drive to boot.

MEMORY PRIMING

Memory test will be carried out after cold-boot.  When there is a lot of memory installed, the test will take some time.  To speed up the test, you may change from  "Full test" to "Quick scan".  The BIOS will only perform a simple memory test which will just need a few seconds.

COLD-BOOT DELAY

Since power-up diagnostics only take a few second, some hard disks may not even complete their setup process and can not be accessed by the BIOS.  In this case, the BIOS has to wait before the hard disk is ready.  Otherwise, hard disk error will be reported.  Maximum cold-boot delay is 30 seconds. Select a proper time delay to suit your hard disk. Set to "none" for no delay.  `None' or a short delay is appropriate for most IDE hard disks.

(7)	KEYBOARD CONFIGURATION

NUMLOCK

In 84-key AT-style keyboards, it lacks a dedicated cursor-control keypad.  The NumLock control key is used on these keyboards to toggle the Numeric Keypad from numeric operation to cursor functions.  In the original AT system, Numlock is always on after booting and thus the keypad is in numeric mode.  You need to press the Numlock key once so as to use the cursor keys. 101/2 key keyboards have a dedicated Cursor Keypad in addition to the Numeric Keypad.  The NumLock key is not necessary.

Through this utility, the boot-time default state of NumLock can be set.  Select "Disable" or "Enable" accordingly.

TYPEMATIC RATE

When a key is pressed on the keyboard for a period of time, the keystroke will begin repeating at a predefined rate.  The delay is by default 0.5 seconds and the repeat rate is 10 characters per second.  This typematic repeat rate feature is a function of the keyboard and is not produced by the system BIOS.  However, most of AT-style keyboards permit overriding the initial delay and subsequent repeat rate.

The BIOS can be configured to issue override typematic parameters to the keyboard at boot-time.  Both the "Delay before repeat" and "typematic Repeat Rate" parameters can be selected.  To accomplish this, "Enable" the "Typematic Override" field, and select the Delay and Rate in the other fields.  We suggest a Delay of 0.5 seconds, and a Repeat Rate of 30.0 cps.

If the keyboard fails to function properly when overriding the default typematic state, disable the "Typematic Override" field.  The Delay and Rate fields will display "Default" in response to this selection.  In this way, no typematic parameters will be issued to the keyboard.

(8)	FIRST AID

The setup utility "First Aid" provides some measures to solve many problems in AT design.

The first one is the keyboard problem with Novell environment in high speed system.  keyboard may not be respond very well because Novell program is running at very fast speed but the keyboard is a slow speed device.  The BIOS allows you to adjust a parameter to make the keyboard work properly.  You need to test your selection.  Set to the lowest value that can solve the problem.

The A20-GATE option is aimed at solving the problem with some applications using protected mode.  The A20-GATE signal is generated from slow speed device.  Sometimes the application crashes when toggling this signal.  Setting this signal to "ON" can eliminate the problem.

(9)	SHADOW RAM CONFIGURATION

In this section, the BIOS allows you to select "shadow RAM" function for the address in 640K to 1M region. The BIOS will detect which address is occupied by adapter ROM and video ROM. The address range of C0000H to CFFFFH, which is usually used by VGA and EGA BIOS, is divided into four 16K blocks.  The memory space from D0000H to DEEEEH is also divided into four 16K blocks.  The last memory block is E0000H to EFFFFH.  The BIOS ROM  is resident at F0000H and is always shadowed.

When no memory block is selected to be shadowed in the range from D0000H to DFFFFH and E0000H to EFFFFH, these memory blocks may be mapped to the top of memory and served as extended memory.  In this case, the memory block originally locates at A0000H to BFFFFH where is already occupied by display memory will also be remapped to the top of memory.  There is additional 256KB available in the extended memory.

Each adapter ROM and video ROM will be assigned a number. If a ROM occupies more than one block, the BIOS will show the same ROM number for each block.

To set a ROM to be shadowed, move the cursor to a particular block and change to the "WP-Shadow".  If the ROM occupies more than one block, just set one block to be shadowed and then other blocks will be set accordingly. It avoids the mistake that you just enable the shadow RAM function for part of a ROM.

The address range that is not used may be available to software by setting to "RW-Shadow". This feature is useful to many memory management utilities and advanced operation systems. These utilities can move the memory resident files and device drivers to these address ranges and hence there are more spaces available in the 640KB range.  If a particular memory block in D0000H to DFFFFH and E0000H to EFFFFH are set to "RW-Shadow", 256KB remapped function is not enabled.

(10)	DMA PARAMETER

DMA timings may be adjusted to adapt to your EtherNet, SCSI and other Bus-Master peripherals, to achieve optimal performance.  However care should be taken when setting these parameters so that it won't violate the specification.  The default values are recommended for reliable operation.  If you have any doubt, ask your dealer.

DMA CLOCK

It is initially set to ATCLK/2.  If set to ATCLK/1, the DMA operation is running at 8MHz.

8-BIT WAITS / 16-BIT WAITS

This option controls the CPU access to the peripherals controller which handles DMA operations and hardware interrupts.  The default setting is 1WS.

COMMAND WIDTH

When "Compress" is selected, all DMA operations will take less cycles and hence the data transfer rate is better.  Only few peripherals can work with compressed command width.  Make sure the system is reliable with this option.  "Normal" is appropriate for normal range.

MEMR# SIGNAL / MEMW# SIGNAL

Setting to the option "Normal" is compatible to standard AT design. The option "Early" allows the faster generation of control signal MEMR# and MEMW#. Since it is reliable to use the option "Normal", it is rarely necessary to choose the other one.

(11)	CHIPSET

This section provides a means to define some parameters for the system, such as memory wait state and the AT-bus speed.  The default settings are proper for normal usage. Changing the setting may lead to better performance, but make sure your system is still reliable and compatible with the peripherals.

DRAM TIMING - READ/WRITE STATE

The number of wait state for read and write operation depends on the clock speed of CPU and the speed rating of the DRAM.  Improper setting can make the system unstable.  Since the specification of DRAM for different manufacturers may vary, you would better consult your local dealer for the detailed information.  In general, select 0 wait state Write and 0 wait state Read with 80ns DRAM for 25MHz and 33MHz system.

EXTRA WAIT STATE

In AT Bus standard, it requires 1 wait state for 16 bit bus operation and 4 wait state for 8 bit bus operation.  For the slower I/O device, it will require one more wait state.

AT BUS CLOCK

Bus clock is used by pripherals on the motherboard and slot, such display and DMA Bus clock is generated from CPU clock-in and the speed of Bus clock is shown below.

| CPU Speed (MHz) |  |  |
| --- | --- | --- |
| AT CLK | 33 | 25 |
| CLKIN/4 | * 8.4 | 6.3 |
| CLKIN/3 | 11.1 | * 8.3 |
| CLKIN/2.5 | 12.5 | 10.0 |
| CLKIN/2 | 16.7 | 12.5 |

The system performance can be improved by selecting a higher Bus clock speed.  To be compatible with general add-on cards, the bus clock must be 8.4MHz or less.  There are many old version add-on cards that can only run at the slow speed.  So, be careful when you want to set to higher speed.

(13)	SECURITY CONFIGURATION

This utility is used to enable or disable Password Security.  In order to use this feature, place a jumper on JP2.

This Security feature offers a measure of protection against unauthorized use of the computer, by requiring a password when the computer is first being powered up.  Three opportunities are given to enter the correct password.  If three unsuccessful attempts are made, the system will be halted and an alarm will be sounded.  The alarm will persist until the power is turned off.  After the correct password is entered, the computer will proceed to boot-up.  Security feature is not applied to warm-boot, pressing CTRL-ALT-DEL.

The Setup-Utility is also password protected when Security feature is enabled.  When entering the BIOS setup via CTRL ALT ESC, the SUMMARY page will be displayed as usual, but the user will be prompted to press "F10 to Exit", or "ENTER for Security Clearance".

When you select to enable the Security feature, a second field will appear on the screen for you to enter a password.  A password consisting of zero to ten characters must be typed in.  Asterisks are echoed to the screen, instead of the character typed, and the only keystroke available for editing is BackSpace.  The password cannot be viewed.  Press ENTER when you have entered the password.  Then a prompt will appear requiring the same password to be entered again.  The BIOS will compare the entries and the process will repeat if the latter entry does not match the initial one.

All keystrokes recognized by BIOS can be used in the password.  So, function keys such as F1 and combinations of keys, such as ALT F1 and SHIFT F1, are valid and considered different.  Also Alphabetic characters are case-sensitive, which means "a" and "A" are different.

Once the password has been defined, it may subsequently be changed using this utility.  To accomplish this, toggle the "Security" field entry from "Enable" to "Change Code", and press ENTER.  The procedure is identical to initially installing the password.

The password entry feature may also be set to "Disable" and the Security feature is disabled.  When disabled, anyone can invoke the BIOS Setup and enable the Security feature with their own password.  Afterward, the system can only boot up with his password.  JP2 is served as a Master-Override Password Security Jumper.  When there is no jumper on JP2, the security function is disabled and no one can enable it in the Setup.  When you need the security function, place a jumper on JP2 and enter your password.

Appendix B

Operation and Maintenance

_______________________________

STATIC ELECTRICITY

When installing or removing any add-on card, DRAM module or coprocessor, you should discharge the static electricity on your body.  Static electricity is dangerous to electronic device and can build-up on your body.  When you touch the add-on card or motherboard, it is likely to damage the device.  To discharge the static electricity, touch the metal of your computer.  When handling the add-on card, don't contact the components on the cards or their "golden finger".  Hold the cards by their edges.

KEEPING THE SYSTEM COOL

The motherboard contains many high-speed components and they will generate heat during operation.  Other add-on cards and hard disk drive can also produce a lot of heat.  The temperature inside the computer system may be very high.  In order to keep the system running stably, the temperature must be kept at a low level.  A easy way to do this is to keep the cool air circulating inside the case.  The power supply contains a fan to blow air out of the case.  If you find that the temperature is still very high, it would be better to install another fan inside the case.  Using a larger case is recommended if there are a number of add-on cards and disk drives in the system.

CLEANING THE "GOLDEN FINGER"

Whenever inserting an add-on card to the motherboard, make sure that there is no dirt on the "golden finger" of the add-on card.  If not, the contact between the "golden finger" and the slot may be poor and thus the add-on card may not work properly.  Use a pencil eraser to clean the "golden finger" if dirt is found.

CLEANING THE MOTHERBOARD

The computer system should be kept clean.  Dust and dirt is harmful to electronic devices.  To prevent dust from accumulating on the mother-board, installing all mounting plates on the rear of the case.  Regularly examine your system, and if necessary, vacuum the interior of the system with a miniature vacuum.

Appendix C

Troubleshooting

_______________________________

MAIN MEMORY ERROR

After power up, the monitor remains blank, and there are beep sounds indicating a main memory failure.  In this case, turn off the power and remove all SIMM modules.  Carefully place the modules back to the sockets and make sure that all the modules are locked by the locking latches firmly.

In some other cases, the total memory found by the BIOS is different from the actual amount of memory on board. ( Note that 128K bytes memory is reserved for the shadow RAM function and will not be counted by the BIOS).  It is also a memory failure and you can follow the instruction above.

IMPROPER SETTING OF WAIT STATE

If the system hangs after memory test, another possible cause is the improper setting of the wait state for memory operation.  The number of wait state must match the speed of the DRAM.  Reset the CMOS RAM and set up the wait state.  Try to increase the number of wait state.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Appendix D

System Board Layout

_______________________________