Supplement

for

CONDOR 386

(AMI BIOS Version)

The system BIOS provides an interface for operating systems and applications to access hardware.  It is fully compatible with standard AT BIOS and works in the network system. It also performs self-test after reset and includes a setup program to setup the system.

SELF-TEST

To ensure the computer hardware is functional, the system BIOS will carry out a self-test upon reset.  The test is very intensive and covers all parts of hardware.  It takes a while before some messages are shown on the screen.  It does not mean that the system is not working when the screen is blank.  So wait for a while after turning on the power and listen carefully to the speaker.  Some errors are reported by a number of beep sounds.   After completing the self-test, the BIOS will display some messages on the screen.

In  case of serious errors, the BIOS will suspend the test.  If the display is not initialized, the BIOS will report the error through a sequence of beep sounds.  Otherwise, error message will be shown on the screen.

These fatal errors are usually communicated through a series of audible beeps.  The numbers on the fatal error list below correspond to the number of beeps for the corresponding error.  All errors listed, with the exception of #8, are fatal errors.

| No. of Beeps | Error Message |
| --- | --- |
| 1 | DRAM Refresh Failure |
| 2 | Base 64KB Memory Parity Error |
| 3 | Base 64KB Memory Failure |
| 4 | System Time Failure |
| 5 | Processor Error |
| 6 | Keyboard Controller Gate A20 Failure |
| 7 | Processor Exception Interrupt Error |
| 8 | Display Memory Read / Write Error (Video Adapter) |
| 9 | ROM Checksum Error |
| 10 | CMOS Shutdown Register Read/Write Error |

If no error is found during self-test, the system BIOS will proceed to boot from floppy disk or hard disk.  The system BIOS will list the system configuration on the screen shown in next page.

System Configuration (C) Copyright 1985-1990, American Megatrends Inc.,

Main Processor	:	80386		Base Memory Size	:	640 KB

Numeric Processor	:	None		Ext. Memory Size	:	7424 KB

Floppy Drive A:	:	1.2 MB, 5¼"		Hard Disk C: Type	:	2

Floppy Drive B:	:	1.44MB, 3½" 	Hard Disk D: Type	:	None

Display Type	:	VGA or EGA	Serial Port(s)	:	None

ROM-BIOS Date	:	04/30/90		Parallel Port(s)	:	3BC

╔════════════════════════════════════════════╗

║                                            ║

╟───────────────────────┬────────────────────╢

║                       │                    ║

║                       │                    ║

║                       │                    ║

║                       │                    ║

║                       │                    ║

║                       │                    ║

╚═══════════════════════╧════════════════════╝

Do check the list to make sure that the configuration is correct.  Sometimes, problems arise because of the incorrect information of the configuration.  For example, if you forget to modify the setup after changing the floppy disk drive from one type to another, it can not boot from floppy disk or may not work properly.  If you check the list, you can find the cause of the problem.

SYSTEM SETUP

The BIOS incorporates FIVE setup sections:

(1)	Standard CMOS Setup

(2)	Advanced CMOS Setup

(3)	Advanced Chipset Setup

(4)	Auto Configuration with BIOS Defaults

(5)	Hard Disk Utilities

It is important that all the setup procedures should be completed before operating the system.  Otherwise, the system will not run properly with the incorrect setup information.  Run the setup again if the configuration is changed.

To enter the setup section, press 'DEL' when the following message is shown during memory test :

Hit <DEL> if you want to run SETUP

Whenever the system BIOS finds that the configuration of the system is altered, error message will be shown and you may press 'F1' to run setup.  Then the following messages are shown on the screen.

BIOS SETUP PROGRAM - AMI BIOS SETUP UTILITIES

<C> 1990 American Megatrends, Inc. All Rights Reserved

STANDARD CMOS SETUP

ADVANCED CMOS SETUP

ADVANCED CHIPSET SETUP AUTO CONFIGURATION WITH BIOS DEFAULTS

HARD DISK UTILITY

WRITE TO CMOS AND EXIT

DO NOT WRITE TO CMOS AND EXIT

Configure system with Power On Default Values for Chipset and Advanced CMOS

( 1 )			CMOS SETUP

The memory size and the numeric processor are detected by the BIOS.  So you are only required to set those options on the left side of the screen.  The system configuration information are shown as follows:

CMOS SETUP (C) Copyright 1985-1990, American Megatrends Inc.,

Date (mn/date/year)	:	Sun, Jul 01, 1990		Base memory size	:	640 KB

Time (hour/min/sec)	:	12  :  05  :  30		Ext. memory size	:	7424 KB

Daylight Saving	:	Disabled			Cyln	Head	WPcom	LZone	Sect	Size

Hard Disk C: type	: 	Not Installed

Hard Disk D: type	:	Not Installed

Floppy Drive A:	:	1.2 MB, 5¼

Floppy Drive B:	:	1.44 MB, 3½

Primary Display	:	VGA or EGA		Sun	Mon	Tue	Wed	Thu	Fri	Sat

Keyboard	:	Installed

1	 2	 3	 4	 5	 6	 7

8	 9	10	11	12	13	14

15	16	17	18	19	20	21

Month	:	Jan, Feb, ..........	Dec	22	23	24	25	26	27	28

Date	:	01, 02, 03, ........	31

Year	:	1901, 1902, .......	2099	29	30	31	 1	 2	 3	 4

ESC=Exit,↓→↑←=Select,PgUp/PgDn=Modify      5	 6	 7	 8	 9	10	11

╔═════════════════════════════════════════════╗

║                                             ║

╟─────────────────────────────────────────────╢

║                                             ║

║                                             ║

║                                             ║

║                                             ║

║                                             ║

║                                             ║

║                        ┌──┬──┬──┬──┬──┬──┬──╢

║                        │  │  │  │  │  │  │  ║

║                        ├──┼──┼──┼──┼──┼──┼──╢

║                        │  │  │  │  │  │  │  ║

║                        ├──┼──┼──┼──┼──┼──┼──╢

║                        │  │  │  │  │  │  │  ║

║                        ├──┼──┼──┼──┼──┼──┼──╢

║                        │  │  │  │  │  │  │  ║

║                        ├──┼──┼──┼──┼──┼──┼──╢

║                        │  │  │  │  │  │  │  ║

║                        ├──┼──┼──┼──┼──┼──┼──╢

║                        │  │  │  │  │  │  │  ║

╟────────────────────────┼──┼──┼──┼──┼──┼──┼──╢

║                        │  │  │  │  │  │  │  ║

╚════════════════════════╧══╧══╧══╧══╧══╧══╧══╝

OPTION 1	TIME AND DATE

Use PgUp and PgDn keys to change the value.  The date and time cannot be entered directly.  An calender is displayed on the lower right corner of the screen for your reference.

OPTION 2	DAYLIGHT SAVING

DISABLED/ENABLED

OPTION 3	FIXED DISK DRIVE

There are 47 types of fixed disks supported by the BIOS.  Consult your fixed disk manual to determine its correct type.  The parameters such as cylinder number, head number, sector number and pre-compensation must match your fixed disk's parameters.

Use PgUp and PgDn keys to change the fixed disk type.  If the type of your fixed disk is not included in the hard disk list, define a new type as type 47.  Use left and right arrow keys to move between the parameter fields and enter the parameters.  The parameters will be stored in the CMOS RAM and your fixed disk can be used afterwards.  Each hard disk can be assigned a different type 47 hard disk.  So two hard disks which are not included in the list can be used together in your system.

If the type of fixed disk is wrong, it takes a while before the BIOS can identify the error.  After setting the fixed disk type, if the system halts after reboot, please wait for a while.  It is most likely that the setting of fixed disk type is incorrect.

When you install a new hard disk, make sure whether it is already formatted.  If not, the BIOS has to check for a while before reporting the hard disk error.  In fact, the error arises only because the hard disk is not formatted.  If the hard disk is formatted, you can run DOS FDISK and DOS FORMAT.

Some fixed disks are specially handled and must be set to 'Not Installed'.  Consult the fixed disk manual for details.

OPTION 4	FLOPPY DISK DRIVE

Four types of floppy disk drives are supported:

1.	5-¼ inch standard drive (360K)

2.	5-¼ inch high-density drive (1.2M)

3.	3-½ inch standard drive (720K)

4.	3-½ inch high-density drive (1.44M)

The system BIOS supports two floppy disk drives and they are recognized as drive A and B.  Select the correct types.  Otherwise the drives cannot work properly.  If one of them is not installed, select 'Not Installed' for that drive.

The BIOS is able to detect the type of the drives automatically.  But remember to check the settings before exit.

OPTION 5	DISPLAY

Four types of display are supported:

1.	CGA 80 column mode

2.	CGA 40 column mode

3.	EGA and VGA

4.	Monochrome

If the type of display is incorrect, the BIOS will prompt you and ask you to set up again.  But the BIOS is still able to display messages on the display attached to the system.  Thus you can enter the setup program.

The jumper JP8 must be set according to this setting.  Otherwise, the BIOS will report error after self-test.

OPTION 6	KEYBOARD

If a keyboard is attached to the system, select 'Installed'.  The BIOS will test the keyboard during self-test.

( 2 )			ADVANCED CMOS SETUP

All the registers of the chipsets are set to default values by the system BIOS.  Usually, there is no need to modify these registers unless the configuration is changed.  Since improper settings of these registers may cause the system malfunction, check your settings carefully before exit.

In ADVANCED CMOS SETUP, the main menu is shown as below:

BIOS SETUP PROGRAM - ADVANCED CMOS SETUP

<C> 1990 American Megatrends Inc., All Rights Reserved

Typematic Rate Programming		:	Disabled	Adapter ROM Shadow D400,16K	:  Disabled

Typematic Rate Delay (Msec)		:	250		Adapter ROM Shadow D800,16K	:  Disabled

Typematic Rate (Chars/Sec)		:	10.0		Adapter ROM Shadow DC00,16K	:  Disabled

Above 1MB Memory Test		:	Disabled	Adapter ROM Shadow E000,16K	:  Disabled

Memory Test Tick Sound		:	Enabled	Adapter ROM Shadow E400,16K	:  Disabled

Memory Parity Error Check		:	Enabled	Adapter ROM Shadow E800,16K	:  Disabled

Hit <DEL> Message Display		:	Enabled	Adapter ROM Shadow EC00,16K		:	  Enabled

Hard Disk Type 47 RAM Area		:	0:300	System ROM Shadow F000,16K		:  Enabled

Wait for <F1> if Any Error		:	Enabled

Numeric Processor			:	Disabled

Weitek Processor				:	Absent

Floppy Drive Seek At Boot		:	Enabled

System Boot Up Sequence		:	A:, C:

Video Rom Shadow C000,16K		:	Disabled

Video ROM shadow C400,16K		:	Disabled

Adapter ROM Shadow C800,16K	:	Disabled

Adapter ROM Shadow CC00,16K	:	Disabled

Adapter ROM Shadow D000,16K	:	Disabled

╔═════════════════════════════════════════════════════╗

║                                                     ║

║                                                     ║

╠══════════════════════════╦══════════════════════════╣

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

╚══════════════════════════╩══════════════════════════╝

After changing the registers' settings, test your system first to make sure that the settings are correct.  It is possible that your system becomes unstable and you need to setup the registers again.

In this section, you simply use the up and down arrow keys to move between options and press PgUp/PgDn to scroll bit value.  After you finish the Setup, press `Esc' to return to main menu.  The BIOS will set the registers accordingly.

A short description follows for each of the options on the Advanced CMOS Setup Screen.  If any problem in some options, press the <F1> Help Key.

Typematic Rate Programming :

By enabling this option, the user can adjust the rate at which a keystroke is repeated.  The options "Typematic Rate Delay" and "Typematic Rate" affect this rate.  When a key is pressed and held down, the character appears on the screen and after a delay set by the Typematic Rate Delay, it keeps on repeating at a rate set by the Typematic Rate Value.  When two or more keys are pressed and held down simultaneously, only the last key pressed will be repeated at the typematic rate.  This stops when the last key pressed is released, even if other keys are depressed.

Above 1MB Memory Test :

This feature, when enabled, will invoke the POST memory routines on the RAM above 1 MB (if present on the system).  If disabled, the BIOS will only check the first 1 MB of RAM.

Memory Test Tick Sound :

This option will enable (turn on) or disable (turn off) the "ticking" sound during the memory test.

Memory Parity Error Check :

If the system board does not have parity RAM, the user may disable the memory parity error checking routines in the BIOS.  The user should check with the manufacturer regarding the proper setting of this option.

Hit <DEL> Message Display :

Disabling this option, will prevent the message:

"Hit <DEL> if you want to run SETUP"

from appearing on the screen when the system boots-up.

Hard Disk Type 47 Data Area :

The AMI BIOS SETUP features two user-definable hard disk types.  Normally, the data for these disk types are stored at 0:300 in lower system RAM.  If a problem occurs with other software, this data can be located at the upper limit of the DOS shell (640 KB).  If the option is set to `DOS 1 KB,' the DOS Shell is shortened to 639 KB, the top KB is used for the hard disk storage.

Wait for <F1> if Any Error :

Before the system boots-up, the BIOS will execute the POST routines, a series of system diagnostic routines.  If any of these tests fail, but a non-fatal error has occurred and the system can still function, the BIOS will respond with an appropriate error message followed by the following statement :

"Press <F1> to continue."

If this option is disabled, any non-fatal error which occurs will not generate the above statement, but the BIOS will still display the appropriate error message.  This will eliminate the need for any user response to a non-fatal error condition message.

Numeric/Weitek Processor(s) :

These options allow the user to mark the numeric processor (Intel 80x87 or compatible) or the Weitek numeric processor (WTL3167 or 4167) as present or absent.

Floppy Drive Seek At Boot :

The default for this option is "Enabled".  If disabled, it allows a fast boot.

System Boot Up Sequence :

The AMI BIOS will normally attempt to boot from floppy drive A: (if present), and if unsuccessful, it will attempt to boot from hard disk C:.  This sequence can be switched using this option.  If the option is set to "C:, A:," the system will attempt to boot from the hard drive C:, and then A:.  If the option is set to "A:, C:," the sequence is reversed.

Video,Adapter and System ROM shadow :

There are two options of shadow RAM for video adapter, ten options of shadow RAM for add-on card and one for the system BIOS.  For the option System ROM, the content of the system at F000H segment BIOS is copied to the on board memory.  For the option Video ROM, the video ROM at C0000H segment or C4000H segment are copied to memory.  If you install an add-on card which ROM BIOS is locate at one of the Adapter ROM shadow options, you may select corresponding option to shadow this ROM.  If there is any problem after enabling the shadow memory on the add-on card, it recommends to enable the shadow RAM function for system BIOS only.

( 3 )			ADVANCE CHIPSET SETUP

In ADVANCED CHIPSET SET, the main screen shown as below:

BIOS SETUP PROGRAM - ADVANCED CHIPSET SETUP

<C> 1990 American Megatrends Inc., All Rights Reserved

DRAM Write Wait State		:	0W/S

DRAM Read Wait State		:	0W/S

Memory Remapping			:	Enabled

Master Mode Byte Dwap		:	Disabled

Singled ALE Enabled  		:	NO

Extra AT Cycle Wait State		:	0W/S

AT Bus Clock Selection		:	Clk 2IN/8

Video BIOS Area Cacheable		:	No

Non-Cacheable block -1 size		:	Disabled

Non-Cacheable block -1 base		:	0KB

Non-Cacheable block -2 size		:	Disabled

Non-Cacheable block -2 base		:	0KB

╔═════════════════════════════════════════════════════╗

║                                                     ║

║                                                     ║

╠══════════════════════════╦══════════════════════════╣

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

║                          ║                          ║

╚══════════════════════════╩══════════════════════════╝

DRAM Read/Write Wait state :

The number of wait state for memory read and write operations depends on the clock speed of CPU and the speed rating of the DRAM.

Improper setting can make the system unstable.  Since the specification of DRAM from different manufacturers may vary, you would better consult your local dealer for the detail information.  In general, select 0 Wait State Write and 0 Wait State Read with 80ns DRAM for 25MHz and 33MHz system.

Memory Remapping :

When 1MByte or more memory are installed, there are some portion of DRAM overlapping with the BIOS which cannot be accessed.  Memory remapping allows these portions of memory to be accessed at other locations and thus can be utilized by the software.  Memory remapping is supported by hardware.

Master Byte Mode Swap :

An add-on card with bus master capability should handle the data swapping on 8-bit operation correctly and this option need not be enabled normally.  However, some bus masters add-on cards is not fully compatible with the ISA bus standard.  By enabling this option, the system logic will instead perform the data swapping.

Single ALE Enable :

During bus conversion in AT bus cycle, such as I/O operation to add-on cards, the system logic will generate multiple ALE signal for add-on cards to latch the address bus status.  A few add-on cards may not work properly with multiple ALE signals.  In this case, select "YES" to solve the problem.  Should you have any question, please contact your local dealer.

Extra At Cycle Wait State :

For the standard AT bus, it requires 1ws for 16 bit AT cycle and 4ws for 8 bit AT cycle.  If you set it to "Enable", an extra wait state will be inserted to every operation to bus.  It is useful when you find that some slow add-on cards are unable to work on CONDOR-386.  Normally, select "Disable" for maximum performance.

At BUS Clock :

Bus clock is used by peripherals on the motherboard and slot, such display and DMA Bus clock is generated from CPU clock-in and the speed of Bus clock is shown below.

| CPU Speed (MHz) |  |  |
| --- | --- | --- |
| AT CLK | 33 | 25 |
| CLKIN/4 | * 8.4 | 6.3 |
| CLKIN/3 | 11.1 | * 8.3 |
| CLKIN/2.5 | 12.5 | 10.0 |
| CLKIN/2 | 16.7 | 12.5 |

The system performance can be improved by selecting a higher Bus clock speed.  To be compatible with general add-on cards, the bus clock must be 8.4MHz or less.  There are many old version add-on cards that can only run at the slow speed.  So, be careful when you want to set to higher speed.

Video BIOS Area Cacheable :

The video BIOS occupies the location 768K (C0000 hexadecimal) and the size is 32KB.  This option lets you set the address range to be non-cacheable easily without concerning about the base address and the size.  The default is "Disable".  If the Video BIOS area is cacheable, the performance of the video BIOS is better, but it is likely that the BIOS may be overwritten by other software.

Non-cacheable Block Size and Non-cacheable Block Base :

Two non-cacheable block, block-1 and block-2, can be defined.  You need to specify the size and base address. The block size may be 64KB, 128KB, 256KB and 512KB.  The base address is where the non-cacheable block will start in the main memory.  Press 'PgDn' and 'PgUp' keys to change the address value to an appropriate value.

( 4 )			AUTO CONFIGURATION WITH BIOS DEFAULTS

The Auto configuration with BIOS default is used for the user to setup the system in higher performance and operate in reliable setting.  Once the Auto Configuration is applied, the user need not to configure the ADVANCED CMOS SETUP.  The STANDARD CMOS SETUP will need to set those options after the Auto Configuration is used.

( 5 )			HARD DISK UTILITY

Hard Disk Diagnostics option is taken by pressing <Enter> at the Main Setup Menu, the screen is shown as below :

BIOS SETUP PROGRAM  -  HARD DISK UTILITY

<C> 1990 American Megatrends Inc,. All Rights Reserved

Cylin	Head	WPcom	LZone	Sect	Size (MB)

Hard Disk C:	Type :	47 USER TYPE	1314	   7	  1314	 1314	 17		 76

Hard Disk D:	Type :	Not Installed

Hard Disk Type can be changed from the STANDARD CMOS SETUP option in Main Menu

Hard Disk Format

Auto Interleave

Media Analysis

╔════════════════════════════════════════════════════╗

║                                                    ║

║                                                    ║

╠════════════════════════════════════════════════════╣

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

╠════════════════════════════════════════════════════╣

║                                                    ║

║                                                    ║

║                                                    ║

╚════════════════════════════════════════════════════╝

There are three options in the hard disk utility :

Hard Disk Format :

performs a "low level" format of the hard drive(s).  The user should check with the system or hard drive manufacturer to determine if this option should be taken.

Auto Interleave :

determines the optimum interleave factor prior to the format of the hard drive(s).

Media Analysis :

performs an analysis of each track of the hard drive to determine whether it is usable, the track is marked as "bad" so that data cannot be stored there in the future.

If you are installing a brand new hard disk (drive), the manufacturer of the hard drive usually provides a list of "bad tracks" with the hard drive.  Your system documentation might also include the optimum interleave factor.

In this case, assuming that you have a list of bad tracks and know the interleave factor, it will not be necessary to take the auto interleave and media analysis options.  Simply follow the instructions in the Hard Disk Format.  If you have a bad track list but have not been provided with the optimum interleave factor, follow the instructions in the Auto Interleave Section.

If you are installing a used hard disk or reformatting an existing hard disk, perform the Media Analysis and then follow the instructions in the Auto Interleave section.

BIOS SETUP PROGRAM  -  HARD DISK UTILITY

<C> 1990 American Megatrends Inc,. All Rights Reserved

Cylin	Head	WPcom	LZone	Sect	Size (MB)

Hard Disk C:	Type :	47 USER TYPE	1314	   7	  1314	 1314	 17		 76

Hard Disk D:	Type :	Not Installed

Hard Disk Format

Disk Drive (C/D)				?  C

Disk Drive Type				?  47

Interleave (1-16)				?  3

Mark Bad Tracks (Y/N)	?

Proceed (Y/N)					?

╔════════════════════════════════════════════════════╗

║                                                    ║

║                                                    ║

╠════════════════════════════════════════════════════╣

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║╔══════════════════╗                                ║

║║                  ║                                ║

║╟──────────────────╢                                ║

║║                  ║                                ║

║║                  ║                                ║

║║                  ║                                ║

║║                  ║                                ║

║║                  ║                                ║

║╚══════════════════╝                                ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

╚════════════════════════════════════════════════════╝

Option 1	Hard Disk Format Utility

Using the Hard Disk Format option to integrate a new hard disk to the system, or to reformat a used hard disk which has developed some bad patches as a result of aging or poor handling.  To find these bad patches o a used drive, you may select the Media Analysis option.

The value for Disk Drive is C for a C: Drive or D for a D: Drive.  If two disk drives have been previously entered at the Standard CMOS Setup Scree, then the ID (C/D) will appear to the right of the question mark following the Disk Drive field.  Choose which drive you wish to format by selecting the appropriate letter and pressing <ENTER>.  If only one drive was selected at the Standard CMOS Setup Screen, the cursor will automatically be placed at the interleave prompt.

The Disk Drive Type is read from the CMOS.  The interleave factor can be selected manually, or can be determined with the Auto Interleave feature of the SETUP program.

The manufacturer of the hard drive usually provides a list of "bad tracks" with the hard drive.  These tracks should be entered with this option, and they will then be marked as "bad" in order to prevent data from being stored there in the future.

The default for the Proceed prompt is <N> to prevent accidental formatting of the hard drive and subsequent loss of data.  Once this prompt is changed to <Y> and the <ENTER> key pressed, andy data residing on the hard disk will be irrevocably lost.

Option 2	Auto Interleave Utility

The Auto Interleave utility calculates the optimum interleave value through trial and error by measuring the transfer rate for four different interleave values.  To determine the best interleave factor, the system will format a portion of the hard disk for each transfer rate calculated.  The cylinders, heads and sectors formatted for each value will be displayed in the actively box on the screen.

BIOS SETUP PROGRAM  -  HARD DISK UTILITY

<C> 1990 American Megatrends Inc,. All Rights Reserved

Cylin	Head	WPcom	LZone	Sect	Size (MB)

Hard Disk C:	Type :	47 USER TYPE	1314	   7	  1314	 1314	 17		 76

Hard Disk D:	Type :	Not Installed

Auto Interleave										   Bad Track %

Disk Drive (C/D)				?  C						   Ms.     Cyln.    Head

Disk Drive Type				?  47

Mark Bad Tracks (Y/N)	?  N

Proceed (Y/N)					?

╔════════════════════════════════════════════════════╗

║                                                    ║

║                                                    ║

╠════════════════════════════════════════════════════╣

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║╔══════════════════╗                ╔════════════╗  ║

║║                  ║                ║            ║  ║

║╟──────────────────╢                ╟────────────╢  ║

║║                  ║                ║            ║  ║

║║                  ║                ║            ║  ║

║║                  ║                ║            ║  ║

║║                  ║                ║            ║  ║

║╚══════════════════╝                ║            ║  ║

║                                    ║            ║  ║

║                                    ║            ║  ║

║                                    ║            ║  ║

║                                    ║            ║  ║

║                                    ║            ║  ║

║                                    ╚════════════╝  ║

║                                                    ║

╚════════════════════════════════════════════════════╝

Option 3	Media Analysis Utility

The Media Analysis utility performs a series of tests to locate bad or damaged patches on the hard disk as a result of aging or poor handling.  This utility locates all bad tracks on the hard disk and lists them in the Bad Track List Box.  Since this test writes to all cylinders and heads on the hard disk to verify andy bad tacks, the test may require several minutes to complete.  For best results, run this test in its entirely.

BIOS SETUP PROGRAM  -  HARD DISK UTILITY

<C> 1990 American Megatrends Inc,. All Rights Reserved

Cylin	Head	WPcom	LZone	Sect	Size (MB)

Hard Disk C:	Type :	47 USER TYPE	1314	   7	  1314	 1314	 17		 76

Hard Disk D:	Type :	Not Installed

Media Analysis

Disk Drive (C/D)				?  C

Disk Drive Type				?  47

Proceed (Y/N)					?

╔════════════════════════════════════════════════════╗

║                                                    ║

║                                                    ║

╠════════════════════════════════════════════════════╣

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║╔══════════════════╗                                ║

║║                  ║                                ║

║╟──────────────────╢                                ║

║║                  ║                                ║

║║                  ║                                ║

║║                  ║                                ║

║╚══════════════════╝                                ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

║                                                    ║

╚════════════════════════════════════════════════════╝

THIS PAGE IS INTENTIONALLY LEFT BLANK