JAGUAR  IV

386

System Manual

The material in this manual is for information only and is subject to change without notice.

REVISION: 1.0

IBM, IBM PC/XT/AT, PC-DOS, MS-DOS, OS/2, INTEL, WEITEK, PHOENIX ARE THE TRADEMARKS OR REGISTERED TRADEMARKS OF THEIR RESPECTIVE OWNERS.

RADIO FREQUENCY INTERFERENCE STATEMENT

This equipment generates and uses radio frequency energy and if not installed and used properly, that is, in strict accordance with the manufacturer's instructions, may cause interference with radio and television reception.

If this equipment does cause interference to radio or TV reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient the receiving antenna.

*	Relocate the computer away from the receiver.

*	Move the computer away from the receiver.

*	Plug the power cord of computer into a different outlet so that computer and receiver are on different branch circuits.

*	Ensure that card slot covers are in place when no card is installed.

*	Ensure that card mounting screws, attachment connector screws, and ground wires are tightly secured.

*	If peripherals are used with this system, it is suggested to use shielded, grounded cables, with in-line filters if necessary.

If necessary, the user should consult the dealer service representative for additional suggestions.

The manufacturer is not responsible for any radio or TV interferences caused by unauthorized modifications to this equipment.  It is the responsibility of the user to correct such interferences.

Note

1.		Electronic components are sensitive to dust and dirt.  Do inspect and clean the computer system regularly.

2.		Turn off the power whenever you install or remove any connector, memory module and add-on card.  Before turning on the power, make sure that all the connectors, memory modules and add-on cards are secured.

3.		After power is on, please wait for a minute.  The system BIOS are going through a self-test during this period and nothing is shown on the screen.  After the self-test, the system BIOS will initialize the display adaptor and show messages.

4.		The SIMM sockets are fragile device.  Do not force the SIMM modules into the sockets.  It may break the locking latches.

Preface

The manual provides information about the installation and maintenance of OCTEK JAGUAR IV-386 motherboard.  In-depth explanations of the functions of this motherboard are provided.  In the appendix, the system BIOS setup is explained.

The content in this manual is only for reference and is intended to provide basic information for the general users.  There are also some technical information for hardware and software engineers.

In this manual, there are 4 chapters.  Chapter 1 contains a brief introduction and specification of OCTEK JAGUAR IV-386 motherboard.  In the Chapter 2, the functions of JAGUAR IV-386 are explained.  It also outlines many advanced features of the CPU and the system architecture.  Chapter 3 explains the installation of coprocessor, DRAM modules and jumpers.  Some technical information are provided in the Chapter 4.

System BIOS is described in the attached BIOS Manual.

Additional information are given in Appendix A and B for the maintenance purpose.

CONTENT

Chapter 1		INTRODUCTION			1-1

Chapter 2		GENERAL FEATURES		2-1

Specification				2-1

Processor					2-3

Math Coprocessor				2-6

Cache Memory				2-7

Memory System				2-9

Dual Bus Design				2-11

System Functions				2-12

Chapter 3		INSTALLING COMPONENTS	3-1

Installing Math Coprocessor		3-1

Installing RAM Modules		3-3

Installing External Battery		3-5

Configuration of Memory		3-6

DRAM Configuration			3-7

Configuration of Cache			3-8

Memory

Control of System Speed		3-10

Reset CMOS Setup			3-10

Information

System Board Jumper Setting		3-11

System Board Connectors		3-12

Chapter 4		TECHNICAL INFORMATION	4-1

Memory Mapping				4-1

I/O Address Map				4-2

System Timers				4-4

System Interrupts				4-6

Direct Memory Access (DMA)		4-7

Real Time Clock and CMOS		4-8

RAM

CMOS RAM Address Map		4-9

Real Time Clock Information		4-10

System Expansion Bus			4-11

APPENDIX A	OPERATION & MAINTENANCE	A-1

Static Electricity				A-1

Keeping the System Cool			A-1

Cleaning the "Golden Finger'		A-2

Cleaning the Motherboard		A-2

APPENDIX B	TROUBLESHOOTING			B-1

Poor Performance				B-1

Main Memory Error			B-1

Cache Memory Failure			B-2

Improper Setting of Wait State		B-2

APPENDIX C	SYSTEM BOARD LAYOUT		C-1

Chapter 1

Introduction

_______________________________

OCTEK JAGUAR IV-386 consists of 32-bit 80386 microprocessor, a large cache memory and highly integrated chipsets to provide high performance, reliability and compatibility.  OCTEK JAGUAR IV-386 is a perfect choice for CAD/CAM workstation and file server and supports sophisticated 32-bit computing applications and multi-user operating systems.

To speed up the performance of the system, a cache memory with maximum size up to 128KB is incorporated.  Frequently used program codes can be fetched by CPU from the high speed cache memory without wait state. Furthermore, access to the main memory is accelerated because the cache controller and the memory controller are integrated together, and operate concurrently.  Thus the overhead of accessing the main memory is minimized.

Aimed at supporting advanced CAD/CAM applications, OCTEK JAGUAR IV-386 supports 80387 or compatible.  The total memory on board is 32MB.

Compatibility and reliability are important issues.  I/O channel is compatible to standard AT bus.  Therefore any AT compatible peripherals may be used on OCTEK JAGUAR IV-386.  On board POWERGOOD generator is essential to ensure the reliability of the system and is well-designed to work with all power supplies.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 2

General Features

_______________________________

SPECIFICATION

Processor :

Intel / AMI 80386DX CPU

with optional 80387DX Math Co-processor

Speed :

Turbo/normal speed

Software/hardware selectable

I/O Slot :

Compatible to standard AT bus

One 8-bit and six 16-bit slots

Standard AT bus clock 8MHz

Cache Memory :

Support 32KB, 64KB and 128KB Cache Memory

2-way set associative cache controllor

Line Burst capability from DRAM to cache RAM

32-bit wide write buffers with byte gathering

Two Non-cacheable regions control

Option for Cacheable video BIOS

Memory :

Shadow RAM for System BIOS and Video BIOS

Fast Page mode with 2-way interleave

SIMM sockets for 256K , 1M or 4M modules

32M bytes on board

CAS# before RAS# refresh to reduce power comsumption

System Support Functions :

8-Channel DMA (Direct Memory Access)

16-level interrupt

3 programmable timers

CMOS RAM for system configuration

Real time clock with battery backup

Other Features :

On board POWERGOOD generation

External battery connector

Hardware turbo switch

Hardware reset switch

PROCESSOR

80386DX is a 32-bit microprocessor with 32-bit external data bus and 32-bit external address bus.  Therefore it processes more data at the same time than 80286 and can access a large memory size which is necessary for 32-bit applications.  To combine the wider bus structure and all the advanced functions on chip, total 275,000 transistors are integrated together.

A advanced pipeline architecture is implemented.  Next four Bytes of instructions are prefetched into the CPU whenever the bus is idle.  The size of the prefetch queue is increased to hold 12 Bytes.   This architecture reduces the overhead of retrieving and decoding of the instructions.

80386DX is aimed to provide advanced facilities to sophisticated software, but still remains compatible with existing software in the market.  It can operate at real mode and protected mode.  In Real mode the 386DX operates as a very fast 8086, but with 32-bit extensions if desired.  Real mode is required primarily to setup the processor for Protected mode operation.  Protected mode provides access to the sophisticated memory management,  paging and privilege capabilities of the processor.  Furthermore, new mechanism allows switching between Real mode and Protected mode at high speed operation.

Hence applications using EMS memory or extended memory can run efficiently although they need software driver to access the memory beyond 1MB.

The Protected mode of 80386 is fully compatible with 80286.  All privilege - level and I/O protection system are supported.  New system control instructions, memory paging, I/O permission bit map are provided to make 80386 ideal for multi-tasking operation systems.

In addition, a virtual 8086 mode is provided.  In this mode, the CPU can be considered by the programs as being divided into several 8086 CPUs and each program has their own CPU and memory space.  Several programs for XT/AT as well as operating systems for 80286 and 80386DX can be executed simultaneously.  Programs are isolated and protected from each other by 80386DX.  Each program can be considered as running at a XT/AT.

Internal memory management unit is complicated, but provides a more flexible addressing scheme for the next generation of operation system.  Multitasking, concurrent operation and manipulating huge data base can be accomplished with excellent performance.  Paging mechanism is employed by 80386DX to allow powerful operating system to implement virtual memory.  Each segment is divided into several pages which are 4K Bytes per page.  Page mechanism is transparent to software and allows software to address up to 64 Terabytes.

Furthermore, the 64KB segment boundary which is an barrier of 8088 and 80286 is removed and the segment length can be increased up to 4GB.  It eliminates the need for the software to manage multiple code and data segments and allows software of other operating system to be transferred to PC more easily.

80386DX includes many new instructions for system control, high level language support and processor control.  These instructions are used in protected mode.  New operation systems and software can make use of these instructions for their advanced features, such as concurrent operation and virtual memory.

MATH COPROCESSOR

The demand for sophisticated, number-crunching scientific and business applications has rapidly increased in recent years.  80386DX features an integer Arithmetic Logic Unit which only handles simple integer operations such as addition and multiplication.  Floating-point operations which are actually utilized by applications must be accomplished through software routines.

To overcome this obstacle, external Math coprocessor is necessary.  The Math coprocessor contains complex hardware and large data registers for floating-point numeric operations.

80387 is upward object-code compatible from 80287 and 8087, but runs 6 to 11 times faster than 80287 used in AT.  It fully implements the IEEE 754 Binary Floating-point Arithmetic standard with a high precision 80-bits internal architecture.

CACHE MEMORY

The system performance can not simply be improved by increasing the clock rate of the system.  The performance depends on many factors, such as system architecture and memory configuration.

A cache memory system with low cost DRAM as main memory and high speed SRAM as cache memory becomes the only choice for high performance system in terms of price and performance.  The frequently used data code instructions are kept in the high speed cache memory.  Therefore, most of the memory operations are carried out in the cache memory instead of the slow speed main memory.

The cache controller of JAGUAR IV-386 is integrated into the chipset, which will simplify the system design and reduce the chip count.  Furthermore, the cache controller is able to run in different size configuration: 32K,64K and 128K Byte.

Write Buffering with Byte Gathering

A sophisticated write-buffering scheme is implemented in JAGUAR IV-386.  Write-buffering is commonly found in cache-based system.  Single CPU write operation is buffered and thus no write state is needed.  The performance improvement is significant mostly because, in cache system, after the CPU code is fetched and resides in the cache, most of the cycles outside of the cache are write cycles.

However, if there is a lot of write operation, the overall performance is down graded, because only a write operation is buffered and subsequent write operations have to be delayed.

The write policy of the JAGUAR IV-386 cache controller is 4-level write-buffering which provided the 0 wait state write during multiple write cycle. An importance feature of the Write-buffer is byte gathering. If data being written by the cache to the write-buffers happens to be a `write buffer write hit' which means the location of the data is already in the write buffers, the data will be directly written to the buffer where the match occurred.

With multi-level write-buffer, the read operation needs to be specially handled.  If the data is still in the write buffer, it will be directly retrieved from write buffer and the main memory is updated later.  If the data is in the main memory, not in the cache, it will be fetched from the main memory, no matter how many data is still in the write buffer.  The latency of the write buffer unload is eliminated.

MEMORY SYSTEM

Two banks of DRAMs can be installed on motherboard and  256K, 1MB and 4MB DRAM SIMM modules are supported. One bank of DRAM refers to four pieces of SIMM modules.  The maximum memory size is 32MB when using 4MB DRAM for all banks.

The DRAM controllor supports Page-interleave mode.  Successive memory accesses within the same page need not require wait state and thus CPU can run at full speed. The DRAM should have fast page mode and CAS# before RAS# refresh capability.

To enhance the system performance, shadow RAM mode is supported.  In shadow RAM mode, system BIOS and video BIOS contained in low speed memory such as EPROM and ROM are copied into DRAM.  Improvement is significant because access to DRAM is much faster than ROM.

Hidden Refresh

In the original PC/AT design, the CPU suspends its operation during memory refresh.  The memory refresh cycle takes about 5% of CPU time which is a short period of time in a slow machine.  Nevertheless, in a 33MHz machine, it will significantly downgrade the system performance since the CPU can complete more operations in the same period.

In JAGUAR IV-386, a hidden refresh function is provided.  When the hidden refresh function is enabled, the CPU will not stop its operation and the memory refresh operation is transparent to the CPU access.  The chipset will monitor the whole system.  If the CPU is accessing the main memory, the memory refresh operation is postponed and will be carried out when there is no access to main memory.  Special refresh mechanism is implemented to reduce the period of memory refresh operation.

DUAL BUS DESIGN

It is very important that a high speed system should be compatible with existing peripherals without lowering the performance. To be compatible, the I/O slot should run at 8MHz or slower. On the other hand, the rest of the system are running at full speed.

A dual bus design is employed. A high speed bus links the CPU, coprocessor, cache memory and main memory. This bus is synchronous with the clock of the CPU and the data transfer is 32 bits. Whenever there is a request for transferring to or from I/O slot, the chipset is responsible for handling the conversion between the buses. The clock rate of the high speed bus will not be reduced, which eliminates many compatibility problem.

CPU

High Speed

bus			8Mhz bus

Chip Set            I/O Slot

Memory bus

Cache			        Main

Memory			      Memory

┌─────┐

│     │

└──┬──┘

│            ┌───────┐      ┌───────┐

├────────────┤       ├──────┤       │

│            └───┬───┘      └───────┘

│                │

┌──┴───┐        ┌───┴───┐

│      │        │       │

│      │        │       │

└──────┘        └───────┘

SYSTEM FUNCTIONS

System functions include :

-  Interrupt

-  DMA

-  Timer

-  Real time clock

-  Clock and ready generation

-  I/O channel control

All system functions are 100% compatible to AT standard.  I/O channel of OCTEK JAGAUR IV-386 is designed to be compatible with standard AT bus.  All the expansion cards conformed to the standard AT bus can be used in OCTEK JAGAUR IV-386 without problem.

Chapter 3

Installing Components

_______________________________

Important Note : Turn off the power before installing or replacing any component.

INSTALLING MATH COPROCESSOR

Math coprocessor 80387 and WEITEK 3167 are PGA devices.  Beside the CPU, there is a 121-pin PGA socket.  To install Math coprocessor, be sure to line up pin 1 of the Math coprocessor with pin 1 of the socket as shown below.  Make sure that the coprocessor is firmly inserted into the socket.

The WEITEK 3167 is a 121-pin chip which matches the pin count of the socket.  Align the pins and gently insert the chip into the socket.  However, 80387 has 69 pins and its package is smaller and different from WEITEK 3167.  So, it only occupies the inner pins of the socket.

Before installing the Math coprocessor, make sure all the pins are straight.  The pins are very fragile.  Once these pins are bent, the coprocessor may be damaged.

Check whether the system BIOS can find the coprocessor after reset.  The system BIOS will display a list of devices on the motherboard after self-test.  If the coprocessor is installed, it should show the type of coprocessor.

INSTALLING RAM MODULES

OCTEK JAGUAR IV-386 has eight sockets for SIMM modules.  Whenever adding memory modules to the motherboard, install four modules at the same time.  Also make sure that the chips on the modules face toward the memory expansion slot as shown in the next page.

To install a module, the module edge is angled into the socket's contact and then the module is pivoted into position, where the locking latches will secure it.  If the module edge is not completely inserted into the socket, it cannot be pivoted to be in vertical position and should be dragged out and inserted again.  Do not force the module into the SIMM socket.  It will damage the locking latches.

The modules should be locked by the locking latches of the sockets firmly.  Please check carefully before turning on the power.  Otherwise, the system will not work properly.

If the BIOS reports an memory error or parity error, drag out the modules and insert them again.   If the locking latches are damaged, contact your dealer to replace the socket.

INSTALLING EXTERNAL BATTERY

To back up the information stored in CMOS RAM, an external battery is needed to provide power after the system is turned off.  The connector (P8) for the battery is located beside the keyboard connector on the rear of the board.  A 3.6V battery is used.  Turn off the power before install the battery.  The location of the connector P8 is shown below.

CONFIGURATION OF MEMORY

The configuration of the memory is very flexible.  It can install 256KB , 1MB or 4MB SIMM modules are acceptable.  There are several combinations of DRAM types you may consider.  So, a basic system can be equipped with fewer memory and the system can be upgraded by installing the extended memory.  The different configurations of memory is illustrated at the next page.

There are totally two banks of DRAM. The memory size is detected automatically by system BIOS.  This detection is performed during memory test and the size is indicated after reset.  No jumper is needed to be set for the memory size and DRAM type.

To determine what DRAM speed rating to be used is depended on the system speed and wait state.  The highest performance is accomplished by using zero wait state, but high speed DRAM has to be used.

The wait state setting is applied to two banks of memory.   Therefore make sure to install DRAM modules with the same speed rating, or accommodate the wait state setting to the new DRAM type.

The number of wait state is assigned in the BIOS setup.  Improper setting may cause the system malfunction.  In this case, reset the CMOS setup using JP4. Then reset the system and go through the system setup again.

DRAM CONFIGURATION

|  | Bank 0 SIMM (1-4) | Bank 1 SIMM (5-8) | Total Memory |
| --- | --- | --- | --- |
| 1 | 256K | X | 1M |
| 2 | 256K | 256K | 2M |
| 3 | 1M | X | 4M |
| 4 | 256K | 1M | 5M |
| 5 | 1M | 1M | 8M |
| 6 | 1M | 1M | 16M |
| 7 | 4M | 4M | 32M |

CONFIGURATION OF CACHE MEMORY

Note : If you have any question about the configuration of cache memory, consult your local dealer.  Improper configuration will cause the system malfunction.

In OCTEK JAGUAR IV-386 SYSTEM, the cache memory have four options: 32KB, 64KB and 128KB. The following table shown the SRAM location in different cache size:

| Cache size | SRAM & Location |
| --- | --- |
| 32KB | 8KX8, U3, U5, U7, U9 |
| 64KB | 8KX8, U3 - U10 |
| 128KB | 32KX8, U3, U5, U7, U9 |

Make sure that the number of SRAMs is correct and they are installed properly.  Otherwise the system will not work.

The speed rating of the SRAM for 33MHz and 40MHz motherboard are listed below:

| System Speed | SRAM |
| --- | --- |
| 33MHz | 25ns |
| 40MHz | 20ns |

The jumper settings for the configuration of cache memory.

Cache Memory Size

|  | JP3 |
| --- | --- |
| 32KB | 2-3 |
| 64KB | 2-3 |
| 128KB | 1-2 |

CONTROL OF SYSTEM SPEED

System speed can be controlled by hardware switch and keyboard.  Connector P3 is connected to the turbo switch of the case.  When the system speed is fast, the turbo LED of the case should be turned on.

To change the speed by keyboard, use '-' and '+' of the numeric keypad.  Press 'Ctrl', 'Alt' and '-' for slow speed and Press  'Ctrl', 'Alt' and '+' for fast speed.

Whenever the system speed is set to slow by turbo switch, it cannot be changed by keyboard, and vice versa.

RESET CMOS SETUP INFORMATION

Sometimes, the improper setting of system setup may make the system malfunction.   In this case, turn off the power and disconnect the external battery.  Then place a jumper on JP4 (2-3) for a while.   The internal CMOS status register will be cleared.  Then remove the jumper and turn on the power.  The BIOS finds the CMOS status register is reset and regards the setup information as invalid.  So it will prompt you to correct the information.  In normal operation, JP4 place in (1-2).

SYSTEM BOARD JUMPER SETTING

There are several options which allow user to select by hardware switches.

Security Selection

| JP1 |  |
| --- | --- |
| 1-2 | Enable |
| 2-3 | Disable |

Note :  * factory setting

SYSTEM BOARD CONNECTORS

Under typical conditions, these connectors should be connected to the indicators and switches of the system unit.

| Connector | Function |
| --- | --- |
| P1 | Hardware reset connector |
| P2 | Speaker connector |
| P3 | Turbo switch connector |
| P4 | Turbo LED connector |
| P5 | Power LED & Ext-Lock connector |
| P6, P7 | Power supply connector |
| P8 | External battery connector |
| KB1 | Keyboard connector |

Pin assignments of the connectors are illustrated as follows:

P 1 - Hardware Reset Connector

| Pin | Assignment |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

P 2 - Speaker Connector

| Pin | Assignment |
| --- | --- |
| 1 | Data out |
| 2 | +5 Vdc |
| 3 | Ground |
| 4 | +5 Vdc |

P 3 - Turbo Switch Connector

| Pin | Assignment |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

P 4 - Turbo LED Connector

| Pin | Assignment |
| --- | --- |
| 1 | +5 Vdc |
| 2 | LED signal |

P 5 - Power LED & Ext-Lock Connector

| Pin | Assignment |
| --- | --- |
| 1 | +5 Vdc |
| 2 | Key |
| 3 | Ground |
| 4 | Keyboard inhibit |
| 5 | Ground |

P6, P7 - Power Supply Connector

| Pin | Assignment |
| --- | --- |
| 1 | POWERGOOD |
| 2 | +5 Vdc |
| 3 | +12 Vdc |
| 4 | -12 Vdc |
| 5 | Ground |
| 6 | Ground |

| Pin | Assignment |
| --- | --- |
| 1 | Ground |
| 2 | Ground |
| 3 | -5 Vdc |
| 4 | +5 Vdc |
| 5 | +5 Vdc |
| 6 | +5 Vdc |

P 8 - External Battery Connector

| Pin | Assignment |
| --- | --- |
| 1 | + Vdc |
| 2 | not used |
| 3 | Ground |
| 4 | Ground |

KB 1 - Keyboard Connector

| Pin | Assignment |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Spare |
| 4 | Ground |
| 5 | +5 Vdc |

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 4

Technical Information

_______________________________

This section provides technical information about OCTEK JAGUAR-IV 386 and is intended for advanced users interested in the basic design and operation of OCTEK JAGUAR-IV 386.

MEMORY MAPPING

| Address | Range | Function |
| --- | --- | --- |
| 000000- 7FFFFF | 000K-512K | System Board Memory (512K) |
| 080000- 09FFFF | 512K-640K | System Board Memory (128K) |
| 0A0000- 0BFFFF | 640K-768K | Display Buffer (128K) |
| 0C0000- 0DFFFF | 768K-896K | Adaptor ROM / Shadow RAM (128K) |
| 0E0000- 0EFFFF | 896K-960K | System ROM / Shadow RAM (64K) |
| 0F0000- 0FFFFF | 960K-1024K | System BIOS ROM / Shadow RAM (64K) |
| 100000- 7FFFFF | 1024K-8192K | System Memory |
| 800000- FFFFFF | 8192K-16318K | System Memory |

I/O ADDRESS MAP

I/O Address Map on System Board

I/O address hex 000 to 0FF are reserved for the system board I/O.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 000-01F | DMA Controller 1, 8237 |
| 020-03F | Interrupt Controller 1, 8259, Master |
| 040-05F | Timer, 8254 |
| 060-06F | Keyboard Controller |
| 070-07F | Real Time Clock, NMI (non-maskable interrupt) mask |
| 080-09F | DMA Page Register, 74LS612 |
| 0A0-0BF | Interrupt Controller 2, 8259 |
| 0C0-0DF | DMA Controller 2, 8237 |
| 0F0 | Clear Math Coprocessor Busy |
| 0F1 | Reset Math Coprocessor |
| 0F8-0FF | Math Coprocessor Port |

I/O address hex 100 to 3FF are available on the I/O channel.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game I/O |
| 278-27F | Parallel Printer Port 2 |
| 2F8-2FF | Serial Port 2 |
| 300-31F | Prototype Card |
| 360-36F | Reserved |
| 378-37F | Parallel Printer Port 1 |
| 380-38F | SDLC, bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Printer Adapter |
| 3C0-3CF | Reserved |
| 3D0-3DF | Color Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 |

SYSTEM TIMERS

OCTEK JAGUAR-IV 386 has three programmable timer/counters controlled by 82C206 and they are defined as channels 0 through 2 :

| Channel 0 | System Timer |
| --- | --- |
| Gate 0 | Tied on |
| Clk in 0 | 1.190 Mhz OSC |
| Clk out 0 | 8259 IRQ 0 |

| Channel 1 | Refresh Request Generator |
| --- | --- |
| Gate 1 | Tied on |
| Clk in 1 | 1.190 Mhz OSC |
| Clk out 1 | Request Refresh Cycle |

| Channel 2 | Tone Generation of Speaker |
| --- | --- |
| Gate 2 | Controlled by bit 0 of port hex 61 PPI bit |
| Clk in 2 | 1.190 Mhz OSC |
| Clk out 2 | Used to drive the speaker |

Note :	Channel 1 is programmed to generate a 15-micro-second period signal.

The 8254 Timer/Counters are treated by system programs as an arrangement of four programmable external I/O ports.  Three are treated as counters and the fourth is a control register for mode programming.

SYSTEM INTERRUPTS

Sixteen levels of system interrupts are provided on OCTEK JAGUAR-IV 386.  The following shows the interrupt-level assignments in decreasing priority.

Level					Function

Microprocessor NMI	Parity or I/O Channel

Check

Interrupt Controllers

CTLR 1	CTLR 2

IRQ0				Timer Output 0

IRQ1				Keyboard

(Output Buffer Full)

IRQ2				Interrupt from CTLR 2

IRQ8		Real-time Clock Interrupt

IRQ9		Software Redirected to

INT 0AH (IRQ2)

IRQ10	Reserved

IRQ11	Reserved

IRQ12	Reserved

IRQ13	Coprocessor

IRQ14	Fixed Disk Controller

IRQ15	Reserved

IRQ3				Serial Port 2

IRQ4				Serial Port 1

IRQ5				Parallel Port 2

IRQ6				Diskette Controller

IRQ7				Parallel Port 1

┌──────────────────────────────────────┐

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│     ─┐                               │

│      │ ┌─                            │

│      │ │                             │

│      │ │                             │

│      │ │                             │

│      └─┤                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        └─                            │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

└──────────────────────────────────────┘

DIRECT MEMORY ACCESS (DMA)

OCTEK JAGUAR-IV 386 supports seven DMA channels.

| Channel | Function |
| --- | --- |
| 0 | Spare (8 bit transfer) |
| 1 | SDLC (8 bit transfer) |
| 2 | Floppy Disk (8 bit transfer) |
| 3 | Spare (8 bit transfer) |
| 4 | Cascade for DMA Controller 1 |
| 5 | Spare (16 bit transfer) |
| 6 | Spare (16 bit transfer) |
| 7 | Spare (16 bit transfer) |

The following shows the addresses for the page register.

| Page Register | I/O Address (HEX) |
| --- | --- |
| DMA Channel 0 | 0087 |
| DMA Channel 1 | 0083 |
| DMA Channel 2 | 0081 |
| DMA Channel 3 | 0082 |
| DMA Channel 5 | 008B |
| DMA Channel 6 | 0089 |
| DMA Channel 7 | 008A |
| Refresh | 008F |

REAL TIME CLOCK AND CMOS RAM

Real time clock and CMOS RAM are contained on board.  Real time clock provides the system date and time.  CMOS RAM stores system information.  Both are backed up by battery and will not lose information after power off.  The following page shows the CMOS RAM Address Map.

CMOS RAM ADDRESS MAP

| Addresses | Description |
| --- | --- |
| 00-0D | * Real-time clock information |
| 0E | * Diagnostic status byte |
| 0F | * Shutdown status byte |
| 10 | Diskette drive type byte - drives A and B |
| 11 | Reserved |
| 12 | Fixed disk type byte - drives C and D |
| 13 | Reserved |
| 14 | Equipment byte |
| 15 | Low base memory byte |
| 16 | High base memory byte |
| 17 | Low expansion memory byte |
| 18 | High expansion memory byte |
| 19-2D | Reserved |
| 2E-2F | 2-byte CMOS checksum |
| 30 | * Low expansion memory byte |
| 31 | * High expansion memory byte |
| 32 | * Date century byte |
| 33 | * Information flags (set during power on) |
| 34-3F | Reserved |

REAL TIME CLOCK INFORMATION

The following table describes real-time clock bytes and specifies their addresses.

| Byte | Function | Address |
| --- | --- | --- |
| 0 | Seconds | 00 |
| 1 | Second alarm | 01 |
| 2 | Minutes | 02 |
| 3 | Minute alarm | 03 |
| 4 | Hours | 04 |
| 5 | Hour alarm | 05 |
| 6 | Day of week | 06 |
| 7 | Date of month | 07 |
| 8 | Month | 08 |
| 9 | Year | 09 |
| 10 | Status Register A | 0A |
| 11 | Status Register B | 0B |
| 12 | Status Register C | 0C |
| 13 | Status Register D | 0D |

SYSTEM EXPANSION BUS

OCTEK JAGUAR-IV 386 provides seven 16-bit slots.

The I/O channel supports:

*	I/O address space from hex 100 to hex 3FF

*	Selection of data access (either 8 or 16 bit)

*	24 bit memory addresses (16MB)

*	Interrupts

*	DMA channels

*	Memory refresh signal

The following figure shows the pin numbering for I/O channel connectors JA1 to JA7.

The following figure shows the pin numbering for I/O channel connectors JB1-JB6.

The following tables summarize pin assignments for the I/O channel connectors.

I/O Channel (A-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| A1 | -I/O CH CK | I |
| A2 | SD7 | I/O |
| A3 | SD6 | I/O |
| A4 | SD5 | I/O |
| A5 | SD4 | I/O |
| A6 | SD3 | I/O |
| A7 | SD2 | I/O |
| A8 | SD1 | I/O |
| A9 | SD0 | I/O |
| A10 | -I/O CH RDY | I |
| A11 | AEN | O |
| A12 | SA19 | I/O |
| A13 | SA18 | I/O |
| A14 | SA17 | I/O |
| A15 | SA16 | I/O |
| A16 | SA15 | I/O |
| A17 | SA14 | I/O |
| A18 | SA13 | I/O |
| A19 | SA12 | I/O |
| A20 | SA11 | I/O |
| A21 | SA10 | I/O |
| A22 | SA9 | I/O |
| A23 | SA8 | I/O |
| A24 | SA7 | I/O |
| A25 | SA6 | I/O |
| A26 | SA5 | I/O |
| A27 | SA4 | I/O |
| A28 | SA3 | I/O |
| A29 | SA2 | I/O |
| A30 | SA1 | I/O |
| A31 | SA0 | I/O |

I/O Channel (B-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| B1 | GND | Ground |
| B2 | RESET DRV | I |
| B3 | +5 Vdc | Power |
| B4 | IRQ9 | I |
| B5 | -5 Vdc | Power |
| B6 | DRQ2 | I |
| B7 | -12 Vdc | Power |
| B8 | 0WS | I |
| B9 | +12 Vdc | Power |
| B10 | GND | Ground |
| B11 | -SMEMW | O |
| B12 | -SMEMR | O |
| B13 | -IOW | I/O |
| B14 | -IOR | I/O |
| B15 | -DACK3 | I |
| B16 | DRQ3 | O |
| B17 | -DACK1 | I |
| B18 | DRQ1 | O |
| B19 | -Refresh | I/O |
| B20 | CLK | O |
| B21 | IRQ7 | I |
| B22 | IRQ6 | I |
| B23 | IRQ5 | I |
| B24 | IRQ4 | I |
| B25 | IRQ3 | I |
| B26 | -DACK2 | O |
| B27 | T/C | O |
| B28 | BALE | O |
| B29 | +5 Vdc | Power |
| B30 | OSC | O |
| B31 | GND | Ground |

I/O Channel (C-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| C1 | SBHE | I/O |
| C2 | LA23 | I/O |
| C3 | LA22 | I/O |
| C4 | LA21 | I/O |
| C5 | LA20 | I/O |
| C6 | LA19 | I/O |
| C7 | LA18 | I/O |
| C8 | LA17 | I/O |
| C9 | -MEMR | I/O |
| C10 | -MEMW | I/O |
| C11 | SD8 | I/O |
| C12 | SD9 | I/O |
| C13 | SD10 | I/O |
| C14 | SD11 | I/O |
| C15 | SD12 | I/O |
| C16 | SD13 | I/O |
| C17 | SD14 | I/O |
| C18 | SD15 | I/O |

I/O Channel (D-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| D1 | -MEM CS16 | I |
| D2 | -I/O CS16 | I |
| D3 | IRQ10 | I |
| D4 | IRQ11 | I |
| D5 | IRQ12 | I |
| D6 | IRQ15 | I |
| D7 | IRQ14 | I |
| D8 | -DACK0 | O |
| D9 | DRQ0 | I |
| D10 | -DACK5 | O |
| D11 | DRQ5 | I |
| D12 | -DACK6 | O |
| D13 | DRQ6 | I |
| D14 | -DACK7 | O |
| D15 | DRQ7 | I |
| D16 | +5 Vdc | Power |
| D17 | -MASTER | I |
| D18 | GND | Ground |

THIS PAGE IS INTENTIONALLY LEFT BLANK

Appendix A

Operation and Maintenance

_______________________________

STATIC ELECTRICITY

When installing or removing any add-on card, DRAM module or coprocessor, you should discharge the static electricity on your body.  Static electricity is dangerous to electronic device and can build-up on your body.  When you touch the add-on card or motherboard, it is likely to damage the device.  To discharge the static electricity, touch the metal of your computer.  When handling the add-on card, don't contact the components on the cards or their "golden finger".  Hold the cards by their edges.

KEEPING THE SYSTEM COOL

The motherboard contains many high-speed components and they will generate heat during operation.  Other add-on cards and hard disk drive can also produce a lot of heat.  The temperature inside the computer system may be very high.  In order to keep the system running stably, the temperature must be kept at a low level.  A easy way to do this is to keep the cool air circulating inside the case.  The power supply contains a fan to blow air out of the case.  If you find that the temperature is still very high, it would be better to install another fan inside the case.  Using a larger case is recommended if there are a number of add-on cards and disk drives in the system.

CLEANING THE "GOLDEN FINGER"

Whenever inserting an add-on card to the motherboard, make sure that there is no dirt on the "golden finger" of the add-on card.  If not, the contact between the "golden finger" and the slot may be poor and thus the add-on card may not work properly.  Use a pencil eraser to clean the "golden finger" if dirt is found.

CLEANING THE MOTHERBOARD

The computer system should be kept clean.  Dust and dirt is harmful to electronic devices.  To prevent dust from accumulating on the mother-board, installing all mounting plates on the rear of the case.  Regularly examine your system, and if necessary, vacuum the interior of the system with a miniature vacuum.

Appendix B

Troubleshooting

_______________________________

POOR PERFORMANCE

If the performance of the system becomes very poor after enabling cache memory, it is likely that the jumper setting for the cache memory is incorrect.  Refer to CONFIGURATION OF CACHE MEMORY in the Chapter 3 for the description of the related jumper.

MAIN MEMORY ERROR

After power up, the monitor remains blank, and there are beep sounds indicating a main memory failure.  In this case, turn off the power and remove all SIMM modules.  Carefully place the modules back to the sockets and make sure that all the modules are locked by the locking latches firmly.

In some other cases, the total memory found by the BIOS is different from the actual amount of memory on board.  It is also a memory failure and you can follow the instruction above.

CACHE MEMORY FAILURE

If the system hangs after memory test, it is likely that the cache memory has some problems.  May be some of the SRAMs are damaged or the contact of the IC pins is poor.  Try to press the SRAM to make sure that the SRAMs are inserted in the sockets, or examine the SRAM to see whether any pins are bent under or out.  If the bent pins are found, remove the SRAM, straighten the pin and place the SRAM again.  You may also check the BIOS setup of the cache configuration.  If the cache controller is enabled, you should select chipset's cache controller.  Otherwise, the system will fail.

IMPROPER SETTING OF WAIT STATE

If the system hangs after memory test, another possible cause is the improper setting of the wait state for memory operation.  The number of wait state must match the speed of the DRAM.  Reset the CMOS RAM and set up the wait state.  Try to increase the number of wait state.

Appendix C

System Board Layout

_______________________________