Jaguar IV 386 Mainboard

Supplement for

System Manual Rev 1.0

(June 2, 92)

=================

There are some typing mistakes in the system manul rev 1.0.  Details are as follow :

INSTALLING MATH COPROCESSOR  (P. 3-1)

Math coprocessor 80387 is PGA device and it has 68 pins.  To install Math coprocessor, be sure to line up pin 1 of the Math coprocessor with pin 1 of the socket as shown below.  Make sure that the coprocessor is firmly inserted into the socket.

Before installing the Math coprocessor, make sure all the pins are straight.  The pins are very fragile.  Once these pins are bent, the coprocessor may be damaged.

Check whether the system BIOS can find the coprocessor after reset.  The system BIOS will display a list of devices on the motherboard after self-test.  If the coprocessor is installed, it should show the type of coprocessor.

DRAM CONFIGURATION  (P. 3-7)

|  | Bank 0 SIMM (1-4) | Bank 1 SIMM (5-8) | Total Memory |
| --- | --- | --- | --- |
| 1 | 256K | X | 1M |
| 2 | 256K | 256K | 2M |
| 3 | 1M | X | 4M |
| 4 | 256K | 1M | 5M |
| 5 | 1M | 1M | 8M |
| 6 | 4M | X | 16M |
| 7 | 1M | 4M | 20M |
| 8 | 4M | 4M | 32M |

IMPROPER SETTING OF WAIT STATE  (P. B-2)

This option is deleted