Appendix A

Operation and Maintenance

_______________________________

STATIC ELECTRICITY

When installing or removing any add-on card, DRAM module or coprocessor, you should discharge the static electricity on your body.  Static electricity is dangerous to electronic device and can build-up on your body.  When you touch the add-on card or motherboard, it is likely to damage the device.  To discharge the static electricity, touch the metal of your computer.  When handling the add-on card, don't contact the components on the cards or their "golden finger".  Hold the cards by their edges.

KEEPING THE SYSTEM COOL

The motherboard contains many high-speed components and they will generate heat during operation.  Other add-on cards and hard disk drive can also produce a lot of heat.  The temperature inside the computer system may be very high.  In order to keep the system running stably, the temperature must be kept at a low level.  A easy way to do this is to keep the cool air circulating inside the case.  The power supply contains a fan to blow air out of the case.  If you find that the temperature is still very high, it would be better to install another fan inside the case.  Using a larger case is recommended if there are a number of add-on cards and disk drives in the system.

CLEANING THE "GOLDEN FINGER"

Whenever inserting an add-on card to the motherboard, make sure that there is no dirt on the "golden finger" of the add-on card.  If not, the contact between the "golden finger" and the slot may be poor and thus the add-on card may not work properly.  Use a pencil eraser to clean the "golden finger" if dirt is found.

CLEANING THE MOTHERBOARD

The computer system should be kept clean.  Dust and dirt is harmful to electronic devices.  To prevent dust from accumulating on the mother-board, installing all mounting plates on the rear of the case.  Regularly examine your system, and if necessary, vacuum the interior of the system with a miniature vacuum.

ENG/SN/039

ENG/SN/039

ENG/SN/039