Hippo 486 Mainboard

Supplementary Note

=====================

The date code of the new version of BIOS is EOBC-6069-101590.  A new extended setup is supplied and allows you to configure the system easier.  You only need to select the desirable option and the BIOS will program the internal registers of the chipset.  The extended setup screen is very comprehensive.  The old settings of each options are listed on the right side of the screen for your reference.  Press up or down arrow keys to move between options and use PgUp or PgDn keys to change the value.  Press ESC key to exit the setup.  If you want to use the new settings, select 'WRITE TO CMOS AND EXIT'.  The new settings will be saved to CMOS RAM and will be used afterwards.

Main and Video BIOS Shadow

The default setting is to enable main BIOS shadow only.  If there is a VGA or EGA display adaptor, you may select 'Both Enabled'.

Internal & External Cache Controller

The internal cache of 486 is enabled first.  If the secondary cache card, HIPPO CACHE card, is installed, select `Both Enabled'.  Then removes the jumpers for secondary cache.  If some programs cannot run at high speed, you may select 'Both Disabled' temporarily.

Non-Cacheable Block

You can specify two address ranges which will be excluded from the cacheable memory address range.  The size of the block is from 64K, up to 8M.  The starting address of each block is set in bit format.

Cacheable Memory Address Range

This range should be equal to the size of the memory on board.

256KB Memory Remapped Area

The 256KB memory remapped area is located beyond the on board memory.  You may select 'Cacheable' to include this area in the cacheable range.

256KB Memory Relocation

If enabled, the unused 256KB memory will be mapped to the top of the memory and is available.

Bus Clock Selection

This option sets the speed of the expansion slot and is same as the ICLK clock selection in the manual in the appendix A.

ATCLK Stretch

The default setting is 'Disable'.  When enabled, the performance can be improved.  But this function may not work with some slow speed add-on cards.

DRAM Write Wait State

One wait state is set.  Many high speed DRAM can work with zero wait state at 25MHz but have to run at 1 wait state at 33MHz.  But make sure that your system is stable before changing to zero wait state.

DRAM Read Wait State

It is recommended to set one wait state for read operation.  70ns DRAM can run at zero wait state at 25MHz and one wait state at 33MHz.  For slower speed DRAM, one more wait state is needed.

GA20 line After Power On

The function is to solve the problem of Windows 3.0.  If you cannot run Windows 3.0 on HIPPO board, set this option to 'On'.  After loading HIMEM.SYS, a warning is shown, but it can be ignored.

RAS* Timeout Counter

The default value is 'Enabled'.  If disabled, the overall performance can be slightly better.  However, some of the DRAM will have problem.  So make sure that no problem arises before permanently enabling this function.