APPENDIX

ADVANCED TECHNICAL INFORMATION

A.1	Memory Address Map

| Address | Size |  | Function |
| --- | --- | --- | --- |
| 0000000-009FFFF | 640 KB |  | System board memory |
| 00A0000-00BFFFF | 128 KB |  | Video RAM display buffer |
| 00C0000-00DFFFF | 128 KB |  | Reserved for add-on cards ROM BIOS |
| 00E0000-00EFFFF | 64 KB |  | System ROM BIOS expansion |
| 00F0000-00FFFFF | 64 KB |  | System ROM BIOS |
| 0100000-0FDFFFF | 15232 KB |  | Extended memory |
| 0FE0000-0FEFFFF | 64 KB |  | Duplicates of system ROM BIOS at 0F0000-0FFFFFF |
| 1000000-5FFFFFF | 80 MB |  | Extended memory |
| 0000000-5FFFFFF | 96 MB |  | Total memory space addressable by Hippo DCA2 VESA System Board |

A.2	I/O Address Map

I/O Address Hex 000 to 0FF are reserved for the system board and Hex 100 to 3FF are for I/O channels.

| Address (Hex) | Device |
| --- | --- |
| 000-01F | DMA Controller 1, 8237A-5 |
| 020-021 | Interrupt Controller 1, 8237A-5 |
| 022-023 | Chip set Address |
| 040-04F | Timer 1, 8254 |
| 050-05F | Timer 2, 8254 |
| 060-06F | 8042 Keyboard/Controller |
| 070-07F | Real Time Clock, Non-Maskable Interrupt Mask |
| 080-09F | DMA Page Registers |
| 0A0-0BF | Interrupt Controller 2,8259A |
| 0C0-0DF | DMA Controller 2, 8237A-5 |
| 0F0 | Clear Math Coprocessor Busy |
| 0F1 | Reset Math Coprocessor |
| 0F8-0FF | Math Coprocessor |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game Ports |
| 278-27F | Parallel Printer Port 2 (PIO-2) |
| 2F8-2FF | Serial Port 2 (SIO-2) |
| 300-31F | Prototype Card/Streaming Tape Adapter |
| 360-363 | PC Network, Low Address |
| 368-36B | PC Network, High Address |
| 378-37F | Parallel Printer Port 1 (PIO-1) |
| 380-38F | SDLC, Bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Primer Adapter |
| 3C0-3CF | EGA Adapter |
| 3D0-3DF | Color/Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 (SIO-1) |

A.3	I/O Extension Pinout

A.3i	8-Bit ISA Pinout

|  | Ground | GND | B1 | A1 | -I/O CHCK |  | I |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | O | RSTDRV | B2 | A2 | SD7 |  | I/O |
|  | Power | +5 VDC | B3 | A3 | SD6 |  | I/O |
|  | I | IRQ9 | B4 | A4 | SD5 |  | I/O |
|  | Power | -5 VDC | B5 | A5 | SD4 |  | I/O |
|  | I | DRQ2 | B6 | A6 | SD3 |  | I/O |
|  | Power | -12 VDC | B7 | A7 | SD2 |  | I/O |
|  | I | OWS | B8 | A8 | SD1 |  | I/O |
|  | Power | +12 VDC | B9 | A9 | SD0 |  | I/O |
|  | Ground | GND | B10 | A10 | -I/O CHRDY |  | I |
|  | O | -SMEMW | B11 | A11 | AEN |  | O |
|  | O | -SMEMR | B12 | A12 | SA19 |  | I/O |
|  | I/O | -IOW | B13 | A13 | SA18 |  | I/O |
|  | I/O | -IOR | B14 | A14 | SA17 |  | I/O |
|  | O | -DACK3 | B15 | A15 | SA16 |  | I/O |
|  | I | DRQ3 | B16 | A16 | SA15 |  | I/O |
|  | O | -DACK1 | B17 | A17 | SA14 |  | I/O |
|  | I | DRQ1 | B18 | A18 | SA13 |  | I/O |
|  | I/O | -Ref | B19 | A19 | SA12 |  | I/O |
|  | O | CLK | B20 | A20 | SA11 |  | I/O |
|  | I | IRQ7 | B21 | A21 | SA10 |  | I/O |
|  | I | IRQ6 | B22 | A22 | SA9 |  | I/O |
|  | I | IRQ5 | B23 | A23 | SA8 |  | I/O |
|  | I | IRQ4 | B24 | A24 | SA7 |  | I/O |
|  | I | IRQ3 | B25 | A25 | SA6 |  | I/O |
|  | O | -DACK2 | B26 | A26 | SA5 |  | I/O |
|  | O | T/C | B27 | A27 | SA4 |  | I/O |
|  | O | BALE | B28 | A28 | SA3 |  | I/O |
|  | Power | +5 VDC | B29 | A29 | SA2 |  | I/O |
|  | O | OSC | B30 | A30 | SA1 |  | I/O |
|  | Ground | GND | B31 | A31 | SA0 |  | I/O |

A.3ii	16-Bit ISA Extension Pinout

|  | I | -MEMCS16 | D1 | C1 | -BHE |  | I/O |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | -I/OCS16 | D2 | C2 | LA23 |  | I/O |
|  | I | IRQ10 | D3 | C3 | LA22 |  | I/O |
|  | I | IRQ11 | D4 | C4 | LA21 |  | I/O |
|  | I | IRQ12 | D5 | C5 | LA20 |  | I/O |
|  | I | IRQ15 | D6 | C6 | LA19 |  | I/O |
|  | I | IRQ14 | D7 | C7 | LA18 |  | I/O |
|  | 0 | -DACK0 | D8 | C8 | LA17 |  | I/O |
|  | I | DRQ0 | D9 | C9 | -MEMR |  | I/O |
|  | 0 | -DACK5 | D10 | C10 | -MEMW |  | I/O |
|  | I | DRQ5 | D11 | C11 | SD08 |  | I/O |
|  | 0 | -DACK6 | D12 | C12 | SD09 |  | I/O |
|  | I | DRQ6 | D13 | C13 | SD10 |  | I/O |
|  | 0 | -DACK7 | D14 | C14 | SD11 |  | I/O |
|  | I | DRQ7 | D15 | C15 | SD12 |  | I/O |
|  | Power | +5 VDC | D16 | C16 | SD13 |  | I/O |
|  | I | -MASTER | D17 | C17 | SD14 |  | I/O |
|  | Ground | GND | D18 | C18 | SD15 |  | I/O |

A.4	VL-BUS Extension Pinout

|  | CD1 | B1 | A1 | CD0 |
| --- | --- | --- | --- | --- |
|  | CD3 | B2 | A2 | CD2 |
|  | Ground | B3 | A3 | CD4 |
|  | CD5 | B4 | A4 | CD6 |
|  | CD7 | B5 | A5 | CD8 |
|  | CD9 | B6 | A6 | Ground |
|  | CD11 | B7 | A7 | CD10 |
|  | CD13 | B8 | A8 | CD12 |
|  | CD15 | B9 | A9 | Power |
|  | Ground | B10 | A10 | CD14 |
|  | CD17 | B11 | A11 | CD16 |
|  | Power | B12 | A12 | CD18 |
|  | CD19 | B13 | A13 | CD20 |
|  | CD21 | B14 | A14 | Ground |
|  | CD23 | B15 | A15 | CD22 |
|  | CD25 | B16 | A16 | CD24 |
|  | Ground | B17 | A17 | CD26 |
|  | CD27 | B18 | A18 | CD28 |
|  | CD29 | B19 | A19 | CD30 |
|  | CD31 | B20 | A20 | Power |
|  | CA30 | B21 | A21 | CA31 |
|  | CA28 | B22 | A22 | Ground |
|  | CD26 | B23 | A23 | CA29 |
|  | Ground | B24 | A24 | CA27 |
|  | CA24 | B25 | A25 | CA25 |
|  | CA22 | B26 | A26 | CA23 |
|  | Power | B27 | A27 | CA21 |
|  | CA20 | B28 | A28 | CA19 |
|  | CA18 | B29 | A29 | Ground |
|  | CA16 | B30 | A30 | CA17 |
|  | CA14 | B31 | A31 | CA15 |
|  | CA12 | B32 | A32 | Power |
|  | CA10 | B33 | A33 | CA13 |
|  | CA8 | B34 | A34 | CA11 |
|  | Ground | B35 | A35 | CA9 |
|  | CA6 | B36 | A36 | CA7 |
|  | CA4 | B37 | A37 | CA5 |
|  | WBack | B38 | A38 | Ground |
|  | BEO | B39 | A39 | CA3 |
|  | Power | B40 | A40 | CA2 |
|  | BE1 | B41 | A41 | n/c |
|  | BE2 | B42 | A42 | Reset |
|  | Ground | B43 | A43 | D/C |
|  | BE3 | B44 | A44 | M/IO |
|  | ADS | B45 | A45 | W/R |
|  | LRDY | B46 | A46 | RDY |
|  | LDEV | B47 | A47 | Ground |
|  | LREQ | B48 | A48 | IRQ9 |
|  | Ground | B49 | A49 | BRDY |
|  | LGNT | B50 | A50 | Blast |
|  | Power | B51 | A51 | ID0 |
|  | ID2 | B52 | A52 | ID1 |
|  | ID3 | B53 | A53 | Ground |
|  | ID4 | B54 | A54 | VLCLK |
|  | LKEN | B55 | A55 | Power |
|  | Leads | B56 | A56 | LBS16 |

A.5	Direct Memory Access Channels

| Channel | Function | 8-Bit | 16-Bit | Transfer Block Size |
| --- | --- | --- | --- | --- |
| 0 | Spare | 8-bit |  | 64 |
| 1 | SDLC | 8-bit |  | 64 |
| 2 | Floppy Disk | 8-bit |  | 64 |
| 3 | Spare | 8-bit |  | 64 |
| 4 | Cascade for DMA controller 1 |  |  |  |
| 5 | Spare |  | 16-bit | 128 |
| 6 | Spare |  | 16-bit | 128 |
| 7 | Spare |  | 16-bit | 128 |

A.6	DMA Controller Registers

| Address (Hex) |  | Command Code |
| --- | --- | --- |
| C0 |  | CH-0 base and current address |
| C2 |  | CH-0 base and current word count |
| C4 |  | CH-1 base and current address |
| C6 |  | CH-1 base and current word count |
| C8 |  | CH-2 base and current address |
| CA |  | CH-2 base and current word count |
| CC |  | CH-3 base and current address |
| CE |  | CH-3 base and current word count |
| D0 |  | Read Status Register/Write Command Register |
| D2 |  | Write Request Register |
| D4 |  | Write Single Mask Register Bit |
| D6 |  | Write Mode Register |
| D8 |  | Clear Byte Pointer Flip-Flop |
| DA |  | Read Temporary Register/Write Master Clear |
| DC |  | Clear Master Register |
| DE |  | Write All Mask Register Bits |

A.7	Page Register Address

| Page Register |  | I/O Address (Hex) |
| --- | --- | --- |
| DMA Channel 0 |  | 0087 |
| DMA Channel 1 |  | 0083 |
| DMA Channel 2 |  | 0081 |
| DMA Channel 3 |  | 0082 |
| DMA Channel 5 |  | 008B |
| DMA Channel 6 |  | 0089 |
| DMA Channel 7 |  | 008A |
| Refresh |  | 008F |

A.8	System Interrupts

| Interrupt Controllers ( NMI level ) |  |  | Parity or I/O Channel Check |
| --- | --- | --- | --- |
| CTLR 1 | CTLR 2 |  |  |
| IRQ0 |  |  | Timer Output 0 |
| IRQ1 |  |  | Keyboard |
|  |  |  | (Output Buffer Full) |
| IRQ2 |  |  | Interrupt from CTLR 2 |
|  | IRQ8 |  | Real-time Clock Interrupt |
|  | IRQ9 |  | Software Redirected to INT 0AH (IRQ2) |
|  | IRQ10 |  | Reserved |
|  | IRQ11 |  | Reserved |
|  | IRQ12 |  | Reserved |
|  | IRQ13 |  | Coprocessor |
|  | IRQ14 |  | Fixed Disk Controller |
|  | IRQ15 |  | Reserved |
| IRQ3 |  |  | Serial Port 2 |
| IRQ4 |  |  | Serial Port 1 |
| IRQ5 |  |  | Parallel Port 2 |
| IRQ6 |  |  | Diskette Controller |
| IRQ7 |  |  | Parallel Port 1 |