USER'S MANUAL

HIPPO DCA 2

VESA LOCAL BUS MOTHERBOARD

Hippo DCA2 486

486 VESA Local Bus Motherboard

User’s Manual

Version 2.25   December 1994

Copyright Notice

Copyright 1994 Ocean Information Systems, Inc.

All rights reserved.

Ocean Information Systems, Inc.

688 Arrow Grand Circle

Covina, CA  91722

Printed in the Hong Kong

All of the information contained in this manual is copyrighted and all rights reserved.  No part of this document, in whole or in part, may be reproduced or copied in any form without prior consent in writing from Ocean Information Systems, Inc.

_____________________________________________________________________________________

Limited Warranty

Buyer agrees  that if this product has any defects, Ocean Information Systems, Inc.  (OISI) is only obligated to replace or refund the purchase price for a period of one year from date of purchase.  OISI is not liable for any loss, direct or indirect, that may be incurred as a result of any defects.  The warranty does not cover any loss or damage caused by shipping, improper installation or maintenance, or any repairs made by anyone other than an authorized Ocean service center.

_____________________________________________________________________________________

Limitations of  Liability

While the information in this manual has been carefully reviewed and is believed to be accurate, OISI assumes no responsibility in the event that any inaccuracies are found.  In no event shall OISI be held liable for any loss or expenses whether directly or indirectly caused by the support materials provided with this product.  It is further acknowledged that OISI is under no obligation to update the manual or to notify purchasers of any forthcoming updates.

_____________________________________________________________________________________

Trademarks

AMD is a registered trademark of Advanced Micro Devices

AMI is a trademark of American Megatrend, Inc.

Fasmath is a registered trademark of Cyrix Corporation

Intel i386, i486, i486SX, i486DX, i486DX2 and Intel are registered trademarks of Intel Corporation.

MS-DOS, Xenix, Windows and Microsoft are registered trademarks of Microsoft Corporation.

MR BIOS is a trademark of Microid Research, Inc.

Novell is a registered trademark of Novell, Inc.

Octek is a registered trademark of  Ocean Information Systems, Inc.

OPTi is a registered trademark of OPTi Inc.

Unix is a registered trademark of  American Telephone and Telegraph Company Bell Laboratories.

VESA and VL-Bus are registered trademarks of Video Electronic Standard Association.

XT, AT, PS/2, OS/2, & IBM are registered trademarks of International Business Machines Corporation.

All other brand and product names mentioned in this manual are trademarks or copyrights of their respective holders.

Preface

About the Hippo DCA2

Thank you for purchasing the Hippo DCA2, Ocean Information Systems’new top-of-the-line 486 motherboard.  The Hippo DCA2 utilizes DynamiCache, an innovative cache memory technology that takes personal computing to a new level of performance.

DynamiCache was designed to eliminate bottlenecks between the CPU and the memory bank.  It does this by replacing traditional DRAM and external L2 cache with a memory bank that runs at CPU speed. With entire memory running at CPU speed the processor can handle multitasking just as easily as it handles single applications.

About the Manual

The content of this manual is for reference only and is intended to provide basic information for the general user.  Technical information is also included in the Appendix for hardware and software engineers.

This manual provides information about the installation and maintenance of the Octek Hippo DCA2 motherboard.  In-depth explanations concerning the functions and features of this motherboard are provided.  Chapter 1 is designed to give the user an overview of DynamiCache, including an in-depth explanation of  what it is as well as how it works.  Chapter 2 describes the features of the Hippo DCA2 motherboard.  It provides important information regarding the main memory system, ISA and VESA slots, the BIOS, and the CPUs supported.  Refer to Chapter 3 for instructions on how to setup and install the motherboard.  This chapter includes the necessary data for properly setting the pin connectors.  Requirements for setting up the BIOS are listed in Chapter 4.  An Appendix is also included that provides address maps as well as advanced technical information for hardware and software professionals.

Table of Contents

Preface	iii

Chapter 1:  DCA2 System Overview

1.1     What is DCA2 ?	1

1.2     What makes DCA2 so powerful ?	1

1.3     What is DynamiCache ?	1

1.4     Why is DCA2 a better choice ?	1

1.5     User advantages of DCA2	2

1.6     Technical advantages of DCA2	2

1.7     DynamiCache Description	3

1.8     DynamiCache Architecture 	3

1.9     Distributed Cache	4

1.10   DynamiCache Applications.	4

1.11   Application Overhead and its effect on External Cache	4

1.12   External Cache Saturation Problem	5

1.13   DynamiCache Excels and External Cache Suffers Under

Heavy Processing Loads	5

1.14   Target Operating Environments	5

1.15   L2/ DCA2 Cache Efficiency Versus Application Size	6

1.16   General Specifications Overview	7

Chapter 2:  Features

2.1    Layout of Hippo DCA2 board.	8

2.2    System Component Map	9

2.3    Jumper Description Table	10

2.4    DynamiCache	11

2.5    DRAM Memory.	11

2.6    Memory Configuration Table	12

2.7    Total System Memory	13

2.8    ISA Slots	13

2.9    VESA Local Bus	13

2.10  CPU	14

2.11  Processor Types Supported	14

2.12  BIOS.	14

2.13  Shadow RAM	15

2.14  Power Management	15

2.15  Software Compatibility	15

Chapter 3:  Setup and Installation

3.1     Setup	16

3.2     Installation Precautions	16

3.3     Operation and Maintenance	16

3.4     CPU Jumper Settings	17

3.5     CPU Installation	18

3.6     DynamiCache / DRAM Installation	19

3.7     Compatible 72-pin SIMM Types	20

3.8     Control of System Speeds	.21

3.9     Fan Voltage.	21

3.10   Fan connector	21

3.11   Reset CMOS buffer	21

3.12   Color / Mono display select.	21

3.13   Connectors	22

3.14   Reset Switch Connector	22

3.15   Turbo Switch Connector	22

3.16   Turbo Led Connector	23

3.17   Speaker Connector.	23

3.18   Keyboard Lock Connector	24

3.19   Keyboard Connector	24

3.20   External Battery Connector.	25

3.21   Power Supply Connector	25

Chapter 4:  BIOS Setup

4.1     BIOS Power-On Self-Test (POST)	27

4.2     AMI BIOS Beep Codes	27

4.3     Microid Research BIOS Beep Codes	28

Appendix:  Advanced Technical Information

A.1     Memory Address Map	29

A.2     I/O Address Map	30

A.3     I/O Extension Pinout	31

A.3.i     18-Bit ISA Pinout	31

A.3.ii     16-Bit ISA Extension Pinout	31

A.4     VL-BUS Extension Pinout	32

A.5     Direct Memory Access Channels	33

A.6     DMA Controller Registers	33

A.7     Page Register Address	34

A.8     System Interrupts	34

CHAPTER 1

SYSTEM OVERVIEW

Listed below are answers to some questions you may have about DCA2 and its phenomenal performance.

1.1	What is DCA2 ?

DCA2 (DynamiCache Architecture - 2nd generation) is a high performance cache memory system that alleviates the traditional bottleneck of mainboards.  This evolutionary system operates up to four times faster than standard DRAM or external cache based boards.  With DCA2 users at last have a means of enhancing their systems without the necessity of processor or external cache upgrades.

1.2	What makes DCA2 so powerful ?

Unlike external cache, which was implemented as a low-cost enhancement to standard DRAM, DCA2 was designed for pure performance.  DCA2 transforms the entire memory bank into DynamiCache, a high speed memory that operates at 15ns rather than 70ns.

1.3	What is DynamiCache ?

DynamiCache is a high speed memory module that boosts the entire memory bank so that it operates at the same speed as the CPU clocking rate.  The entire memory bank appears as cache to the CPU.

1.4	Why is DCA2 a better choice ?

DCA2 has a far greater impact and offers more benefits dollar for dollar than the best external cache based boards.  Programs such as Windows 3.11, OS/2 and Windows NT place extreme stress on external cache based systems, often causing bottlenecks between the CPU and the memory bank.  When the cache bank becomes full, the CPU must wait for new instructions to be loaded up from the slower DRAM memory.  With DCA2, the memory operates at the same speed as the CPU, allowing the CPU to retrieve instructions from any location within the main memory at zero wait states.

1.5	Users advantages of DCA2

- The entire memory bank acts as if its a cache subsytem.

- 15ns memory (standard DRAM only operates at 70ns).

- Zero wait states between the CPU and memory.

- Up to 200% memory performance increase with applications running in a DOS environment or other 16-bit operating systems.

- Up to 400% memory performance increase with applications run in an OS2 and NT environment or other 32-bit operating systems.

- DCA2 memory operates at CPU speed, making it four times faster than any other DRAM memory currently on the market.

1.6	Technical advantages of DCA2

- 15ns Access Time during Hit Cycle

- 15ns Posted Random Writes

- 35ns Random Read Access Page Miss

- Transparent Refresh during Cache Reads

- Hidden DRAM precharge during Page Reads

- Consumes 1/3 the power of SRAM plus DRAM solutions

- On-board cache Hit/Miss Comparator

- Write Posting Register with Direct Path to Memory Array

1.7	DynamiCache Description

The benefits of DynamiCache are easy to understand.  New operating systems such as Windows 3.11, Windows NT, OS/2, UNIX and their associated applications generate a new level of high stresses on conventional external caches, preventing the power of these Oss from being fully exploited.  Although these operating systems do offer the ability to multitask applications, these features are rarely used in practice as it causes even the best designed L2 cache motherboard to perform at unacceptably low levels.  DynamiCache, being able to access any instruction or data string from any physical address in memory at full cache speed, enables the CPU to handle multitasked operations as smoothly as it handles single applications.

1.8	DynamiCache Architecture

The DynamiCache chips are physically similar to a standard 4MB page mode or static column DRAM with the addition of an integrated Row Register Register and an internal controller that allows it to operate much like page mode or static column DRAM. DynamiCache’s Row Register register is tightly coupled with the Memory Array.  Memory Reads always occur from the Row Register Registers.  When the internal comparitor detects a page hit, only the Row Register register is accessed and data is made available in 15ns from the column address.  When a page read miss is detected, the new Memory Array row is loaded into the cache and data is available at the output all within a single 35ns access.  Subsequent reads within the page (burst read, local instructions or data) can continue at a 15ns cycle time.

Figure 1

Since reads occur from the Row Register register, the Memory Array Precharge can occur simultaneously without degrading performance.  The on chip refresh counter with an independent refresh bus allows the DynamiCache to be refreshed during cache reads. Memory Writes are internally posted in 15 nSecs and are directed to the DRAM array.During a write hit, the on chip comparator activates a parallel write path to the Row Register register to maintain coherency.  The DynamiCache delivers 15nSec page mode memory writes.  This high level of performance is achieved with a single non-interleaved memory consisting of as few as eight 1M x 4 components or a single 72-pin 4Mbyte DynamiCache module.

1.9	Distributed Cache

DynamiCache uses what is referred to as distributed cache rather than lumped cache. The 15 ns access time is available throughout the entire range of installed DynamiCache, not just in a small external cache.  External cache only provides benefit to the system while the instructions are housed in the cache.  The penalties that are usually associated with external cache, such as write back latencies, cache thrashings and cache saturation are not present in DynamiCache.  This architecture provides all of the cache benefits for the entire range of installed memory with none of the cache penalties.

1.10	DynamiCache Applications

DynamiCache was designed with the new emerging 32-bit protected mode operating systems in mind.  External cache, prior to 1994 was an acceptable cost based solution for enhancing DOS applications that typically occupied a memory space that was less than the size of the installed external cache.  If the CPU requested an instruction or a string of data, this information was made available to the CPU at cache speed (zero wait state).  With the advent of Windows, however, the typical application became larger and much more complex supporting virtual disks, DLLs and GUI interfaces.

1.11	Application Overhead and its effect on External Cache

These new applications subsequently brought with them tremendous overhead for the operating systems.   OS/2 and Windows NT were created to handle the demands of the new applications with the hopes that a pre-emptive multitasking, multithreading 32 bit OS would break the bottleneck to productivity.  It was quickly found that these operating systems were only effective on high end systems, such as DX2, Pentium or RISC platforms.  Even with top end CPUs, a large main DRAM bank was needed, typically 16Mbytes or more and a very expensive L2 cache was often employed, usually 256K to 512K.

1.12	External Cache Saturation Problem

Cache saturation is an easy concept to understand.  If the executing program is smaller than the external cache I have installed, then no saturation occurs.  There will always be memory address space available for the execution of this program.  If the program is slightly larger than my L2 cache, saturation may still not occur, provided that the cache is well designed.  There is a point, however, when the L2 cache will remain constantly saturated with instructions.  Some of these may be in the process of being read from the main memory and some will be waiting to be written back to memory.  The net effect is that if the program is large enough, the L2 cache is kept filled with data on a continuous basis.  It is the maintenance of this traffic flow that dramatically reduces the amount of time the CPU would otherwise have for executing software instructions.

1.13	Dynamicache Excels and External Cache Suffers Under Heavy Processing Loads

Heavy processing loads can easily reduce a 66Mhz CPU and L2 cache combination to effective processing speeds as little as 20% of the CPU’s capabilities. A CPU under these circumstances spends as much as 80% of its time squeezing large amounts of data to and from memory through a very small SRAM cache.  External cache, under these high stress conditions is kept in a continuous state of flux.  This is the point at which a cache mechanism ceases to be effective in providing any benefit to system performance.

This is the state in which the cache has become saturated.  If the processing demands from the CPU increase any further, the external cache actually creates a penalty in performance by virtue of it’s memory management overhead.  DynamiCache does not suffer from these penalties.

1.14	Targeted Operating Environments

DCA2 is designed with the following target operating systems, and application environments:

- Multitasking Mutlithreding ( Windows 3.11, Windows NT, OS/2 2.11, etc.)

- Large Applications ( Desktop publishers, graphic designers, databases/spreadsheets )

- I/O Intensive computing ( Graphics applications, multimedia, disk and data transfers )

- CPU Bound Computing ( CAD/CAM, MATHCad and other scientific number crunchers )

- Software Compilers

- Application Servers ( Networked applications accessed from a central server )

- File Servers ( Novel, Lantastic, LANManager, Vines )

- DynamiCache is fully compatible with all the major operating systems and application.

1.15	L2/ DCA2 Cache Efficiency Versus Application Size

.

Where do the software applications fit on this curve ?

- Typical Benchmarks usually utilize less than 5% memory load.

- DOS applications typically generate less than 25% memory load.

- Windows 3.11 applications typically generate approximately 50% memory load.

- OS/2 2.11, UNIX and Windows NT easily generate 50% to 100% memory load

- Windows NT Advanced Server and Novell Application Servers routinely use 90% to 100% load.

* Memory Loads do not include virtual disk accesses.

1.16	General Specifications Overview

Processor:

w Processor Type		Intel  486SX-25/33, 486DX-25/33, 486DX2-50/66

Intel   486DX4-75/100, Intel P24T

AMD  486DX-40, 486DX2-50/66

Cyrix  486DX-33, 486DX2-50/66

w Speed			25 / 33 / DX2-50 / DX2-66 / DX4-100

w CPU voltage detection	 Automatic voltage conversion circuitry

Cache Architecture:

w Internal Cache 		8KB 4-way set associative cache (SX/DX/DX2)

16KB 4-way set associative cache (DX4)

w External Cache 		DynamiCache Architecture II (DCA2) running at CPU speed

DynamiCache running at 15ns

Memory Subsystem:

w DynamiCache Simm sockets	2 x 72 pin  4MB / 8MB / 16MB DynamiCache modules

w DRAM Simm sockets		2 x 72 pin  4MB / 8MB / 16MB / 32MB DRAM modules

w Max. Memory Size		96MB

w Mixed Memory Type		Support mixed DynamiCache / DRAM modules

w Memory Width		32-bit DynamiCache & DRAM

w Enhancement			Fast Page Mode and Hidden Refresh supported

BIOS Subsystem

w BIOS Type			AMI / Microid Research

w BIOS Shadowing		Shadow RAM for System and Video BIOS

w BIOS Features		Built-in setup, Power-on self test, Drive table optimization, User-					definable drive types, Password protection, Shadowing options

Input/Output Subsystem

w VESA bus slots		3 x 32-bit VL-Bus slots (2 slave, 1 master/slave)

w ISA bus slots			6 x 16-bit  ISA slots

2 x 8-bit  ISA slots

w I/O bus speed			Up to 33MHz (VESA bus)

Power Management

w Green functions		Support Green Functions (Hardware controlled)

Voltage regulation for DX4 CPUs

System Support Functions

w System functions		7 DMA channels, 16 level interrupts, Programmable timers

w Support functions		Fast A20 gate and Fast Reset

w Clock				Enhanced real time clock/calendar with battery back-up

Other Features

w Power good			On board power good signal generation

w Switches			Hardware Turbo, Reset, Keylock  switches

w Size				8.5" (W) x 11" (L)