CHAPTER 4

BIOS SET UP

4.1	BIOS Power-On Self-Test (POST)

During the self-test, errors may occur.  These errors are communicated through a series of beeps.  The method of beeps used depends on the BIOS program.  For example, Microid Research BIOS uses a combination of low and high tones.  AMI BIOS uses a series of beeps with different error messages that correspond to the number of beeps heard.

4.2	AMI BIOS Error Beep Codes

| # of Beeps | Error Message | Explanation |
| --- | --- | --- |
| 1 | DRAM Refresh Failure | The memory refresh circuitry is faulty |
| 2 | Parity Error | A parity error occured in the first 64KB block of memory |
| 3 | Base 64K RAM Failure | A memory failure occurred in the first 64KB of memory |
| 4 | System Timer Failure | Timer 1 is not functioning |
| 5 | Processor Failure | An error occurred in the CPU |
| 6 | Gate A20 Failure | The BIOS is unable to switch the CPU into protected mode |
| 7 | Processor Exception Error | An exception interrupt occured in the CPU |
| 8 | Display Memory Read/Write Error | The system video adapter is missing or its memory is faulty |
| 9 | ROM Checksum Error | The ROM checksum value does not match the BIOS code |
| 10 | Read/Write Error | The CMOS shutdown register memory has failed |

4.3	Microid Research BIOS Error Beep Codes

| Beep Tones | Error Message |
| --- | --- |
| L H L H H | Real time clock is not updating |
| L H-L L L | ROM BIOS checksum test |
| L H-H L L | Page register test |
| L H-L H L | Keyboard controller self-test |
| L H-H H L | Memory refresh circuit test |
| L H-L L H | Master DMA controller failure |
| L H-H L H | Slave DMA controller failure |
| L H-L L L L | Memory bank 0 pattern test failure |
| L H-H L L L | Memory bank 0 parity circuitry failure |
| L H-L H L L | Memory bank 0 parity error |
| L H-H H L L | Memory bank 0 data bus failure |
| L H-L L H L | Memory bank 0 address failure |
| L H-H L H L | Memory bank 0 block access read failure |
| L H-L H H L | Memory bank 0 block access read/wrfitge failure |
| L H-H H H L | Master 8259 failure |
| L H-L L L H | Slave 8259 failure |
| L H-H L L H | Master 8259 interrupt address error |
| L H-L H L H | Slave 8259 interrupt address error |
| L H-H H L L | 8259 interrupt address error |
| L H-L L H H | Master 8259 stock interrupt error |
| L H-H L H | Slave 8259 stock interrupt error |
| L H-L H H H | System timer 8254 interrupt failure |
| L H-H H H H | 8254 channel 0 test and initialization |
| L H-L L L L H | 8254 channel 2 speaker failure |

|  | After the power is on, wait for a minute before proceeding. The system BIOS are going through a self-test and nothing is displayed on screen. After the self-test, the system BIOS initialize the display adaptor and show messages. |
| --- | --- |