Hippo II 486 Motherboard

Supplementary

==================

1.	The following options are cancelled :

|  | Reference |
| --- | --- |
| Memory Parity Error Check | Page A-12 |
| Wait for F1 if any error | Page A-13 |
| Keyboard Reset Control | Page A-17 |
| Cache Write w/s | Page A-18 |

2.	The following options are modified :

a.	Hit <ESC> Message Display   (Page A-12)

It is changed to <DEL> key, and the message is :

"Hit <Del> if you want to run SETUP"

b.	Hard Disk Type 47 Data Area   (Page A-12)

The statement shown below is appended to the description :

"When System ROM shadowed (F000) is enabled, the BIOS will add the user-defined hard disk types to the hard disk type table in the system BIOS.  It is no need to store the data for the user-defined hard disk type in the separate area.  So the option will be ignored."

c.	Hidden Refresh   (Page A-16)

The statement below is appended to the description :

"When 4 MB DRAM is used, the hidden refresh should be disabled."

3.		The following option is improved :

Memory configuration

256KB SIMM is supported.  The following memory configurations are added :

| Bank 0 | Bank 1 | Total |
| --- | --- | --- |
| 256K | --- | 1M |
| 256K | 256K | 2M |
| 256K | 1M | 5M |

IMPORTANT NOTE :

Using 4MB SIMM

Hidden refresh is not supported for 4MB DRAM.  So you must disable hidden refresh in the BIOS Setup when using 4MB DRAM.