Hippo II 486 Motherboard

Supplementary

Oct. 01,91.

==================

1.	The following options are cancelled :

|  | Reference |
| --- | --- |
| Memory Parity Error Check | Page A-12 |
| Wait for F1 if any error | Page A-13 |
| Keyboard Reset Control | Page A-17 |
| Cache Write w/s | Page A-18 |

2.	The following options are modified :

a.	Hit <ESC> Message Display   (Page A-12)

It is changed to <DEL> key, and the message is :

"Hit <Del> if you want to run SETUP"

b.	Hard Disk Type 47 Data Area   (Page A-12)

The statement shown below is appended to the description :

"When System ROM shadowed (F000) is enabled, the BIOS will add the user-defined hard disk types to the hard disk type table in the system BIOS.  It is no need to store the data for the user-defined hard disk type in the separate area.  So the option will be ignored."

c.	Hidden Refresh   (Page A-16)

The statement below is appended to the description :

"When 4 MB DRAM is used, the hidden refresh should be disabled."

d.	At Bus Clock Selection

There are four options :

| CLK I | AT BUS CLOCK |
| --- | --- |
| /6 | 5.56 MHz |
| /4 | 8.33 MHz |
| /3 | 11.11 MHz |
| /2.5 | 13.33 MHz |

At Bus Clock is not recommended to be faster than 8.33 MHz.

3.		The following option is improved :

Memory configuration

256KB SIMM is supported.  The following memory configurations are added :

| Bank 0 | Bank 1 | Bank 2 | Bank 3 | Total |
| --- | --- | --- | --- | --- |
| 256K | --- | --- | --- | 1M |
| 256K | 256K | --- | --- | 2M |
| 1M | --- | --- | --- | 4M |
| 256K | 1M | --- | --- | 5M |
| 256K | 256K | 1M | --- | 6M |
| 1M | 1M | --- | --- | 8M |
| 256K | 1M | 1M | --- | 9M |
| 256K | 256K | 1M | 1M | 10M |
| 1M | 1M | 1M | --- | 12M |
| 256K | 1M | 1M | 1M | 13M |
| 1M | 1M | 1M | 1M | 16M |
| 4M | --- | --- | --- | 16M |
| 1M | 4M | --- | --- | 20M |
| 4M | 1M | --- | --- | 20M |
| 1M | 1M | 4M | --- | 24M |
| 1M | 4M | 1M | --- | 24M |
| 4M | 1M | 1M | --- | 24M |
| 1M | 1M | 1M | 4M | 28M |
| 1M | 4M | 1M | 1M | 28M |
| 4M | 1M | 1M | 1M | 28M |
| 1M | 4M | --- | --- | 32M |
| 1M | 4M | 4M | --- | 36M |
| 4M | 1M | 4M | --- | 36M |
| 4M | 4M | 1M | --- | 36M |
| 4M | 1M | 4M | 4M | 40M |
| 1M | 4M | 4M | 1M | 40M |
| 4M | 1M | 4M | 1M | 40M |
| 4M | 4M | 1M | 1M | 40M |
| 4M | 4M | 4M | --- | 46M |
| 1M | 4M | 4M | 4M | 52M |
| 4M | 1M | 4M | 4M | 52M |
| 4M | 4M | 4M | 1M | 52M |
| 4M | 4M | 4M | 4M | 64M |

Bank 2 and Bank 3 are on the memory expansion card

Important Note :

Using 4MB SIMM

Hidden refresh is not supported for 4MB DRAM.  So you must disable hidden refresh in the BIOS Setup when using 4MB DRAM.

4.		Jumper Setting :

a.	JP5 at Bus Clock after Power Up

| JP5 |  |
| --- | --- |
| 1-2 | 5.55 MHz |
| 2-3 | 8.33 MHz |

b.	JP10 Reset COMS Setup

| JP10 |  |
| --- | --- |
| 1-2 | Reset CMOS Setup |
| 2-3 | Normal |

c.	JP9 Display Selection

| JP9 |  |
| --- | --- |
| 1-2 | Color |
| 2-3 | Monochrome |

d.	Reserved Jumper

| JP2 | JP3 | JP4 | JP6 |
| --- | --- | --- | --- |
| 1-2 | 1-2 | 2-3 | 1-2 |

5.		Cache Memory Installation

These jumpers are ignored if " External CACHE Memory " option in the BIOS setup is disabled.

| Memory Cache Size | JP1 | JP7 | JP8 |
| --- | --- | --- | --- |
| 64 KB | 2-3 | 1-2 | 1-2 |
| 256 KB | 1-2 | 2-3 | 2-3 |

For 64 KB, place 8K x 8  25 ns SRAM on U2, U3, U4, U5, U10, U11, U12 and U13.

For 256 KB, place 32K x 8  25 ns SRAM on these locations.

The IC on U28, U29 and U30 may be installed according to the CACHE Memory size.  The speed is 15 ns.

|  | U28 16Kx4 | U29 16Kx4 | U30 8Kx8 |
| --- | --- | --- | --- |
| 64K | Y | Y | - |
| 64K | - | - | Y |
| 256K | Y | Y | - |

A 64Kx1 15 ns SRAM must be place on U31 if CACHE Memory is enabled.

6.		System Board Connector

|  | Description |
| --- | --- |
| P1 | Hardware reset connector |
| P2 | Speaker connector |
| P3 | Turbo switch |
| P4 | Turbo LED |
| P5 | Power LED & Ext-lock connector |
| P6,P7 | Power supply connector |
| P8 | External battery connector |

7.		Board Layout (Rev. 2.0 and above)