Hippo III 486 Mainboard

Erratum for User's Manual

Rev. 1.0

(March 23, 92)

================

There are typing mistakes in the User's Manual Rev. 1.0 for Hippo III 486.  Please correct as the following shown.  It is also true in the Summary of Jumper Setting.

Configuration of Cache Memory  (P. 3-3)

| Cache Size | JP2 | JP7 | JP8 |
| --- | --- | --- | --- |
| 64KB | 2-3 | 2-3 | 2-3 |
| 256KB | 1-2 | 1-2 | 1-2 |

System Board Jumper Setting  (P. 3-9)

CPU Type

|  | 486DX | 486SX | 487SX |
| --- | --- | --- | --- |
| JP4 | 1-2 | 2-3 | 1-2 |
| JP3 | 1-2 | 2-3 | 1-2 |
| JP1 | 2-3 | NO | 1-2 |