HIPPO OP

486

The material in this manual is for information only and is subject to change without notice.

REVISION: 1.0

IBM, IBM PC/XT/AT, PC-DOS, MS-DOS, OS/2, INTEL, AMI ARE THE TRADEMARKS OR REGISTERED TRADEMARKS OF THEIR RESPECTIVE OWNERS.

RADIO FREQUENCY INTERFERENCE STATEMENT

This equipment generates and uses radio frequency energy and if not installed and used properly, that is, in strict accordance with the manufacturer's instructions, may cause interference with radio and television reception.

If this equipment does cause interference to radio or TV reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient the receiving antenna.

*	Relocate the computer away from the receiver.

*	Move the computer away from the receiver.

*	Plug the computer into a different outlet so that computer and receiver are on different branch circuits.

*	Ensure that card slot covers are in place when no card is installed.

*	Ensure that card mounting screws, attachment connector screws, and ground wires are tightly secured.

*	If peripherals are used with this system, it is suggested to use shielded, grounded cables, with in-line filters if necessary.

If necessary, the user should consult the dealer service representative for additional suggestions.

The manufacturer is not responsible for any radio or TV interference caused by unauthorized modifications to this equipment.  It is the responsibility of the user to correct such interference.

Table of Content

Chapter 1	Introduction	1-1

Chapter 2	General Features	2-1

Chapter 3	Installation

Installing DRAM modules	3-1

Installing the cache module	3-1

Jumper setting	3-2

System board connectors	3-8

Chapter 4	Technical Information

Memory map	4-1

I/O map	4-2

System interrupts	4-4

Direct memory access	4-5

Real time clock and CMOS RAM	4-6

VL slot pinout	4-7

ISA slot pinout	4-7

Chapter 1

Introduction

___________________________________________

The usage of PC systems in the office environment has seen dramatic increase over this few years. With increase in the power consumption due to growth of the PC installation base, the United States Environmental Protection Agency (EPA) issued a guideline stating that systems must have the capability  to be put into a low power state (less than 30 W) during period of inactivity to conserve energy. Therefore the major benefit of incorporating power management to desktop PCs is to reduce electrical operating cost over the lifetime of the system.

OCTEK HIPPO OP brings the new technology of power saving to desktop computing. Compliance with the EPA requirement, high throughput VESA Local Bus, advanced chipset with power user-programmable features and integrated on-board I/O are some of the real benefits of HIPPO OP.

The key components of the advanced power-saving capabilities are HIPPO OP's built-in support of DPMS functions (Display Power Management Standard) of VESA (Video Electronics Standard Association), support for Windows APM and CPU clock step-down hardware. The energy saving feature of HIPPO OP is compatible with MS-DOS 3.3 (or later versions) and Windows 3.1.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 2

General Features

___________________________________________

Specifications:

Processor:

80486DX, 80486DX2, 80486SX or 80487SX

Processor Clock:

25/33/40/50 MHz clock selectable

Speed:

Turbo/normal speed

Expansion Bus:

Six 16-bit ISA slots

Three VESA Local Bus slots (supporting one 	VL master)

On-board VESA local bus device:

VL-IDE controller

DRAM upgrade capacity:

1  MB (min. config.)

32 MB (max. config.)

DRAM configuration:

256K X 9, 1M X 9, 4M X 9 SIMM sockets

Cache :

8 KB internal cache in CPU

SIMM socket for installation of a 256KB 	secondary cache module

On-board I/O facility:

floppy controller,

2 serial ports,

1 parallel port,

1 game port,

Power Saving Feature:

CPU clock step down hardware

BIOS support of VESA DPMS

Others:

Fast GateA20 and reset emulation

The 80486 CPU

In simple terms, an 80486 is a combination of a microprocessor (80386), a floating point math coprocessor (80387), a cache controller and 8KB of cache RAM, all in one chip.

The 80486 CPU is  binary-compatible  with the  8086, 8088 , 80186, 80286, 80386DX and 80386SX CPUs. Software developed for older members of the 80x86 family (such as the 8086 and the 80286) can be run without modification on the 80486.  Frequently used instructions can be executed in  one clock cycle owing to architectural enhancement:

1. 	an 8 KB on-chip cache

2. 	pipelined instruction execution

3. 	most 486s (DX, DX2)  contain on-chip floating point units

The burst bus interface of the 80486 enables it to attain a transfer rate  of 106 MB/sec at 33  MHz CPU clock.  Burst cycles are done at the  continuous rate of one 32-bit  transfer per clock. Another advanced feature is the implementation of the write-buffers. If the bus is currently busy, memory writes are directed to one of the four write-buffers. This write buffering allows the 80486 to continue the next instruction.

In summary, the 80486 CPU provides a high performance level and retains compatibility with a large base of existing software.

The DRAM system

The memory controller supports fast page mode DRAM accesses. This feature is instrumental in sustaining a high memory bandwidth between the DRAM array and the 486 operating in burst mode.

Both normal and hidden refresh are supported.  Normal refresh refers to the classical refresh scheme in which the CPU is held while refresh cycles take place in the local DRAM and the ISA bus memory. Hidden refresh scheme allows the CPU to continue executing instructions during DRAM refresh period as long as the CPU does NOT access DRAM or the ISA bus. Refresh overhead is therefore minimized.

Shadow RAM is available as an option. System BIOS and video BIOS residing in slow EPROM can be copied to local DRAM to speed up accesses to BIOS code. Shadow RAM addresses range from C0000h to FFFFFh. 16K granularity is provided for address range C0000h to EFFFFh, while the area F0000h - FFFFFh can only be shadowed as a whole. Video BIOS at C0000h - C7FFFh can be cached in the external cache after shadowing.

DRAM CONFIGURATION

| Bank 0 | Bank 1 | Total |
| --- | --- | --- |
| 256K x 32 | --- | 1M |
| 256K x 32 | 256K x 32 | 2M |
| 256K x 32 | 1M x 32 | 5M |
| 256K x 32 | 4M x 32 | 17M |
| 1M x 32 | --- | 4M |
| 1M x 32 | 1M x 32 | 8M |
| 1M x 32 | 4M x 32 | 20M |
| 4M x 32 | --- | 16M |
| 4M x 32 | 4M x 32 | 32M |

80ns DRAM can be used in all operating frequencies.

The external cache system

An external cache module is available to further boost the system performance. The external cache is direct-mapped and employs a write-back scheme. The cache size is 256 KB and organized in interleaved banks.

VESA  Local  Bus

Connecting devices to a CPU local bus can dramatically  increase the speed of the I/O bound peripherals. The Video Electronics Standards Association (VESA) VL Bus is a widely accepted local bus standard.

Maximum number of  VL-Bus devices and slots

The following guidelines are recommended by VESA:

| Speed | Max. no. of VL adapter cards installed |
| --- | --- |
| 40 MHz | 2 |
| 50 MHz | 1 |

One VL master adapter card may be installed in slot J2 OR slot J3

The on-board VL IDE

The VL Local Bus IDE controller supports high speed data transfer (ANSI ATA standard mode0, mode 1, mode 2). The cycle time  of the IDE  cycle is jumper selectable.  The chip incorporates 4-level write FIFO and 4-level prefetch FIFO with write-posting. This allows IDE cycles and CPU cycles to run concurrently.  This parallelism (or cycle overlapping) is a  key component in speeding up transfer rate. Additional fine-tuning of the IDE cycle timing via registers allow further performance optimization.

Fast GATEA20 and RESET Emulation

Commands to ports 60h and 64h are intercepted so that the the functions of the GATEA20 and RESET of the keyboard controller are emulated. This decode sequence is software transparent and requires no BIOS modifications. The result is that the protected mode switching operation is much faster.

The on-board I/O  controller

The on-board I/O controller offers

1. 	support for two floppy drives ( 360K  5 1/4", 1.2M  5 1/4", 720K  3 1/2", 144M 3 1/2" )

2. 	one parallel port

3. 	two serial ports

4. 	game port

Control of system speed

System speed can be controlled by the keyboard and the turbo switch. When the keys "Ctrl", "Alt", "-" are pressed simultaneously, the system switches to low speed. When the keys "Ctrl", "Alt", "+" are pressed at the same time, the system returns to high speed mode (turbo mode).

The header P4 should be connected to the turbo switch of the cabinet. Whenever the system speed is set low by the turbo switch, the key strobes "Ctrl", "Alt", "+" and "Ctrl", "Alt", "-" will have NO impact on the system speed.

Power Saving Features

Whenever the system is not in used  (e.g. No key has been hit in the keyboard for a certain period of time),  the clock chip of the main board will be signaled to slow down the CPU clock. The system subsequently enters a "sleep" mode. When an external request for system usage is detected (as the pressing of a key or the movement of  a mouse), the clock is stepped up to full operating frequency. The time limit  for entry to "sleep" mode is selectable in CMOS setup.

IDE drive spin-down is also controllable.  The fixed disk will stop spinning after a pre-selected time interval of inactivity.

The Video Electronics Standards Association (VESA) has  developed a  preliminary standard Display Power Management Signaling (DPMS) specification, which utilizes the synchronization signals as an indication that the monitor should power down.

| State | Horizontal sync | Vertical sync, | Video |
| --- | --- | --- | --- |
| ON | pulses | pulses | active |
| standby | no pulses | pulses | blanked |
| Suspend | pulses | no pulses | blanked |
| OFF | no pulses | no pulses | blanked |

New VGA adapters with video BIOS supporting VESA DPMS can be installed in the HIPPO OP to reduce power consumption in the monitors.

Reset CMOS setup information

Sometimes it may be desirable to clear the existing content of the CMOS RAM. This may be accomplished by allowing the shunt connector to join pin 2 and pin 3 of the header at JP29 for a while. When the system is re-booted, CMOS RAM content is regarded as invalid and the user is prompted to setup the CMOS again.

Chapter 3

Installation

______________________________________________

Installing DRAM modules

Follow  the steps below to install DRAM SIMMs :

(a) 	Turn off the PC.

(b) 	The SIMMs (in groups of four) are inserted in Bank 0 first. The memory modules should be carefully installed into the SIMM sockets in such a way that the modules are properly oriented and aligned.

(c) 	Allow the two metal latches at each end of the socket to lock onto the memory module.

(d) 	Further memory capacity upgrade should be directed to Bank 1.

Installing the cache module

Follow the steps below to install the cache module.

(a) 	Turn off the PC

(b) 	The cache module is inserted the 72-pin cache socket.

(c) 	Allow the two metal latches at each end of the socket to lock onto the cache module.

(d) 	Install the jumper JP5.

(e) 	Power up the PC.

(f) 	Enable the external cache in the CMOS setup.

(g) 	Allow the system to boot.

Jumper Setting

Cache configuration

| JP5 |  |
| --- | --- |
| 1-2 | Single cache bank |
| 2-3 | Two cache banks (default) |

Clock selection

| JP3 | JP4 | JP30 | Clock Frequency | CPU |
| --- | --- | --- | --- | --- |
| 2-3 | 1-2 | 1-2 | 50MHz | DX50 |
| 2-3 | 2-3 | 2-3 | 33MHz | DX33,DX2-66 |
| 1-2 | 2-3 | 2-3 | 40MHz | DX40 |
| 1-2 | 1-2 | 2-3 | 25MHz | DX25,DX2-50 |

CMOS content of the Real Time Clock

| JP29 |  |
| --- | --- |
| 1-2 | CMOS content preserved (default) |
| 2-3 | CMOS content cleared |

Color/mono selection

| JP6 |  |
| --- | --- |
| 1-2 | mono |
| 2-3 | color (default) |

on-board floppy controller

| JP21 |  |
| --- | --- |
| 1-2 | enabled (default) |
| 2-3 | disabled |

on-board game port

| JP22 |  |
| --- | --- |
| 1-2 | enabled (default) |
| 2-3 | disabled |

on-board printer port

| JP23 |  |
| --- | --- |
| 1-2 | enabled (default) |
| 2-3 | disabled |

Printer port selection

| JP24 |  |
| --- | --- |
| 1-2 | LPT1 (378H) (default) |
| 2-3 | LPT2 (278H) |

Serial port 1

| JP25 |  |
| --- | --- |
| 1-2 | enabled (default) |
| 2-3 | disabled |

| JP26 |  |
| --- | --- |
| 1-2 | COM1 (3F8 - 3FFh) (default) |
| 2-3 | COM3 (3E8 - 3EFh) |

Serial port 2

| JP28 |  |
| --- | --- |
| 1-2 | enabled (default) |
| 2-3 | disabled |

| JP27 |  |
| --- | --- |
| 1-2 | COM2 (2F8 - 2FFh) |
| 2-3 | COM4 (2E8 - 2EFh) |

ROM

| JP7 |  |
| --- | --- |
| 1-2 | EPROM installed (default) |
| 2-3 | Flash ROM installed |

VL slot configuration

| JP1 |  |
| --- | --- |
| 1-2 | 1 WS VL-write cycle |
| 2-3 | 0 WS VL-write cycle |

| JP31 | VL-bus ID3 |
| --- | --- |
| 1-2 | VL slot clock greater than 33.333MHz |
| 2-3 | VL slot clock less than or equal to 33.333MHz (default) |

CPU Type

|  | JP18 | JP19 | JP20 |
| --- | --- | --- | --- |
| 486DX, 486DX2 | 2-3 | 1-2 | 1-2 (default) |
| 486SX | 1-2 | 2-3 | 2-3 |

Reserved jumpers

JP8		2-3	(DEFUALT)

JP11		1-2	(DEFAULT)

JP10		2-3	(DEFAULT)

JP33		1-2	(DEFAULT)

on-board IDE

| JP12 | JP17 |  |
| --- | --- | --- |
| 1-2 | 1-2 | enabled (default) |
| 2-3 | 2-3 | disabled |

| JP13 |  |
| --- | --- |
| 1-2 | primary IDE (1FXh, 3FXh) |
| 2-3 | secondary IDE (17Xh, 37Xh) |

| JP14 | JP15 | 16-bit cycle time |
| --- | --- | --- |
| 1-2 | 1-2 | > 200ns (mode 3) |
| 1-2 | 2-3 | > 240ns (mode 2) |
| 2-3 | 1-2 | > 383ns (mode 1) |
| 2-3 | 2-3 | > 480ns (mode 0) default setting |

| JP16 | 8-bit cycle time |
| --- | --- |
| 1-2 | > 600ns |
| 2-3 | > 840ns (default) |

| JP2 | on-board local bus IDE write wait state |
| --- | --- |
| 1-2 | wait state determined by the setting of JP1 |
| 2-3 | forced to 1 wait state (default) |

System board connectors

|  | Description |
| --- | --- |
| P1 | Hardware reset switch |
| P2 | Speaker connector |
| P3 | Turbo LED connector |
| P4 | Turbo switch |
| P5 | Hard disk LED connector |
| P6 | Power LED and keylock connector |
| P7 | IDE hard disk connector |
| P8 | Floppy drive connector |
| P9 | Parallel port connector |
| P10,P11 | Power connectors |
| P12 | Primary Serial port connector |
| P13 | Secondary Serial port connector |
| P14 | Game port connector |
| P15 | External Battery connector |
| KB1 | Keyboard connector |

P1	Hardware reset switch

| Pin | Assigment |
| --- | --- |
| 1 | Signal pin |
| 2 | GND |

P2	Speaker connector

| Pin | Assignment |
| --- | --- |
| 1 | Data out |
| 2 | +5V |
| 3 | Ground |
| 4 | +5V |

P3	Turbo LED connector

| Pin | Assignment |
| --- | --- |
| 1 | +5V |
| 2 | LED signal |

P4	Turbo switch

| Pin | Assignment |
| --- | --- |
| 1 | Signal pin |
| 2 | Ground |

P6	Power LED & keylock connector

| Pin | Assignment |
| --- | --- |
| 1 | +5V |
| 2 | Key |
| 3 | Ground |
| 4 | Keyboard inhibit |
| 5 | Ground |

P10, P11	Power connectors

| Pin | Assignment |
| --- | --- |
| 1 | Power Good |
| 2 | +5V |
| 3 | +12V |
| 4 | -12V |
| 5 | Ground |
| 6 | Ground |

| Pin | Assignment |
| --- | --- |
| 1 | Ground |
| 2 | Ground |
| 3 | -5V |
| 4 | +5V |
| 5 | +5V |
| 6 | +5V |

Chapter 4

Technical Information

__________________________________________________

This section provides technical information about HIPPO OP and is particularly useful to advanced users.

Memory map

| Address | Range | Function |
| --- | --- | --- |
| 0 - 9FFFFh | 0 - 640K | Base Memory |
| A0000 - BFFFFh | 640 - 768K | Video Display Buffer |
| C0000 - DFFFFh | 768 - 896K | Adapter ROM |
| E0000 - EFFFFh | 896 - 960K | System ROM (optional) |
| F0000 - FFFFFh | 960 - 1024K | System ROM |
| 100000 - 1FFFFFFh | 1024 - 3276K | Extended Memory |

I/O Map

| ADDRESS | DEVICE |
| --- | --- |
| 0-1Fh | DMA Controller 1 |
| 20-3Fh | Interrupt Controller 1 |
| 40-5Fh | Timer |
| 60-6Fh | Keyboard Controller |
| 70-7Fh | Real Time Clock |
| 80-9Fh | DMA Page Register |
| A0-BFh | Interrupt Controller |
| C0-DFh | DMA Controller 2 |
| F0 | Clear Math Coprocessor Busy flag |
| F1 | Reset Math Coprocessor |
| F8-FFh | Math Coprocessor Port |

I/O Map

| ADDRESS | DEVICE |
| --- | --- |
| 170-177h | Fixed Disk (secondary) |
| 1F0-1F7h | Fixed Disk (primary) |
| 200-207h | Game port |
| 278-27Fh | Parallel port 2 |
| 2E8-2EFh | Serial port 4 |
| 2F8-2FFh | Serial port 2 |
| 300-31Fh | Prototype card |
| 360-36Fh | Reserved |
| 370-377h | Floppy Disk Controller (secondary) |
| 378-37Fh | Parallel port 1 |
| 380-38Fh | SDLC, bisync 2 |
| 3A0-3AFh | Bisync 1 |
| 3B0-3BFh | Monochrome Display and Printer Adapter |
| 3C0-3CFh | Reserved |
| 3D0-3DFh | Color Graphics Adapter |
| 3E8-3EFh | Serial port 3 |
| 3F0-3F7h | Floppy Disk Controller (primary) |
| 3F8-3FFh | Serial port 1 |

Level					Function

NMI		Parity or I/O Channel Check

Interrupt Controllers

Controller 1	Controller 2

IRQ0				Timer Output 0

IRQ1				Keyboard

IRQ2

IRQ8		Real-time Clock Interrupt

IRQ9		Software Redirected to

INT 0AH (IRQ2)

IRQ10	Reserved

IRQ11	Reserved

IRQ12	Reserved

IRQ13	Coprocessor

IRQ14	Fixed Disk

IRQ15	Reserved

IRQ3				Serial Port 2

IRQ4				Serial Port 1

IRQ5				Parallel Port 2

IRQ6				Diskette Controller

IRQ7				Parallel Port 1

┌──────────────────────────────────────┐

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│     ─┐                               │

│      │ ┌─                            │

│      │ │                             │

│      │ │                             │

│      │ │                             │

│      └─┤                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        └─                            │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

└──────────────────────────────────────┘

DMA channel

| Channel | Function |
| --- | --- |
| 0 | Spare (8 bit) |
| 1 | SDLC (8 bit) |
| 2 | Floppy Disk (8 bit) |
| 3 | Spare (8 bit) |
| 4 | Cascade for DMA Controller 1 |
| 5 | Spare (16 bit) |
| 6 | Spare (16 bit) |
| 7 | Spare (16 bit) |

CMOS RAM address map

| Addresses | Description |
| --- | --- |
| 00-0Dh | Real-time clock information |
| 0Eh | Diagnostic status byte |
| 0Fh | Shutdown status byte |
| 10h | Diskette drive type byte - drives A and B |
| 11h | Reserved |
| 12h | Fixed disk type byte - drives C and D |
| 13h | Reserved |
| 14h | Equipment byte |
| 15h | Low base memory byte |
| 16h | High base memory byte |
| 17h | Low expansion memory byte |
| 18h | High expansion memory byte |
| 19-2Dh | Reserved |
| 2E-2Fh | 2-byte CMOS checksum |
| 30h | Low expansion memory byte |
| 31h | High expansion memory byte |
| 32h | Date century byte |
| 33h | Information flags |
| 34-3Fh | Reserved |

VL-slot pinout

The "A" side of the connector is the add-in board component side. The "B" side of the connector is the add-in board solder side.

| I/O Pin | Signal Name |
| --- | --- |
| A1 | D1 |
| A2 | D3 |
| A3 | GND |
| A4 | D5 |
| A5 | D7 |
| A6 | D9 |
| A7 | D11 |
| A8 | D13 |
| A9 | D15 |
| A10 | GND |
| A11 | D17 |
| A12 | VCC |
| A13 | D19 |
| A14 | D21 |
| A15 | D23 |
| A16 | D25 |
| A17 | GND |
| A18 | D27 |
| A19 | D29 |
| A20 | D31 |
| A21 | A30 |
| A22 | A28 |
| A23 | D26 |
| A24 | GND |
| A25 | A24 |
| A26 | A22 |
| A27 | VCC |
| A28 | A20 |

VL-slot (side A)

| I/O Pin | Signal Name |
| --- | --- |
| A29 | A18 |
| A30 | A16 |
| A31 | A14 |
| A32 | A12 |
| A33 | A10 |
| A34 | A8 |
| A35 | GND |
| A36 | A6 |
| A37 | A4 |
| A38 | WBACK# |
| A39 | BEO# |
| A40 | VCC |
| A41 | BE1# |
| A42 | BE2# |
| A43 | GND |
| A44 | BE3# |
| A45 | ADS# |
| A48 | LRDY# |
| A49 | LDEV# |
| A50 | LREQ# |
| A51 | GND |
| A52 | LGNT# |
| A53 | VCC |
| A54 | ID2 |
| A55 | ID3 |
| A56 | ID4 |
| A57 | LKEN# |
| A58 | LEADS# |

VL-slot (side B)

| I/O Pin | Signal Name |
| --- | --- |
| B1 | D0 |
| B2 | D2 |
| B3 | D4 |
| B4 | D6 |
| B5 | D8 |
| B6 | GND |
| B7 | D10 |
| B8 | D12 |
| B9 | VCC |
| B10 | D14 |
| B11 | D16 |
| B12 | D18 |
| B13 | D20 |
| B14 | GND |
| B15 | D22 |
| B16 | D24 |
| B17 | D26 |
| B18 | D28 |
| B19 | D30 |
| B20 | VCC |
| B21 | A31 |
| B22 | GND |
| B23 | A29 |
| B24 | A27 |
| B25 | A25 |
| B26 | A23 |
| B27 | A21 |
| B28 | A19 |

VL-slot (side B)

| I/O Pin | Signal Name |
| --- | --- |
| B29 | GND |
| B30 | A17 |
| B31 | A15 |
| B32 | VCC |
| B33 | A13 |
| B34 | A11 |
| B35 | A9 |
| B36 | A7 |
| B37 | A5 |
| B38 | GND |
| B39 | A3 |
| B40 | A2 |
| B41 | N.C. |
| B42 | RESET# |
| B43 | D/C# |
| B44 | M/IO# |
| B45 | W/R# |
| B48 | RDYRTN# |
| B49 | GND |
| B50 | IRQ9 |
| B51 | BRDY# |
| B52 | BLAST# |
| B53 | ID0 |
| B54 | ID1 |
| B55 | GND |
| B56 | VLCLK |
| B57 | VCC |
| B58 | LBS16# |

ISA slot (side A)

| I/O Pin | Signal Name |
| --- | --- |
| A1 | IOCHCK |
| A2 | SD7 |
| A3 | SD6 |
| A4 | SD5 |
| A5 | SD4 |
| A6 | SD3 |
| A7 | SD2 |
| A8 | SD1 |
| A9 | SD0 |
| A10 | IOCHRDY |
| A11 | AEN |
| A12 | SA19 |
| A13 | SA18 |
| A14 | SA17 |
| A15 | SA16 |
| A16 | SA15 |
| A17 | SA14 |
| A18 | SA13 |
| A19 | SA12 |
| A20 | SA11 |
| A21 | SA10 |
| A22 | SA9 |
| A23 | SA8 |
| A24 | SA7 |
| A25 | SA6 |
| A26 | SA5 |
| A27 | SA4 |
| A28 | SA3 |
| A29 | SA2 |
| A30 | SA1 |
| A31 | SA0 |

ISA slot (side B)

| I/O Pin | Signal Name |
| --- | --- |
| B1 | GND |
| B2 | RESET DRV |
| B3 | +5 V |
| B4 | IRQ9 |
| B5 | -5 V |
| B6 | DRQ2 |
| B7 | -12 V |
| B8 | OWS |
| B9 | +12 V |
| B10 | GND |
| B11 | SMEMW |
| B12 | SMEMR |
| B13 | I0W |
| B14 | I0R |
| B15 | DECK3 |
| B16 | DRQ3 |
| B17 | DECK1 |
| B18 | DRQ1 |
| B19 | REFRESH |
| B20 | CLK |
| B21 | IRQ7 |
| B22 | IRQ6 |
| B23 | IRQ5 |
| B24 | IRQ4 |
| B25 | IRQ3 |
| B26 | DECK2 |
| B27 | T/C |
| B28 | BALE |
| B29 | +5 V |
| B30 | OSC |
| B31 | GND |

ISA slot (side C)

| I/O Pin | Signal Name |
| --- | --- |
| C1 | SBHE |
| C2 | LA23 |
| C3 | LA22 |
| C4 | LA21 |
| C5 | LA20 |
| C6 | LA19 |
| C7 | LA18 |
| C8 | LA17 |
| C9 | MEMR |
| C10 | MEMW |
| C11 | SD8 |
| C12 | SD9 |
| C13 | SD10 |
| C14 | SD11 |
| C15 | SD12 |
| C16 | SD13 |
| C17 | SD14 |
| C18 | AS15 |

ISA slot (side D)

| I/O Pin | Signal Name |
| --- | --- |
| D1 | MEMCS16 |
| D2 | I0CS16 |
| D3 | IRQ10 |
| D4 | IRQ11 |
| D5 | IRQ12 |
| D6 | IRQ15 |
| D7 | IRQ14 |
| D8 | DECK0 |
| D9 | DRQ0 |
| D10 | DECK5 |
| D11 | DRQ5 |
| D12 | DECK6 |
| D13 | DRQ6 |
| D14 | DECK7 |
| D15 | DRQ7 |
| D16 | +5 V |
| D17 | MASTER |
| D18 | GND |