H I P P O - S X

The material in this manual is for information only and is subject to change without notice.

REVISION: 1.0

IBM, IBM PC/XT/AT, PC-DOS, MS-DOS, OS/2, INTEL, AMI ARE THE TRADEMARKS OR REGISTERED TRADEMARKS OF THEIR RESPECTIVE OWNERS.

RADIO FREQUENCY INTERFERENCE STATEMENT

This equipment generates and uses radio frequency energy and if not installed and used properly, that is, in strict accordance with the manufacturer's instructions, may cause interference with radio and television reception.

If this equipment does cause interference to radio or TV reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient the receiving antenna.

*	Relocate the computer away from the receiver.

*	Move the computer away from the receiver.

*	Plug the computer into a different outlet so that computer and receiver are on different branch circuits.

*	Ensure that card slot covers are in place when no card is installed.

*	Ensure that card mounting screws, attachment connector screws, and ground wires are tightly secured.

*	If peripherals are used with this system, it is suggested to use shielded, grounded cables, with in-line filters if necessary.

If necessary, the user should consult the dealer service representative for additional suggestions.

The manufacturer is not responsible for any radio or TV interference caused by unauthorized modifications to this equipment.  It is the responsibility of the user to correct such interference.

Note

1.		Electronic components are sensitive to dust and dirt.  Do inspect and clean the computer system regularly.

2.		Turn off the power whenever you install or remove any connector, memory module and add-on card.  Before turning on the power, make sure that all the connectors, memory modules and add-on cards are secured.

3.		After power is on, wait for a minute.  The system BIOS are going through a self-test during this period and nothing is shown on the screen.  After the self-test, the system BIOS will initialize the display adaptor and show messages.

4.		The SIMM sockets are fragile device.  Do not force the SIMM modules into the sockets.  It may break the locking latches.

Preface

This manual provides information about the installation and maintenance of OCTEK Hippo-SX motherboard.  In-depth explanations of the functions of this motherboard are provided.

The content in this manual is only for reference and is intended to provide the basic information for the general users.  There are also technical information for hardware and software engineers.

In this manual, there are 4 chapters.  Chapter 1 contains a brief introduction.  In Chapter 2, the specification and functions of OCTEK Hippo-SX motherboard are explained.  It also outlines many advanced features of the CPU and the system architecture.  Chapter 3 explains the installation of coprocessor, DRAM modules and jumpers.  Technical information is provided in Chapter 4.

Memory expansion card is described in Appendix A.  Appendix B contains the informa-tion about the Hippo Cache Card.  Additional information is given in the Appendix C, D, E, F, and G for maintenance purpose.

Table of Content

Chapter 1	INTRODUCTION

Chapter 2	GENERAL FEATURES

Specification					2-1

Processor						2-3

Secondary Cache Memory			2-6

Memory System					2-8

Dual Bus Design					2-10

Chapter 3	CONFIGURING THE SYSTEM

Installing Math Coprocessor			3-1

Installing RAM Modules			3-3

Configuration of Memory			3-5

DRAM Configuration				3-7

Control of System Speed			3-8

Reset CMOS Setup Information		3-8

System Board Jumper Setting			3-9

System Board Connectors			3-10

Chapter 4	TECHNICAL INFORMATION

Memory Mapping					4-1

I/O Address Map					4-2

System Timers					4-4

System Interrupts					4-6

Direct Memory Access (DMA)			4-7

Real Time Clock and CMOS RAM		4-9

CMOS RAM Address Map			4-10

Real Time Clock Information			4-11

System Expansion Bus				4-12

Appendix A	MEMORY EXPANSION CARD

Appendix B	HIPPO CACHE CARD

Appendix C	OPERATION AND MAINTENANCE

Static Electricity					C-1

Keeping The System Cool			C-1

Cleaning The `Golden Finger'			C-2

Cleaning The Motherboard			C-2

Appendix D	TROUBLESHOOTING

Main Memory Error				D-1

Cache Memory Failure				D-1

Improper Setting of Wait State		D-2

Appendix E	SYSTEM BOARD LAYOUT

Appendix F	MEMORY EXPANSION CARD

LAYOUT

Appendix G	HIPPO CACHE CARD LAYOUT

Chapter 1

Introduction

_______________________________

OCTEK Hippo-SX provides excellent computing performance at low cost.  The heart of Hippo-SX is the 80486SX which combines CPU and internal cache memory on a single chip.  Incorporated with a highly integrated chipset, OCTEK Hippo-SX fully takes advantage of the power of 80486SX and provides high performance, reliability and compatibility to the user.  OCTEK Hippo-SX is suitable for those users who need power for the daily computing applications and network operations.

The burst read mode is supported and thus the data is transferred to CPU at full speed.  Furthermore, the chipset supports a optional secondary 128K bytes cache memory.  Installation is very simple.

If you want more speed in numeric operation, WEITEK 4167 can be installed.  256KB, 1MB and 4MB SIMM RAM modules are supported.  32MB memory can be installed on board.

Fast A20 gate and fast reset generation are incorporated to improve the performance of advanced operation system and expanded memory managers.

Compatibility and reliability are important issues.  I/O channel is compatible to standard AT bus.  Therefore any peripheral may be used on OCTEK Hippo-SX.  On board POWERGOOD generator is essential to ensure the reliability of the system and is well-designed to work with all power supplies.

Chapter 2

General Features

_______________________________

SPECIFICATION

Processor :

Intel 80486SX CPU

with optional WEITEK 4167 Co-processor

Speed :

Turbo/normal speed

I/O Slot :

Compatible to standard AT bus

Eight 16-bit slots

Optional Secondary Cache Memory :

128KB Direct mapped/Write through

Memory :

Shadow RAM and Memory remapping

Page/Interleaved mode

Flexible configuration

SIMM sockets for 256K, 1M and 4M modules

Maximum 32MB on board

System support functions :

-	8-Channel DMA (Direct Memory Access)

-	16-level interrupt

-	3 programmable timers

-	CMOS RAM for system configuration

-	Real time clock with battery back-up

-	Fast A20 gate and fast reset

Other Features :

-	On board POWERGOOD generation

-	External battery connector

PROCESSOR

80486SX is the powerful microprocessor which merges many innovative features on a single chip, suitable for commercial applications and advanced operation systems.  80486SX is a 32-bit microprocessor with 32-bit external data bus and 32-bit external address bus.  It is fully binary compatible with existing Intel's CPU.  All existing software for PC XT/AT can be used on OCTEK Hippo-SX.

Cache memory can improve the overall performance of a computer system.  Nevertheless, if the cache memory is separated from CPU, CPU still needs to fetch code and data from external bus.  That means the data transfer rate should not be too fast to allow the external devices to keep pace with the CPU.  In 80486SX, the cache controller and cache memory are integrated into the chip.  Most of the operations can be carried out inside the CPU, which reduces the bus operation on external data and address bus and thus speeds up the internal execution.

The cache memory is a 8K bytes, 16 bytes line size, four-way set associative configuration.  The hit rate of this configuration is much better than 32K bytes two-way set associative external cache because a four-way set associative architecture provides better performance in a multitasking and multi-processor environment.

If a read miss occurs, the CPU will initiate a burst mode read operation.  In burst mode read operation, CPU performs four successive read operations each of which takes only one cycle.  Total 128 bits data are fetched into the CPU's internal cache.  Since burst mode read operation is very fast, the traffic of the CPU bus is greatly reduced and the bus is available to other bus masters, such as DMA controller.

By eliminating the access to external bus, operations with the internal cache can be completed in a single cycle.  80386DX at least needs two cycles for an operation.  To further increase the rate of data transfer inside the CPU, the internal bus of the cache memory is increased to 128 bits, which is four times of the external bus.  Since, in most of the time, the CPU is using the internal cache, the large bus size substantially improves the overall performance.

When the CPU writes data to the main memory, the data is first stored in a write buffer.  There are four write buffers.  When the external bus is idle, data will be sent to the main memory.  If all buffers are filled, it can start write operation in burst mode.  Since the internal cache is updated immediately, the CPU need not suspend its operation and there is no need to wait for the external device to update the main memory.

Many often-used instructions are executed in a clock cycle and some instructions are modified to take fewer cycles than in 80386DX.  On the contrary, 80386DX may take two to three more cycles for the same instruction.  The CPU contains an advanced instruction pipeline structure and a 32-byte code queue to speed up the execution.

80486SX includes all the functions of 80386DX and is able to support sophisticated software and operation systems which are widely employed now.  It is able to operate in real mode, protected mode and virtual 8086 mode.  A new addressing mechanism allows switching between modes more efficiently.  Hence applications using EMS memory can run efficiently although they only use software drivers to emulate EMS memory.

Internal memory management unit provides a flexible addressing scheme for the next generation operation system.  Multitasking, concurrent operation and manipulating huge data base can be accomplished with excellent performance.  Paging mechanism is employed to allow powerful operating system to implement virtual memory.  Each segment is divided into several pages which are 4K bytes per page.  Page mechanism is transparent to software and allows software to address 64 terabytes.  Furthermore, the 64KB segment boundary which is an barrier of 8088 and 80286 is removed and the segment length can be increased up to 4GB.

SECONDARY CACHE MEMORY

Although there is 8K bytes internal cache memory inside the CPU, a secondary cache memory is still necessary when the size of main memory is large.

In a single user system, when only DOS is used and few megabytes memory is installed, the internal cache memory is adequate and the performance is still very high.

In a multi-user or multitasking environment, CPU has to switch from one process to another, which involves a large mount of data transfer.  The performance to a great extent depends on the data transfer rate.  In a 80486SX system, a large portion of the internal cache memory has to be updated to the main memory and then the data for another process is retrieved into the cache memory.  The low speed main memory will substantially slow down the system.

In order to shorten the time required for switching between processes, a secondary cache memory is needed as a buffer between the slow speed main memory and the internal cache memory of 80486SX.  Wait state is thus eliminated when the CPU reads external data from the secondary cache memory.  Furthermore, the secondary cache memory can support the burst mode of 80486SX.  The CPU is able to run at full speed.

Since there is already a 8K bytes cache memory in 80486SX, a small cache memory, such as 32K bytes, can not substantially improved the performance.  On the other hand, the secondary cache should be able to contain a copy of the internal cache memory or more, which means data of different processes can coexist in the secondary cache memory.  So, an optimal size is 128K or 512K bytes and this size is adequate for over 8M bytes main memory.

In Hippo-SX, the secondary cache controller is incorporated in the chipset.  There is no need to install an external cache controller.  If the secondary cache memory is needed, just plug in the secondary cache memory card and enable secondary card in the BIOS setup.  It reduces the cost for upgrading the system.

MEMORY SYSTEM

Four banks of DRAMs can be installed.  Two banks are available on motherboard and others are on optional memory expansion board.  The maximum memory size is 32MB when using 4MB DRAM.

The memory system provides a flexible memory configuration.  256KB and 1MB DRAM can be mixed together.  Several combinations of DRAM types are allowed.  So, a basic system is equipped with 1MB using 256KB DRAM and then memory size is increased to 5MB by putting another bank of 1MB DRAM.

When using 4MB modules, you may install up to two banks.  So the memory size is either 16MB or 32MB.  4MB module cannot work with other memory types.

The memory system supports page-inter-leaved mode.  The memory is divided into pages.  Successive memory accesses within the same page need not require wait state.  When more than one bank of memory are installed, the page size is effectively increased because several banks of memory can be kept active.  If four banks are filled, the page size is increased by a factor of four.

To enhance the system performance, shadow RAM mode is supported.  In shadow RAM mode, system BIOS and video BIOS contained in low speed memory such as EPROM and ROM are copied into DRAM.  Improvement is significant because access to DRAM is mush faster than ROM.

Another useful feature is memory remapping.  640KB is allocated as base memory.  System BIOS and video BIOS occupy some locations between 640K and 1M.  When 1M bytes or more are installed, there are some parts of DRAM overlapping with the BIOS and cannot be accessed.  Memory remapping allows these parts of memory to be accessed at other locations and thus is available to software.

1.256M

256KB

1M				         1M

256KB

640K				       640K

0K				         0K

before remapping	         after remapping

┌─────────┐

│▒▒▒▒▒▒▒▒▒│

┌─────────┐        ├─────────┤

├─────────┤        │         │

│▒▒▒▒▒▒▒▒▒│        │         │

├─────────┤        ├─────────┤

│         │        │         │

│         │        │         │

│         │        │         │

└─────────┘        └─────────┘

Memory remapping

DUAL BUS DESIGN

It is very important that a high speed system should be compatible with existing peripherals without lowering the performance.  To be compatible, the I/O slot should run at 8MHz or slower.  On the other hand, the rest of the system are running at full speed.

In Hippo-SX, a dual bus design is employed.  A high speed bus links the CPU, coprocessor, cache memory and main memory.  This bus is synchronous with clock of the CPU and the data transfer is 32 bits.  Whenever there is a request for transferring data to or from I/O slot, the chipset is responsible for handling the conversion between the bus.  The clock rate of the high speed bus will not be reduced, which eliminates many compatibility problem.

CPU

High Speed

bus			8Mhz bus

Chip Set            I/O Slot

Cache         Main

Memory     Memory

┌─────┐

│     │

└──┬──┘

│            ┌───────┐      ┌───────┐

├───────┬────┤       ├──────┤       │

│       │    └───────┘      └───────┘

│       │

┌──┴───┐┌──┴───┐

│      ││      │

│      ││      │

└──────┘└──────┘

Chapter 3

Configuring The System

_______________________________

Important Note : Turn off the power before installing or replacing any component.

INSTALLING MATH COPROCESSOR

Math coprocessor WEITEK 4167 is a PGA devices.  Beside the CPU, there is a 144-pin PGA socket.  To install Math Coprocessor, be sure to line up pin 1 of the Math coprocessor with pin 1 of the socket as shown below.

Before installing the Math coprocessor, make sure that all the pins are straight.  The pins are very fragile.  Once these pins are bent, the coprocessor may be damaged.

The Math coprocessor is automatically detected by the system and the applications.  No jumper is needed to be set.

INSTALLING RAM MODULES

OCTEK Hippo-SX has eight sockets for SIMM modules.  Whenever you add memory to the motherboard, install four modules at the same time.  Also make sure that the chips on the modules face towards CPU.  The modules should be locked by the sockets.  Please check carefully before turning on the power.  Otherwise, the system will not work properly.

To install a module, the module edge is angled into the socket's contact and then the module is pivoted into position, where the locking latches will secure it.  If the module edge is not completely inserted into the socket, it cannot be pivoted to be in vertical position and should be dragged out and re-inserted again.  Do not force the module into the SIMM socket.  It will damage the locking latches.

The modules should be locked by the locking latches of the sockets firmly.  Please check carefully before turning on the power.  Otherwise, the system will not work properly.

If the BIOS reports an memory error or parity error, drag out the modules and insert them again.  If the locking latches are damaged, contact your dealer to replace the socket.

CONFIGURATION OF MEMORY

The configuration of the memory is very flexible.  There are several combinations of DRAM types you may consider.  Either 256KB or 1MB SIMM are acceptable.

There are totally four banks of DRAM.  Two banks (bank 0 and 1) are on the motherboard and the others (bank 2 and 3) are on the memory expansion board.  If bank 0 and 1 are filled, you have to use bank 2 and 3.  If you use 4MB module, bank 2 and bank 3 are not used.

Page mode is always active.  Interleaved mode is only enabled for those banks with similar pairs or quartet of DRAMs.  The memory size is detected automatically by system BIOS and indicated during memory test after reset.  No jumper is needed to be set for the memory size and DRAM type.

To determine what DRAM speed rating should be used depends on the system speed and wait state.  The highest performance is accomplished by using zero wait state, but high speed DRAM has to be used.  If zero wait state is selected, fast page mode DRAM is needed.

Select zero wait state for 100ns DRAM or faster.  120ns DRAM can be used for 1 wait state.  The wait state setting is applied to four banks of memory.  Therefore make sure to install DRAM with the same speed rating, or accom-modate the wait state setting to the new DRAM type.

Because of the shadow RAM and memory remapping feature, the memory size may not equal to  the actual memory size.  For example, there is 1MB on board.  If memory remapping is disabled, the BIOS will show 640KB.  After enabled, the memory size is increased to 896KB.  The rest of the memory (128KB) is assigned for shadow RAM and can not be used by software.

The number of wait state is assigned in the BIOS setup.  Improper setting may make the system malfunction.  In this case, reset the CMOS setup using JP11.  Then reset the system and go through the system setup again.

DRAM CONFIGURATION

|  | Bank | Bank | Bank | Bank | Total | Inter- |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | Memory | leaved |
| 1 | 0 | 0 | 0 | 0 | 0 |  |
| 2 | 256K | 0 | 0 | 0 | 1M | 2W |
| 3 | 256K | 256K | 0 | 0 | 2M | 2W |
| 4 | 256K | 256K | 256K | 0 | 3M |  |
| 5 | 256K | 256K | 256K | 256K | 4M | 4W |
| 6 | 1M | 0 | 0 | 0 | 4M |  |
| 7 | 256K | 1M | 0 | 0 | 5M |  |
| 8 | 1M | 256K | 0 | 0 | 5M |  |
| 9 | 256K | 256K | 1M | 0 | 6M | 2W |
| 10 | 256K | 1M | 256K | 0 | 6M |  |
| 11 | 1M | 256K | 256K | 0 | 6M |  |
| 12 | 256K | 256K | 1M | 256K | 7M | 2W |
| 13 | 256K | 1M | 256K | 256K | 7M | 2W |
| 14 | 1M | 256K | 256K | 256K | 7M | 2W |
| 15 | 1M | 1M | 0 | 0 | 8M | 2W |
| 16 | 256K | 1M | 1M | 0 | 9M |  |
| 17 | 1M | 1M | 256K | 0 | 9M | 2W |
| 18 | 1M | 256K | 1M | 0 | 9M |  |
| 19 | 256K | 256K | 1M | 1M | 10M | 2W |
| 20 | 256K | 1M | 1M | 256K | 10M |  |
| 21 | 1M | 1M | 256K | 256K | 10M | 2W |
| 22 | 1M | 256K | 1M | 256K | 10M |  |
| 23 | 1M | 1M | 1M | 0 | 12M | 2W |
| 24 | 256K | 1M | 1M | 1M | 13M | 2W |
| 25 | 1M | 1M | 1M | 256K | 13M | 2W |
| 26 | 1M | 256K | 1M | 1M | 13M | 2W |
| 27 | 1M | 1M | 1M | 1M | 16M | 4W |
| 28 | 4M |  |  |  | 16M |  |
| 29 | 4M | 4M |  |  | 32M | 2W |

CONTROL OF SYSTEM SPEED

System speed can be controlled by keyboard.  To change the speed by keyboard, use `-' and `+' of the numeric keypad.  Press `Ctrl' `Alt' and `-' for slow speed and press `Ctrl' `Alt' and `+' for fast speed.

RESET CMOS SETUP INFORMATION

Sometimes, the improper setting of system setup may make the system malfunction.  In this case, turn off the power and set JP1 to 1-2 for a while.  The internal CMOS status register is reset.  Then set the jumper to 2-3 and turn on the power.  The BIOS finds the CMOS status register is reset and regards the setup information is invalid.  So it will prompt you to correct the information.

If you just want the BIOS to discard the register settings, you select the XCMOS SETUP, press `INS' on the keyboard for a few seconds after power up.  The BIOS will detect the key stroke and use the default setting for booting.

SYSTEM BOARD JUMPER SETTING

There are several options which allows user to select by hardware switches.  JP3 and JP4 are reserved.

Display Selection

| JP2 |  |
| --- | --- |
| ON | CGA, EGA, VGA |
| OFF | Monochrome display * |

Secondary Cache Installation

|  | Secondary | Cache |
| --- | --- | --- |
| JP5 & JP6 | Installed | Not installed |
|  | OFF | ON * |

Note :  * factory setting

SYSTEM BOARD CONNECTORS

Under typical conditions, these connectors should be connected to the indicators and switches of the system unit.  The functions of connectors on the version 2.0 motherboard are listed below.

| Connector | Function |
| --- | --- |
| SW1 | Hardware reset connector |
| J26 | Speaker connector |
| J27 | Power LED & Ext-lock connector |
| J11, J12 | Power supply connector |
| J1 | External battery connector |
| J2 | Keyboard connector |
| J25 | NOT Used |

Pin assignment of the connector are illustrated as follows:

SW 1 - Hardware Reset Connector

| Pin | Assignment |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

J 1 - External Battery Connector

| Pin | Assignment |
| --- | --- |
| 1 | + Vdc |
| 2 | not used |
| 3 | Ground |
| 4 | Ground |

J 2 - Keyboard Connector

| Pin | Assignment |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Spare |
| 4 | Ground |
| 5 | +5 Vdc |

J 26 - Speaker Connector

| Pin | Assignment |
| --- | --- |
| 1 | Data out |
| 2 | +5 Vdc |
| 3 | Ground |
| 4 | +5 Vdc |

J 27 - Power LED & Ext-Lock Connector

| Pin | Assignment |
| --- | --- |
| 1 | +5 Vdc |
| 2 | Key |
| 3 | Ground |
| 4 | Keyboard inhibit |
| 5 | Ground |

J 11, J 12 - Power Supply Connector

| Pin | Assignment |
| --- | --- |
| 1 | POWERGOOD |
| 2 | +5 Vdc |
| 3 | +12 Vdc |
| 4 | -12 Vdc |
| 5 | Ground |
| 6 | Ground |

| Pin | Assignment |
| --- | --- |
| 1 | Ground |
| 2 | Ground |
| 3 | -5 Vdc |
| 4 | +5 Vdc |
| 5 | +5 Vdc |
| 6 | +5 Vdc |

Chapter 4

Technical Information

_______________________________

This section provides technical information about OCTEK Hippo-SX and is intended for advanced users interested in the basic design and operation of OCTEK Hippo-SX.

MEMORY MAPPING

| Address | Range | Function |
| --- | --- | --- |
| 000000-7FFFFF | 000K-512K | System Board Memory (512K) |
| 080000-09FFFF | 512K-640K | System Board Memory (128K) |
| 0A0000-0BFFFF | 640K-768K | Display Buffer (128K) |
| 0C0000-0DFFFF | 768K-896K | Adaptor ROM / Shadow RAM (128K) |
| 0E0000-0EFFFF | 896K-960K | System ROM / Shadow RAM (64K) |
| 0F0000-0FFFFF | 960K-1024K | System BIOS ROM / Shadow RAM (64K) |
| 100000-7FFFFF | 1024K-8192K | System Memory |
| 800000-FFFFFF | 8192K-16318K | System Memory |

I/O ADDRESS MAP

I/O Address Map on System Board

I/O address hex 000 to 0FF are reserved for the system board I/O.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 000-01F | DMA Controller 1, 8237 |
| 020-03F | Interrupt Controller 1, 8259, Master |
| 040-05F | Timer, 8254 |
| 060-06F | Keyboard Controller |
| 070-07F | Real Time Clock, NMI (non-maskable interrupt) mask |
| 080-09F | DMA Page Register, 74LS612 |
| 0A0-0BF | Interrupt Controller 2, 8259 |
| 0C0-0DF | DMA Controller 2, 8237 |
| 0F0 | Clear Math Coprocessor Busy |
| 0F1 | Reset Math Coprocessor |
| 0F8-0FF | Math Coprocessor Port |

I/O address hex 100 to 3FF are available on the I/O channel.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game I/O |
| 278-27F | Parallel Printer Port 2 |
| 2F8-2FF | Serial Port 2 |
| 300-31F | Prototype Card |
| 360-36F | Reserved |
| 378-37F | Parallel Printer Port 1 |
| 380-38F | SDLC, bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Printer Adapter |
| 3C0-3CF | Reserved |
| 3D0-3DF | Color Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 |

SYSTEM TIMERS

OCTEK Hippo-SX has three programmable timer/counters controlled by 82C206 and they are defined as channels 0 through 2:

| Channel 0 | System Timer |
| --- | --- |
| Gate 0 | Tied on |
| Clk in 0 | 1.190 Mhz OSC |
| Clk out 0 | 8259 IRQ 0 |

| Channel 1 | Refresh Request Generator |
| --- | --- |
| Gate 1 | Tied on |
| Clk in 1 | 1.190 Mhz OSC |
| Clk out 1 | Request Refresh Cycle |

| Channel 2 | Tone Generation of Speaker |
| --- | --- |
| Gate 2 | Controlled by bit 0 of port hex 61 PPI bit |
| Clk in 2 | 1.190 Mhz OSC |
| Clk out 2 | Used to drive the speaker |

Note :	Channel 1 is programmed to generate a 15-micro-second period signal.

The 8254 Timer/Counters are treated by system programs as an arrangement of four programmable external I/O ports.  Three are treated as counters and the fourth is a control register for mode programming.

SYSTEM INTERRUPTS

Sixteen levels of system interrupts are provided on OCTEK Hippo-SX.  The following shows the interrupt-level assignments in decreasing priority.

Level					Function

Microprocessor NMI	Parity or I/O Channel

Check

Interrupt Controllers

CTLR 1	CTLR 2

IRQ0				Timer Output 0

IRQ1				Keyboard

(Output Buffer Full)

IRQ2				Interrupt from CTLR 2

IRQ8		Real-time Clock Interrupt

IRQ9		Software Redirected to

INT 0AH (IRQ2)

IRQ10	Reserved

IRQ11	Reserved

IRQ12	Reserved

IRQ13	Coprocessor

IRQ14	Fixed Disk Controller

IRQ15	Reserved

IRQ3				Serial Port 2

IRQ4				Serial Port 1

IRQ5				Parallel Port 2

IRQ6				Diskette Controller

IRQ7				Parallel Port 1

┌──────────────────────────────────────┐

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│     ─┐                               │

│      │ ┌─                            │

│      │ │                             │

│      │ │                             │

│      │ │                             │

│      └─┤                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        └─                            │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

└──────────────────────────────────────┘

DIRECT MEMORY ACCESS (DMA)

OCTEK Hippo-SX supports seven DMA channels.

| Channel | Function |
| --- | --- |
| 0 | Spare (8 bit transfer) |
| 1 | SDLC (8 bit transfer) |
| 2 | Floppy Disk (8 bit transfer) |
| 3 | Spare (8 bit transfer) |
| 4 | Cascade for DMA Controller 1 |
| 5 | Spare (16 bit transfer) |
| 6 | Spare (16 bit transfer) |
| 7 | Spare (16 bit transfer) |

The following shows the addresses for the page register.

| Page Register | I/O Address (HEX) |
| --- | --- |
| DMA Channel 0 | 0087 |
| DMA Channel 1 | 0083 |
| DMA Channel 2 | 0081 |
| DMA Channel 3 | 0082 |
| DMA Channel 5 | 008B |
| DMA Channel 6 | 0089 |
| DMA Channel 7 | 008A |
| Refresh | 008F |

REAL TIME CLOCK AND CMOS RAM

Real time clock and CMOS RAM are contained on board.  Real time clock provides the system date and time.  CMOS RAM stores system information.  Both are backed up by battery and will not lose information after power off.  The following page shows the CMOS RAM Address Map.

CMOS RAM ADDRESS MAP

| Addresses | Description |
| --- | --- |
| 00-0D | * Real-time clock information |
| 0E | * Diagnostic status byte |
| 0F | * Shutdown status byte |
| 10 | Diskette drive type byte - drives A and B |
| 11 | Reserved |
| 12 | Fixed disk type byte - drives C and D |
| 13 | Reserved |
| 14 | Equipment byte |
| 15 | Low base memory byte |
| 16 | High base memory byte |
| 17 | Low expansion memory byte |
| 18 | High expansion memory byte |
| 19-2D | Reserved |
| 2E-2F | 2-byte CMOS checksum |
| 30 | * Low expansion memory byte |
| 31 | * High expansion memory byte |
| 32 | * Date century byte |
| 33 | * Information flags (set during power on) |
| 34-3F | Reserved |

REAL TIME CLOCK INFORMATION

The following table describes real-time clock bytes and specifies their addresses.

| Byte | Function | Address |
| --- | --- | --- |
| 0 | Seconds | 00 |
| 1 | Second alarm | 01 |
| 2 | Minutes | 02 |
| 3 | Minute alarm | 03 |
| 4 | Hours | 04 |
| 5 | Hour alarm | 05 |
| 6 | Day of week | 06 |
| 7 | Date of month | 07 |
| 8 | Month | 08 |
| 9 | Year | 09 |
| 10 | Status Register A | 0A |
| 11 | Status Register B | 0B |
| 12 | Status Register C | 0C |
| 13 | Status Register D | 0D |

SYSTEM EXPANSION BUS

OCTEK Hippo-SX provides eight 16-bit slots.

The I/O channel supports:

*	I/O address space from hex 100 to hex 3FF

*	Selection of data access (either 8 or 16 bit)

*	24 bit memory addresses (16MB)

*	Interrupts

*	DMA channels

*	Memory refresh signal

The following figure shows the pin numbering for I/O channel connectors (side A and B).

The following figure shows the pin numbering for I/O channel connectors (side C and D).

The following tables summarize pin assignments for the I/O channel connectors.

I/O Channel (A-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| A1 | -I/O CH CK | I |
| A2 | SD7 | I/O |
| A3 | SD6 | I/O |
| A4 | SD5 | I/O |
| A5 | SD4 | I/O |
| A6 | SD3 | I/O |
| A7 | SD2 | I/O |
| A8 | SD1 | I/O |
| A9 | SD0 | I/O |
| A10 | -I/O CH RDY | I |
| A11 | AEN | O |
| A12 | SA19 | I/O |
| A13 | SA18 | I/O |
| A14 | SA17 | I/O |
| A15 | SA16 | I/O |
| A16 | SA15 | I/O |
| A17 | SA14 | I/O |
| A18 | SA13 | I/O |
| A19 | SA12 | I/O |
| A20 | SA11 | I/O |
| A21 | SA10 | I/O |
| A22 | SA9 | I/O |
| A23 | SA8 | I/O |
| A24 | SA7 | I/O |
| A25 | SA6 | I/O |
| A26 | SA5 | I/O |
| A27 | SA4 | I/O |
| A28 | SA3 | I/O |
| A29 | SA2 | I/O |
| A30 | SA1 | I/O |
| A31 | SA0 | I/O |

I/O Channel (B-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| B1 | GND | Ground |
| B2 | RESET DRV | I |
| B3 | +5 Vdc | Power |
| B4 | IRQ9 | I |
| B5 | -5 Vdc | Power |
| B6 | DRQ2 | I |
| B7 | -12 Vdc | Power |
| B8 | 0WS | I |
| B9 | +12 Vdc | Power |
| B10 | GND | Ground |
| B11 | -SMEMW | O |
| B12 | -SMEMR | O |
| B13 | -IOW | I/O |
| B14 | -IOR | I/O |
| B15 | -DACK3 | I |
| B16 | DRQ3 | O |
| B17 | -DACK1 | I |
| B18 | DRQ1 | O |
| B19 | -Refresh | I/O |
| B20 | CLK | O |
| B21 | IRQ7 | I |
| B22 | IRQ6 | I |
| B23 | IRQ5 | I |
| B24 | IRQ4 | I |
| B25 | IRQ3 | I |
| B26 | -DACK2 | O |
| B27 | T/C | O |
| B28 | BALE | O |
| B29 | +5 Vdc | Power |
| B30 | OSC | O |
| B31 | GND | Ground |

I/O Channel (C-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| C1 | SBHE | I/O |
| C2 | LA23 | I/O |
| C3 | LA22 | I/O |
| C4 | LA21 | I/O |
| C5 | LA20 | I/O |
| C6 | LA19 | I/O |
| C7 | LA18 | I/O |
| C8 | LA17 | I/O |
| C9 | -MEMR | I/O |
| C10 | -MEMW | I/O |
| C11 | SD8 | I/O |
| C12 | SD9 | I/O |
| C13 | SD10 | I/O |
| C14 | SD11 | I/O |
| C15 | SD12 | I/O |
| C16 | SD13 | I/O |
| C17 | SD14 | I/O |
| C18 | SD15 | I/O |

I/O Channel (D-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| D1 | -MEM CS16 | I |
| D2 | -I/O CS16 | I |
| D3 | IRQ10 | I |
| D4 | IRQ11 | I |
| D5 | IRQ12 | I |
| D6 | IRQ15 | I |
| D7 | IRQ14 | I |
| D8 | -DACK0 | O |
| D9 | DRQ0 | I |
| D10 | -DACK5 | O |
| D11 | DRQ5 | I |
| D12 | -DACK6 | O |
| D13 | DRQ6 | I |
| D14 | -DACK7 | O |
| D15 | DRQ7 | I |
| D16 | +5 Vdc | Power |
| D17 | -MASTER | I |
| D18 | GND | Ground |

Appendix A

Memory Expansion Card

_______________________________

Memory expansion card contains bank 2 and bank 3 of memory.  There are 8 SIMM modules on the card and total memory on this card is 8MB.  Please refer to Chapter 3 for the configuration of the memory.

After installing the memory card, the system BIOS will determine the type of DRAM and the amount of total memory.  There is no need to set any jumper.  The system BIOS will prompt you to setup the memory size after re-boot.

However, you should make sure that the memory on the memory expansion board can be used reliably with the current setting of wait state. If there is any problem, increase the number of wait state.

There is a mounting plate on rear of the card.  This mounting plate is used to keep the card on the slot firmly.  Use a screw to fasten the card to the case.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Appendix B

Hippo Cache Card

_______________________________

There are 128K bytes cache memory on the Hippo cache card.  The expansion slot beside the CPU is dedicated to this card.

To setup the cache controller, follow the steps described below.

1.	Turn off the power and plug in the card;

2.	Turn on the power and enter CMOS SETUP;

3.	Enable the cache memory;

4.	Turn off the power again and then remove the mini jumper on JP5 and JP6;

5.	Turn on the power and the cache card can be used.

If you want to remove the cache memory card, disable the cache memory in CMOS SETUP.  Then you should turn off the power and place mini jumpers on JP5 and JP6 before removing the cache memory card.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Appendix C

Operation and Maintenance

_______________________________

STATIC ELECTRICITY

When installing or removing any add-on card, DRAM module or coprocessor, you should discharge the static electricity on your body.  Static electricity is dangerous to electronic device and can build-up on your body.  When you touch the add-on card or motherboard, it is likely to damage the device.  To discharge the static electricity, touch the metal of your computer.  When handling the add-on card, don't contact the components on the cards or their "golden finger".  Hold the cards by their edges.

KEEPING THE SYSTEM COOL

The motherboard contains many high-speed components and they will generate heat during operation.  Other add-on cards and hard disk drive can also produce a lot of heat.  The temperature inside the computer system may be very high.  In order to keep the system running stably, the temperature must be kept at a low level.  A easy way to do this is to keep the cool air circulating inside the case.  The power supply contains a fan to blow air out of the case.  If you find that the temperature is still very high, it would be better to install another fan inside the case.  Using a larger case is recommended if there are a number of add-on cards and disk drives in the system.

CLEANING THE "GOLDEN FINGER"

Whenever inserting an add-on card to the motherboard, make sure that there is no dirt on the "golden finger" of the add-on card.  If not, the contact between the "golden finger" and the slot may be poor and thus the add-on card may not work properly.  Use a pencil eraser to clean the "golden finger" if dirt is found.

CLEANING THE MOTHERBOARD

The computer system should be kept clean.  Dust and dirt are harmful to electronic devices.  To prevent dust from accumulating on the mother-board, installing all mounting plates on the rear of the case.  Regularly examine your system, and if necessary, vacuum the interior of the system with a miniature vacuum.

Appendix D

Troubleshooting

_______________________________

MAIN MEMORY ERROR

After power up, the monitor remains blank, and there are beep sounds indicating a main memory failure.  In this case, turn off the power and remove all SIMM modules.  Carefully place the modules back to the sockets and make sure that all the modules are locked by the locking latches firmly.

In some other cases, the total memory found by the BIOS is different from the actual amount of memory on board.  (Note that 128K bytes memory is reserved for the shadow RAM function and will not be counted by the BIOS).  It is also a memory failure and you can follow the instruction above.

CACHE MEMORY FAILURE

If the system hangs after memory test, it is likely that the cache memory has some problems when the secondary cache memory card is installed.  Maybe some of the SRAMs are damaged or the contact of the IC pins is poor.  Try to press the SRAM to make sure that the SRAMs are inserted in the sockets, or examine the SRAM to see whether any pins are bent under or out.  If the bent pins are found, remove the SRAM, straighten the pin and place the SRAM again.  You may also check the BIOS setup of the cache configuration.  If the cache controller is enabled, you should select chipset's cache controller.  Otherwise, the system will fail.

IMPROPER SETTING OF WAIT STATE

If the system hangs after memory test, another possible cause is the improper setting of the wait state for memory operation.  The number of wait state must match the speed of the DRAM.  Reset the CMOS RAM and set up the wait state.  Try to increase the number of wait state.

Appendix E

System Board Layout

_______________________________

Appendix F

Memory Expansion Card Layout

_______________________________

Appendix G

Hippo Cache Card Layout

_______________________________