Section Four :

System BIOS

*************************************************

4.1	Setup Utility Description

4.2	Edit-Page/Utilities

*************************************************

All microcomputer systems use a Basic Input Output System.  This is software that has been permanently recorded in a ROM (Read Only Memory) chip and functions as the basic point of communication between the system board and the rest of the computer.

The BIOS provides an operational interface to the system and relieve the programmer from worrying about the characteristics of hardware devices.  Thus, hardware modifications and enhancements become transparent to user's programs, access to BIOS is through the program interrupts of the microprocessor.  Each BIOS entry point is available through its own interrupt.

Upon you turn on the power of your system, it will go through a self-test routine which checks all of its internal devices.  Complete testings will be carried out on the CPU, base 640K RAM, extended RAM, ROM, system board, CMOS memory, video controllers, parallel and serial subsystems, floppy and fixed disk subsystems and the keyboard.

When the self-test is completed, the system will search for the DOS (disk operating system) system file in drive A.  If no system diskette is put in drive A, it will check the fixed disk (if installed).

1.			SETUP UTILITY DESCRIPTION

A Setup Utility is incorporated into the BIOS which allows the user to view the system configuration, and to select a variety of powerup/runtime options.

Entrance to Setup can occur in three ways:

(1)	A configuration change detected during POST forces entrance, or

(2)	ESC is pressed during cold-boot, or

(3)	CTRL+ALT+ESC is pressed to warm-boot into Setup.

With the exception of a few special features (eg, low level format), the Setup Utility is nothing more than an interactive, menu-driven editor for the system CMOS.  After an initial CMOS configuration session in Setup, the system will be able to powerup and warm boot without any further assistance from Setup.  The typical end user will rarely (if ever) access Setup again, unless hardware is changed or CMOS is corrupted by a runaway program.

The Setup utility gives the appearance of being a single application program, but is actually comprised of a group of separate, dedicated utilities ("screens").  A different utility exists for each of the subsystems which require configuration.  Each such utility is designed to look and operate in a fashion consistent with the rest, yielding an integrated result which is coherent and easily understood.  Access to the individual utilities is accomplished through Menu selection.  It is this Menu which bonds the otherwise disjoint configuration screens into a single utility.

The Setup screen is broadcast simultaneously to both CRTs regardless of CMOS settings, so it will never happen that the user will be without video when (re) configuration is necessary.  The screen format consists of these four fields:

Copyright/OEM Banner

on the top two lines.  The BIOS version number and OEM Porting description also appear in this field.  Directly below the Copyright/OEM banner, a three-segment graphic window is displayed which consumes the remaining 23 lines of the screen.  A Menu resides on the top line, a dynamic keystroke Prompt is found on the bottom line, and the remaining region is referred to as the Edit Page:

Menu Line

A list of utility names appears on this line, from which a specific selection may be made.  (Eg, Video, Floppy, etc.)  A reverse-video cursor is present which can be moved leftward or rightward to highlight a desired entry.  As the cursor is moved from entry to entry, the Edit-Page (see #3 below) is simultaneously updated to show the corresponding configuration utility.  When ENTER is pressed, that utility will be invoked within the Edit-Page window.  The cursor leaves the Menu-Line, and reappears upon the first editable field within the utility.  Because there are more utility names than can be fit onto the Menu Line, only a subset of the entire menu is displayed.  The rightmost entry displays "More-->", and an attempt to move the cursor onto it causes a new portion of the menu to be revealed.  This new section begins with "<--More", allowing leftward cursor movement back to the original menu.

Edit Page

This "window" displays the utility whose title is illuminated on the Menu Line (#2 above).  As already explained, the utility becomes activated by pressing ENTER (or PgDn), and the cursor appears on an editable field within this Edit-Page.  To further distinguish that this utility is active, the Menu-Line is erased with only this utility's title remaining.  Until exited (back to the Menu), all fields within this Edit-Page are accessible for editing.  Deactivation of the utility occurs when PgUp is pressed.  The Menu-Line is re-displayed and the cursor leaves the Edit-Page, reappearing on the Menu-Line.

While the utility is activated, all keystrokes required to accomplish the edit are indicated on the PROMPT-LINE, discussed next.

Prompt Line

The Setup-Utility is designed to be usable without the aid of this manual.  To this end, a list of acceptable key-strokes and effects is provided on this line, and that list is updated on a keystroke by keystroke basis.  Even an unfamiliar user should be able to maneuver around and successfully configure the system, upon realization that this PROMPT LINE updates dynamically per keystroke.

Depending on BIOS version, a menu similar to next page will be shown upon the SETUP is invoked.

MR BIOS (tm) Copyright (c) 1991, Microid Research Ver 1.0F Port OPTI317

| CPU Type ......................... 386SX-08 | Floppy 0 (A:) ............... 1.2M 3mS |
| --- | --- |
| CPU MHz .................................. 20.0 | Floppy 1 (B:) ................360K 4mS |
| Boot Speed ............................... High | Fixed 80 (C:) ................... Type 17 |
| Coprocessor ............................ 387SX | Fixed 81 (D:) .................. Type 17 |
| RAM Cache ................................ n/a | Boot Sequence .................... C: 1st |
| Shadow RAM ........................ Enable | Cold-Boot Delay .................. 0 Sec |
| Memory-Base ........................... 640K |  |
| Memory-Extended ................. 1280K | Keyboard ................................ AT |
| Memory-System ....................... 128K | NumLock ................................ On |
| Memory-Total ........................ 2048K | Typematic ............................. 30.0 |
| COM1 .......... 3F8 ¦ LPT1 ........ 378 | Video-Primary ...... V/EGA-Color |
| COM2 .......... n/a ¦ LPT2 ........ n/a | Video-Secondary .................... n/a |
| COM3 .......... n/a ¦ LPT3 ........ n/a |  |
| COM4 .......... n/a ¦ LPT4 ........ n/a | Security ............................ Disable |

Summary	Clock	Video	Floppy	Fixed	Boot-Seq	Keyboard	More-->

F10 to Record and Exit		Home End Moves Cursor

MR BIOS (tm) Copyright (c) 1991, Microid Research Ver 1.0F Port OPTI317

<---	More	First-Aid	Speed	Shadow	DMA	Chipset	Security

Certain PC design advances may reveal/cause difficulties with existing software.  These special options may correct/enhance system operation.

Novell Keyboard Management ........	No

A20-Gate Always Enabled .............	No

F10 to Record and Exit     <--- to Select      Home End  Moves Cursor

╔══════════════════════════════════════╗

║                                      ║

╠══════════════════════════════════════╣

║┌────────────────────────────────────┐║

║│                                    │║

║│                                    │║

║└────────────────────────────────────┘║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

║                                      ║

╠══════════════════════════════════════╣

║                                      ║

╚══════════════════════════════════════╝

Prompt-Line Text

The purpose of this section is to further explain the meanings of the keystroke prompts.  They are somewhat abbreviated due to screen space limitations:

F10 To Record and Exit

Press F10 to record the new configuration to CMOS, and terminate the Setup session.  The system will proceed to boot-up.

Home End (Left/Right Arrows) Moves Cursor

The Menu-cursor can be moved respectively to the first entry, last entry, or next leftward/rightward entry.

(Enter) to Select

The Menu-cursor currently illuminates an entry, such as CLOCK, VIDEO, FLOPPY, etc., and the Edit-Window currently shows the configuration related to that Menu entry.  Press ENTER to commence editing that Edit Page.  The cursor will move from the Menu-Line into the Edit-Page, upon the first editable field.  Note:PgDn key can be used instead of ENTER in this context.

ESC For Menu

The cursor is currently in the Edit-Window.  ESC (or PgUp) returns it to the Menu-Line.  Note:the newly edited configuration is not yet recorded to CMOS.  See F10 key description above.

(Up/Down/Left/Right Arrows) Moves Cursor

The cursor is currently illuminating a field within an Edit-Page.  It may be moved to another field via these cursor keys.

(Enter) to Edit

The cursor is currently illuminating a field within and Edit-Page.  This particular field can be edited keying-in numbers or letters.  To invoke the editor, press ENTER.  The field remains illuminated, and a small blinking underline (hardware cursor) will appear under the leftmost editable character in that field.  In general, Left-Arrow, Right-Arrow, Space, Backspace, and AlphaNumerics are accepted in edit mode.  ESC will restore the field to its pre-edit state and the blinking underline will disappear.  ENTER will finalize the edit and the blinking underline will disappear.  All "edit-mode" keystrokes are prompted.

+- Scrolls Choices

Spacebar +- to Change

Spacebar +- Scrolls Choices

The cursor is currently illuminating a field within the Edit-Page which may be changed.  SpaceBar and "+" make visible (select) other available options.  The options are rolled through a list in the forward direction.  BackSpace and "-" roll the options in reverse order.

Esoteric Prompts

A few special-case prompts also exist.  Generally, they specify a range of numbers or a particular set of AlphaNumeric characters that will be accepted in the field.  For example, the CLOCK Time-Of-Day subfield accepts Alphabetic "a" and "p" to indicate am and pm.

The SECURITY utility requires pressing ENTER after selecting a new configuration.  This additional step is not consistent with behaviour of the other utilities, but is necessary so that a new password can be prompted when appropriate, and so the current password is not dismissed should the user simply scroll through available options.

Due to the severity of consequences, the LOW-LEVEL-FORMAT field column within the FIXED disk configuration utility cannot be accessed until CTRL-F is pressed.  Pressing ESC while the cursor is in that column will move it safely to a non-Format column on the screen.  While the format is in progress, ESC will immediately terminate (abort) the process.

2.		EDIT-PAGE / UTILITIES

There are currently 12 utilities contained in Setup, each accessible in a separate, dedicated Edit Page.  As the cursor is moved horizontally across the Menu Line, new Edit Pages appear in the Edit Window which correspond to that menu entry.  A quick summary of the utilities is given here, and full descriptions are given in the remainder of this section.

| Utility | Description |
| --- | --- |
| Summary | The entire hardware/feature config is summarized |
| Clock | Time, Date, and daylight Savings config utility |
| Video | Video configuration Utility |
| Floppy | Floppy Disk subsystem configuration utility |
| Fixed | Fixed Disk subsystem config and format utility |
| Boot-Seq | Boot sequence configuration utility |
| Keyboard | Powerup NumLock and Typematic configuration utility |
| Speed | CPU speed configuration utility |
| Shadow | Shadow RAM config and adaptor ROM display utility |
| DMA | DMA timing parameters |
| Chipset | Memory Wait States and Bus Speed Selection |
| Security | Password Security configuration utility |

(1)	SUMMARY UTILITY PAGE

This Page serves as both a summary utility and as the signon screen of the Setup-Utility.  Most every characteristic of the Computer can be viewed here, but editing the fields is not permitted from this Page.  (Should a configuration change be desired, select the desired utility by moving the Menu cursor).  Each of the fields are discussed below, and an example given of how they might appear:

CPU

The type and "stepping" (revision) number of the CPU is shown.

| CPU 80386SX-08 | indicates a '386SX, rev 08 processor chip |
| --- | --- |

MHz

The oscillator frequency driving the CPU (max speed if multi-speed).

| Mhz 20.0 | indicates 20 MegaHertz speed |
| --- | --- |

NPX

Numeric-Coprocessor-Extension type found in the system.

| NPX | Description |
| --- | --- |
| n/a | if none found |
| 80387SX | if '387SX |

CPU SPEED

The programmable boot-time CPU speed is indicated by this field.

| CPU Speed | Description |
| --- | --- |
| Low | if slow speed |
| High | if fast speed |

SHADOW RAM

Indicates if any part of the (384K) ROM space is mapped to shadow RAM.

For example, Shadow RAM ..... n/a

Shadow RAM ..... Enable

Shadow RAM ..... Disable

MEMORY-BASE

Indicates the amount of base memory (below 1 Megabyte boundary) confirmed to be in working order.  Possible range is 64K to 640K.

MEMORY-EXTENDED

Indicates the amount of extended memory (above 1 Megabyte boundary) found to be in working order.  Possible range is OK to 15360K.

For example, Memory-Extended ...... 1024K

MEMORY-SYSTEM

Indicates the amount of special OEM memory found to be in working order.  Typically, this field will be un-implemented (OK), or will represent the 384K available for shadow RAM or relocation to the Extended Memory pool.

For example, Memory-System ..... 384K

MEMORY-TOTAL

This is simply a sum of the three preceding quantities.

For example, Memory-Total ..... 2048K

COM1

Indicates if RS232 serial port COM1 is present, and its I/O address.

For example, COM1 ..... n/a

COM1 ..... 3F8

COM1 ..... 2F8

COM2

See COM1 description above.

LPT1

Indicates if parallel (printer) port LPT1 is preset, and its I/O address.

For example, LPT1 ..... n/a

LPT1 ..... 3BC

LPT1 ..... 378

LPT1 ..... 278

LPT2

LPT3

See LPT1 description above.

FLOPPY 0 (A:)

Indicates floppy drive A: type and step rate.  The "type" can be 360K, 1.2M, 720K, or 1.4M.  The "step rate" is given as an amount of time (in milliseconds) required for the read/write head to be moved to an adjacent cylinder.  There are two step-rate settings possible for each drive type, a slow and a fast rate.  (The lower number is the faster rate, and provides improved performance).

| Floppy 0 (A:) | Description |
| --- | --- |
| N/A | No controller card found |
| None | Card present but no drive |
| 1.4M 3mS | 3.5 inch hi-cap, fast step |

FLOPPY 1 (B:)

See description of drive A: field, above.

FIXED 80 (C:)

Indicates fixed disk C: type, step rate encoding if nonstandard, and if "Translation Mode" is in effect.  The drive "type" may be any value from 1 to 47, and is dependent on the fixed disk present in the system.  If this drive has more than 1024 cylinders, a special feature "Translation Mode" may be selected (via FIXED menu entry) to make use of the cylinders in excess of 1023.  If selected, then "T" will be appended to the drive type number as a reminder.  Another special feature allows the selection of non-standard step rates to gain higher performance.  Selection of a non-zero steprate encoding is non-standard, and as a reminder, is shown between braces.

| Fixed 80 (C:) | Description |
| --- | --- |
| N/A | Fixed controller card not present |
| None | Drive type "0", no drive present |
| 2 | Drive type "2", no special features |
| 47T | Drive type "47T", Translation Mode |
| 8 {F} | Drive type "8", special Steprate |
| 46T {E} | Drive type "46", Translate+Steprate |

FIXED 81 (D:)

See description of drive C: field, above.

BOOT SEQUENCE

Specifies the selected order in which the disk(s) will be booted.

| Boot-Sequence | Description |
| --- | --- |
| A: 1st | Try A:first, if failure, try C: |
| C: 1st | Try C:first, if failure, try A: |
| Prompt | Screen prompt for drive A: - D: |

KEYBOARD

The keyboard type/protocol is shown in this field.

Eg, Keyboard ... AT  PC/AT type keyboard

Keyboard ..... XT  PC/XT type keyboard

NUMLOCK

The programmable powerup NumLock state is shown in this field.

Note: This is only meaningful for AT-type keyboards.

For example, NumLock ..... Off

NumLock ..... On

TYPEMATIC

The programmable powerup "typematic" repeat rate is shown here.  Many values are possible, up to 30.0 characters per second.

Note: This is only meaningful for AT-type keyboards.

| Typematic | Description |
| --- | --- |
| Default | No rate programmed, speed approx 10 cps |
| 30.0 | 30 cps rate is programmed |

VIDEO-PRIMARY

Indicates the video adaptor which will be in use when the system boots (DOS).

| Video-Primary | Description |
| --- | --- |
| None | Special Support, see VIDEO menu |
| Monochrome | B/W card |
| CGA - Snow | CGA, slow access due to "snow" |
| CGA - Fast | CGA, "snow" isn't problem |
| V/EGA-Mono | Advanced Graphics, B/W monitor |
| V/EGA-Color | Advanced Graphics, Color monitor |

VIDEO-SECONDARY

Indicates if a second video card is present in the system.  (Typically, two CRT systems are used in CAD applications).  If present, this monitor will only be supported by special software aware of its presence.  In particular, it will not be seen by DOS.  Most systems will NOT have dual monitor configurations, and this will be displayed:

Video-Secondary ..... n/a

Otherwise, refer to VIDEO-PRIMARY above.

SECURITY

The state of password-Security is shown in this field.

For example, Security ..... Enable

Security ..... Disable

(2)	CLOCK CONFIGURATION UTILITY

The battery backed Real-Time-Clock (RTC) time, date, and daylight savings feature are programmed through this utility.

NOTE:

Changing the setting of any one field at 11:59:59 pm may permit unexpected roll-over of another field, and especially unintended results may occur during daylight-savings transition periods (*).

TIME

The time field is shown in USA 12 hour format, followed by a time-of-day indicator "a" or "p" (am/pm).  When changing the time setting, all values are checked.  if necessary, Setup will enforce (impose) legal limits.

| Time hh/mm/ss t | Military |
| --- | --- |
| 12:00:00 a | 00:00:00 |
| 09:10:11 p | 21:10:11 |

DATE

The date field is shown in USA mm/dd/yyyy format.  When editing the date setting, all values are checked (include leap years).  If necessary, Setup will enforce (impose) legal limits.

| Date (mm/dd/yyyy) |  |
| --- | --- |
| 01/23/1990 | January 23, 1990 |

DAYLIGHT SAVINGS

The RTC has a built-in capability to automatically adjust the time on the two daylight savings days of the year (*).  If this is desired, set the field to "Enable".  Otherwise, set the field to "Disable".  Note that in general, nothing will be immediately observable by setting the field to either state.

For example, Daylight Savings ..... Enable

Daylight Savings ..... Disable

(*)	On the last Sunday in April, the time increments from 1:59:59 am to 3:00:00 am.  On the last Sunday in October, when the time first reaches 1:59:59 am, it is rolled-back to 1:00:00 am.

(3)		VIDEO CONFIGURATION UTILITY

The primary video adaptor is declared through this utility.

A "primary" adaptor is defined to be the video card which will be recognized by an operation system (such as DOS) when it receives control of the system after a powerup or warm-boot.  If there are two video cards present, the "other" one becomes (by default) the "secondary" video adaptor.  The secondary adaptor is placed into a standby state, waiting to be activated by specialized software which knows of its existence.  Ordinarily, there will be one and only one video card in the system, and it is (trivially) assigned "primary".

No jumper setting is required, all that need be done is to select an adaptor from those available in this utility, and the only choices made available will be consistent with the equipment found in the computer.  In systems with only a single video card (the usual case), the choices are limited to that present card, or "none".

As suggested, a video-less system may be configured.  This is useful in certain specialized monitoring/control applications.  These steps should be taken to configure a video-less system: First, while some video card is present in the system, enter the Setup utility and select the VIDEO configuration utility (here).  Select the "none" option, and exit this screen (press PgUp).  Then press F10, which records the new configuration and boots the computer.  Turn the system off, and remove the video card.  The computer is now ready to be run without video.

When two video cards are present in the system, one must be color, and the other B/W.  MR BIOS TM will identify these cards, and make both choices available for primary selection.  If one of the cards is V/EGA, its operation in the capacity of monochrome or color will automatically be detected and reported.  Although V/EGA cards generally require setting dipswitches, MR BIOS TM will override those settings according to the primary adaptor selected via this utility.  Note well that "some" valid dipswitches state must still be set on the V/EGA card so that it may correctly initialize itself.

If a CGA card is present in the system, a second field will appear on the screen.  Its purpose is to allow selection of video access speed.  Some CGA cards will produce screen "snow" (also called "hash") when accessed too quickly in 80-column AlphaNumeric mode (the usual mode).  Such cards require synchronization with video retrace/sync signals to prevent this undesirable effect.  Most CGA cards do not require this synchronization, and dramatic speed improvement (and a lack of scroll "flicker") is realized through unrestrained access.  Try selecting the fast mode - either the "snow" problem won't exist, or it will be intolerably obvious.

(4)	FLOPPY DISK CONFIGURATION UTILITY

This utility configures the floppy drive subsystem, drives A: and B:.

The BIOS must be informed of the types of floppy drives in the system, their step rates, and in the case of 720K drives, whether or not a "media-change-line" is present.

If the drive controller card is absent, no choices will be available, and there is nothing to do here.  that is rarely the case, but is supported configuration.  usually, a controller card is present along with at least one diskette drive.  In this more usual case, one of four drive types can be specified for each, drive A: and drive B:.  The choices include two 5.25 inch drive types: 360K and 1.2M drives, and two 3.5 inch drives types: 720K and 1.4M drives.  If a drive is not connected to either the A: or B: card connector, then "none" should be selected for that drive.  Otherwise, an error message will be generated each time the computer is booted.

A "media-change-line" field is also displayed with each drive type, but is only programmable in the 720K case.  This "media-change-line" is an electrical signal produced by a mechanical switch in the disk drive.  It is activated when a disk is inserted.  (You can feel the spring-loaded switch actuate when inserting a disk in a 1.2M drive).  Operating systems like DOS check this signal each time the disk is accessed, saving the time required to re-read the disk directory into memory if the switch is not tripped.  There were difficulties during the transition period from PC-XTs to PC-ATs regarding this, because the switch was NOT present on the XT-720K drive, but WAS present on the AT-720K drive.  This BIOS supports either kind of 720K drive, by allowing (requiring) the change-line to be specified.

Follow these steps to determine which type of 720K drive you have: Configure the system for Change-Line present (ie, select "Yes"), and exit the Setup Utility.  Insert a (non-blank) disk into the drive, and type DIR to see that directory.  Remove the disk, and insert a different one known to have a different directory.  Type DIR again, and examine the new directory.  If this drive lacks the Change-Line, then the latter directory display will (incorrectly) be identical to the original disk's.  In such a case, invoke the Setup-Utility again and correct the Change-Line field to "No".  NOTE: If a 720K drive is improperly configured to indicate "No" Change-Line when it is in fact present, the drive will operate correctly, but with diminished performance.

(5)	FIXED DISK CONFIGURATION / LOW LEVEL FORMAT UTILITY

This utility configures the fixed disk subsystem, drives C: and D:

DRIVE TYPES

If one or two fixed disks are present in the system (and connected to a controller card), then the drive(s) must be configured via this utility.  The main purpose is to assign a drive "type" to the drive(s).  The "type" in turn is used to select a table of parameters which completely describe the drive characteristics.  The drive parameter tables are comprised of these entries:

(1)   Number of Cylinders

(2)   Number of Heads

(3)   Starting Precompensation Cylinder

(4)   Landing Zone Cylinder (Park)

(5)   Number of Sectors per Track

Up to 47 drive tables/types can be defined within the BIOS in a compatible fashion, and the first 23 drive types (1-23) are standardized throughout the industry.  The remaining 24 drive types/tables are BIOS implementation dependent, of course chosen in an attempt to service the widest variety of drive types.  However, there are many hundreds of drives being marketed, and it is thus impossible to explicitly support all of them.

PROGRAMMABLE TYPES 46 & 47

This BIOS provides tables for types 1-45, reserving types 46 and 47 as "user programmable" drive types.  The parameters for each drive type can be viewed by scrolling they "Type" field through the entire sequence 1-47.  (See the bottom "Prompt" line on the screen.  Pressing + or - while the cursor is on the "Type" field will cause the fields to scroll).  If it happens that your particular drive parameter table is not among the 45 explicitly defined in this BIOS, then the option exists to program them "by hand" into either drive table 46 or 47.

To create a "user programmed" drive parameter table, first move the cursor to the "Type" field and select 46 or 47.  Then move the cursor down to the next field, "Cylinders", and press ENTER.  Key in the number of cylinders, and press ENTER when done.  Continue in this fashion, editing the "Heads", "Precomp", "Landing", and "Sector" fields.  It's as simple as that.

TRANSLATION MODE

Larger capacity drives with higher track density are becoming increasingly available, but the standard BIOS interface limits support to 1024 tracks.  The standard BIOS interface cannot take advantage of these new technologies.  It is possible through translation schemes, however, to support drives with capacity in excess of a GigaByte.

MR BIOS TM provides such a Translation Mode to surpass this 1024 (1K) cylinder limitation.  up to 16384 (16K) cylinders can be addressed via the Translation Mode... but MOST STATE OF THE ART DRIVE CONTROLLER CARDS SUPPORT ONLY 4K CYLINDERS to date.  Many older cards only support 2K cylinders.  The Translation Mode implementation is designed to be compatible both with programs which exclusively use the BIOS interface, and programs which interpret the drive tables and run the drives directly.  If Translate Mode is not selected (ie, the field is set to "No"), then only the first 1024 cylinders will be accessible through the BIOS interface.

LOW LEVEL FORMAT

The large drive support is complemented by a built-in Low-level-Format utility.  Be very careful when using this utility.  As with all format programs, the data currently on the drive will be forever lost.  Most typical uses of the format utility will involve formatting the entire disk, but it is not required here.  A range of cylinders may be specified as the target of the operation, and the sequence may be increasing or decreasing order.  Also, the "interleave" (also called "skew factor") is selectable.  In general, select an interleave of "3" (default) unless your drive card has 8K or more RAM on board, or specifically claims to be a "1:1" card.  In that case, select an interleave of "1".

NOTE:

Be wary of products which calculate the "best" interleave factor for a drive/card combination.  Multi-sector reads can often be (optimally) accomplished with a tight interleave, but programs employing sequential single-sector reads (including DOS 3.3 and earlier) will suffer greatly from such an "optimization".

(6)	BOOT SEQUENCE CONFIGURATION UTILITY

The order in which the disks are searched for a "boot sector" is configurable via this utility.

In a typical BIOS implementation, whenever CTRL ALT DEL is pressed, an attempt to boot drive A: always occurs first, and if the drive is found to be empty, then drive C: is booted.  If an unbootable disk is present in drive A:, the system pauses and a message is displayed to that effect.

In most installations, drive A: is used simply to transfer files, and C: is the general purpose storage and boot device.  If a disk is present in drive A: when the computer is booted, it is usually there by mistake.

For convenience, MR BIOS TM supports a programmable boot order.  In addition to the usual "A: first, then C:" sequence, the order may be reversed so that drive C: is accessed first.  This promotes a swifter boot process, and eliminates the annoyance of having to remove a floppy from drive A: prior to booting.

A special "hot-key" warm-boot sequence CTRL ALT ENTER is available to override the default sequence established here in this utility.  The effect is identical to CTRL ALT DEL, except a prompt appears on the screen asking which drive should be booted.  In most circumstances, we suggest setting the default boot sequence to "C: 1st", and when the rare occasion arises requiring a floppy boot, use CTRL ALT ENTER to override to drive A:.  Note that drives B: and D: may also be specified at the prompt, but this is incompatible with current DOS revisions.

During cold-boot (powerup or pushbutton reset), the Screen-Prompted boot can be invoked by pressing ENTER during the memory test.

The default boot sequence may also be set to Screen-Prompt, in which case booting always requires explicit selection of the boot drive.

(7)	KEYBOARD CONFIGURATION UTILITY

This utility is used to configure the boot-time NumLock state and the keyboard Typematic rate.

NUMLOCK

Many users have become accustomed to the 84-key AT-style keyboard, which lacks a dedicated cursor-control keypad.  The NumLock control key is used on these keyboards to toggle the Numeric Keypad from numeric operation to cursor functions.  The standard powerup default state of NumLock is "off", so the Numeric Keypad is in the cursor mode of operation after booting.

The newer generation 101/2 key keyboards have a dedicated Cursor Keypad (in addition to the Numeric Keypad).  Although the NumLock key is present on this keyboard, it is only there for downward compatibility with the earlier 84 key keyboards.  The standard powerup default state of NumLock is "on", so the Numeric Keypad is in the intuitively correct numeric mode.

Through this utility, the boot-time default state of NumLock can be set to individual preference.  Select "Disable" or "Enable" accordingly.

TYPEMATIC RATE

When a key is held pressed on the keyboard for some period of time, that keystroke will begin repeating at a predefined rate.  The initial delay is by default 0.5 seconds, and the repeat rate defaults to 10 characters per second.  This "typematic" feature is a function of the keyboard, and is not produced by the system BIOS.  However, (most) AT-style keyboards permit overriding the initial delay and subsequent repeat rate.

MR BIOS TM can be configured to issue override typematic parameters to the keyboard at boot-time.  Both the Delay and Rate parameters can be selected, per personal preference.  To accomplish this, "Enable" the "Typematic Override" field, and select the Delay and Rate in the other fields.  We suggest a Delay of 0.5 seconds, and a Repeat Rate of 30.0 cps.

Should it happen that the keyboard fails to function properly when overriding the default typematic state, then "Disable" the "Typematic Override" field.  The Delay and Rate fields will display "Default" in response to this selection.  When configured this way, no typematic parameters will be issued to the keyboard at boot-time.

(8)	SPEED CONFIGURATION UTILITY

This utility allows the boot-time speed of the system to be selected, either high-speed or low-speed.

It is very unusual to configure the system to boot the computer into the slow-speed state, since the only effect is degraded performance.  Two instances where it may be necessary are: An add-on card or other hardware device malfunctions when running at full speed, or, a software program that is always used fails at full speed.  In general, set the default system speed to "High".

A convenient method is available to change system speed "on the fly", without affecting the boot-time default speed.  The hot-key sequences CTRL ALT + and CTRL ALT - set the speed to high and low, respectively.

(9)	SHADOW RAM CONFIGURATION UTILITY

This utility performs two functions.  Its main purpose is to allow the boot-time state of the shadow RAM to be configured.  It also provides a means to view the ROMs found in the 640K - 1Meg region, which is useful for users of EMS drivers and Virtual-86 programs.

Explanation of the term "shadow RAM":  The system BIOS, V/EGA ROM, and a variety of other programming is stored in one-time programmable devices called ROMs (Read-Only-Memory).  Two important properties of ROMs which lend them to this use are:

(1)	The data is protected, it cannot be changed by mistake, and

(2)  	the data is retained when the power is shut off

A notable deficiency in ROM technology is its speed - ROM access time is typically two to three times slower than normal memory (RAM) access.  to further exaggerate this difference, ROM data is generally accessible only in 8-bit quantities, whereas general purpose memory can be read in 16-bit or 32-bit quantities (286/386 respectively).  A simple-minded scheme to improve the access speed is to copy data from slow ROMs to fast RAM, and thereafter using the RAM image instead of the ROM.  When RAM is dedicated to this purpose, it is called "shadow RAM".

In nearly all cases, the shadowed regions should be configured as "Write-Protected".  This will prevent the shadowed data from becoming corrupted should a programming error write data into that region.  Although very unusual, several board products are known to possess Read/Write memory embedded in the same region as the ROM.  Such "ROMs", if shadowed, must be configured "Read/Write" to function correctly.

The entire 384K ROM space is subdivided into ten regions, depicted by the ten fields shown on the screen.  The Video and Adaptor ROM region is comprised of eight equal 16K segments, and the System and BIOS ROM region is divided into two 64K segments.  This breakdown is generic, and may suggest a finer granularity of shadow capability than is actually available in a particular computer.  When this is the case, enabling shadow in one region will automatically cause other regions to become shadowed.  The screen will be updated accordingly.  A similar situation exists with the "Read/Write" vs "Write-Protect" attribute assignment.

ROMs of varying capacities may be present in the computer, and a single one may (and often does) extend beyond a 16K segment boundary.  Also, a single ROM might not be aligned on a 16K boundary, and consequently resides in two adjacent segments even though it is smaller than 16K.  This utility scans the entire 384K ROM space, and assigns a number to each ROM found (beginning with ROM #1).  Each of the ten fields on the screen show the ROM #n found in the respective segment.  If a single ROM spans two or more (adjacent) 16K segments, each associated field will display the identical ROM #n.

It would be bizarre and most likely troublesome to shadow anything less than an entire ROM.  Therefore, this utility automatically extends the shadow state set in a field to all other fields possessing the same ROM #n.  For example, if two adjacent fields show ROM #1 present, and then one of the fields is set to "WP-Shadow", the other field will simultaneously be updated to display the identical state (WP-Shadow).

Note that when all fields are set to non-shadowed, they will either display "Vacant" (no ROM present there), or they will show the "ROM #n" residing at that location.

(10)	DMA PARAMETER UTILITY

DMA timings may be adjusted to increase the performance of EtherNet, SCSI and other Bus-Master peripherals.  However care should be taken when altering these parameters so that it won't violate the specification, otherwise problem may arise.  The default values are recommended for reliable operation.

DMA CLOCK

ATCLK/2 *

ATCLK/1

8-BIT WAITS / 16-BIT WAITS

1 WS *

2 WS

3 WS

4 WS

COMMAND WIDTH

Normal *

Compress

MEMR# SIGNAL / MEMW# SIGNAL

Normal *

Early

*   Default

(11)	CHIPSET PARAMETER UTILITY

This utility identifies the chipset revision and provides a means to define the system memory wait state and the AT-bus speed.  BIOS defaults are conservative settings, these allow the system to boot at the CPU's rated speed.  Operational capability may be temporarily reduced until these configuration parameters are set to mirror the true system configuration.

Reg10: Wait States - Write

Reg10: Wait States - Read

1 *

0

For 20MHz system, select '0' if you use DRAM with access time of 80ns or faster.

For 25MHz system, select '0' if you use DRAM with access time of 70ns or faster.

A setting of '0' will allow the system to performance zero wait state memory access under the page/interleave scheme.

Reg 14: AT-Bus Speed

| 20MHz System | 25MHz System |
| --- | --- |
| * 6.7 | * 8.3 |
| 10 | 12.5 |

*   Default

It is strongly recommended that the AT-Bus speed should not exceed 8 MHz so as to comply with ISA AT-Bus specification.

(12)	SECURITY CONFIGURATION UTILITY

This utility is used to enable or disable Password Security.

This Security feature offers a measure of protection against unauthorized use of the computer, by requiring a password be entered when the computer is first being powered up.  Three opportunities are given to key-in the correct-password (ie, two mistakes are allowed).  If three unsuccessful attempts are made to guess the password, the system will be halted and an alarm will be sounded.  The alarm will persist until the power is cycled off.

After the correct password is provided, the computer will boot-up and operate in the normal fashion.  CTRL ALT DEL will not re-invoke the Security Clearance procedure encountered at powerup.

The Setup-Utility is also password protected when Security is active.  When entering the Setup-Utility via CTRL ALT ESC, the SUMMARY page will be displayed as usual, but the user will be prompted to press "F10 to Exit", or "ENTER for Security Clearance".  Ie, the user must leave, or enter the correct password.  The configurations cannot be modified until the password is correctly typed.  As is the case during powerup, three failed attempts to supply the correct password results in an alarm being sounded and the system halting.

When enabling the Security feature (ie, a transition from "Disable" to "Enable"), a second field will appear on the screen prompting entry of a password.  A password consisting of zero to ten characters, followed by ENTER, must be typed in.  Asterisks are echoed to the screen (instead of the character typed), and the only keystroke available for editing is BackSpace.  Thus, the password cannot be viewed.  Not even by the person who creates it.  Upon completing the password (ie, after ENTER is pressed), a prompt will appear requiring the same password again be entered.  This step ensures that no typographical errors exist in the original password.  The entire process will reloop if the latter entry does not match the initial password.

All keystrokes normally recognized by BIOS are available for use in the password.  Ie, F1, ALT F1, SHIFT F1 are all valid and considered different.  Also Alphabetic characters are case-sensitive, which means for example that "a" and "A" are distinguished from one another.

Once the password has been defined, it may subsequently be changed using this utility.  To accomplish this, toggle the "Security" field entry from "Enable" to "Change Code", and press ENTER.  The resulting steps are identical to initially installing the password.

The password entry feature may also be set to "Disable", which turns off Security.  Be aware that this configuration allows anyone who invokes the Setup-Utility to re-Enable the Security feature with their own password.  It could happen for example, that a curious but otherwise unfamiliar user accidentally sets a new password.  The situation may not be discovered until too late, when the computer cannot be powered up because the correct password is unknown.  It is therefore STRONGLY ADVISED that if this Security feature is going to be left disabled, then the Master-Override Password Security Jumper `JP2' be set to position '1-2' (Master-Disable).

Section Five :

Software

*************************************************

5.1	VGA Mode Switch Utility

5.2	Installing Procedure

5.3	Installing Software Applications

5.3.1		AutoCAD

5.3.2		AutoSHADE

5.3.3		Lotus 1-2-3

5.3.4		Windows 3.0

5.3.5		Ventura Publisher

5.3.6		GEM

5.3.7		CADVANCE

5.3.8		PCAD

5.3.9		Framework II

5.3.10	WordStar Professional

5.3.11	WordPerfect

5.3.12	Generic CADD Level 3

5.3.13	VersaCAD Design Ver. 5.3 & 5.4

*************************************************

1.			VGA MODE SWITCH UTILITY

VGAMODE.EXE is a menu-driven program designed to help the user select the right video mode for special application or for game programs which can't run in standard VGA mode. It also lets you LOCK the VGA port into a particular configuration in order to reboot that configuration rather than the default VGA mode.  To invoke this program:

1.	Type : VGAMODE  at the DOS prompt

A simple menu will appear listing some of the options available.

2.	Use the up and down arrow keys to select the feature you wish to access. When the desired feature has been selected (highlighted), press the [ENTER] key.

3.	Choose "Exit to DOS: to quit.

VGAMODE.EXE may be executed directly from the DOS prompt line to bypass the menu. This is useful for incorporating VGAMODE commands into a batch file.  You just need to type VGAMODE and the parameters listed in the following table.

The following table explains how to use VGAMODE parameters.

| Command | Explanation |
| --- | --- |
| VGAMODE | Brings up the VGAMODE Menu |
| VGAMODE CGA | Sets the VGA Port to CGA operation |
| VGAMODE MDA | Sets the VGA Port to emulate the Hercules Graphics VGA Port with no graphics memory allocated |
| VGAMODE HERC0 | Sets the VGA Port to emulate the Hercules Graphics VGA Port with one page of graphics memory allocated |
| VGAMODE HERC1 | Sets the VGA Port to emulate the Hercules Graphics VGA Port with both pages of graphics memory allocated |
| VGAMODE COLOR | Changes the VGA Port to color VGA mapping as needed |
| VGAMODE MONO | Changes the VGA Port to monochrome VGA mapping as needed |
| VGAMODE 13225 | Switches the VGA Port to 25 line, 132 column text mode |
| VGAMODE 13243 | Switches the VGA Port to 43 line, 32 column text mode |
| VGAMODE LOCK | Locks the current video standard so it will survive a warm boot |
| VGAMODE 200 | Sets the number of lines used for VGA text mode to 200 |
| VGAMODE 350 | Sets the number of lines used for VGA text mode to 350 |
| VGAMODE 400 | Sets the number of lines used for VGA text mode to 400 |
| VGAMODE 600 | Switches the VGA Port to 800 by 600 resolution |
| VGAMODE 768 | Switches the VGA Port to 1024 by 768 resolution |
| VGAMODE 400256 | Switches the VGA Port to 640 by 400-256 color |
| VGAMODE 480256 | Switches the VGA Port to 640 by 480-256 color |

Note :	800 x 600 -256 color mode cannot be switched by using VGAMODE, as this mode does not support scrolling at DOS prompt. So, this mode must be used with the application drivers.

2.		INSTALLATION PROCEDURE

Before beginning a software installation, please note the following:

1.	All of the drivers and utilities provided on the diskettes must be installed by using the install program.

2.	The install program must be executed from a floppy drive because it will not allow you to install to the source drive.

3.	You should have the command processor (command.com) in the path, or you can copy it to the current directory.  You must use the diskcopy command to make working copies of the VGA Utilities Disk.

4.	If the installation process fails, follow the instructions provided in the software manual to complete the installation.

5.	In a normal installation, the required driver(s) are decompressed and copied to the directory that you specify.  The directory is created if it does not already exist.  This process can be verified by the following screen messages:  Installing, Reading, Verifying, Writing, Decompressing.

Use the following steps to install drivers/utilities:

1.	Insert the VGA Utilities Diskette into drive A.  At the DOS prompt type

A:install

Important Messages:  General warnings and any recent changes or updates will be displayed on the monitor.  Read this information carefully.  The main selection menu will display.

2.	Use the arrow keys [Page Up], [Page Down], [Home] and [End] to access options not shown on the screen.  Press [Enter] to make a selection.

3.	When a selection has been made, the install program will prompt you for the target drive and directory (if applicable).  Remember to select only a floppy drive as the target drive when you have two or more floppy drives.  (You can not install to drive A because it is the source drive. Moreover, do not copy the drivers directly to the hard disk and install from it.)

4.	Follow the screen prompts to install the drivers and utilities.

To avoid problems due to the variety of possible configurations, it is recommended that you first read the appropriate section in the software manual.  Some applications can run in the new resolution immediately after the installation of your application.  Other applications may require you to run a configuration program in order to recognize the new driver, while others require a complete reinstallation of the application itself.

NOTE :

1.	If you install the driver(s) into the directory where the application is installed, the install program will simplify the procedure.

2.	Be sure to specify the full path name when prompted for the driver(s).  Note that some applications require the driver(s) to be in the same directory as the application itself, yet others require the driver to be on a floppy disk.  Simply copy the driver(s) to the appropriate directory/drive and install the driver manually.

3.		INSTALLING SOFTWARE APPLICATIONS

3.1		AutoCAD

The VGA Utilities Disk supports Display and Non-Display List Drivers for AutoCAD 10, and Non-Display List Drivers for AutoCAD9.  Expanded memory is required to use Display List Drivers.

To configure AutoCAD 10 for 800 by 600 or 1024 by 768 graphics mode, follow these instructions:

1.	Use the install utility to automatically copy the Display List and Non-Display List Drivers from the VGA Utilities Diskette to the AutoCAD directory.

These files load an ADI driver, which will allow AutoCAD to work in the extended resolution mode.  For specific driver names, please refer to the README.DOC text on the Utilities Diskette.

2.	Go into the directory to which you have just copied the file and type :

[filename]

The AutoCAD drivers has two types :

(i)   Display List Drivers and

(ii)  Non-Display List Drivers

For Display List Driver :

The user can change the parameters of the driver by using the CONF.EXE program.

e.g.	CONF WDVGADL8.EXE

where WDVGADL8.EXE is a Display List Driver for 800x600 mode.  CONF.EXE will ask you to select display parameters for WDVGAL8.EXE.  These parameters will then be saved to the display driver.

After configuring the driver, type

WDVGADL8 f[n]

where [n] is the number of kilobytes of expanded memory that the driver should leave free for use of AutoCAD.

Note :  For Display List Drivers, EMS driver must be installed first.

For Non-Display List Driver :

The user type the filename which just have copied.

3.	Execute ACAD to start the AutoCAD program.

Option:   type:  [file name] i

(i =  used to run AutoCAD with an inverted black background)

You may need to change AutoCAD vector interrupt code to something other than the default "7A".

type:  [file name] xx

(xx = a two digit hexadecimal vector interrupt code)

4.	Additional readme files may be automatically copied from the Utilities Diskette for you (depending on the driver selected).

3.2		AutoSHADE

This section describes how to configure AutoShade to utilize the extended modes with the VGA Port.

1.	Delete the configuration file SHADE.CFG.

2.	Copy the ADI rendering driver by running INSTALL from the supplied VGA Utilities Diskette.

3.	To run AutoShade with the new ADI driver, run "SHADE" and answer the following questions as indicated.

a.	Question:	Select Display Device

Chose:		IBM Video Graphics Array

b.	Question:	Select Rendering Display Device

Chose:		Autodesk Device Interface Rendering Driver

c.		Question:	Shade a Single Screen

Chose:		Yes

d.	Question:	Does Flip Screen Require a Redraw?

Chose:		Yes

3.3		LOTUS 1-2-3

This section describes how to configure Lotus 1-2-3 to use 640 by 480, 800 by 600 graphics or 132 column text modes for the VGA Port.

Caution:  When adding new drivers to the 1-2-3 or Symphony install program, there will be a file created called SINGLE.LBR. If using Lotus 1-2-3 Release 2.0 or Symphony Release 1.1, make sure there is not already a file called SINGLE.LBR on the Utilities/Install Disk or in the Lotus hard disk subdirectory. If there is such a file, delete it prior to running INSTALL.  For 1-2-3 Release 2.1 or Symphony 1.2, it is not necessary to delete the SINGLE.LBR file.

The following instructions are used to copy files from the VGA Utilities subdirectory.

1.	Use the install utility to copy the  drivers from the VGA Utilities Diskette to the 1-2-3 program disk or directory.  For specific driver names please refer to the readme file on the Utilities Diskette.

2.	Please see your Lotus manual for installation procedures for the drivers. When using Lotus's INSTALL.EXE file, choose:  ADVANCED OPTIONS.  Add New Drivers to Library  -  to add the .DRV files to the SINGLE.LBR files.

3.	Choose: Modify Current Drivers Set - to list the new drivers on the menu.

3.4		WINDOWS 3.0

This section describes how to configure Microsoft Windows 3.0 to use the extended modes with the VGA Port.

You must install Window 3.0 prior to beginning this procedure.

1.	Use the VGA Utilities Diskette to create the Windows 3.0 subdirectory on your hard disk (or diskette).

2.	Go to the directory where Microsoft Windows 3.0 is installed and type SETUP.

3.	Use the Up/Down arrow keys to select the DISPLAY option.  When highlighted, press [Enter].

4.	Use the arrow keys to scroll to the bottom of the list and select OTHER.  You will be asked to insert the Display Driver Disk (VGA Utilities Diskette) in a floppy drive.

5.	Enter the pathname to the display driver subdirectory built by INSTALL on the hard disk (or the  diskette).

6.	Select the desired driver from the list defined on the screen.  When the driver has been changed, you will return to the configuration screen.

7.	Choose "Accept configuration shown above" and press [Enter] to execute the "Complete Changes" process.  Windows will return to DOS when the configuration has been completed.

Note:  COLORS.EXE is a Windows program which will display the colors used with the configured driver.

Additional information regarding display driver installation can be found in the text file WIN3_DRV.TXT in the display driver subdirect-ory created by INSTALL.

3.5		VENTURA PUBLISHER

This section describes how to configure Ventura Publisher release 2.0 to use 800 by 600 or 1024 by 768 graphics with the VGA Port.

1.	Run "INSTALL.EXE" by typing INSTALL and then press [Enter].  Use this install utility to automatically copy the drivers supporting the selected resolution from the supplied VGA Utilities Diskette to the filename "CONFIG.DAT" which is in the Ventura directory on the hard disk.

For specific driver names please refer to the readme file on the Utilities Diskette.

2.	Change to the drive and directory which contains the CONFIG.DAT file.

3.	Select Ventura 1.1 or 2.0 in the menu by highlighting the name and pressing [Enter].  Exit the utility through the menu.

4.	When finished with the INSTALL program, choose 5.  Exit to DOS and press [Enter].

5.	Go to the directory that contains the VP.BAT file   (usually in the root directory).

6.	Change the "DRVRMRGR . . ." line in this file to one of the following:

"DRVRMRGR VP %1 /S=SD_800_C.VGA"

for the 800 by 600 by 16 driver, or

"DRVRMRGR VP %1 /S=SD_800_M.VGA"

for the 800 by 600 by 2 driver, or

"DRVRMRGR VP %1 /S=SD_1K_C.VGA"

for <D>the 1024 by768 by 16 driver , or

"DRVRMRGR VP %1 /S=SD_1K_M.VGA"

for the 1024 by 768 by 2 driver

7.	Type VP and Ventura Publisher will display in the selected resolution.

3.6		GEM

This section describes how to configure GEM to use 800 by 600 or 1024 by 768 graphics to utilize the extended modes with the VGA Port.

1.	Run "INSTALL.EXE" by typing INSTALL and then press [Enter].   Use the install utility to automatically copy the drivers supporting the selected resolution from the supplied VGA Utilities Diskette to the filename "CONFIG.DAT" which usually resides in the GEMapps directory.

For specific driver names please refer to the readme file on the Utilities Diskette.

2.	Change to the drive and directory which contains the CONFIG.DAT file.

3.	Select GEM/3 in the menu by highlighting the name and pressing  [Enter].  When finished with the INSTALL program, choose " 5.  Exit to DOS " and press [Enter].

4.	Go back to the root directory and type GEM.

3.7		CADVANCE

To install Cadvance for use with the VGA Port in the 1024 by 768 or 800 by 600 graphics modes, copy the appropriate file from the VGA Utilities Diskette to the Cadvance disk or directory.

Note: If modifying the program on a floppy disk, be sure to use a duplicate of the Cadvance disk.

3.8		PCAD

To install PCAD:

1.	Run the install program from the supplied VGA Utilities Diskette to extract the selected driver.

2.	Edit the file 'PCADDRV.SYS' located in the root directory.

3.	Change the display line to reflect the change in the driver.

E.g.     Display  \pcad\tkcards\dwdpvga.sys

where DWDPVGA.SYS is 800 by 600 driver, and DPVGA1K.SYS is 1024 by 768 driver.

3.9		FRAMEWORK II

This section describes how to configure the drivers that support 132 columns by 25 or 43 lines of text for the "desktop" as well as drivers that support the 800 by 600 graphics mode for the "desktop" and the "zoom" [F9] function.

Use the utilities to extract the desired driver before using the setup procedure as described on the following page.  Have a blank diskette available to copy the Framework drivers that you have extracted from the Utilities Diskette.

Refer to the following table to select the appropriate driver depending on your choice of screen formats and "zoom" screen formats.

| Screen Format | Zoom Screen Format | Driver Name |
| --- | --- | --- |
| 132x25 text | 640x480 graphics | PVGA1.SC |
| 132x43 text | 640x480 graphics | PVGA2.SC |
| 132x25 text | 800x600 graphics | PVGA3.SC |
| 132x43 text | 800x600 graphics | PVGA4.SC |
| 800x600 graphics | 800x600 graphics | PVGA5.SC |

Before installing Framework II for any of these new drivers, first install Framework II with a screen driver chosen from the standard IBM Enhanced Graphics Adapter, VGA and PS/2 Display Drivers according to the instructions in the Framework II Getting Started manual. Then make sure Framework is functioning properly before proceeding with the installation of the extended mode drivers.

1.	To install the new Framework II drivers, run the SETUP program on the Framework II setup disk or from the  Framework II directory if it is installed on your hard disk. Type: SETUP [Enter]

2.	At the "Welcome to Framework II's 'Setup' program" menu, choose option 2,  "All other uses of the setup program."

3.	Inform the SETUP program of the location of the FWSETUP file that stores the current configuration. If using a floppy system, insert the System Disk  in drive B and type "1". If using a hard disk system, choose option "2" since FWSETUP should be in the current default directory.

4.	At the Main Menu choose "2" to change individual configuration settings.

5.	From the Change Configuration menu choose "1" to change primary hardware configurations which include the screen driver.

6.	Choose "1" to change the screen driver. Then choose "7 - I want to enter my own driver file name." Insert the name of one of the drivers from the following table and press [Enter].

| Screen Format | Zoom Screen Format | Driver Name |
| --- | --- | --- |
| 132x25 text | 640x480 graphics | PVGA1.SC |
| 132x43 text | 640x480 graphics | PVGA2.SC |
| 132x25 text | 800x600 graphics | PVGA3.SC |
| 132x43 text | 800x600 graphics | PVGA4.SC |
| 800x600 graphics | 800x600 graphics | PVGA5.SC |

For example, to choose the driver for 132 column by 43 line format for the "desktop" and 800 by 600 graphics for the "zoom" function, type

A: PVGA4.SC		[ENTER]

Be sure to type the complete driver name including the .SC extension.

7.	Type M to return to the Main Menu.

8.	At the Main Menu type 7 to "Save All New Settings."

9.	You will then be prompted to insert the setup disk in drive A.  Insert the driver disk in drive A, then press the spacebar.

10.	Again, the SETUP program will want to know where to store the FWSETUP configuration file.  Follow step 3 and choose the option for your setup :

1 -  for floppy systems

2 -  for hard disk systems

Then strike any key.  You have now installed Framework II for the new drivers.  Start Framework II in the normal manner, and Framework II will use the selected capabilities of your VGA Port.

3.10	WORDSTAR PROFESSIONAL

WordStar Professional Release 4.0 can make use of 132 column text modes without special modification to the software. Simply tell the software the screen size in number of rows and columns of text and the software will take advantage of the new configuration.

1.	Copy WordStar Professional Release 4.0 to the working floppy disk or hard disk as described in the WordStar Professional documentation.

2.	Start WordStar's WSCHANGE installation program.

3.	Type in the name of the WordStar program file.  For example WS.EXE.

Next you will be prompted to enter the name of the file to which you wish to save the changes. You may wish to call this file WS132.EXE.

4.	At the Main Installation Menu choose option A for Console options.

5.	At the Console Menu choose option A for Monitor options.

6.	At the Monitor Menu choose option C for Screen sizing.

7.	At the Screen Sizing Menu choose option A for Height. You will then be asked to enter the correct value for the screen format you will use.  This should be:

25 - for a 132 column by 25 row screen, or

43 - for a 132 column by 43 row screen.

You will then be returned to the Screen Sizing Menu.

8.	At the Screen Sizing Menu, choose option B for Width. You will be asked to enter the correct value for the screen format. This should be 132 for either of the 132 column text modes.  Then return to the Screen Sizing Menu.

9.	Type X to return to the Monitor Menu. Type X from the Monitor Menu to return to the Console menu. Type X from the Console Menu to return to the Main  Installation menu. Type  X at the Main Installation Menu to indicate that you have completed the installation.

10.	You will be asked to confirm your choices and the WSCHANGE program will modify and save the WordStar program.  This procedure will not have to be run again unless changes to the WordStar program configuration are needed.

Running WordStar Professional in 132 Column Text

1.	Use the VGAMODE.EXE utility program on the Utilities to switch to the 132 column mode of your choice. Use the chart below to determine the correct command.

| Switch To | Command |
| --- | --- |
| 132x25 line text | VGAMODE 13225 |
| 132x43 line text | VGAMODE 13243 |
| 80x25 line text | VGAMODE VGA |

2.	Start your modified copy of WordStar.

Note: You may want to incorporate all the commands.  Example batch file contents:

VGAMODE 13243		[ENTER]

WS132					[ENTER]

VGAMODE VGA		[ENTER]

3.11	WORDPERFECT

WordPerfect 5.0 with Graphics Screen Driver

The following instructions are used to configure WordPerfect 5.0 for the supported graphics screen drivers.  The install program will copy the graphics screen drivers for the selected resolution to the directory where WordPerfect 5.0 (WP.EXE) is located.

To complete the WordPerfect 5.0 configuration using the selected graphics screen drivers, perform the following steps:

1.	Type:  WP

2.	Enter the Setup Key (Shift + F1)

3.	Choose the display

4.	Select the driver

Running WordPerfect 4.2 in 132 Column Text Mode

Some programs such as WordPerfect version 4.2 can make use of 132 column text modes without any special modification to the software.  Simply tell the software the number of rows and columns of text you can display on your monitor and the software will take advantage of the new configuration.

1.	Use the appropriate 132 column mode switching program on the VGA Utilities to switch to the desired 132 column mode.  Use the chart below to determine the correct command.

| Switch To | Command |
| --- | --- |
| 132x25 line text | VGAMODE 13225 |
| 132x43 line text | VGAMODE 13243 |
| 80x25 line text | VGAMODE VGA |

Run the 132 column mode switching software prior to starting WordPerfect.

2.	Start WordPerfect with a /S after the name of the program to get to the Setup Menu.

WP/S					[ENTER]

3.	Choose option 3 (Set Screen and Beep Options).

4.	Enter the number of rows (25 or 43) and the number of columns (132). Press the [Enter] key until you return to the Setup Menu.

5.	Choose option 0 to accept the configuration and enter WordPerfect.

6.	When you exit WordPerfect, run the command VGAMODE.EXE VGA to return the screen to standard 80 column by 25 line text.

Once WordPerfect has been configured, the Setup Menu will not need to be used again except to make configuration changes.

When in the WordPerfect program, the margins ([shift] [F8]) can be set to be as wide as 132 characters.

Note:  You may want to incorporate all the commands needed to start WordPerfect in 132 column mode in a batch file.  Example batch file contents:

VGAMODE 13243		[ENTER]

WP						[ENTER]

CLS						[ENTER]

VGAMODE VGA		[ENTER]

3.12	GENERIC CADD LEVEL 3

The following instructions are provided to configure Generic Cadd Level 3 for use with the extended modes  drivers:

1.	Use the install utility to copy the driver and then run the Generic Cadd program CONFIG.EXE.  You will be asked to press any key after the "Current Configuration" is displayed.

2.	Select "1" for video graphics display.

3.	Choose the appropriate VGA driver.

4.	Select "Exit to DOS".

5.	Respond "Y" to save the selected drivers.

The Generic Cadd Level 3 installation is now completed for the appropriate extended mode and can be started in a normal manner to utilize the 1024 by 768 or 800 by 600 graphics mode of the VGA Port.

3.13	VERSACAD DESIGN Version 5.3 and 5.4

The instructions provided below are used to configure the VersaCAD Design Version 5.3 and 5.4 for use with the extended mode software drivers:

1.	Use the install utility to copy the driver and run the ENVIRO.EXE program to set up the new screen driver.

2.	Environment menu 3 is used to select the 1024 by 768 or 800 by 600 single screen drivers.

3.	Depending on the resolution desired, start VersaCAD by executing one of the following:

a.	VCAD53 1024  for 1024x768  16 color

b.	VCAD53 800  for 800x600  16 color

c.	VCAD54 1024  for 1024x768  16 color

d.	VCAD54 800  for 800x600  16 color

e.	VCAD54 256  for 256 colors

The VersaCAD Design Version 5.3 or 5.4 has now been installed for either the 1024 by 768 or 800 by 600 graphics driver.

Section Six :

System Maintenance

*************************************************

6.1	Maintenance of SL-100 System

6.2	Hardware Problem

*************************************************

1.			MAINTENANCE OF SL-100 SYSTEM

This appendix helps you find the solution to possible problems experienced when working with your SL-100 system.  Usually, the cause of the problem will be easily traced and fixed.

The Computer System Does Not Start

If the SL-100 system does not start or the screen remains blank, switch the computer off, reset the `CMOS SETUP' (see Configuring the System), power back on the SL-100 system.  This puts the system in a defined basic condition.  If this does not help, check each of the following:

Power Switches

Make sure that the power switches on the SL-100 system, the monitor and any connected peripherals are switched on.  Check that the brightness and contrast settings of the monitor are correct.

Connecting Cables

Check all the connecting cables for correct location and tight fit.  Check that the power sockets to which you have connected your equipment are working.  Plug a table lamp into the plug and turn on the lamp to check that the power plug is working.

Monitor

Check that the monitor is switched on and that its selector switches and display modes are correctly set.

Keyboard and Mouse

If the keyboard or the mouse does not react when used, it may be that the software being run does not accept input from the keyboard or mouse.  Check the software's instruction manual.

Memory Expansion

After installing memory expansion RAM, make sure that this memory area is checked with the rest of RAM during system start-up.  If the entire available memory is not displayed (640 Kbytes plus any expansion RAM installed, switch the computer off.  Remove the electrical plug from the wall.  Now check to see that the memory expansion RAM are correctly seated.  If in doubt, re-insert the RAMs.

Expansion Cards And Internal Drives

If an expansion card or internal drive does not work, switch off the system.  Remove the power plug from the wall.  Remove the unit cover and check that the card is properly seated in the expansion slot.  Also check the power and data connecting cables between the controller, power supply, and internal drives.

2.			HARDWARE PROBLEM

| Symptom |  | Check |
| --- | --- | --- |
| "Blank Screen and No BEEP" | 1. | Is the system unit power cable is plugged into a working electrical outlet? |
|  | 2. | Is the power cable connected to the system properly? |
|  | 3. | Is the computer and the AC switched on? |
| "Blank Screen or poor quality display" | 1. | Is the VGA monitor video cable connected properly? |
|  | 2. | The VGA monitor turned on? or, the brightness and contrast controls adjusted? |
| "Blank Screen with BEEPS" |  | The VGA sub-system bad, check: |
|  |  | Is another VGA card plugged into the system? If so,, this VGA card can not set with the same operating mode as the on board VGA sub-system. |
| "System option not set" Display on screen |  | Run the setup program |
| "Keyboard error" Display on screen | 1. | Is keyboard plugged into the system? |
|  | 2. | Run the System diagnostic program to check for any defect in the keyboard. |
| "Memory size error" Display on screen |  | Set the memory size by the setup program. |

| Symptom |  | Check |
| --- | --- | --- |
| "Parity error" Display on screen |  | Your system memory bad and must be serviced by qualified Technician. |
| "No response form the Keyboard" Display on screen | 1. | Keyboard cable not properly connected. |
|  | 2. | System crashed. Restart your system by pressing the RESET button or, turn off the system, wait few seconds, then on again. |
| "Disk error" Display on screen | 1. | Replace the diskette and try the others. |
|  | 2. | Is the Power and control/data cable connected properly on you disk drives? |
|  | 3. | Run the "SETUP" program to check the options are set correctly. |
| Can Not READ or WRITE on floppy disk operation | 1. | Replace the diskette and try the others. |
|  | 2. | The diskette not inserted properly. |
|  | 3. | Diskette write-protected - Make sure the write protect label not in place. |
|  | 4. | The diskette is damaged. |
|  | 5. | The diskette is unformatted. |
|  | 6. | Your system needs service if all above can not solve the problem |
| Can not reset the computer while pressing the "RESET" button |  | Turn off your system, wait few seconds, turn it back on again. After DOS prompt appear, press RESET button again, if the system still can not reset, your system needs service. |

THIS PAGE IS INTENTIONALLY LEFT BLANK