Appendix B :

Technical Data

for on Board Connectors

*************************************************

B.1	Keyboard (KB1)

B.2	VGA Display (J1)

B.3	Game Port (J2)

B.4	Printer Port (J3)

B.5	Serial Port (J4, J5) or

(COM A, COM B)

B.6	Speaker Header (P1)

B.7	External Battery (P2)

B.8	Power Supply (P3)

B.9	Hard Disk Drive (P4)

B.10	Floppy Disk Drive (P5)

B.11	Panel LED (JP9)

B.12	I/O Channel

*************************************************

1.		KEYBOARD CONNECTOR (KB1)

| Pin | Signal |
| --- | --- |
| 1 | Keyboard Clock |
| 2 | Keyboard Data |
| 3 | N.C. |
| 4 | Ground |
| 5 | + 5V Supply |

2.		VGA DISPLAY CONNECTOR (J1)

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Red Video | 9 | No Pin |
| 2 | Green Video | 10 | Sync Return (Ground) |
| 3 | Blue Video | 11 | N. C. |
| 4 | N. C. | 12 | N. C. |
| 5 | Ground | 13 | Horizontal Sync |
| 6 | Red Return (Ground) | 14 | Vertical Sync |
| 7 | Green Return (Ground) | 15 | N. C. |
| 8 | Blue Return (Ground) |  |  |

3.		GAME CONNECTOR (J2)

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | + 5V Supply | 9 | + 5V Supply |
| 2 | Button 4 | 10 | Button 6 |
| 3 | Position 0 | 11 | Position 2 |
| 4 | Ground | 12 | Ground |
| 5 | Ground | 13 | Position 3 |
| 6 | Position 1 | 14 | Button 7 |
| 7 | Button 5 | 15 | + 5V Supply |
| 8 | + 5V Supply |  |  |

4.		PRINTER PORT CONNECTOR (J3)

| Pin | Signal | I/O | Pin | Signal | I/O |
| --- | --- | --- | --- | --- | --- |
| 1 | -STROBE | O | 14 | -AUTO FEED | O |
| 2 | DATA 0 | O | 15 | -ERROR | I |
| 3 | DATA 1 | O | 16 | -INIT | O |
| 4 | DATA 2 | O | 17 | SLCT IN | O |
| 5 | DATA 3 | O | 18 | GROUND | - |
| 6 | DATA 4 | O | 19 | GROUND | - |
| 7 | DATA 5 | O | 20 | GROUND | - |
| 8 | DATA 6 | O | 21 | GROUND | - |
| 9 | DATA 7 | O | 22 | GROUND | - |
| 10 | -PACK | I | 23 | GROUND | - |
| 11 | BUSY | I | 24 | GROUND | - |
| 12 | PAPER END | I | 25 | GROUND | - |
| 13 | SLCT | I |  |  |  |

5.		SERIAL PORT CONNECTOR (J4, J5) or (COM A, COM B)

| Pin | Signal | I/O | Pin | Signal | I/O |
| --- | --- | --- | --- | --- | --- |
| 1 | N. C. | - | 14 | N. C. | - |
| 2 | Transmit Data | O | 15 | N. C. | - |
| 3 | Receive Data | I | 16 | N. C. | - |
| 4 | Request to Send | O | 17 | N. C. | - |
| 5 | Clear to Send | I | 18 | N. C. | - |
| 6 | Data Send Ready | I | 19 | N. C. | - |
| 7 | Ground | - | 20 | Data Terminal Ready | O |
| 8 | Carrier Detect | I | 21 | N. C. | - |
| 9 | N. C. | - | 22 | Ring Indicator | I |
| 10 | N. C. | - | 23 | N. C. | - |
| 11 | N. C. | - | 24 | N. C. | - |
| 12 | N. C. | - | 25 | N. C. | - |
| 13 | N. C. | - |  |  |  |

6.		SPEAKER HEADER (P1)

| Pin | Signal |
| --- | --- |
| 1 | Data Out |
| 2 | + 5V Supply |
| 3 | Ground |
| 4 | + 5V Supply |

7.		EXTERNAL BATTERY CONNECTOR (P2)

| Pin | Signal |
| --- | --- |
| 1 | + Volt Supply |
| 2 | N.C. |
| 3 | Ground |
| 4 | Ground |

8.		POWER SUPPLY CONNECTOR (P3)

| Pin | Signal |
| --- | --- |
| 1 | - 5v Supply |
| 2 | Ground |
| 3 | - 12V Supply |
| 4 | + 12 V Supply |
| 5 | Ground |
| 6 | + 5V Supply |

9.		HARD DISK DRIVE CONNECTOR (P4)

| Pin | Signal | I/O | Pin | Signal | I/O |
| --- | --- | --- | --- | --- | --- |
| 1 | Host Reset | I | 21 | N.C. | - |
| 2 | Ground | - | 22 | Ground | - |
| 3 | Host Data 7 | I/O | 23 | -Host IOW | I |
| 4 | Host Data 8 | I/O | 24 | Ground | - |
| 5 | Host Data 6 | I/O | 25 | -Host IOR | I |
| 6 | Host Data 9 | I/O | 26 | Ground | - |
| 7 | Host Data 5 | I/O | 27 | Host IORDY | I |
| 8 | Host Data 10 | I/O | 28 | Host ALE | I |
| 9 | Host Data 4 | I/O | 29 | N.C. | - |
| 10 | Host Data 11 | I/O | 30 | Ground | - |
| 11 | Host Data 3 | I/O | 31 | Host IRQ14 | O |
| 12 | Host Data 12 | I/O | 32 | -Host IOCS16 | O |
| 13 | Host Data 2 | I/O | 33 | Host ADR 1 | I |
| 14 | Host Data 13 | I/O | 34 | N.C. | - |
| 15 | Host Data 1 | I/O | 35 | Host ADR 0 | I |
| 16 | Host Data 14 | I/O | 36 | Host ADR 2 | I |
| 17 | Host Data 0 | I/O | 37 | -Host CSO | I |
| 18 | Host Data 15 | I/O | 38 | -Host CS1 | I |
| 19 | Ground | - | 39 | -Active | O |
| 20 | Ground | - | 40 | Ground | - |

10.		FLOPPY DISK DRIVE CONNECTOR (P5)

| Pin | Signal | I/O | Pin | Signal | I/O |
| --- | --- | --- | --- | --- | --- |
| 1 | Ground | - | 18 | -Direction | O |
| 2 | -Reduced Write | O | 19 | Ground | - |
| 3 | Ground | - | 20 | -Step | O |
| 4 | N.C. | - | 21 | Ground | - |
| 5 | Ground | - | 22 | -Write Data | O |
| 6 | N.C. | - | 23 | Ground | - |
| 7 | Ground | - | 24 | -Write Enable | O |
| 8 | -Index | I | 25 | Ground | - |
| 9 | Ground | - | 26 | -Track 00 | I |
| 10 | -Motor Enable 0 | O | 27 | Ground | - |
| 11 | Ground | - | 28 | -Write Protect | I |
| 12 | -Drive Select 1 | O | 29 | Ground | - |
| 13 | Ground | - | 30 | -Read Data | I |
| 14 | -Drive Select 0 | O | 31 | Ground | - |
| 15 | Ground | - | 32 | -Head Select 1 | O |
| 16 | -Motor Enable 1 | O | 33 | Ground | - |
| 17 | Ground | - | 34 | -Disk Change | I |

11.		PANEL LED CONNECTOR (JP9)

| Pin | Signal |
| --- | --- |
| 1 | + 5V Supply |
| 2 | Floppy Access LED |
| 3 | Hard Disk Access LED |
| 4 | Turbo LED |
| 5 | Turbo Switch |
| 6 | Reset Switch |
| 7 | Ground |

12.		I/O CHANNEL

The following tables summarize pin assignments for the I/O channel connectors.

I/O Channel (A-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| A1 | -I/O CH CK | I |
| A2 | SD7 | I/O |
| A3 | SD6 | I/O |
| A4 | SD5 | I/O |
| A5 | SD4 | I/O |
| A6 | SD3 | I/O |
| A7 | SD2 | I/O |
| A8 | SD1 | I/O |
| A9 | SD0 | I/O |
| A10 | -I/O CH RDY | I |
| A11 | AEN | O |
| A12 | SA19 | I/O |
| A13 | SA18 | I/O |
| A14 | SA17 | I/O |
| A15 | SA16 | I/O |
| A16 | SA15 | I/O |
| A17 | SA14 | I/O |
| A18 | SA13 | I/O |
| A19 | SA12 | I/O |
| A20 | SA11 | I/O |
| A21 | SA10 | I/O |
| A22 | SA9 | I/O |
| A23 | SA8 | I/O |
| A24 | SA7 | I/O |
| A25 | SA6 | I/O |
| A26 | SA5 | I/O |
| A27 | SA4 | I/O |
| A28 | SA3 | I/O |
| A29 | SA2 | I/O |
| A30 | SA1 | I/O |
| A31 | SA0 | I/O |

I/O Channel (B-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| B1 | GND | Ground |
| B2 | RESET DRV | O |
| B3 | +5 Vdc | Power |
| B4 | IRQ9 | I |
| B5 | -5 Vdc | Power |
| B6 | DRQ2 | I |
| B7 | -12 Vdc | Power |
| B8 | 0WS | I |
| B9 | +12 Vdc | Power |
| B10 | GND | Ground |
| B11 | -SMEMW | O |
| B12 | -SMEMR | O |
| B13 | -IOW | I/O |
| B14 | -IOR | I/O |
| B15 | -DACK3 | O |
| B16 | DRQ3 | I |
| B17 | -DACK1 | O |
| B18 | DRQ1 | I |
| B19 | -Refresh | I/O |
| B20 | CLK | O |
| B21 | IRQ7 | I |
| B22 | IRQ6 | I |
| B23 | IRQ5 | I |
| B24 | IRQ4 | I |
| B25 | IRQ3 | I |
| B26 | -DACK2 | O |
| B27 | T/C | O |
| B28 | BALE | O |
| B29 | +5 Vdc | Power |
| B30 | OSC | O |
| B31 | GND | Ground |

I/O Channel (C-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| C1 | SBHE | I/O |
| C2 | LA23 | I/O |
| C3 | LA22 | I/O |
| C4 | LA21 | I/O |
| C5 | LA20 | I/O |
| C6 | LA19 | I/O |
| C7 | LA18 | I/O |
| C8 | LA17 | I/O |
| C9 | -MEMR | I/O |
| C10 | -MEMW | I/O |
| C11 | SD8 | I/O |
| C12 | SD9 | I/O |
| C13 | SD10 | I/O |
| C14 | SD11 | I/O |
| C15 | SD12 | I/O |
| C16 | SD13 | I/O |
| C17 | SD14 | I/O |
| C18 | SD15 | I/O |

I/O Channel (D-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| D1 | -MEM CS16 | I |
| D2 | -I/O CS16 | I |
| D3 | IRQ10 | I |
| D4 | IRQ11 | I |
| D5 | IRQ12 | I |
| D6 | IRQ15 | I |
| D7 | IRQ14 | I |
| D8 | -DACK0 | O |
| D9 | DRQ0 | I |
| D10 | -DACK5 | O |
| D11 | DRQ5 | I |
| D12 | -DACK6 | O |
| D13 | DRQ6 | I |
| D14 | -DACK7 | O |
| D15 | DRQ7 | I |
| D16 | +5 Vdc | Power |
| D17 | -MASTER | I |
| D18 | GND | Ground |