SL-150 Computer

Supplement for Operation Manual

(July 8, 1992)

=====================

VGA DISPLAY JUMPER SETTINGS

Scanning Rate Control in 800 x 600 Mode

The jumper JP2 and JP4 control the scanning rate in 800 x 600 mode.

| JP2 | JP4 | Scanning Rate Control in 800 x 600 Mode |  |  |
| --- | --- | --- | --- | --- |
|  |  | No. of Color | Hsync | Vsync |
| 1-2 * | 1-2 * | 16/256 | 35.5K | 56 |
| 1-2 | 2-3 | 16 | 48K | 72 |
|  |  | 256 | 35.5K | 56 |
| 2-3 | 1-2 | 16/256 | 40K | 60 |
| 2-3 | 2-3 | 16 | 48K | 72 |
|  |  | 256 | 38K | 60 |

*  Default

Note :	Before setting the jumper, please check your monitor can meet the scanning rate requirement.  Selecting a scanning rate higher than the monitor can handle may damage the monitor.