PX4_PAT

=======

Introduction

------------

"PX4_PAT" is a Microsoft Windows 95 .inf file upgrade utility for the Intel

PIIX4 which writen by Award Software International Inc.. Its expanded Windows 95

functionality to support the Intel 440LX chipset. WINP2X4 runs under Windows 95

to automatically upgrade IDE controller .inf files to support the Intel PIIX4

processor.

If you want to transfer a hard drive from an Intel 430VX-based motherboard to

an Intel 430TX/LX based board, you need to upgrade support for the IDE

controller from PIIX3 to PIIX4. The upgrade requires modifying hidden .inf

files, such as machine.inf and mshdc.inf, that are required for the PIIX4 IDE

controller. To upgrade system software to support the PIIX4 IDE controller,

you can choose from three avenues:

- Using the IDE controller upgrade driver currently available in the market-

place

- Following midification directions from Intel

- Using "PX4_PAT to upgrade automatically

To use PX4_PAT.EXE, your hard drive must currently be running the Windows95

operating system and configured for an Intel PIIX3 IDE controller. The current

host system must contain a PIIX4 IDE controller. If you try to run PX4_PAT on

a non-PIIX4 platform, you receive an error message such as:

"Sorry, this utility is for PIIX4 platform only."

Q. How to upgrade to a PIIX4 platform ?

A. 1. Start PX4_PAT

- Open a DOS windows inside Windows 95

- At the DOS prompt, type PX4_PAT. Then ask for your Win95 directory.

- Respond "Y" if Windows 95 directory is correct. If the Windows95

directory is other than the default directory, respond "N", and follow

the on-screen prompt to enter the correct path:

- Follow the on-screen prompt to restart your system to let the changes

take effect.

- You may be prompted to update device drivers after the system restarts.

Keep your Windows 95 Installation medium handy.

2. Now under Win95/My Computer/Control Panel/Device Manager/View Device by

Type

Hard Disk Controller

Intel 82371AB PCI Bus Master IDE controller

Primary IDE Controller (Single FIFO)

Secondary IDE Controller (Single FIFO)

Universal Serial Bus Controller

Intel 82371AB PCI to USB Universal Host Controller.