BISON II

The material in this manual is for information only and is subject to change without notice.

REVISION: 1.0

IBM, IBM PC/XT/AT, PC-DOS, MS-DOS, OS/2, INTEL, AMI, Pentium ARE THE TRADEMARKS OR REGISTERED TRADEMARKS OF THEIR RESPECTIVE OWNERS.

RADIO FREQUENCY INTERFERENCE STATEMENT

This equipment generates and uses radio frequency energy and if not installed and used properly, that is, in strict accordance with the manufacturer's instructions, may cause interference with radio and television reception.

If this equipment does cause interference to radio or TV reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient the receiving antenna.

*	Relocate the computer away from the receiver.

*	Move the computer away from the receiver.

*	Plug the computer into a different outlet so that computer and receiver are on different branch circuits.

*	Ensure that card slot covers are in place when no card is installed.

*	Ensure that card mounting screws, attachment connector screws, and ground wires are tightly secured.

*	If peripherals are used with this system, it is suggested to use shielded, grounded cables, with in-line filters if necessary.

If necessary, the user should consult the dealer service representative for additional suggestions.

The manufacturer is not responsible for any radio or TV interference caused by unauthorized modifications to this equipment.  It is the responsibility of the user to correct such interference.

Note

1.		Electronic components are sensitive to dust and dirt.  Do inspect and clean the computer system regularly.

2.		Turn off the power whenever you install or remove any connector, memory module and add-on card.  Before turning on the power, make sure that all the connectors, memory modules and add-on cards are secured.

3.		After power is on, wait for a minute.  The system BIOS are going through a self-test during this period and nothing is shown on the screen.  After the self-test, the system BIOS will initialize the display adaptor and show messages.

4.		The SIMM sockets are fragile device.  Do not force the SIMM modules into the sockets.  It may break the locking latches.

Table of Content

Chapter 1	INTRODUCTION	1-1

Chapter 2	GENERAL FEATURES

Specifications	2-1

The Central Processing Unit	2-3

Cache Subsystem	2-4

DRAM Subsystem	2-4

DRAM Configuration	2-6

Chapter 3	CONFIGURING THE SYSTEM

Installing the Processor	3-1

Installing RAM Modules	3-2

Configuring the Cache Memory	3-3

Control of System Speed	3-4

Reset CMOS Setup Information	3-4

System Board Jumper and Connector	3-6

Summary

System Board Jumper Setting	3-7

System Board Connectors	3-8

Chapter 4	TECHNICAL INFORMATION

Memory Mapping	4-1

I/O Address Map	4-2

System Timers	4-4

System Interrupts	4-6

Direct Memory Access (DMA)	4-7

Real Time Clock and CMOS RAM	4-8

CMOS RAM Address Map	4-9

Real Time Clock Information	4-10

System Expansion Bus	4-11

Appendix A	OPERATION AND MAINTENANCE

Static Electricity	A-1

Keeping The System Cool	A-1

Cleaning The "Golden Finger"	A-2

Cleaning The Motherboard	A-2

Appendix B	TROUBLESHOOTING

Main Memory Error	B-1

Chapter 1

Introduction

____________________________________________________

OCTEK BISON II is a workstation class platform that can meet the demand of most time critical applications nowadays. With the Pentium processor and 64 bit interleaved memory system, it delivers the highest performance among the PC/AT class machines that ever been.

ISA bus and VESA bus are incorporated to adapt the most popular add-on card standard.

Next-generation design remains 100% binary and PC/AT compatible that boosts up the existing applications without modification.

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 2

General Features

____________________________________________________

Specifications:

Processor:

Pentium 60MHz or 66MHz CPU

Speed:

Turbo/normal speed

I/O Slot:

Six 16 bit ISA slots

One 8 bit ISA slot

Two VESA VL-bus slots (supporting one VL master)

Cache:

Internal cache inside Pentium:

8K instruction cache

8K data cache

External cache:

64K external cache expandable to 512K

Both caches can be programmed as write back or write through mode.

DRAM:

Supports 4 banks of 32 bit wide SIMM modules with 1MB, 4MB or 16MB page-mode DRAMs.

Supports DRAM configurations from 2MB to 64MB.

Two non-cacheable regions for VL/ISA bus masters.

DRAM post write buffer.

Supports pipeline DRAM burst cycles.

Others:

Fast GateA20 and reset emulations.

The Central Processing Unit

The Pentium processor is the next-generation member of 80486 family of microprocessors. It contains all of the features of the 80486 and provides significant enhancments and additions. It is 100% binary compatible with the X86 CPU.

The superscalar architecture of the Pentium processor contains two instruction pipelines and floating-point unit on the Pentium processor are capable of independent operation. Each pipeline issues hit instructions in a single clock. The dual pipes can issue two integer instructions in one clock, or one floating-point instruction in one clock.

The branch prediction unit includes two prefech buffers, one to prefetch code in a linear fashion, and one to prefetch code according to the Branch Target Buffer so that the needed code is almost always prefetched before it is needed for execution.

The floating point unit is redesigned and runs at least three times faster than 80486.

The Pentium processor includes separate 8K code cache and 8K write-back data cache. Each cache is 32 byte line size and is 2-way set associative. The data cache is configurable to be write-back or write-through on a line-by-line basis and follows the MESI protocol. The code cache is a write-protected cache.

The Pentium processor has widen the data bus to 64 bits to improve the data transfer rate. Burst read and burst writeback cycles are supported by the Pentium processor.

In summary, the Pentium processor provides an ultimate performance levels and retains compatibility with the existing applications.

Cache Subsystem

The external cache of BISON II is write-back, direct-mapped with sizes of 64K, 128K, 256K or 512K. It is organized by single or dual bank. Dual bank is accessed in interleaved manner and it can support 3-2-2-2 burst read cycle.

DRAM Subsystem

The memory controller is 64 bit wide and has the following features:

-	Hidden refresh is used to increase the CPU bandwidth. The DRAM can be refreshed in the background while the CPU is accessing the internal cache.

-	Page Mode is used for faster data access from the DRAMs.

-	Posted-Write to the DRAM improves the write-cycle timing. One quad-word deep data buffer is used to hold the data from the CPU without waiting for the external DRAM cycle.

Shadow RAM is available as an option. System BIOS and video BIOS residing in slow EPROM can be copied to local DRAM to speed up accesses to BIOS code. Shadow RAM addresses range from C0000h to FFFFFh. 16K granularity is provided for address range C0000h to EFFFFh, while the area F0000h - FFFFFh can only be shadowed as a whole. Video BIOS at C0000h - C7FFFh can be cached in the external cache after shadowing.

Two DRAM control regions can be selected to adapt the bus mastering add-on cards. For the memory region that bus master would write to, set it to non-cacheable or write-through would improve bus bandwidth owing to the elimination of the time-consuming cache invalidation cycle. For the card with its own memory mapping overlaid to the system memory, holes can be enabled to allow accessing to the memory on the ISA or VESA bus.

DRAM CONFIGURATION

| Bank 0 + Bank 1 (bits) | Bank 2 + Bank 3 (bits) | Total (bytes) |
| --- | --- | --- |
| 256K x 64 | --- | 2M |
| 256K x 64 | 256K x 64 | 4M |
| 256K x 64 | 1M x 64 | 10M |
| 256K x 64 | 4M x 64 | 34M |
| 1M x 64 | --- | 8M |
| 1M x 64 | 1M x 64 | 16M |
| 1M x 64 | 4M x 64 | 40M |
| 4M x 64 | --- | 32M |
| 4M x 64 | 4M x 64 | 64M |

70ns 32-bit wide SIMM modules can be used.

Chapter 3

Configuring The System

____________________________________________________

Important Note: Turn off the power before installing or replacing any component.

Installing the processor

Pentium is a 273 pin PGA device. Make sure the pin 1 of Pentium (with a notch at the corner) is line up with the pin 1 of the socket.

Before installing the processor, make sure that all the pins are straight. The pins are very fragile. Once these pins are bent, the processor may be damaged.

Installing RAM modules

BISON II has four banks on board for SIMM modules. Bank 0 and bank 1 must be installed first. Make sure pin 1 of the SIMM module inserted near the power connector. Lock it firmly with the latches on the socket.

Extra SIMM module should be inserted at bank 2 and bank 3.

Configuring the cache memory

Note: 	If you have any question about the configuration of the cache memory, consult your local dealer. Improper configuration will cause the system malfunction.

The external cache is organized by single bank or dual banks with sizes of 64KB to 512KB. Follow the tables below to configure the system.

| Cache size | JP6 | JP23 | JP22 | JP21 | JP20 | JP19 |
| --- | --- | --- | --- | --- | --- | --- |
| 64K | 2-1 | 2-1 | 2-1 | OPEN | OPEN | OPEN |
| 128K | 2-3 | 2-3 | 2-3 | OPEN | OPEN | CLOSE |
| 256K | 2-1 | 2-1 | 2-1 | OPEN | CLOSE | CLOSE |
| 512K | 2-3 | 2-3 | 2-3 | CLOSE | CLOSE | CLOSE |

| Cache size | U30 | U31 | BANK 0 | BANK 1 |
| --- | --- | --- | --- | --- |
| 64K | OPEN | 8K x 8 | 8K x 8 | OPEN |
| 128K | OPEN | 8K x 8 | 8K x 8 | 8K x 8 |
| 256K | OPEN | 8K x 8 | 32K x 8 | OPEN |
| 512K | 8K x 8 | 8K x 8 | 32K x 8 | 32K x 8 |

CONTROL OF SYSTEM SPEED

System speed can be controlled by keyboard and turbo switch.  To change the speed by keyboard, use `-' and `+' of the numeric keypad.  Press `Ctrl' `Alt' and `-' for slow speed and press `Ctrl' `Alt' and `+' for fast speed.

RESET CMOS SETUP INFORMATION

Sometimes, the improper setting of system setup may make the system malfunction.  In this case, turn off the power and set JP1 to 1-2 for a while.  The internal CMOS status register is reset.  Then set the jumper to 2-3 of JP1 and turn on the power.  The BIOS finds the CMOS status register is reset and regards the setup information is invalid.  So it will prompt you to correct the information.

SYSTEM BOARD JUMPER AND CONNECTOR SUMMARY

|  | Description |
| --- | --- |
| J1 | Keyboard Connector |
| JP1 | Reset CMOS |
| J10,J11 | Power Supply Connector |
| J9 | External Battery Connector |
| JP2 | Display Selection |
| JP6 | Cache Size Selection |
| JP19-JP23 | Cache Size Selection |
| JP20 | Power LED 6 Ext-Lock Connector |
| JP18 | Speaker Connector |
| JP24 | Power-Good Selection |
| JP27 | Cooling Fan Power Selection |
| JP26 | Cooling Fan Connector |
| JP25 | Hardware Reset Connector |

SYSTEM BOARD JUMPER SETTING

There are several options which allows user to select by hardware switches.

JP2 - Display Selection

| JP2 |  |
| --- | --- |
| CLOSE | CGA, EGA, VGA (default) |
| OPEN | Monochrome display |

JP24 - Power-good Selection

| PIN |  |
| --- | --- |
| 1-2 | Power-good signal from power supply (default) |
| 2-3 | On board power-good signal |

JP27 - Cooling Fan Power Selection

| PIN |  |
| --- | --- |
| 1-2 | 12Vdc (default) |
| 2-3 | 15Vdc |

SYSTEM BOARD CONNECTORS

Under typical conditions, these connectors should be connected to the indicators and switches of the system unit.  The functions and the pin assignment of the connectors on the motherboard are listed below.

JP18 - Speaker Connector

| Pin | Assignment |
| --- | --- |
| 1 | Data out |
| 2 | +5 Vdc |
| 3 | Ground |
| 4 | +5 Vdc |

JP25 - Hardware Reset Connector

| Pin | Assignment |
| --- | --- |
| 1 | Selection Pin |
| 2 | Ground |

J20 - Power LED & Ext-Lock Connector

| Pin | Assignment |
| --- | --- |
| 1 | +5 Vdc |
| 2 | Key |
| 3 | Ground |
| 4 | Keyboard inhibit |
| 5 | Ground |

J10,J11 - Power Supply Connector

| Pin | Assignment |
| --- | --- |
| 1 | POWERGOOD |
| 2 | +5 Vdc |
| 3 | +12 Vdc |
| 4 | -12 Vdc |
| 5 | Ground |
| 6 | Ground |

| Pin | Assignment |
| --- | --- |
| 1 | Ground |
| 2 | Ground |
| 3 | -5 Vdc |
| 4 | +5 Vdc |
| 5 | +5 Vdc |
| 6 | +5 Vdc |

J9 - External Battery Connector

| Pin | Assignment |
| --- | --- |
| 1 | + Vdc |
| 2 | not used |
| 3 | Ground |
| 4 | Ground |

JP26 - Cooling Fan Connector

| Pin | Assignment |
| --- | --- |
| 1 | + 5Vdc |
| 2 | Ground |

J1 - Keyboard Connector

| Pin | Assignment |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Spare |
| 4 | Ground |
| 5 | +5 Vdc |

THIS PAGE IS INTENTIONALLY LEFT BLANK

Chapter 4

Technical Information

____________________________________________________

This section provides technical information about BISON II and is intended for advanced users interested in the basic design and operation of BISON II.

MEMORY MAPPING

| Address | Range | Function |
| --- | --- | --- |
| 000000-7FFFFF | 000K-512K | System Board Memory (512K) |
| 080000-09FFFF | 512K-640K | System Board Memory (128K) |
| 0A0000-0BFFFF | 640K-768K | Display Buffer (128K) |
| 0C0000-0DFFFF | 768K-896K | Adaptor ROM / Shadow RAM (128K) |
| 0E0000-0EFFFF | 896K-960K | System ROM / Shadow RAM (64K) |
| 0F0000-0FFFFF | 960K-1024K | System BIOS ROM / Shadow RAM (64K) |
| 100000-7FFFFF | 1024K-8192K | System Memory |
| 800000-FFFFFF | 8192K-16318K | System Memory |

I/O ADDRESS MAP

I/O Address Map on System Board

I/O address hex 000 to 0FF are reserved for the system board I/O.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 000-01F | DMA Controller 1, 8237 |
| 020-03F | Interrupt Controller 1, 8259, Master |
| 040-05F | Timer, 8254 |
| 060-06F | Keyboard Controller |
| 070-07F | Real Time Clock, NMI (non-maskable interrupt) mask |
| 080-09F | DMA Page Register, 74LS612 |
| 0A0-0BF | Interrupt Controller 2, 8259 |
| 0C0-0DF | DMA Controller 2, 8237 |
| 0F0 | Clear Math Coprocessor Busy |
| 0F1 | Reset Math Coprocessor |
| 0F8-0FF | Math Coprocessor Port |

I/O address hex 100 to 3FF are available on the I/O channel.

| ADDRESS (HEX) | DEVICE |
| --- | --- |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game I/O |
| 278-27F | Parallel Printer Port 2 |
| 2F8-2FF | Serial Port 2 |
| 300-31F | Prototype Card |
| 360-36F | Reserved |
| 378-37F | Parallel Printer Port 1 |
| 380-38F | SDLC, bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Printer Adapter |
| 3C0-3CF | Reserved |
| 3D0-3DF | Color Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 |

SYSTEM TIMERS

BISON II has three programmable timer/counters controlled by Headland chipset and they are defined as channels 0 through 2:

| Channel 0 | System Timer |
| --- | --- |
| Gate 0 | Tied on |
| Clk in 0 | 1.190 Mhz OSC |
| Clk out 0 | 8259 IRQ 0 |

| Channel 1 | Refresh Request Generator |
| --- | --- |
| Gate 1 | Tied on |
| Clk in 1 | 1.190 Mhz OSC |
| Clk out 1 | Request Refresh Cycle |

| Channel 2 | Tone Generation of Speaker |
| --- | --- |
| Gate 2 | Controlled by bit 0 of port hex 61 PPI bit |
| Clk in 2 | 1.190 Mhz OSC |
| Clk out 2 | Used to drive the speaker |

Note :	Channel 1 is programmed to generate a 15-micro-second period signal.

The 8254 Timer/Counters are treated by system programs as an arrangement of four programmable external I/O ports.  Three are treated as counters and the fourth is a control register for mode programming.

SYSTEM INTERRUPTS

Sixteen levels of system interrupts are provided on BISON II. The following shows the interrupt-level assignments in decreasing priority.

Level					Function

Microprocessor NMI	Parity or I/O Channel

Check

Interrupt Controllers

CTLR 1	CTLR 2

IRQ0				Timer Output 0

IRQ1				Keyboard

(Output Buffer Full)

IRQ2				Interrupt from CTLR 2

IRQ8		Real-time Clock Interrupt

IRQ9		Software Redirected to

INT 0AH (IRQ2)

IRQ10	Reserved

IRQ11	Reserved

IRQ12	Reserved

IRQ13	Coprocessor

IRQ14	Fixed Disk Controller

IRQ15	Reserved

IRQ3				Serial Port 2

IRQ4				Serial Port 1

IRQ5				Parallel Port 2

IRQ6				Diskette Controller

IRQ7				Parallel Port 1

┌──────────────────────────────────────┐

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

╞══════════════════════════════════════╡

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

│     ─┐                               │

│      │ ┌─                            │

│      │ │                             │

│      │ │                             │

│      │ │                             │

│      └─┤                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        │                             │

│        └─                            │

│                                      │

│                                      │

│                                      │

│                                      │

│                                      │

└──────────────────────────────────────┘

DIRECT MEMORY ACCESS (DMA)

BISON II supports seven DMA channels.

| Channel | Function |
| --- | --- |
| 0 | Spare (8 bit transfer) |
| 1 | SDLC (8 bit transfer) |
| 2 | Floppy Disk (8 bit transfer) |
| 3 | Spare (8 bit transfer) |
| 4 | Cascade for DMA Controller 1 |
| 5 | Spare (16 bit transfer) |
| 6 | Spare (16 bit transfer) |
| 7 | Spare (16 bit transfer) |

The following shows the addresses for the page register.

| Page Register | I/O Address (HEX) |
| --- | --- |
| DMA Channel 0 | 0087 |
| DMA Channel 1 | 0083 |
| DMA Channel 2 | 0081 |
| DMA Channel 3 | 0082 |
| DMA Channel 5 | 008B |
| DMA Channel 6 | 0089 |
| DMA Channel 7 | 008A |
| Refresh | 008F |

REAL TIME CLOCK AND CMOS RAM

Real time clock and CMOS RAM are contained on board.  Real time clock provides the system date and time.  CMOS RAM stores system information.  Both are backed up by battery and will not lose information after power off.  The following page shows the CMOS RAM Address Map.

CMOS RAM ADDRESS MAP

| Addresses | Description |
| --- | --- |
| 00-0D | * Real-time clock information |
| 0E | * Diagnostic status byte |
| 0F | * Shutdown status byte |
| 10 | Diskette drive type byte - drives A and B |
| 11 | Reserved |
| 12 | Fixed disk type byte - drives C and D |
| 13 | Reserved |
| 14 | Equipment byte |
| 15 | Low base memory byte |
| 16 | High base memory byte |
| 17 | Low expansion memory byte |
| 18 | High expansion memory byte |
| 19-2D | Reserved |
| 2E-2F | 2-byte CMOS checksum |
| 30 | * Low expansion memory byte |
| 31 | * High expansion memory byte |
| 32 | * Date century byte |
| 33 | * Information flags (set during power on) |
| 34-3F | Reserved |

REAL TIME CLOCK INFORMATION

The following table describes real-time clock bytes and specifies their addresses.

| Byte | Function | Address |
| --- | --- | --- |
| 0 | Seconds | 00 |
| 1 | Second alarm | 01 |
| 2 | Minutes | 02 |
| 3 | Minute alarm | 03 |
| 4 | Hours | 04 |
| 5 | Hour alarm | 05 |
| 6 | Day of week | 06 |
| 7 | Date of month | 07 |
| 8 | Month | 08 |
| 9 | Year | 09 |
| 10 | Status Register A | 0A |
| 11 | Status Register B | 0B |
| 12 | Status Register C | 0C |
| 13 | Status Register D | 0D |

SYSTEM EXPANSION BUS

BISON II provides six 16-bit slots and two VL-bus slots.

The I/O channel supports:

*	I/O address space from hex 100 to hex 3FF

*	Selection of data access (either 8 or 16 bit)

*	24 bit memory addresses (16MB)

*	Interrupts

*	DMA channels

*	Memory refresh signal

The following figure shows the pin numbering for I/O channel connectors (A-side and B-side).

The following figure shows the pin numbering for I/O channel connectors (C-side and D-side).

The following tables summarize pin assignments for the I/O channel connectors.

I/O Channel (A-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| A1 | -I/O CH CK | I |
| A2 | SD7 | I/O |
| A3 | SD6 | I/O |
| A4 | SD5 | I/O |
| A5 | SD4 | I/O |
| A6 | SD3 | I/O |
| A7 | SD2 | I/O |
| A8 | SD1 | I/O |
| A9 | SD0 | I/O |
| A10 | -I/O CH RDY | I |
| A11 | AEN | O |
| A12 | SA19 | I/O |
| A13 | SA18 | I/O |
| A14 | SA17 | I/O |
| A15 | SA16 | I/O |
| A16 | SA15 | I/O |
| A17 | SA14 | I/O |
| A18 | SA13 | I/O |
| A19 | SA12 | I/O |
| A20 | SA11 | I/O |
| A21 | SA10 | I/O |
| A22 | SA9 | I/O |
| A23 | SA8 | I/O |
| A24 | SA7 | I/O |
| A25 | SA6 | I/O |
| A26 | SA5 | I/O |
| A27 | SA4 | I/O |
| A28 | SA3 | I/O |
| A29 | SA2 | I/O |
| A30 | SA1 | I/O |
| A31 | SA0 | I/O |

I/O Channel (B-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| B1 | GND | Ground |
| B2 | RESET DRV | I |
| B3 | +5 Vdc | Power |
| B4 | IRQ9 | I |
| B5 | -5 Vdc | Power |
| B6 | DRQ2 | I |
| B7 | -12 Vdc | Power |
| B8 | 0WS | I |
| B9 | +12 Vdc | Power |
| B10 | GND | Ground |
| B11 | -SMEMW | O |
| B12 | -SMEMR | O |
| B13 | -IOW | I/O |
| B14 | -IOR | I/O |
| B15 | -DACK3 | I |
| B16 | DRQ3 | O |
| B17 | -DACK1 | I |
| B18 | DRQ1 | O |
| B19 | -Refresh | I/O |
| B20 | CLK | O |
| B21 | IRQ7 | I |
| B22 | IRQ6 | I |
| B23 | IRQ5 | I |
| B24 | IRQ4 | I |
| B25 | IRQ3 | I |
| B26 | -DACK2 | O |
| B27 | T/C | O |
| B28 | BALE | O |
| B29 | +5 Vdc | Power |
| B30 | OSC | O |
| B31 | GND | Ground |

I/O Channel (C-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| C1 | SBHE | I/O |
| C2 | LA23 | I/O |
| C3 | LA22 | I/O |
| C4 | LA21 | I/O |
| C5 | LA20 | I/O |
| C6 | LA19 | I/O |
| C7 | LA18 | I/O |
| C8 | LA17 | I/O |
| C9 | -MEMR | I/O |
| C10 | -MEMW | I/O |
| C11 | SD8 | I/O |
| C12 | SD9 | I/O |
| C13 | SD10 | I/O |
| C14 | SD11 | I/O |
| C15 | SD12 | I/O |
| C16 | SD13 | I/O |
| C17 | SD14 | I/O |
| C18 | SD15 | I/O |

I/O Channel (D-Side)

| I/O Pin | Signal Name | I/O |
| --- | --- | --- |
| D1 | -MEM CS16 | I |
| D2 | -I/O CS16 | I |
| D3 | IRQ10 | I |
| D4 | IRQ11 | I |
| D5 | IRQ12 | I |
| D6 | IRQ15 | I |
| D7 | IRQ14 | I |
| D8 | -DACK0 | O |
| D9 | DRQ0 | I |
| D10 | -DACK5 | O |
| D11 | DRQ5 | I |
| D12 | -DACK6 | O |
| D13 | DRQ6 | I |
| D14 | -DACK7 | O |
| D15 | DRQ7 | I |
| D16 | +5 Vdc | Power |
| D17 | -MASTER | I |
| D18 | GND | Ground |

The following table summery pin assignments for VESA VL-bus connector.

VL-bus (side A)

| I/O Pin | Signal Name |
| --- | --- |
| A1 | CD1 |
| A2 | CD3 |
| A3 | GROUND |
| A4 | CD5 |
| A5 | CD7 |
| A6 | CD9 |
| A7 | CD11 |
| A8 | CD13 |
| A9 | CD15 |
| A10 | GROUND |
| A11 | CD17 |
| A12 | POWER |
| A13 | CD19 |
| A14 | CD21 |
| A15 | CD23 |
| A16 | CD25 |
| A17 | GROUND |
| A18 | CD27 |
| A19 | CD29 |
| A20 | CD31 |
| A21 | CA30 |
| A22 | CA28 |
| A23 | CD26 |
| A24 | GROUND |
| A25 | CA24 |
| A26 | CA22 |
| A27 | POWER |
| A28 | CA20 |

VL-bus (side A)

| I/O Pin | Signal Name |
| --- | --- |
| A29 | CA18 |
| A30 | CA16 |
| A31 | CA14 |
| A32 | CA12 |
| A33 | CA10 |
| A34 | CA8 |
| A35 | GROUND |
| A36 | CA6 |
| A37 | CA4 |
| A38 | WBACK- |
| A39 | BEO- |
| A40 | POWER |
| A41 | BE1- |
| A42 | BE2- |
| A43 | GROUND |
| A44 | BE3- |
| A45 | ADS- |
| A46 | LRDY- |
| A47 | LDEV- |
| A48 | LREQ- |
| A49 | GROUND |
| A50 | LGNT- |
| A51 | POWER |
| A52 | ID2 |
| A53 | ID3 |
| A54 | ID4 |
| A55 | LKEN- |
| A56 | LEADS- |

VL-bus (side B)

| I/O Pin | Signal Name |
| --- | --- |
| B1 | CD0 |
| B2 | CD2 |
| B3 | CD4 |
| B4 | CD6 |
| B5 | CD8 |
| B6 | GROUND |
| B7 | CD10 |
| B8 | CD12 |
| B9 | POWER |
| B10 | CD14 |
| B11 | CD16 |
| B12 | CD18 |
| B13 | CD20 |
| B14 | GROUND |
| B15 | CD22 |
| B16 | CD24 |
| B17 | CD26 |
| B18 | CD28 |
| B19 | CD30 |
| B20 | POWER |
| B21 | CA31 |
| B22 | GROUND |
| B23 | CA29 |
| B24 | CA27 |
| B25 | CA25 |
| B26 | CA23 |
| B27 | CA21 |
| B28 | CA19 |

VL-bus (side B)

| I/O Pin | Signal Name |
| --- | --- |
| B29 | GROUND |
| B30 | CA17 |
| B31 | CA15 |
| B32 | POWER |
| B33 | CA13 |
| B34 | CA11 |
| B35 | CA9 |
| B36 | CA7 |
| B37 | CA5 |
| B38 | GROUND |
| B39 | CA3 |
| B40 | CA2 |
| B41 | n/c |
| B42 | RESET- |
| B43 | D/C- |
| B44 | M/IO- |
| B45 | W/R- |
| B46 | RDY- |
| B47 | GROUND |
| B48 | IRQ9 |
| B49 | BRDY- |
| B50 | BLAST- |
| B51 | ID0 |
| B52 | ID1 |
| B53 | GROUND |
| B54 | VLCLK |
| B55 | POWER |
| B56 | LBS16- |

THIS PAGE IS INTENTIONALLY LEFT BLANK

Appendix A

Operation and 	Maintenance

____________________________________________________

STATIC ELECTRICITY

When installing or removing any add-on card, DRAM module or processor, you should discharge the static electricity on your body.  Static electricity is dangerous to electronic device and can build-up on your body.  When you touch the add-on card or motherboard, it is likely to damage the device.  To discharge the static electricity, touch the metal of your computer.  When handling the add-on card, don't contact the components on the cards or their "golden finger".  Hold the cards by their edges.

KEEPING THE SYSTEM COOL

The motherboard contains many high-speed components and they will generate heat during operation.  Other add-on cards and hard disk drive can also produce a lot of heat.  The temperature inside the computer system may be very high.  In order to keep the system running stably, the temperature must be kept at a low level. A easy way to do this is to keep the cool air circulating inside the case.  The power supply contains a fan to blow air out of the case.  If you find that the temperature is still very high, it would be better to install another fan inside the case.  Using a larger case is recommended if there are a number of add-on cards and disk drives in the system.

CLEANING THE "GOLDEN FINGER"

Whenever inserting an add-on card to the motherboard, make sure that there is no dirt on the "golden finger" of the add-on card.  If not, the contact between the "golden finger" and the slot may be poor and thus the add-on card may not work properly.  Use a pencil eraser to clean the "golden finger" if dirt is found.

CLEANING THE MOTHERBOARD

The computer system should be kept clean.  Dust and dirt is harmful to electronic devices.  To prevent dust from accumulating on the mother-board, installing all mounting plates on the rear of the case.  Regularly examine your system, and if necessary, vacuum the interior of the system with a miniature vacuum.

Appendix B

Troubleshooting

____________________________________________________

MAIN MEMORY ERROR

After power up, the monitor remains blank, and there are beep sounds indicating a main memory failure.  In this case, turn off the power and remove all SIMM modules.  Carefully place the modules back to the sockets and make sure that all the modules are locked by the locking latches firmly.

In some other cases, the total memory found by the BIOS is different from the actual amount of memory on board.  ( Note that 128K bytes memory is reserved for the shadow RAM function and will not be counted by the BIOS).  It is also a memory failure and you can follow the instruction above.

THIS PAGE IS INTENTIONALLY LEFT BLANK