Bios   Manual

for

BISON   III

The material in this manual is for information only and is subject to change without notice.

REVISION: 1.0

IBM, IBM PC/XT/AT, PC-DOS, MS-DOS, OS/2, INTEL, AWARD, Pentium ARE THE TRADEMARKS OR REGISTERED TRADEMARKS OF THEIR RESPECTIVE OWNERS.

The system BIOS provides an interface for the software to control the hardware and is recorded in the ROM chip.  Upon power up, it will perform a self test to ensure the system is functional.  It will then initialize the chipset with setting stored in the CMOS RAM.

Then it proceeds to load the disk operating system and you can start to work your applications.

The BIOS has a built-in Setup program that allows users to modify the basic system configuration.  The information is stored in the CMOS RAM.

Entering Setup

Power on the computer, when the below message appears briefly at the bottom of the screen during the POST (Power On Self Test), press <Del> key or simultaneously press <Ctrl>, <Alt>, and <Esc> keys.

PRESS DEL TO ENTER SETUP

If the message disappears before you respond and you still wish to enter Setup, restart the system to try again by turning it OFF then ON or pressing the “RESET” button on the system case.  You may also restart by simultaneously press <Ctrl>, <Alt>, and <Delete> keys.  If you do not press the keys at the correct time and the system does not boot, an error message will be displayed and you will again be asked to,

PRESS F1 TO CONTINUE, DEL TO ENTER SETUP

Control Keys

| Up Arrow | Move to previous item |
| --- | --- |
| Down Arrow | Move to next item |
| Left Arrow | Move to the item in the left hand |
| Right Arrow | Move to the item in the right hand |
| Esc key | Main Menu - Quit and not save changes into CMOS Status Page Setup Menu and Option Page Setup Menu - Exit current page and return to Main Menu |
| PgUp key | Increase the numeric value or make changes |
| PgDn key | Decrease the numeric value or make changes |
| F1 key | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| F2 key | Change color from total 16 colors |
| F3 key | Calendar, only for Status Page Setup Menu |
| F4 key | Reserved |
| F5 key | Restore the previous CMOS value from CMOS, only for Option Page Setup Menu |
| F6 key | Load the default CMOS value from BIOS default table, only for Option Page Setup Menu |
| F7 key | Load the default |
| F8 key | Reserved |
| F9 key | Reserved |
| F10 key | Save all the CMOS changes, only for Main Menu |

Getting Help

Main Menu

The on-line description of the highlighted setup function is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes  the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help Window press <Esc>.

The Main Menu

Once you enter the BIOS CMOS Setup Utility, the Main Menu will appear on the screen.  The Main Menu allows you to select from ten setup functions and two exit choices.  Use arrow keys to select the items and press <Enter> to accept or enter the sub-menu.

ROM PCI/ISA BIOS (XXXXXXX)

CMOS Setup Utility

Award Software, Inc.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER-ON WITH BIOS DEFAULTS POWER-ON WITH CHIPSET DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVE |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¬®¯ : Select item F2 : Change Color |
| Description of each functions |  |

Standard CMOS Setup

This setup page includes all the items in a standard BIOS.

BIOS Features Setup

This setup page includes all the items of Enhanced Features.

Chipset Features Setup

This setup page includes all the items of chipset special features.

PCI Configuration Setup

This setup page includes all the items of the PCI related setup.

Power-on with BIOS Defaults

BIOS defaults indicates the most appropriate value of the system parameter which the system would be in maximum performance.  However, you may change the parameter value through the Option Page Setup Menu.

Power-on with Chipset Defaults

Chipset defaults indicates the values required by the system for the minimum performance.  However, you may change the parameter through the Option Page Setup Menu.

Password Setting

Change, set, or disable password.  It allows you to limit access to the system and Setup, or just to Setup.

IDD HDD Auto Detection

Automatically configure hard disk parameters.

Save & Exit Setup

Save CMOS value changes to CMOS and exit setup.

Exit without Save

Abandon all CMOS value changes and exit setup.

Standard CMOS Setup Menu

The items in Standard CMOS Setup Menu are divided into 10 categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

ROM PCI/ISA BIOS (XXXXXXXX)

CMOS SETUP UTILITY

Award Software, Inc.

Date	:   Mon, 01 Jan 1992

Time	:   00:00:00

Daylight saving	:   Disabled

CYLS.	HEADS	PRECOMP	LANDZONE	SECTORS

Drive C	:   10	(20MB)	  820	 3	None	 820	       17

Drive D	:   35	(110MB)	 1024	13	None	1023	       17

Drive A	:   1.2MB, 5-1/4 in.

Drive B	:   1.44MB, 3-1/2 in.	Base Memory	:	640K

Extended Memory	:	1024K

Video	:   EGA/VGA	Expanded Memory	:	0K

Other Memory	: 	384K

Halt on Error	:   No Errors

Total Memory	:	2048K

Esc	:	Quit	¬®¯ 	:	Select Item	PgUp/PgDn	:	Modify

F1	:	Help	    F2	:	Change Color		F3	:	Calendar

Date

The date format is <day>, <date> <month> <year>, Press <F3> to show the calendar.

| day | The day, from Sun to Sat, determined by the BIOS and is display-only |
| --- | --- |
| date | The date, from 1 to 31 (or the maximum allowed in the month) |
| month | The month, Jan through Dec |
| year | The year, from 1900 through 2099 |

Time

The time format is <hour> <minutes> <second>.  The time is calculated based on the 24-hour military-time clock.  For example, 1 p.m. is 13:00:00.

Daylight Saving

The category adds one hour to the clock when daylight-saving time begins.  It also subtracts one hour when standard time begins.

Drive C Type / Drive D Type

The category identifies the types of hard disk drive C or drive D that has been installed in the computer.  There are 46 predefined types and a user definable type.  Type 1 to Type 46 are predefined.  Type User is user-definable.

Press PgUp or PgDn to select a numbered hard disk type or type the number and press <Enter>.  Note that the specifications of your drive must match with the drive table.  The hard disk will not work properly if you enter improper information for this category.  If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually.

If you select Type User, related information is asked to be entered to the following items.  Enter the information directly from the keyboard and press <Enter>.  Those information should be provided in the documentation from your hard disk vendor or the system manufacturer.

| CYLS. | Number of Cylinders |
| --- | --- |
| HEADS | Number of heads |
| PRECOMP | Write Precom |
| LANDZONE | Landing Zone |
| SECTORS | Number of Sectors |

If a hard disk has not been installed select NONE and press <Enter>.

Drive A Type / Drive B Type

The category identifies the types of floppy disk drive A or drive B that has been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5.25 in | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K, 3.5 in | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3.5 in | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3.5 in | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

Video

The category selects the type of adapter used for the primary system monitor that must matches your video display card and monitor.  Although secondary monitors are supported, you do not have to select the type in Setup.

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, or PGA monitor adapters |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

Error Halt

The category determines whether the computer will stop if an error is detected during power up.

| No Errors | Whenever the BIOS detects a non-fatal error the system will be stopped and you will be prompted |
| --- | --- |
| All Errors | The system boot will not be stopped for any error that may be detected |
| All, But Keyboard | The system boot will not stop for a keyboard error; it will stop for all other errors |
| All, But Diskette | The system boot will not stop for a disk error; it will stop for all other errors |
| All, But Disk / Key | The system boot will not stop for a keyboard or disk error; it will stop for all other errors |

Memory

The category is display-only which is determined by POST (Power On Self Test) of the BIOS.

Base Memory

The POST of the BIOS will determine the amount of base (or conventional) memory installed in the system.

Extended Memory

The BIOS determines how much extended memory is present during the POST.  This is the amount of memory located above 1MB in the CPU’s memory address map.

Other Memory

This refers to the memory located in the 640K to 1024K address space.  This is memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free for application programs.  Most use for this area is Shadow RAM.

BIOS Features Setup Menu

Virus Warning

This category flashes on the screen.  During and after the system boots up, any attempt to write to the boot sector or partition table of the hard disk drive will halt the system and the following error message will appear.

!  WARNING  !

Disk boot sector is to be modified

Type “Y” to accept write or “N” to abort write

Award software, Inc.

| Enabled | Activate automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table |
| --- | --- |
| Disabled | No warning message to appear when anything attempts to access the boot sector or hard disk partition table |

CPU Internal Cache / External Cache

These two categories speed up memory access.  The default value is enabled.

Quick Power On Self Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If it is set to Enable, BIOS will shorten the check list during POST.

Boot Sequence

This category determines which drive computer searches first for the disk operating system (i.e. DOS).  Default value is A, C.

| C, A | System will first search for hard disk drive then floppy disk drive |
| --- | --- |
| A, C | System will first search for floppy disk drive then hard disk drive |

Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks.  360K type is 40 tracks while 760K, 1.2M and 1.44M are all 80 tracks.

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks. Note that BIOS can not tell from 720K, 1.2M or 1.44M drive type as they are all 80 tracks |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number. Note that there will not be any warning message if the drive installed is 360K |

Boot Up NumLock Status

The default value is On.

| On | Keypad is number keys |
| --- | --- |
| Off | Keypad is arrow keys |

Boot Up System Speed

It selects the default system speed - the speed that the system will run at immediately after power up.

IDE HDD Block Mode

Block mode improves the disk transfer rate by reducing the number of interrupts.  Most of the new IDE drives have this feature.

Gate A20 Option

| Normal | Reserved |
| --- | --- |
| Fast | Chipset |

Memory Parity Check

| Enabled | Normal memory parity check |
| --- | --- |
| Disabled | Ignore memory parity check |

Typematic Rate Setting

This determines the typematic rate.

| Enabled | Enable typematic rate |
| --- | --- |
| Disabled | Disable typematic rate |

Typematic Rate (Chars/Sec)

6-30 characters per second can be selected.

Typematic Delay (Msec)

When hold a key, the time between the first and second character displayed.  250-1000 msec can be selected.

Security Option

This category allows you to limit access to the system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt |

Note :   To disable security, select PASSWORD SETTING at Main menu and then you will be asked to enter password.  Do not type anything and just press <Enter>, it will disable security.  Once the security is disabled, the system will boot and you can enter Setup freely.

System BIOS Shadow

It determines whether system BIOS will be copied to RAM.  System Shadow will improve the system performance.

Video BIOS Shadow

It determines whether video BIOS will be copied to RAM.  Video Shadow ill increase the video speed.

C8000-CBFFF Shadow / EC000-EFFFF Shadow

These categories determine whether optional ROM will be copied to RAM by 16K bytes.

Chipset Features Setup Menu

The items in this menu are configured in factory.  Changing the defaults may lead malfunction.

Password Setting

When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD

Type the password, up to eight characters, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the selection and not enter a password.

To disable password, just press <Enter> when you are prompted to enter password.  A message will confirm the password being disabled.  Once the password is disabled, the system will boot and you can enter Setup freely.

PASSWORD DISABLED

If you select System at Security Option of BIOS Features Setup Menu, you will be prompted for the password every time the system is rebooted or any time you try to enter Setup.  If you select Setup at Security Option of BIOS Features Setup Menu, you will be prompted only when you try to enter Setup.

PCI Configuration Setup Menu

This menu allows users to route the interrupts on the PCI Bus to the ISA Bus.

Slot x Using INT# :

x  is the slot number of the PCI slot

A - D, four PCI interrupts on each PCI slot can be selected as the source with level and edge triggered

Slot x Using IRQ :     Y By Z

Y is the ISA interrupt number

5, 9-15, eight ISA interrupts can be selected as the output of the PCI interrupts.

Z is trigger mode of the PCI interrupts.  Either level or edge can be chosen.  Other options in this menu are configured in factory.  Changing the defaults may lead malfunctions.

Power-On Boot

After you have made all the changes to CMOS values and the system can not boot with the CMOS values selected in Setup, restart the system by turning if OFF then ON or pressing the “RESET” button on the system case.  You may also restart by simultaneously press <Ctrl>, <Alt> and <Delete> keys.  Upon restart the system, immediately press <Insert> to load BIOS default CMOS value for boot up.

BIOS Reference - POST Message

When the BIOS encounters an error that requires the user to correct something, either a beep code will sound or a message will be displayed in a box in the middle of the screen and the message PRESS F1 TO CONTINUE, DEL TO ENTER SETUP will be shown in the information box at the bottom.

POST Beep

Currently there is only one beep code in BIOS.  This code indicates that a video error has occurred and the BIOS cannot initialize the video screen to display any additional information.  This beep code consists of a single long beep followed by two short beeps.

Error Messages

One or more of the following messages may be displayed if the BIOS detects an error during the POST.

CMOS BATTERY HAS FAILED

CMOS battery is no longer functional.  It should be replaced.

CMOS CHECKSUM ERROR

Checksum of CMOS is incorrect.  This can indicate that CMOS has become corrupt.  This error may have been caused by a weak battery.  Check the battery and replace if necessary.

DISK BOOT FAILURE, INSERT SYSTEM DISK AND PRESS ENTER

No boot device was found.  Insert a system disk into Drive A: and press <Enter>.  If you assumed the system would boot from the hard drive, make sure the controller is inserted correctly and all cables are properly attached.  Also be sure the disk is formatted as a boot device.  Then reboot the system

DISKETTE DRIVES OR TYPES MISMATCH ERROR - RUN SETUP

Type of diskette drive installed in the system is different from the CMOS definition.  Run Setup to reconfigure the drive type correctly.

DISPLAY SWITCH IS SET INCORRECTLY

Display switch on the motherboard can be set to either monochrome or color.  This indicates the switch is set to a different setting than indicated in Setup.  Determine which setting is correct, and then either turn off the system and change the jumper, or enter Setup and change the VIDEO selection.

DISPLAY TYPE HAS CHANGED SINCE LAST BOOT

Since last powering off the system, the display adapter has been changed.  You must configure the system for the new display type.

ERROR ENCOUNTERED INITIALIZING HARD DRIVE

Hard drive cannot be initialized.  Be sure the adapter is installed correctly and all cables are correctly and firmly attached.  Also be sure the correct hard drive type is selected in Setup.

ERROR INITIALIZING HARD DISK CONTROLLER

Cannot initialize controller.  Make sure the cord is correctly and firmly installed in the bus.  Be sure the correct hard drive type is selected in Setup.  Also check to see if any jumper needs to be set correctly on the hard drive.

FLOPPY DISK CNTRLR ERROR OR NO CNTRLR PRESENT

Cannot find or initialize the floppy drive controller.  Make sure the controller is installed correctly and firmly.  If there are no floppy drives installed, be sure the Diskette Drive selection in Setup is set to NONE.

KEYBOARD ERROR OR NO KEYBOARD PRESENT

Cannot initialize the keyboard.  Make sure the keyboard is attached correctly and no key are being pressed during the boot.

If you are purposely configurating the system without a keyboard, set the error halt condition in Setup to HALT ON ALL, BUT KEYBOARD.  This will cause the BIOS to ignore the missing keyboard and continue the boot.

Memory Address Error at ...

Indicates a memory address error at a specific location.  You can use this location along with the memory map for your system to find and replace the bad memory chips.

Memory Parity Error at ...

Indicates a memory parity error at a specific location.  You can use this location along with the memory map for your system to find and replace the bad memory chips.

MEMORY SIZE HAS CHANGED SINCE LAST BOOT

Memory has been added or removed since the last boot.  Enter the new memory size in the memory fields.

Memory Verify Error at ...

Indicates an error verifying a value already written to memory.  Use the location along with your system’s memory map to locate the bad chip.

OFFENDING ADDRESS NOT FOUND

This message is used in conjunction with the I/O CHANNEL CHECK and RAM PARITY ERROR messages when the segment that has caused the problem cannot be isolated.

OFFENDING SEGMENT

This message is used in conjunction with the I/O CHANNEL CHECK and RAM PARITY ERROR messages when the segment that has caused the problem cannot be isolated.

PRESS A KEY TO REBOOT

This will be displayed at the bottom screen when an error occurs that requires you to reboot.  Press any key and the system will reboot.

PRESS F1 TO DISABLE NMI, F2 TO REBOOT

When BIOS detects a Non-maskable Interrupt condition during boot, this will allow you to disable the NMI and continue to boot, or you can reboot the system will the NMI enabled.

RAM PARITY ERROR - CHECKING FOR SEGMENT

Indicates a parity error in Random Access Memory.

BIOS Reference - BIOS Default Drive Table

| Type | Size (MB) | Cylinders | Heads | Sectors | Write Precomp | Land Zone | Example Model |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 306 | 4 | 17 | 128 | 305 | TEAC SD510, MMI 112, 5412 |
| 2 | 20 | 615 | 4 | 17 | 300 | 615 | Seagate ST225, ST4026 |
| 3 | 30 | 615 | 6 | 17 | 300 | 615 |  |
| 4 | 62 | 940 | 8 | 17 | 512 | 940 |  |
| 5 | 46 | 940 | 6 | 17 | 512 | 940 |  |
| 6 | 20 | 615 | 4 | 17 | None | 615 | Seagate ST125, Tandon TM262 |
| 7 | 30 | 462 | 8 | 17 | 256 | 511 |  |
| 8 | 30 | 733 | 5 | 17 | None | 733 | Tandon TM703 |
| 9 | 112 | 900 | 15 | 17 | None | 901 |  |
| 10 | 20 | 820 | 3 | 17 | None | 820 |  |
| 11 | 35 | 855 | 5 | 17 | None | 855 |  |
| 12 | 49 | 855 | 7 | 17 | None | 855 |  |
| 13 | 20 | 306 | 8 | 17 | 128 | 319 | Disctron 526, MMI M125 |
| 14 | 42 | 733 | 7 | 17 | None | 733 |  |
| 15 |  | Reserved |  |  |  |  |  |
| 16 | 20 | 612 | 4 | 17 | 0 | 663 | Microscience HH725, Syquest 3250, 3425 |
| 17 | 40 | 977 | 5 | 17 | 300 | 977 |  |
| 18 | 56 | 977 | 7 | 17 | None | 977 |  |
| 19 | 59 | 1024 | 7 | 17 | 512 | 1023 |  |
| 20 | 30 | 733 | 5 | 17 | 300 | 732 |  |
| 21 | 42 | 733 | 7 | 17 | 300 | 732 |  |
| 22 | 30 | 306 | 5 | 17 | 300 | 733 | Seagate ST4038 |
| 23 | 10 | 977 | 4 | 17 | 0 | 336 |  |
| 24 | 40 | 1024 | 5 | 17 | None | 976 | Seagate ST4051 |
| 25 | 76 | 1224 | 9 | 17 | None | 1023 | Seagate ST4096 |
| 26 | 71 | 1224 | 7 | 17 | None | 1223 | Maxtor 2085 |
| 27 | 111 | 1224 | 11 | 17 | None | 1223 | Maxtor 2140, Priam S14 |
| 28 | 152 | 1024 | 15 | 17 | None | 1223 | Maxtor 2190, Priam S19 |
| 29 | 68 | 1024 | 8 | 17 | None | 1023 | Maxtor 1085, Micropolis 1325 |
| 30 | 93 | 918 | 11 | 17 | None | 1023 | Maxtor 1105, 1120, 4780 |
| 31 | 83 | 925 | 11 | 17 | None | 1023 | Maxtor 1170 |
| 32 | 69 | 1024 | 9 | 17 | None | 926 | CDC 9415 |
| 33 | 85 | 1024 | 10 | 17 | None | 1023 |  |
| 34 | 102 | 1024 | 12 | 17 | None | 1023 |  |
| 35 | 110 | 1024 | 13 | 17 | None | 1023 |  |
| 36 | 119 | 1024 | 14 | 17 | None | 1023 |  |
| 37 | 17 | 1024 | 2 | 17 | None | 1023 |  |
| 38 | 136 | 1024 | 16 | 17 | None | 1023 |  |
| 39 | 114 | 918 | 15 | 17 | None | 1023 | Maxtor 1140, 4380 |
| 40 | 40 | 820 | 6 | 17 | None | 820 | Seagate ST251 |
| 41 | 42 | 1024 | 5 | 17 | None | 1023 | Seagate 4053, Miniscribe 3053/6053 |
| 42 | 65 | 1024 | 5 | 26 | None | 1023 | Miniscribe 3053/6053 RLL |
| 43 | 40 | 819 | 6 | 17 | None | 852 | Miniscribe 3650 |
| 44 | 61 | 809 | 6 | 26 | None | 852 | Miniscribe 3675 RLL |
| 45 | 100 | 776 | 8 | 33 | None | 775 | Conner CP3104 |
| 46 | 203 | 684 | 16 | 38 | None | 685 | Conner CP3204 |
| User |  |  |  |  |  |  |  |