### 2.02	CPU Related Settings

CPU Voltage Core Selection

RHINO 15 supports Intel Pentium (P54C) & Pentium with MMX (P55C), AMD K5/K6, Cyrix/IBM 6x86/6x86L, CyrixMX. Both single & dual voltage CPUs are supported. For dual voltage CPUs, JP4/5/6/19 must be set as 1-2 to separate the core voltage & I/O voltage. The voltage selection for core voltage is as follows :

| JP4/5/6/19 | JP9 | CPU Core Voltage | CPU Type |
| --- | --- | --- | --- |
| 2-3* | 9-10 | 3.5V | Cyrix/IBM 6x86, AMD K5 |
|  | 7-8* | 3.3V | Intel P54C |
| 1-2 | 5-6 | 3.2V | AMD K6/233 |
|  | 3-4 | 2.9V | AMD K6/166/200, CyrixMX |
|  | 1-2 | 2.8V | Intel P55C, Cyrix/IBM 6x86L |
|  | Open | 2.5V | Reserved |

F Be careful to select the appropriate Core voltage for different CPUs. Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU ! FF The Official Name of P55C is “Pentium Processor with MMX Technology”.

CPU Type

| JP1 | JP2 | JP3 | JP15 (For AMD K6) | JP10 | JP11 (Option) | CPU Clock | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2 1-2 1-2 1-2 1-2 1-2 1-2 1-2 1-2 | 1-2 1-2 2-3 2-3 1-2 2-3 1-2 1-2 2-3 | 1-2 2-3 2-3 1-2 2-3 2-3 1-2 2-3 2-3 | Open Open Open Open Open Open Open Open Open | 1-2 1-2 1-2 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 60 MHz | Intel P54C-90 * Intel P54C-120 Intel P54C-150 Intel P54C-180 Cyrix 6x86-P150+ Cyrix 6x86MX-PR166 AMD K5-PR90 AMD K5-PR120 AMD K5-PR150 |
| 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 2-3 2-3 2-3 2-3 1-2 1-2 1-2 2-3 1-2 1-2 2-3 2-3 2-3 1-2 | 1-2 2-3 2-3 2-3 1-2 1-2 1-2 2-3 2-3 2-3 1-2 1-2 2-3 2-3 1-2 1-2 | Open Open Open Open Open Open Open Open Open Open Open Open Open Open Open Open | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 66 MHz | Intel P54C-100 Intel P54C-133 Intel P54C-166 Intel P55C-166 Intel P54C-200 Intel P55C-200 Intel P55C-233 Cyrix/IBM 6x86-P166+ Cyrix/IBM 6x86L-P166+ Cyrix 6x86MX-PR200 AMD K5-PR100 AMD K5-PR133 AMD K5-PR166 AMD K6/166 AMD K6/200 AMD K6/233 |

NOTE: All factory default settings are marked by “*”.

## Jumper Setting