USER'S  MANUAL

RHINO 5

Pentium PCI Local Bus Motherboard

RHINO 5

Pentium PCI  Local Bus Motherboard

User’s Manual

Version 1.0   August 1995

Copyright Notice

Copyright 1995 Ocean Office Automation Ltd.

All rights reserved.

Ocean Office Automation Ltd.

5th Floor, Kader Industrial Building,

22 Kai Cheung Road, Kowloon Bay,

Kowloon, Hong Kong.

All of the information contained in this manual is copyrighted and all rights reserved.  No part of this document, in whole or in part, may be reproduced or copied in any form without prior consent in writing from Ocean Office Automation Ltd..

_____________________________________________________________________

Limitations of  Liability

While the information in this manual has been carefully reviewed and is believed to be accurate, Ocean Office Automation Ltd. (Ocean) assumes no responsibility in the event that any inaccuracies are found.  In no event shall Ocean be held liable for any loss or expenses whether directly or indirectly caused by the support materials provided with this product.  It is further acknowledged that Ocean is under no obligation to update the manual or to notify purchasers of any forthcoming updates.

_____________________________________________________________________

Trademarks

Intel i486, Pentium and Intel are registered trademarks of Intel Corporation.

Octek is a registered trademark of  Ocean Office Automation Ltd.

OPTi is a registered trademark of OPTi Inc.

XT, AT, PS/2, OS/2, & IBM are registered trademarks of International Business Machines Corporation.

All other brand and product names mentioned in this manual are trademarks or copyrights of their respective holders.

Preface

About the Rhino 5

Thank you for purchasing the Rhino 5, Ocean’s new top-of-the-line Pentium motherboard.  The Rhino 5 utilizes advanced PCI architecture that takes personal computing to a new level of performance.

About the Manual

The content of this manual is for reference only and is intended to provide basic information for the general user.  Technical information is also included in the Appendix for hardware and software engineers.

This manual provides information about the installation and maintenance of the Octek  Rhino 5 motherboard.

Table of Contents

Preface

Chapter 1:  System Overview

1.1      General Specification Overview

1.2      Central Processing Unit

1.3      External Cache Subsystem

1.4      DRAM Subsystem

1.5      PCI Bus

1.6      Super I/O Subsystem

1.7      Input/Output Subsystem

1.8      BIOS Subsystem

Chapter 2: Installation and Upgrade

2.1      Setup

2.2      Installation Precautions

2.3      Operation and Maintenance

2.4      CPU Installation

2.5      CPU Upgrade/ VRM Header

2.6      Fast Page mode / EDO DRAM Installation

2.7      Asynchronous Cache RAM Installation

2.8      Control of System Speed

2.9      Fan voltage

2.10    Fan Connector

2.11    Reset CMOS

Appendix-A: Advanced Technical Information

A.1      Memory Address Map

A.2      I/O Address Map

A.3      I/O Extension Pinout

A.3i      8-Bit ISA Pinout

A.3ii    16-Bit ISA Extension Pinout

A.4      PCI Bus Pinout

A.5      Direct Memory Access Channels

A.6      DMA Controller Registers

A.7      Page Register Address

A.8      System Interrupts

Appendix-B: Connectors Pinout

B.1      Reset Connector (P1)

B.2      Turbo LED Connector (P2)

B.3      Keylock Connector (P3)

B.4      Fan Connector (P4)

B.5      EPMI/T.B. Switch (P5)

B.6      Speaker Connector (P6)

B.7      Header 7 (P8)

B.8      External Battery Connector (P9)

B.9      IDE LED Connector (P10)

B.10    IDE Connector (P11,P12)

B.11    Floppy Connector (P13)

B.12    Printer Port Connector (P14)

B.13    Serial Port Connector (P15,P16)

B.14    Power Connector (P17,P18)

B.15    PS/2 Mouse Connector (P19)

B.16    Keyboard Connector (KB1)

Appendix-C: Hardware Settings

C.1      System Component Map

C.2      Layout of RHINO 5  Main Board

C.3      Jumper Settings

C.3.1      CPU related settings

C.3.2      External cache (L2 cache) setting

C.3.3      DRAM

C.3.4      Peripheral setup

C.3.5    Miscellaneous

# CHAPTER 1

# SYSTEM OVERVIEW

OCTEK RHINO 5 is a powerful PC machine offering unparalleled performance. The advanced external cache system implemented meets the demand of the most memory-intensive applications today. With the Pentium processor and a high bandwidth 32-bit PCI expansion bus, the I/O  bottleneck that plagues most PC systems is now removed.

All of the I/O is integrated inside the mainboard to further facilitate system installation. The built-in IDE can support up to 4 fast Enhanced IDE devices whereas it can  also support up to two  floppy drives. The  mainboard also includes two serial ports and one parallel port as the basic configuration for end user.  All that is needed is just a VGA card plugged into a PCI or ISA slot to complete the whole system.

## 1.1	General Specifications Overview

Processor:

w Processor Type		Intel  Pentium CPU including

P54C,  P54CQS, P54CS, P55C 					and future upgrades CPU

w External CPU clock		50/60/66 Mhz

Chipset:

w Motherboard  chipset		Opti Viper-M PCI/ISA Pentium 					motherboard chipset

w Super I/O chipset		SMC 665/669 super I/O chipset

Cache Architecture:

w Internal Cache 	 	8KB data cache

8KB code cache

w External Cache                        256KB/ 512KB/ 1MB asynchronous 				3V/Mixed voltage SRAM

Memory Subsystem:

w DRAM SIMM sockets		4 x 72 pin  4MB / 8MB / 16MB / 					32MB DRAM modules

w Max. Memory Size		128MB

w Enhancement			Fast Page Mode or EDO DRAM 					supported

Input/Output Subsystem

w PCI bus slots		             3 x 32-bit PCI Bus slots (3 masters)

w ISA bus slots			3 x 16-bit  ISA slots

w shared bus slots		1 x 32 bit PCI bus slot (master) OR 1 				x 16-bit ISA slot

w I/O bus speed			Up to 33MHz (PCI bus)

Integrated IDE, Super I/O Subsystem

w IDE support			Chipset built-in PCI IDE support up to 				4 IDE Drives

w On board I/O 			One Floppy Port supporting 2 floppy 				drives of 360 / 720 / 1.44M/ 2.88M 				capacity.

Two serial ports (16550 Fast UART 				compatibles)

One parallel Port (Standard, ECP, 				EPP support)

PS/2 Mouse

w PS/2 Mouse			Supports PS/2 Mouse through a 2x4 				header

Power Management

w Green functions		Support various Power Management 				schemes

BIOS Subsystem

w BIOS Type			AWARD

w BIOS Shadowing		Shadow RAM for System and Video 				BIOS

w BIOS Features		Built-in setup, Power-on self test, 				Drive table optimization, User-					definable drive types, Password 					protection, Shadowing options

Plug & Play / BIOS Update

w Plug & Play BIOS		Support Plug & Play for easy 					installation

w Flash EEPROM		Use Flash EEPROM (1M bits)  to 				allow easy BIOS update

System Support Functions

w System functions		7 DMA channels, 16 level interrupts, 				Programmable timers

w Support functions		Fast A20 gate and Fast Reset

w Clock				Enhanced real time clock/calendar 				with battery back-up

Other Features

w Power good			On board power good signal 					generation

w 3.3V supply			On board 3.3V supply to eliminate 				the need for special power supply for

3.3V component e.g. CPU, SRAM.

Maximum rating : 30 W.

w Switches			EPMI, Reset, Keylock  switches

w Size				8.5" (W) x 13" (L)

## 1.2	Central Processing Unit

The Pentium processor is a superscalar,  pipelined CPU that provides next generation performance for the existing PC compatible software.

The processor is equipped with an 8K code cache and an 8K data cache . Each cache is organized in a 2-way set-associative architecture, offering higher hit rates. The data cache can be configured in write-back or write-through modes.

The internal numeric coprocessor  is redesigned to give three times the performance of the 80486 FPU. It is backward compatible with i486DX math coprocessor and complying to ANSI/IEEE standard 754-1985.

## 1.3	External Cache Subsystem

The external cache of RHINO 5 is organized in a direct-mapped configuration with sizes of  256KB, or 512KB or 1MB. It can operate in write-through mode or write-back mode

.

Either  3.3V asynchronous SRAM or mixed mode voltage asynchronous SRAM can be used in RHINO 5.

## 1.4	DRAM Subsystem

The main memory in RHINO 5 is organized as a 64-bit memory pool. Both fast-page mode and EDO DRAMs are supported.

EDO DRAM stands for Extended Data Out DRAM and is designed to improve the DRAM read performance. EDO DRAM holds the memory data valid until the next CAS# falling edge, unlike standard fast page mode DRAM which tri-states the memory data when CAS# is deasserted to precharge for the next cycle. As a result, the CAS# precharge can now overlap with the data valid time to allow CAS# to negate earlier while still satisfying the memory data valid window time.

In addition, Advanced Posted-Write buffers are implemented to improve memory write timing. Shadow RAM is available as an option . System BIOS and video BIOS residing in slow EPROM can be copied to local DRAM to speed up accesses  to BIOS code.

## 1.5	PCI Bus

The Peripheral Component Interconnect (PCI) local bus was specified to establish a high performance local bus standard. It is a 32-bit wide bus supporting burst transactions. PCI local bus devices residing in add-on cards can be configured automatically.

The PCI local bus implemented in RHINO 5 is fully compliant to v2.0 specification. Up to four PCI bus masters are supported. Advanced PCI  bus buffer management logic speeds to the transfer rate.

## 1.6	Super I/O Subsystem

To facilitate system implementation, included in RHINO 5 are two fast Enhanced IDE ports that can dramatically boost the system performance if fast  IDE drives are used.

Furthermore, floppy drives supporting various  format are also supported through the floppy connector on board. The motherboard is also equipped with two serial ports (16550 Fast UART compatibles) and one parallel port that operates in standard, ECP or EPP mode.

In addition, RHINO 5 is designed to support the PS/2 mouse using specialized keyboard controller and the use of interrupt IRQ 12. Nevertheless, user can disable the PS/2 mouse function by means of jumpers.

## 1.7 	Input/Output Subsystem

To allow greater system feasibility, RHINO 5 has four ISA bus expansion connectors  and four PCI expansion connectors. One of the expansion slots is shared by connectors that will accommodate either an ISA or a PCI expansion, but not both at the same time. Therefore, up to seven expansion slots can be populated on RHINO 5 . Furthermore, all the PCI slots can accept PCI bus master cards.

## 1.8 	BIOS Subsystem

RHINO 5 uses Award ROM BIOS and is stored in Flash EEPROM (1 M bits) which allows easy upgrade through the utility found inside the diskette shipped with RHINO 5.

Inside the diskette you can find the BIOS update utility as well as the most updated version of  the BIOS for RHINO 5, together with the BIOS reference manual in TEXT format. The BIOS contains Setup utility, Power-On Self Tests (POST), and PCI auto-configuration  utility during boot-up.

# CHAPTER 2

# INSTALLATION AND UPGRADE

## 2.1	Setup

Before installing the Rhino 5 motherboard in your system, you must attach the peripheral connectors. Follow the instructions outlined in this chapter for proper installation.

## 2.2	Installation Precautions

- Turn off the power whenever installing or removing any connectors, memory module or add-on cards.

- Before turning on the power, make sure that all the connectors, memory modules and add-on cards are secured.

- Eliminate any static electricity when handling the motherboard or any peripherals to prevent  damage to electrical components.  To eliminate electricity, touch a grounded metal object before removing the board from the anti-static bag and wear a grounded wrist or ankle strap when handling the components.

- Put the board and peripherals back into the anti-static bags when you are not using them.

- Always handle the board by its edges, taking care not to touch any of its components.

## 2.3	Operation and Maintenance

- Keep the system cool.  The motherboard contains many high-speed components and they will generate heat during operation.  Other add-on cards and the hard disk can also produce a lot of heat.  The temperature inside the computer system may be very high.  In order to keep the system running stable, the temperature must be kept at a low level.  An easy way to do this is to keep the cool air circulating inside the case.  If the temperature is still very high, install another fan inside the case.  Using a larger case is recommended if there a number of add-on cards and disk drives in the system.

- Clean the “Gold Finger.”  Whenever inserting an add-on card to the motherboard, make sure that there is no dirt on the “gold finger” of the add-on card.  If dirt is present, the contact between the “golden finger” and the slot may be poor, causing the add-on card to work improperly.  Use a pencil eraser to clean the “gold finger” if dirt is found.

- Clean the motherboard. The computer system should be kept clean . Dust and dirt are harmful to electronic devices. To prevent from accumulating on the motherboard, install all mounting plates on the rear of the case. Regularly examine your system and , if necessary, vacuum the interior of the system with a miniature vacuum.

## 2.4	CPU Installation

The CPU is composed of pins that can easily be bent during installation, causing permanent damage to the processor.  It is therefore very important that you make sure the pins are straight before installing the CPU onto the SPGA socket located on RHINO 5 (refer to layout for exact location).  To properly align the CPU  with the socket, align pin 1 of the CPU ( with a notch at the corner) with pin 1 of the CPU socket as demonstrated below.

Figure  1   Socket 7 for Pentium CPU

## 2.5	CPU Upgrade/ VRM Header

In case that you would like to upgrade your CPU, make sure that you have followed the hardware setting described in Appendix-C  for new CPU setting. If you upgrade your Pentium with CPU such as P55C , care must be taken to install the VRM used in connection with the CPU.

VRM stands for Voltage Regulator Module and is specially designed to provide a steady power source (with voltage of around 2.5V)  for use by P55C CPU. CPU such as P54C, P54CT, P54CS, P54CQS do not need to use the VRM as all the voltage required (about 3.3.V) has already provided by the mainboard.

Inside RHINO 5 motherboard, you will find the VRM header (Header 7) which is stuffed with four jumper shunts. To install the VRM , remove the four jumper shunts and insert the VRM into the header. Make sure that it is securely stuffed by the header.

|  | Never change the jumper shunts in the VRM header if you are not upgrading the P55C CPU. |
| --- | --- |

## 2.6	Fast Page mode / EDO DRAM Installation

There are four memory banks located on the RHINO 5 motherboard, marked SM1, SM2, SM3 and SM4. The SM1, SM2, SM3 and SM4 are counted starting from right to left consecutively. Start to install the SIMM modules ( IN PAIRS) from the right hand side first ( SM1/SM2). Depending on how your memory is configured, you may not need to use all the memory banks. Either X32 or X36 of  72 pins SIMM can be installed.

SM4

SM     3

SM

2

SM 1

Figure 2     SIMM  Sockets Location

|  | Never force the SIMM modules into the SIMM sockets. The sockets are fragile and the locking latches may break. |
| --- | --- |

To insert the modules into the banks follow these steps:

- Locate the notch (near pin 1) on the corner of the module.

- Hold the module so that the notch is at the bottom left corner.

- Insert the bottom edge of the module into the bank at an angle, then pull the module in the direction towards yourself so that it is locked into place by the latches located on the sides of the bank.  The latches should be locked tightly and the holes in the module should be aligned with the tabs on the bank.

1

72

®  Near KB

Connector

Figure 3      Proper Memory Module Installation

Below is the memory configuration table for RHINO 5.

| SM4 | SM3 | SM2 | SM1 | Total |
| --- | --- | --- | --- | --- |
| -------- | -------- | 256Kx32 | 256Kx32 | 2MB |
| -------- | -------- | 512Kx32 | 512Kx32 | 4MB |
| -------- | -------- | 1Mx32 | 1Mx32 | 8MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 16MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 32MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 64MB |
| 256Kx32 | 256Kx32 | 256Kx32 | 256Kx32 | 4MB |
| 512Kx32 | 512Kx32 | 256Kx32 | 256Kx32 | 6MB |
| 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 8MB |
| 1Mx32 | 1Mx32 | 256Kx32 | 256Kx32 | 10MB |
| 1Mx32 | 1Mx32 | 512Kx32 | 512Kx32 | 12MB |
| 1Mx32 | 1Mx32 | 1Mx32 | 1Mx32 | 16MB |
| 2Mx32 | 2Mx32 | 256Kx32 | 256Kx32 | 18MB |
| 2Mx32 | 2Mx32 | 512Kx32 | 512Kx32 | 20MB |
| 2Mx32 | 2Mx32 | 1Mx32 | 1Mx32 | 24MB |
| 2Mx32 | 2Mx32 | 2Mx32 | 2Mx32 | 32MB |
| 4Mx32 | 4Mx32 | 256Kx32 | 256Kx32 | 34MB |
| 4Mx32 | 4Mx32 | 512Kx32 | 512Kx32 | 36MB |
| 4Mx32 | 4Mx32 | 1Mx32 | 1Mx32 | 40MB |
| 4Mx32 | 4Mx32 | 2Mx32 | 2Mx32 | 48MB |
| 4Mx32 | 4Mx32 | 4Mx32 | 4Mx32 | 64MB |
| 8Mx32 | 8Mx32 | 256Kx32 | 256Kx32 | 66MB |
| 8Mx32 | 8Mz32 | 512Kx32 | 512Kx32 | 68MB |
| 8Mx32 | 8Mx32 | 1Mx32 | 1Mx32 | 72MB |
| 8Mx32 | 8Mx32 | 2Mx32 | 2Mx32 | 80MB |
| 8Mx32 | 8Mx32 | 4Mx32 | 4Mx32 | 96MB |
| 8Mx32 | 8Mx32 | 8Mx32 | 8Mx32 | 128MB |

Table 1      Memory Configuration Table

## 2.7 	Asynchronous Cache RAM Installation

RHINO 5 provides the option for changing the size of the asynchronous cache RAM from cacheless up to 1M bytes cache RAM.  To do this, besides the jumper settings mentioned in the Appendix C has to be set properly, care must be taken to stuff the cache RAM in the proper place and orientation.

Cache size of 256K bytes requires 8 pieces of 32Kx8 data RAM. As they are 28 pin DIP IC, they should be stuffed at U2-U6 and U9-U11 as shown below :

Pin  #1 of IC Socket

32 Pin DIP socket at U2-U6, U9-U11

Pin  #1 of  32Kx8

32K x8

SRAM

Align at the bottom  side of the IC.

For cache size of  512K bytes or 1M bytes, 8 pieces of 64Kx8 or 128Kx8 data SRAM should be used respectively. As both of them are 32-pin DIP IC, just stuff them at U2-U6 and U9-U11.

## 2.8	Control of System Speed

System speed can be controlled by keyboard. To change the speed by keyboard, use the minus sign (-) and the plus sign (+).  Press <control> + <alt> + <“-”> for slow speed and  <control> + <alt> + <“+”> for fast speed.

## 2.9	Fan voltage

There are two different voltages available for CPU fans; +5 volts and + 12 volts. Select a proper setting for jumper JP1 before you turn on the computer.

## 2.10	Fan Connector

This Fan connector (P4) allows the CPU fan to draw current directly from the motherboard, without having to rely on a separate power connector.

|  | A CPU fan is highly recommended for today’s high speed CPUs. Extreme high CPU temperature are generated during system operations, and may cause abnormal hardware behaviour if system is not cool down. |
| --- | --- |

## 2.11	Reset CMOS

If the setting of the system setup is done improperly, it may make the system malfunction. If this happens, turn off the power and set jumper JP21 to 1-2 to clear the internal CMOS status register.  Next, set the jumper JP21 back to 2-3 and turn on the power.  The BIOS will find the CMOS status register is  reset and will regard the setup information invalid, so it will prompt you to correct the information.

# APPENDIX-A

# ADVANCED TECHNICAL INFORMATION

## A.1	Memory Address Map

| Address | Size |  | Function |
| --- | --- | --- | --- |
| 0000000- 009FFFF | 640 KB |  | System board memory |
| 00A0000-00BFFFF | 128 KB |  | Video RAM display buffer |
| 00C0000-00DFFFF | 128 KB |  | Reserved for add-on cards ROM BIOS |
| 00E0000-00EFFFF | 64 KB |  | System ROM BIOS expansion |
| 00F0000-00FFFFF | 64 KB |  | System ROM BIOS |
| 0100000-7FFFFFF | 127MB |  | Extended memory |

## A.2	I/O Address Map

I/O Address Hex 000 to 0FF are reserved for the system board and Hex 100 to 3FF are for I/O channels.

| Address (Hex) | Device |
| --- | --- |
| 000-01F | DMA Controller 1, 8237A-5 |
| 020-021 | Interrupt Controller 1, 8237A-5 |
| 022-023 | Chip set Address |
| 040-04F | Timer 1, 8254 |
| 050-05F | Timer 2, 8254 |
| 060-06F | 8042 Keyboard/Controller |
| 070-07F | Real Time Clock, Non-Maskable Interrupt Mask |
| 080-09F | DMA Page Registers |
| 0A0-0BF | Interrupt Controller 2,8259A |
| 0C0-0DF | DMA Controller 2, 8237A-5 |
| 0F0 | Clear Math Coprocessor Busy |
| 0F1 | Reset Math Coprocessor |
| 0F8-0FF | Math Coprocessor |
| 1F0-1F8 | Fixed Disk |
| 200-207 | Game Ports |
| 278-27F | Parallel Printer Port 2 (PIO-2) |
| 2F8-2FF | Serial Port 2 (SIO-2) |
| 300-31F | Prototype Card/Streaming Tape Adapter |
| 360-363 | PC Network, Low Address |
| 368-36B | PC Network, High Address |
| 378-37F | Parallel Printer Port 1 (PIO-1) |
| 380-38F | SDLC, Bisynchronous 2 |
| 3A0-3AF | Bisynchronous 1 |
| 3B0-3BF | Monochrome Display and Primer Adapter |
| 3C0-3CF | EGA Adapter |
| 3D0-3DF | Color/Graphics Monitor Adapter |
| 3F0-3F7 | Diskette Controller |
| 3F8-3FF | Serial Port 1 (SIO-1) |

## A.3	I/O Extension Pinout

### A.3i	8-Bit ISA Pinout

|  | Ground | GND | B1 | A1 | -I/O CHCK |  | I |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | O | RSTDRV | B2 | A2 | SD7 |  | I/O |
|  | Power | +5 VDC | B3 | A3 | SD6 |  | I/O |
|  | I | IRQ9 | B4 | A4 | SD5 |  | I/O |
|  | Power | -5 VDC | B5 | A5 | SD4 |  | I/O |
|  | I | DRQ2 | B6 | A6 | SD3 |  | I/O |
|  | Power | -12 VDC | B7 | A7 | SD2 |  | I/O |
|  | I | OWS | B8 | A8 | SD1 |  | I/O |
|  | Power | +12 VDC | B9 | A9 | SD0 |  | I/O |
|  | Ground | GND | B10 | A10 | -I/O CHRDY |  | I |
|  | O | -SMEMW | B11 | A11 | AEN |  | O |
|  | O | -SMEMR | B12 | A12 | SA19 |  | I/O |
|  | I/O | -IOW | B13 | A13 | SA18 |  | I/O |
|  | I/O | -IOR | B14 | A14 | SA17 |  | I/O |
|  | O | -DACK3 | B15 | A15 | SA16 |  | I/O |
|  | I | DRQ3 | B16 | A16 | SA15 |  | I/O |
|  | O | -DACK1 | B17 | A17 | SA14 |  | I/O |
|  | I | DRQ1 | B18 | A18 | SA13 |  | I/O |
|  | I/O | -Ref | B19 | A19 | SA12 |  | I/O |
|  | O | CLK | B20 | A20 | SA11 |  | I/O |
|  | I | IRQ7 | B21 | A21 | SA10 |  | I/O |
|  | I | IRQ6 | B22 | A22 | SA9 |  | I/O |
|  | I | IRQ5 | B23 | A23 | SA8 |  | I/O |
|  | I | IRQ4 | B24 | A24 | SA7 |  | I/O |
|  | I | IRQ3 | B25 | A25 | SA6 |  | I/O |
|  | O | -DACK2 | B26 | A26 | SA5 |  | I/O |
|  | O | T/C | B27 | A27 | SA4 |  | I/O |
|  | O | BALE | B28 | A28 | SA3 |  | I/O |
|  | Power | +5 VDC | B29 | A29 | SA2 |  | I/O |
|  | O | OSC | B30 | A30 | SA1 |  | I/O |
|  | Ground | GND | B31 | A31 | SA0 |  | I/O |

### A.3ii	16-Bit ISA Extension Pinout

|  | I | -MEMCS16 | D1 | C1 | -BHE |  | I/O |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I | -I/OCS16 | D2 | C2 | LA23 |  | I/O |
|  | I | IRQ10 | D3 | C3 | LA22 |  | I/O |
|  | I | IRQ11 | D4 | C4 | LA21 |  | I/O |
|  | I | IRQ12 | D5 | C5 | LA20 |  | I/O |
|  | I | IRQ15 | D6 | C6 | LA19 |  | I/O |
|  | I | IRQ14 | D7 | C7 | LA18 |  | I/O |
|  | 0 | -DACK0 | D8 | C8 | LA17 |  | I/O |
|  | I | DRQ0 | D9 | C9 | -MEMR |  | I/O |
|  | 0 | -DACK5 | D10 | C10 | -MEMW |  | I/O |
|  | I | DRQ5 | D11 | C11 | SD08 |  | I/O |
|  | 0 | -DACK6 | D12 | C12 | SD09 |  | I/O |
|  | I | DRQ6 | D13 | C13 | SD10 |  | I/O |
|  | 0 | -DACK7 | D14 | C14 | SD11 |  | I/O |
|  | I | DRQ7 | D15 | C15 | SD12 |  | I/O |
|  | Power | +5 VDC | D16 | C16 | SD13 |  | I/O |
|  | I | -MASTER | D17 | C17 | SD14 |  | I/O |
|  | Ground | GND | D18 | C18 | SD15 |  | I/O |

## A.4	PCI Bus Pinout

| -12V | B1 | A1 | TRST# |
| --- | --- | --- | --- |
| TCK | B2 | A2 | +12V |
| GROUND | B3 | A3 | RESERVED |
| RESERVED | B4 | A4 | RESERVED |
| +5V | B5 | A5 | +5V |
| +5V | B6 | A6 | INTA# |
| INTB# | B7 | A7 | INTC# |
| INTD# | B8 | A8 | +5V |
| PRSNT1# | B9 | A9 | RESERVED |
| RESERVED | B10 | A10 | +5V |
| PRSNT2# | B11 | A11 | RESERVED |
| GROUND | B12 | A12 | GROUND |
| GROUND | B13 | A13 | GROUND |
| RESERVED | B14 | A14 | RESERVED |
| GROUND | B15 | A15 | RST# |
| CLK | B16 | A16 | +5V |
| GROUND | B17 | A17 | GNT# |
| REQ# | B18 | A18 | GROUND |
| +5V | B19 | A19 | RESERVED |
| AD[31] | B20 | A20 | AD[30] |
| AD[29] | B21 | A21 | RESERVED |
| GROUND | B22 | A22 | AD[28] |
| AD[27] | B23 | A23 | AD[26] |
| AD[25] | B24 | A24 | GROUND |
| RESERVED | B25 | A25 | AD[24] |
| C/BE[3]# | B26 | A26 | IDSEL |
| AD[23] | B27 | A27 | RESERVED |
| GROUND | B28 | A28 | AD[22] |
| AD[21] | B29 | A29 | AD[20] |
| AD[19] | B30 | A30 | GROUND |
| RESERVED | B31 | A31 | AD[18] |
| AD[17] | B32 | A32 | AD[16] |
| C/BE[2]# | B33 | A33 | RESERVED |
| GROUND | B34 | A34 | FRAME# |
| IRDY# | B35 | A35 | GROUND |
| RESERVED | B36 | A36 | TRDY# |
| DEVSEL# | B37 | A37 | GROUND |
| GROUND | B38 | A38 | STOP# |
| LOCK# | B39 | A39 | RESERVED |
| PERR# | B40 | A40 | SDONE |
| RESERVED | B41 | A41 | SBO# |
| SERR# | B42 | A42 | GROUND |
| RESERVED | B43 | A43 | PAR |
| C/BE[1]# | B44 | A44 | AD[15] |
| AD[14] | B45 | A45 | RESERVED |
| GROUND | B46 | A46 | AD[13] |
| AD[12] | B47 | A47 | AD[11] |
| AD[10] | B48 | A48 | GROUND |
| GROUND | B49 | A49 | AD[09] |
| AD[08] | B50 | A50 | C/BE[0]# |
| AD[07] | B51 | A51 | RESERVED |
| RESERVED | B52 | A52 | AD[06] |
| AD[05] | B53 | A53 | AD[04] |
| AD[03] | B54 | A54 | GROUND |
| GROUND | B55 | A55 | AD[02] |
| AD[01] | B56 | A56 | AD[00] |
| RESERVED | B57 | A57 | +5V |
| ACK64# | B58 | A58 | REQ64# |
| +5V | B59 | A59 | +5V |
| +5V | B60 | A60 | +5V |

PCI  Bus Pinout  (Cont.)

## A.5	Direct Memory Access Channels

| Channel | Function | 8-Bit | 16-Bit | Transfer Block Size |
| --- | --- | --- | --- | --- |
| 0 | Spare | 8-bit |  | 64 |
| 1 | SDLC | 8-bit |  | 64 |
| 2 | Floppy Disk | 8-bit |  | 64 |
| 3 | Spare | 8-bit |  | 64 |
| 4 | Cascade for DMA controller 1 |  |  |  |
| 5 | Spare |  | 16-bit | 128 |
| 6 | Spare |  | 16-bit | 128 |
| 7 | Spare |  | 16-bit | 128 |

## A.6	DMA Controller Registers

| Address (Hex) |  | Command Code |
| --- | --- | --- |
| C0 |  | CH-0 base and current address |
| C2 |  | CH-0 base and current word count |
| C4 |  | CH-1 base and current address |
| C6 |  | CH-1 base and current word count |
| C8 |  | CH-2 base and current address |
| CA |  | CH-2 base and current word count |
| CC |  | CH-3 base and current address |
| CE |  | CH-3 base and current word count |
| D0 |  | Read Status Register/Write Command Register |
| D2 |  | Write Request Register |
| D4 |  | Write Single Mask Register Bit |
| D6 |  | Write Mode Register |
| D8 |  | Clear Byte Pointer Flip-Flop |
| DA |  | Read Temporary Register/Write Master Clear |
| DC |  | Clear Master Register |
| DE |  | Write All Mask Register Bits |

## A.7	Page Register Address

| Page Register |  | I/O Address (Hex) |
| --- | --- | --- |
| DMA Channel 0 |  | 0087 |
| DMA Channel 1 |  | 0083 |
| DMA Channel 2 |  | 0081 |
| DMA Channel 3 |  | 0082 |
| DMA Channel 5 |  | 008B |
| DMA Channel 6 |  | 0089 |
| DMA Channel 7 |  | 008A |
| Refresh |  | 008F |

## A.8	System Interrupts

| Interrupt Controllers ( NMI level ) |  |  | Parity or I/O Channel Check |
| --- | --- | --- | --- |
| CTLR 1 | CTLR 2 |  |  |
| IRQ0 |  |  | Timer Output 0 |
| IRQ1 |  |  | Keyboard |
|  |  |  | (Output Buffer Full) |
| IRQ2 |  |  | Interrupt from CTLR 2 |
|  | IRQ8 |  | Real-time Clock Interrupt |
|  | IRQ9 |  | Software Redirected to INT 0AH (IRQ2) |
|  | IRQ10 |  | Reserved |
|  | IRQ11 |  | Reserved |
|  | IRQ12 |  | PS/2 Mouse |
|  | IRQ13 |  | Coprocessor |
|  | IRQ14 |  | Fixed Disk Controller |
|  | IRQ15 |  | Reserved |
| IRQ3 |  |  | Serial Port 2 |
| IRQ4 |  |  | Serial Port 1 |
| IRQ5 |  |  | Parallel Port 2 |
| IRQ6 |  |  | Diskette Controller |
| IRQ7 |  |  | Parallel Port 1 |

# APPENDIX-B

# CONNECTORS  PINOUT

## B.1	Reset Connector (P1)

| Pin | Signal Name |
| --- | --- |
| 1 | Reset |
| 2 | Ground |

## B.2	Turbo LED Connector (P2)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_150 |
| 2 | LED_Turbo- |

## B.3	Keylock Connector (P3)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | Mechanical Key |
| 3 | Ground |
| 4 | Keyboard Inhibit |
| 5 | Ground |

## B.4	Fan Connector (P4)

| Pin | Signal Name |
| --- | --- |
| 1 | +5/+12dc Power |
| 2 | Ground |

## B.5	EPMI/T.B. Switch (P5)

| Pin | Signal Name |
| --- | --- |
| 1 | EPMI |
| 2 | Ground |

## B.6 	Speaker Connector (P6)

| Pin | Signal Name |
| --- | --- |
| 1 | Speaker Data_Out |
| 2 | N.C. |
| 3 | Ground |
| 4 | +5Vdc |

## B.7	Header 7 (P8)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Ground | 1 | 2 | Ground |
| Ground | 3 | 4 | Ground |
| +12Vdc | 5 | 6 | V/IO |
| V/IO | 7 | 8 | V/IO |
| +3.3Vdc | 9 | 10 | +3.3Vdc |
| +3.3Vdc | 11 | 12 | +3.3Vdc |
| VCORE | 13 | 14 | VCORE |
| VCORE | 15 | 16 | VCORE |
| Ground | 17 | 18 | VCORE |
| VCORE | 19 | 20 | VCORE |
| N.C. | 21 | 22 | UPVRM# |
| Sense | 23 | 24 | N.C. |
| Ground | 25 | 26 | Ground |
| +5Vdc | 27 | 28 | +5Vdc |
| +5Vdc | 29 | 30 | +5Vdc |

## B.8	External Battery Connector (P9)

| Pin | Signal Name |
| --- | --- |
| 1 | +3.6Vdc |
| 2 | N.C. |
| 3 | Ground |
| 4 | Ground |

## B.9	IDE LED Connector (P10)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | HD_LED- |

## B.10 	IDE Connector (P11,P12)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Reset | 1 | 2 | Ground |
| Data 7 | 3 | 4 | Data 8 |
| Data 6 | 5 | 6 | Data 9 |
| Data 5 | 7 | 8 | Data 10 |
| Data 4 | 9 | 10 | Data 11 |
| Data 3 | 11 | 12 | Data 12 |
| Data 2 | 13 | 14 | Data 13 |
| Data 1 | 15 | 16 | Data 14 |
| Data 0 | 17 | 18 | Data 15 |
| Ground | 19 | 20 | Key |
| DRQx | 21 | 22 | Ground |
| I/O Write- | 23 | 24 | Ground |
| I/O Read- | 25 | 26 | Ground |
| IOCHRDY | 27 | 28 | BALE |
| DACKx | 29 | 30 | Ground |
| IRQ14/15* | 31 | 32 | IOCS16- |
| Signal Name | Pin | Pin | Signal Name |
| Address 1 | 33 | 34 | N.C. |
| Address 0 | 35 | 36 | Address 2 |
| Chip Select 0- | 37 | 38 | Chip Select 1- |
| Activity | 39 | 40 | Ground |

IDE Connector Pinout ( Cont.)

* Note : For Primary IDE, IRQ 14 is used whereas for Secondary IDE, IRQ 15 is used.

## B.11 	Floppy Connector (P13)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Ground | 1 | 2 | FDHDIN |
| Ground | 3 | 4 | Reserved |
| Key | 5 | 6 | FDEDIN |
| Ground | 7 | 8 | Index- |
| Ground | 9 | 10 | Motor Enable 0 |
| Ground | 11 | 12 | Drive Select 1 |
| Ground | 13 | 14 | Drive Select 0 |
| Ground | 15 | 16 | Motor Enable 1 |
| Ground | 17 | 18 | DIR- |
| Ground | 19 | 20 | STEP- |
| Ground | 21 | 22 | Write Data- |
| Ground | 23 | 24 | Write Gate- |
| Ground | 25 | 26 | Track 00- |
| Ground | 27 | 28 | Write Protect- |
| Ground | 29 | 30 | Read Data- |
| Ground | 27 | 28 | Write Protect- |
| Ground | 29 | 30 | Read Data- |
| Ground | 31 | 32 | Side 1 Select- |
| Ground | 33 | 34 | Disk Change |

## B.12 	Printer Port Connector (P14)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| STROBE- | 1 | 14 | AUTO FEED- |
| Data bit 0 | 2 | 15 | ERROR- |
| Data bit 1 | 3 | 16 | INIT- |
| Data bit 2 | 4 | 17 | SLCT IN- |
| Data bit 3 | 5 | 18 | Ground |
| Data bit 4 | 6 | 19 | Ground |
| Data bit 5 | 7 | 20 | Ground |
| Data bit 6 | 8 | 21 | Ground |
| Data bit 7 | 9 | 22 | Ground |
| ACK- | 10 | 23 | Ground |
| BUSY | 11 | 24 | Ground |
| PE (Paper End) | 12 | 25 | Ground |
| SLCT | 13 | 26 | N.C. |

## B.13	Serial Port Connector (P15,P16)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| DCD | 1 | 2 | RXD |
| TXD | 3 | 4 | DTR |
| Ground | 5 | 6 | DSR |
| RTS | 7 | 8 | CTS |
| RI | 9 | 10 | N.C. |

## B.14	Power Connector (P17,P18)

| Pin | Signal Name |
| --- | --- |
| 1 | Power Good |
| 2 | +5Vdc |
| 3 | +12Vdc |
| 4 | -12V dc |
| 5 | Ground |
| 6 | Ground |
| 7 | Ground |
| 8 | Ground |
| 9 | -5Vdc |
| 10 | +5Vdc |
| 11 | +5Vdc |
| 12 | +5Vdc |

## B.15	PS/2 Mouse Connector (P19)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| +5V dc | 1 | 2 | N.C. |
| N.C. | 3 | 4 | N.C. |
| Data | 5 | 6 | N.C. |
| Ground | 7 | 8 | Clock |

## B.16	Keyboard Connector (KB1)

| Note: KB1 is a DIN5 socket. Pin | Signal Name |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Not used |
| 4 | Ground |
| 5 | VCC |

# APPENDIX-C

# HARDWARE  SETTINGS

## C.1	System Component Map

| Jumper Connectors | Function |
| --- | --- |
| P1 | Reset |
| P2 | Turbo LED |
| P3 | Keylock |
| P4 | Fan connector |
| P5 | EPMI/TB Switch |
| P6 | Speaker |
| P8 | Header 7 |
| P9 | External Battery Connector |
| P10 | IDE LED Connector |
| P11 | Secondary IDE Connector |
| P12 | Primary IDE Connector |
| P13 | Floppy Drive Connector |
| P14 | Printer Port |
| P15 | Serial Port 1 |
| P16 | Serial Port 2 |
| P17 | Power Connector |
| P18 | Power Connector |
| P19 | PS/2 Mouse Connector |
| KB1 | Keyboard Connector |

## C.2 	Layout of RHINO 5  Main Board

.

## C.3	Jumper Settings

All factory settings are marked by * in the following sections.

### C.3.1 	CPU related settings

Header 7 setting

The factory settings are as follows:

P8  (7-9)

P8  (8-10)

P8  (11-13)

P8  (12-14)

The four pieces of shunt connectors are to be left in factory setting. This configuration works well with all Pentium CPUs P54C, P54CQS and P54CS.

For P55C CPU, the processor will be used in connection with an Voltage Regulator Module (VRM). All the four shunt connectors installed in P8 should be removed. An VRM should be plugged in P8 instead.

TOP VIEW OF HEADER 7 (BESIDE THE CPU SOCKET)

30                          29

28                          27

26                          25

24                          23

22                          21

20                          19

18                          17

16                          15

14                          13

12                          11

10                           9

8                            7

6                            5

4                            3

2                            1

Figure 4   Top View of Header 7

CPU Clock

| JP29 | JP30 | CPU Clock | CPU |
| --- | --- | --- | --- |
| 1-2 | 2-3 | 50MHz | Intel P54C-75 Cyrix M1 |
| 2-3 | 1-2 | 60MHz | Intel P54C-90 Intel P54C-120 Intel P54CS-150 |
| 1-2 | 1-2 | 66MHz | Intel P54C-100 Intel P54C-133 |

Clock multiplier

| JP2 | JP39 | P54C P54CQS | P54CS B-STEP | P54CS C-STEP | P55C |
| --- | --- | --- | --- | --- | --- |
| 1-2 | 1-2 | reserved | reserved | x2.5 | x2.5 |
| 2-3 | 1-2 | reserved | reserved | reserved | reserved |
| 1-2 | 2-3 | x2 | x2 | x2 | x2 |
| 2-3 | 2-3 | x1.5 | x1.5 | x1.5 | reserved |

Multiplier selection based on CPU type

|  |  | JP2 | JP39 |
| --- | --- | --- | --- |
| Intel | P54C-75 | 2-3 | 2-3 |
| Intel | P54C-90 | 2-3 | 2-3 |
| Intel | P54C-100 | 2-3 | 2-3 |
| Intel | P54C-120 | 1-2 | 2-3 |
| Intel | P54C-133 | 1-2 | 2-3 |
| Cyrix | M1 | 1-2 | 2-3 |

Power saving mode

| CPU | JP11 |
| --- | --- |
| Intel | 1-2 |
| Cyrix | 2-3 |

CPU Fan voltage select

|  | JP1 |
| --- | --- |
| +5V | 1-2 * |
| +12V | 2-3 |

### C.3.2 	External cache (L2 cache) setting

Async Cache/ Sync Cache selection

|  | JP42 |
| --- | --- |
| Asynchronous Cache | 1-2 |
| Synchronous Cache | 2-3 |

Asynchronous L2 cache size

|  | JP3 | JP4 | JP10 |
| --- | --- | --- | --- |
| 256KB | 2-3 | 1-2 | 2-3 |
| 512KB | 1-2 | 1-2 | 1-2 |
| 1MB | 1-2 | 2-3 | 1-2 |

Voltage source for asynchronous L2 cache

|  | JP12 | JP13 | JP14 | JP15 |
| --- | --- | --- | --- | --- |
| +5V supply | 2-3 | 2-3 | 2-3 | 2-3 |
| +3.3V supply | 1-2 | 1-2 | 1-2 | 1-2 |

Async SRAM device requirement

256KB cache

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U8 | 32Kx8 | 15ns | 5V I/0 |
| data RAM | U2 - U6, U9 - U11 | 32Kx8 | 15ns | 3.3V I/O |

512KB cache

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U8 | 32Kx8 | 15ns | 5V I/0 |
| data RAM | U2 - U6, U9 - U11 | 64Kx8 | 15ns | 3.3V I/O |

1MB cache

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U8 | 32Kx8 | 15ns | 5V I/0 |
| data RAM | U2 - U6, U9 - U11 | 128Kx8 | 15ns | 3.3V I/O |

Synchronous SRAM module

Install a 32Kx8  5V tag SRAM at location U8.

### C.3.3	 DRAM

DRAM Type

|  | JP28 |
| --- | --- |
| Fast page mode DRAM(60ns or 70ns) | 1-2 |
| EDO DRAM(60ns or 70ns) | 2-3 |

EDO DRAM SIMMs are NOT allowed to be mixed with Fast-page mode DRAM SIMMs.

### C.3.4 	 Peripheral setup

PS/2 MOUSE support

|  | JP17 | JP18 | JP19 |
| --- | --- | --- | --- |
| Enabled | 1-2 * | 2-3 * | 2-3 * |
| Disabled | 2-3 | 1-2 | 1-2 |

Printer port interrupt

|  | JP31 |
| --- | --- |
| IRQ5 | 1-2 |
| IRQ7 | 2-3 * |

DMA request for printer port ECP mode

|  | JP32 |
| --- | --- |
| DRQ1 | 1-2 * |
| DRQ3 | 2-3 |

DMA acknowledge for printer port ECP mode

|  | JP33 |
| --- | --- |
| DACK1 | 1-2 * |
| DACK3 | 2-3 |

Installation of a 2nd FLOPPY drive in PS/2 mode

|  | JP36 |
| --- | --- |
| 2nd drive absent | 1-2 * |
| 2nd drive present | 2-3 |

### C.3.5 	Miscellaneous

Power Good Signal select

|  | JP40 |
| --- | --- |
| External Power Good | 1-2 * |
| On-board Power Good | 2-3 |

CMOS discharge

|  | JP21 |
| --- | --- |
| Clear CMOS | 1-2 |
| Preserve CMOS | 2-3 * |

Battery select

|  | JP20 |
| --- | --- |
| External Battery | 1-2 |
| On-board Battery | 2-3 * |

Reserved Jumpers

| JP22 | 2-3 |
| --- | --- |
| JP27 | 2-3 PCI clock = CPUclock/2 |