AWARD Bios Manual for RHINO 5

REV 1.1

The system BIOS provides an interface for the software to

control the hardware and is recorded in the ROM chip. Upon

power up, it will perform a self test to ensure the system

is functional.It will then initialize the chipset with the

table values stored in the CMOS RAM.

Then it proceeds to load the disk operating system and you

can start to work with your applications.

The BIOS has a built-in Setup program that allows users to

modify the basic system configuration.  The information is

stored in the CMOS RAM.

Entering Setup

==============

Power on the computer, when the message below appears briefly

at the bottom of the screen during the POST (Power On Self Test),

press <Del> key or simultaneously press <Ctrl>, <Alt>, and

<Esc> keys.

PRESS DEL TO ENTER SETUP

If the message disappears before you respond and you still wish

to enter Setup, restart the system to try again by turning it

OFF then ON or pressing the "RESET" button on the system case.

You may also restart by simultaneously press <Ctrl>, <Alt>, and

<Delete> keys. If you do not press the keys at the correct time

and the system does not boot, an error message will be displayed

and you will again be asked to,

PRESS F1 TO CONTINUE, DEL TO ENTER SETUP

Control Keys

============

Up Arrow        Move to previous item

Down Arrow      Move to next item

Left Arrow      Move to the item in the left hand

Right Arrow     Move to the item in the right hand

Esc key         Main Menu - Quit and not save changes into CMOS

Status Page Setup Menu and Option Page Setup Menu

- Exit current page and return to Main Menu

PgUp key        Increase the numeric value or make changes

PgDn key        Decrease the numeric value or make changes

F1 key          General help, only for Status Page Setup Menu

and Option Page Setup Menu

F2 key          Change color from total 16 colors

F3 key          Calendar, only for Status Page Setup Menu

F4 key          Reserved

F5 key          Restore the previous CMOS value from CMOS, only

for Option Page Setup Menu

F6 key          Load the default CMOS value from BIOS default table,

only for Option Page Setup Menu

F7 key          Load the default

F8 key          Reserved

F9 key          Reserved

F10 key         Save all the CMOS changes, only for Main Menu

Getting Help

============

Main Menu

---------

The on-line description of the highlighted setup function is

displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the

appropriate keys to use and the possible selections for the

highlighted item.  To exit the Help Window press <Esc>.

The Main Menu

=============

Once you enter the BIOS CMOS Setup Utility, the Main Menu will

appear on the screen. The Main Menu allows you to select from

ten setup functions and two exit choices. Use arrow keys to select

the items and press <Enter> to accept or enter the sub-menu.

ROM PCI/ISA BIOS (2A5UNO09)

CMOS Setup Utility

Award Software, Inc.

---------------------------------------------------------------------------

STANDARD CMOS SETUP             PASSWORD SETTING

BIOS FEATURES SETUP             IDE HDD AUTO DETECTION

CHIPSET FEATURES SETUP          SAVE & EXIT SETUP

POWER MANAGEMENT SETUP          EXIT WITHOUT SAVE

PCI CONFIGURATION SETUP

LOAD SETUP DEFAULTS

---------------------------------------------------------------------------

STANDARD CMOS SETUP

-------------------

This setup page includes all the items in a standard BIOS.

BIOS FEATURES SETUP

-------------------

This setup page includes all the items of Enhanced Features.

CHIPSET FEATURES SETUP

----------------------

This setup page includes all the items of chipset special features.

POWER MANAGEMENT SETUP

------------------------

This setup page provides all the select items for power saving

feature & time event.

PCI CONFIGURATION SETUP

------------------------

This setup page provides some selection items for PCI setup.

LOAD SETUP DEFAULTS

-------------------

SETUP defaults are the most appropriate values (optimized) of the

system parameters with which the system would be in highest performance.

However, you may change the parameter value through the Option

Page Setup Menu.

PASSWORD SEETING

----------------

User can create a password in the system. During boot up the system,

user must enter the password in boot up screen.

IDD HDD Auto Detection

----------------------

Automatically configure hard disk parameters.

SAVE & EXIT SETUP

------------------

Save CMOS value changes to CMOS and exit setup.

EXIT WITHOUT SAVE

-----------------

Abandon all CMOS value changes and exit setup.

STANDARD CMOS SETUP MENU

========================

The items in Standard CMOS Setup Menu are divided into 10

categories. Each category includes no, one or more than one

setup items. Use the arrow keys to highlight the item and then

use the <PgUp> or <PgDn> keys to select the value you want in

each item.

ROM PCI/ISA BIOS (2A5UNO09)

CMOS SETUP UTILITY

Award Software, Inc.

----------------------------------------------------------------------------

Date            :   Mon, 01 Jan 1992

Time            :   00:00:00

TYPE  SIZE  CYLS.  HEADS  PRECOMP  LANDZ  SECTORS  MODE

Primary master  : USER  541   1120   16     65535    1119   59       NOR

Primary slave   : none  0     0      0      0        0      0        --

Secondary master: none  0     0      0      0        0      0        --

Secondary slave : none  0     0      0      0        0      0        --

Drive A         :   1.2MB, 5-1/4 in.

Drive B         :   1.44MB, 3-1/2 in.

Base Memory     : 640K

Video           :   EGA/VGA                   Extended Memory : 15360K

Halt on Error   :   ALL Errors                Other Memory    : 384K

Total Memory    : 16384K

------------------------------------------------------------------------

DATE

----

The date format is <day>, <date> <month> <year>, Press <F3>

to show the calendar.

DAY     The day, from Sun to Sat, determined

by the BIOS and is display-only

DATE    The date, from 1 to 31 (or the maximum allowed in

the month)

MONTH   The month, Jan through Dec

YEAR    The year, from 1900 through 2099

Time

----

The time format is <hour> <minutes> <second>. The time is

calculated based on the 24-hour military-time clock. For

example, 1 p.m. is 13:00:00.

Primary/Secondary  Master/Slave

-------------------------------

The category identifies the types of master drive or slave

that has been installed in the computer. There are 46 pre-defined

types and a user definable type. Type 1 to Type 46 are pre-defined.

Type User is user-definable.

Press PgUp or PgDn to select a numbered hard disk type or type

the number and press <Enter>. Note that the specifications of

your drive must match with the drive table. The hard disk will

not work properly if you enter improper information for this

category. If your hard disk drive type is not matched or listed,

you can use Type User to define your own drive type manually.

If you select Type User, related information is asked to be

entered to the following items.  Enter the information directly

from the keyboard and press <Enter>.  Those information should be

provided in the documentation from your hard disk vendor or the

system manufacturer.

CYLS.      Number of Cylinders

HEADS      Number of heads

PRECOMP    Write Precom

LANDZONE   Landing Zone

SECTORS    Number of Sectors

If the user selects the Type "Auto", the hard disk will be detected

and checked prior to system-boot.

If a hard disk has not been installed select NONE and press

<Enter>.

Drive A Type / Drive B Type

---------------------------

The category identifies the types of floppy disk drive A or

drive B that has been installed in the computer.

No floppy drive installed

360K, 5.25 in   5-1/4 inch PC-type standard drive; 360 kilobyte

capacity

1.2M, 5.25 in   5-1/4 inch AT-type high-density drive; 1.2

megabyte capacity

720K, 3.5 in    3-1/2 inch double-sided drive; 720 kilobyte

capacity

1.44M,3.5 in    3-1/2 inch double-sided drive; 1.44 megabyte

capacity

2.88M, 3.5 in   3-1/2 inch double-sided drive; 2.88 megabyte

capacity

Video

-----

The category selects the type of adapter used for the primary

system monitor that must matches your video display card and

monitor. Although secondary monitors are supported, you do not

have to select the type in Setup.

EGA/VGA         Enhanced Graphics Adapter/Video Graphics Array.

For EGA, VGA, SEGA, or PGA monitor adapters

CGA 40Color     Graphics Adapter, power up in 40 column mode

CGA 80Color     Graphics Adapter, power up in 80 column mode

MONO            Monochrome adapter, includes high resolution

monochrome adapters

Error Halt

----------

The category determines whether the computer will stop if an

error is detected during power up.

No Errors  ==> Whenever the BIOS detects a non-fatal error the

system will be stopped and you will be prompted

All Errors ==> The system boot will not be stopped for any error

that may be detected

All, But Keyboard ==> The system boot will not stop for a keyboard

error; it will stop for all other errors

All, But Diskette ==> The system boot will not stop for a disk

error; it will stop for all other errors

All, But Disk / Key ==> The system boot will not stop for a keyboard

or disk; it will stop for all other errors.

It is recommended that you leave the setting on the default "ALL ERRORS"

MEMORY

------

The category is display-only which is determined by POST (power

on self test) of BIOS.

Base Memory

The POST of the BIOS will determine the amount of base (or

conventional) memory installed in the system.

Extended Memory

The BIOS determines how much extended memory is present during

the POST. This is the amount of memory located above 1MB in the

CPU's memory address map.

Other Memory

This refers to the memory located in the 640K to 1024K address

space. This is memory that can be used for different applications.

DOS uses this area to load device drivers to keep as much base

memory free for application programs. This area is mostly used for

Shadow RAM.

BIOS FEATURES SETUP

===================

Virus Warning

-------------

This category flashes on the screen. During and after the system

boots up, any attempt to write to the boot sector or partition

table of the hard disk drive will halt the system and the following

error message will appear.

!  WARNING  !

Disk boot sector is to be modified

Type "Y" to accept write or "N" to abort write

Award software, Inc.

Enabled    Activate automatically when the system boots up

causing a warning message to appear when anything

attempts to access the boot sector or hard disk

partition table

Disabled   No warning message to appear when anything attempts

to access the boot sector or hard disk partition table

CPU Internal Cache / External Cache

-----------------------------------

These two categories speed up memory access. The default value

is enabled.

Quick Power On Self Test

------------------------

This category speeds up Power On Self Test (POST) after you power

on the computer.  If it is set to Enable, BIOS will shorten the

check list during POST.

Boot Sequence

-------------

This category determines which drive computer searches first for

the disk operating system (i.e. DOS).  Default value is A, C.

Swap Floppy Drive

-----------------

This option exchanges the identities of floppy disk drive A and B

WITHOUT the need to re-install the floppy cable.

Boot Up NUMlock System

----------------------

The default is enabled for NUumlock the keybaord switch.

Boot Up System Speed

--------------------

This option is set to High normally.

Gate A20 Option

---------------

The default value is selected to fast.

Typematic Rate Setting

------------------------

This determines the typematic rate. The default value is select

enable.

Typematic Rate (Chars/Sec)

--------------------------

6-30 characters per second can be selected.

Typematic Delay (Msec)

----------------------

The delay refers to the time elapsed between the first and second characters

are displayed when the key is held.  250-1000 msec can be selected.

Security Option

---------------

This category allows you to limit access to the system and Setup,

or just to Setup.

System  The system will not boot and access to Setup will be

denied if the correct password is not entered at the prompt.

Setup   The system will boot, but access to Setup will be denied if

the correct password is not entered at the prompt

Note :  To disable security, select PASSWORD SETTING at Main menu

and then you will be asked to enter password.  Do not type

anything and just press <Enter>, it will disable security.

Once the security is disabled, the system will boot and you

can enter Setup freely.

IDE Second Channel Control

--------------------------

The on board PCI IDE controller provided the second channel of IDE

interface. Normally, set this option to "enabled".

Video BIOS Shadow

-----------------

It determines whether video BIOS will be copied to RAM. Video

Shadow will increase the video speed.

C8000 - EFFFF Shadow

--------------------

These categories determine whether optional ROM will be copied

to RAM by 16K bytes.

CHIPSET FEATURES SETUP

======================

Most items in this menu are configured in factory. Changing the

defaults may lead to malfunction.

A few items can be modified optionally by the users:

SRAM

----

SRAM type can be set to Async or Sync.

The default value is Async. This applies to the case when

8 pcs. of DIP SRAMs and 1 pc of tag RAM are installed in

the nine 32-pin sockets next to the CPU.

If you install a Synchronous SRAM module in the 160-pin socket

in location DM1, select the option "Sync"

SYNC SRAM support

-----------------

There are two options: Pipeline,

Standard

(This option is valid provided that you have installed a Sync

SRAM module AND you have set SRAM type = Sync in CMOS setup.)

Check the specifications of the Sync SRAM module purchased

before selecting the "Sync SRAM support" options.

Cache Read Burst

----------------

The wait-state should be set as follows:

3-2-2-2 for Async SRAM

3-1-1-1 for  Sync SRAM

Cache Write Burst

-----------------

The wait-state should be set as follows:

4-2-2-2 for Async SRAM

3-1-1-1 for  Sync SRAM

Onboard FDC controller

----------------------

OPTIONS - Enabled/Disabled

Onboard Serial Port 1

---------------------

OPTIONS - 3F8/IRQ4

2F8/IRQ3

3E8/IRQ4

2E8/IRQ3

Disabled

Onboard Serial Port 2

---------------------

OPTIONS - 3F8/IRQ4

2F8/IRQ3

3E8/IRQ4

2E8/IRQ3

Disabled

Onboard Parallel Port

---------------------

OPTIONS - 378/IRQ7

278/IRQ5

3BC/IRQ7

Disabled

POWER MANAGEMENT SETUP

======================

For compliance with the EPA Energy Star program, various energy

conservation methods can be integrated into your computer. The

objective is to automatically reduce power to devices like the

fixed disk and CRT when they are idle, and to restore their full

operation (with minimum delay ) upon detection of activity. This

is accomplished through use of idle timers and event monitoring

techniques. Within this utility, you can select how rapidly the

timer will expire, or you can disable them altogether if you find

they are interfering with your computer.

Industry Standards:

APM  - Intel/Microsoft Advanced Power Management

ATA  - AT Attachments Specification (IDE Drive)

DPMS - Display Power Management Signaling (VESA/Video)

PCI CONFIGURATION SETUP

=======================

Most items in this menu are configured in factory. Changing the

defaults may lead to malfunction.

A few items can be optionally modified by the users:

Bank 0 DRAM type

----------------

You can select Paged or EDO

Bank 1 DRAM type

----------------

You can select Paged or EDO

On-Chip PCI/IDE

---------------

The on-board PCI IDE controller can be disabled or enabled.

IDE HDD Block mode

------------------

IDE HDD Block mode improves the disk transfer rate by initiating

mult-sector transfers instead of data transfers in single-sector

units. Most of the new IDE drives have this feature.

LOAD BIOS DEFAULTS

==================

This category optimizes the system option for CHIPSET FEATURES

SETUP & BIOS FEATURES SETUP.

Password Setting

================

When you select this function, the following message will appear

at the center of the screen to assist you in creating a password.

ENTER PASSWORD

Type the password, up to eight characters, and press <Enter>.

The password typed now will clear any previously entered password

from CMOS memory.  You will be asked to confirm the password. Type

the password again and press <Enter>.  You may also press <Esc> to

abort the selection and not enter a password.

To disable password, just press <Enter> when you are prompted to

enter password. A message will confirm the password being disabled.

Once the password is disabled, the system will boot and you can

enter Setup freely.

PASSWORD DISABLED

If you select System at Security Option of BIOS Features Setup

Menu, you will be prompted for the password every time the system

is rebooted or any time you try to enter Setup. If you select

Setup at Security Option of BIOS Features Setup Menu, you will

be prompted only when you try to enter Setup.

IDE HDD AUTO DETECTION

======================

Most IDE drives can be interrogated for their parameters. To

select this option, the desired parameters will appear in their

respective fields.

BIOS Reference - POST Message

=============================

When the BIOS encounters an error that requires the user to correct

something, either a beep code will sound or a message will be

displayed in a box in the middle of the screen and the message

PRESS F1 TO CONTINUE, DEL TO ENTER SETUP will be shown in the

information box at the bottom.

POST Beep

---------

Currently there is only one beep code in BIOS. This code indicates

that a video error has occurred and the BIOS cannot initialize the

video screen to display any additional information. This beep code

consists of a single long beep followed by two short beeps.

Error Messages

--------------

One or more of the following messages may be displayed if the BIOS

detects an error during the POST.

CMOS BATTERY HAS FAILED

CMOS battery is no longer functional.  It should be replaced.

CMOS CHECKSUM ERROR

Checksum of CMOS is incorrect.  This can indicate that CMOS has

become corrupt. This error may have been caused by a weak battery.

Check the battery and replace if necessary.

DISK BOOT FAILURE, INSERT SYSTEM DISK AND PRESS ENTER

No boot device was found.  Insert a system disk into Drive A: and

press <Enter>. If you assumed the system would boot from the hard

drive, make sure the controller is inserted correctly and all

cables are properly attached.  Also be sure the disk is formatted

as a boot device.  Then reboot the system

DISKETTE DRIVES OR TYPES MISMATCH ERROR - RUN SETUP

Type of diskette drive installed in the system is different from

the CMOS definition. Run Setup to reconfigure the drive type

correctly.

DISPLAY SWITCH IS SET INCORRECTLY

Display switch on the motherboard can be set to either monochrome

or color. This indicates the switch is set to a different setting

than indicated in Setup. Determine which setting is correct, and

then either turn off the system and change the jumper, or enter

Setup and change the VIDEO selection.

DISPLAY TYPE HAS CHANGED SINCE LAST BOOT

Since last powering off the system, the display adapter has been

changed.  You must configure the system for the new display type.

ERROR ENCOUNTERED INITIALIZING HARD DRIVE

Hard drive cannot be initialized. Be sure the adapter is installed

correctly and all cables are correctly and firmly attached.  Also

be sure the correct hard drive type is selected in Setup.

ERROR INITIALIZING HARD DISK CONTROLLER

Cannot initialize controller. Make sure the cable is correctly and

firmly installed in the drive. Be sure the correct hard drive type is

selected in Setup. Also check to see if any jumper needs to be set

correctly on the hard drive.

FLOPPY DISK CONTROLLER ERROR OR NO CONTROLLER PRESENT

Cannot find or initialize the floppy drive controller. Make sure

the cable is installed correctly and firmly. If there are no

floppy drives installed, be sure the Diskette Drive selection in

Setup is set to NONE.

KEYBOARD ERROR OR NO KEYBOARD PRESENT

Cannot initialize the keyboard. Make sure the keyboard is attached

correctly and no keys are being pressed during the boot.

If you are purposely configurating the system without a keyboard,

set the error halt condition in Setup to HALT ON ALL, BUT KEYBOARD.

This will cause the BIOS to ignore the missing keyboard and continue

the boot.

Memory Address Error at ...

Indicates a memory address error at a specific location.  You can

use this location along with the memory map for your system to find

and replace the bad memory chips.

Memory Parity Error at ...

Indicates a memory parity error at a specific location.  You can

use this location along with the memory map for your system to find

and replace the bad memory chips.

MEMORY SIZE HAS CHANGED SINCE LAST BOOT

Memory has been added or removed since the last boot. Enter the

new memory size in the memory fields.

Memory Verify Error at ...

Indicates an error verifying a value already written to memory.

Use the location along with your system's memory map to locate

the bad chip.

OFFENDING ADDRESS NOT FOUND

This message is used in conjunction with the I/O CHANNEL CHECK

and RAM PARITY ERROR messages when the segment that has caused

the problem cannot be isolated.

OFFENDING SEGMENT

This message is used in conjunction with the I/O CHANNEL CHECK

and RAM PARITY ERROR messages when the segment that has caused

the problem cannot be isolated.

PRESS A KEY TO REBOOT

This will be displayed at the bottom screen when an error occurs

that requires you to reboot. Press any key and the system will

reboot.

PRESS F1 TO DISABLE NMI, F2 TO REBOOT

When BIOS detects a Non-maskable Interrupt condition during boot,

this will allow you to disable the NMI and continue to boot, or

you can reboot the system will the NMI enabled.

RAM PARITY ERROR - CHECKING FOR SEGMENT

Indicates a parity error in Random Access Memory.

BIOS Reference - BIOS Default Drive Table

=========================================

Type  Size  Cyl.  Heads  Sectors  Write  LandZ  Example

pre.          Model

1     10    306   4      17       128    305    TEAC SD510,

MMI 112, 5412

2     20    615   4      17       300     615   Seagate ST225,

ST4026

3     30    615   6      17       300     615

4     62    940   8      17       512     940

5     46    940   6      17       512     940

6     20    615   4      17       None    615   Seagate ST125,

Tandon TM262

7     30    462   8      17       256     511

8     30    733   5      17       None    733    Tandon TM703

9     112   900   15     17       None    901

10    20    820   3      17       None    820

11    35    855   5      17       None    855

12    49    855   7      17       None    855

13    20    306   8      17       128     319     Disctron 526,

MMI M125

14    42    733   7      17       None    733

15          Reserved

16    20    612   4      17       0       663     Microscience HH725,

Syquest 3250,3425

17    40    977   5      17       300     977

18    56    977   7      17       None    977

19    59    1024  7      17       512     1023

20    30    733   5      17       300     732

21    42    733   7      17       300     732

22    30    306   5      17       300     733     Seagate ST4038

23    10    977   4      17       0       336

24    40    1024  5      17       None    976     Seagate ST4051

25    76    1224  9      17       None    1023    Seagate ST4096

26    71    1224  7      17       None    1223    Maxtor 2085

27    111   1224  11     17       None    1223    Maxtor 2140,

Priam S14

28    152   1024  15     17       None    1223    Maxtor 2190,

Priam S19

29    68    1024  8      17       None    1023    Maxtor 1085,

Micropolis 1325

30    93    918   11     17       None    1023    Maxtor 1105, 1120

4780

31    83    925   11     17       None    1023    Maxtor 1170

32    69    1024  9      17       None    926     CDC9415

33    85    1024  10     17       None    1023

34    102   1024  12     17       None    1023

35    110   1024  13     17       None    1023

36    119   1024  14     17       None    1023

37    17    1024  2      17       None    1023

38    136   1024  16     17       None    1023

39    114   918   15     17       None    1023     Maxtor 1140,4380

40    40    820   6      17       None    820      Seagate ST251

41    42    1024  5      17       None    1023     Seagate 4053,

Miniscribe 3053/

6053

42    65    1024  5      26       None    1023     Miniscribe 3053/

6053 RLL

43    40    819   6      17       None    852      Miniscribe 3650

44    61    809   6      26       None    852      Miniscribe 3675

45    100   776   8      33       None    775      Conner CP3104

46    203   684   16     38       None    685      Conner CP3204

USER

MEMORY WRITER PROGRAM

=====================

The RHINO 5 main board contains a flash memory device, allowing updates

of BIOS content. The utility diskette contains a flash memory writer

program called AWDFLASH.EXE and a ROM BIOS binary file R5-95016.BIN

Step 1

------

Make sure that the CPU is running in real mode.

This can be achieved by allowing the system to boot from a

bootable floppy disk (DOS-formatted). There should be no

config.sys and autoexec.bat in the floppy disk.

Step 2

------

Insert the utility diskette into the floppy drive, and at the DOS prompt

key in the command:

AWDFLASH

and hit the <RETURN> key.

Step 3

------

Enter the name of the ROM BIOS binary file with extension .BIN

Step 4

------

Respond to the questions in the following way:

Do you want to save BIOS? (y/n) ----- n

Are you sure to program? (y/n)  ----- y

Step 5

------

Wait until the program finishes writing to the flash memory device.

(1FF00 O.K.)

Power down the machine.

Turn on the machine once again.