## B.3	Jumper Settings

All factory settings are marked by * in the following sections.

### B.3.1 	CPU related settings

CPU Voltage Core Selection

RHINO 6VX has on board voltage regulators to directly support Pentium CPU such as P54C, P54CQS, P54CS, Cyrix 6x86 and AMD K5 CPU will need a Core voltage of 3.5V. The voltage selection for Core voltage is done by JP40~JP45, JP49 as follows :

| JP40~45 | JP49 | CPU Core voltage | CPU Type |
| --- | --- | --- | --- |
| 1-2* | 2-3* | 3.5V | Intel P54C, P54CQS, P54CS, Cyrix 6x86, AMD K5 |

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !