USER'S  MANUAL

RHINO 6VX

Pentium PCI Local Bus Motherboard

Copyright Notice

Copyright 1996 Ocean Office Automation Ltd.

All rights reserved.

Ocean Office Automation Ltd.

5th Floor, Kader Industrial Building,

22 Kai Cheung Road, Kowloon Bay,

Kowloon, Hong Kong.

All of the information contained in this manual is copyrighted and all rights reserved.  No part of this document, in whole or in part, may be reproduced or copied in any form without prior consent in writing from Ocean Office Automation Ltd..

___________________________________________________________________

Limitations of  Liability

While the information in this manual has been carefully reviewed and is believed to be accurate, Ocean Office Automation Ltd. (Ocean) assumes no responsibility in the event that any inaccuracies are found.  In no event shall Ocean be held liable for any loss or expenses whether directly or indirectly caused by the support materials provided with this product.  It is further acknowledged that Ocean is under no obligation to update the manual or to notify purchasers of any forthcoming updates.

___________________________________________________________________

Trademarks

Intel i486, Pentium and Intel are registered trademarks of Intel Corporation.

Octek is a registered trademark of  Ocean Office Automation Ltd.

XT, AT, PS/2, OS/2, & IBM are registered trademarks of International Business Machines Corporation.

All other brand and product names mentioned in this manual are trademarks or copyrights of their respective holders.

___________________________________________________________________

REVISION 1.31  Sept. 1996

Table of Contents

Chapter 1: System Overview

1.1      General Specification Overview

1.2      Central Processing Unit

1.3      External Cache Subsystem

1.4      DRAM Subsystem	6

1.5      PCI Bus

1.6      Universal Serial Bus	6

1.7      Super I/O Subsystem

1.8      Input/Output Subsystem

1.9      BIOS Subsystem	7

Chapter 2: Installation and Upgrade

2.1      CPU Installation

2.2      Fast Page mode / EDO DRAM Installation

2.3      Control of System Speed

2.4      Reset CMOS	11

Appendix-A: Connectors Pinout

A.1      Reset Connector (P1)	12

A.2      Turbo LED Connector (P2)	12

A.3      Speaker Connector (P3)	12

A.4      Keylock Connector (P4)	12

A.5      HD LED Connector (P5)	13

A.6      External Battery Connector (P9)	13

A.7      Sleep Switch (P10)	13

A.8      USB Connector (P24)	13

A.9      Power Connector (P18,P19)	14

A.10    PS/2 Mouse Connector (P22)	14

A.11    Keyboard Connector (KB1)	14

Appendix-B: Hardware Settings

B.1      System Component Map	15

B.2      Layout of RHINO 6VX Main Board	16

B.3      Jumper Settings	17

B.3.1      CPU related settings	17

B.3.2      External cache (L2 cache) setting	18

B.3.3      Peripheral setup	20

B.3.4      Miscellaneous	20

# CHAPTER 1

# SYSTEM OVERVIEW

OCTEK RHINO 6VX is a powerful PC machine offering unparalleled performance. The advanced external cache system implemented meets the demand of the most memory-intensive applications today. The support of Synchronous Cache RAM (pipelined burst SRAM) and EDO DRAM results in better performance when compared with traditional asynchronous SRAM and Fast Page Mode DRAM. With the Pentium processor and a high bandwidth 32-bit PCI expansion bus, the I/O  bottleneck that plagues most PC systems is now removed.

All of the I/O is integrated inside the mainboard to further facilitate system installation. The built-in IDE can support up to 4 fast Enhanced IDE devices whereas it can  also support up to two floppy drives. The  mainboard also includes two serial ports and one parallel port as the basic configuration for end user.  All that is needed is just a VGA card plugged into a PCI or ISA slot to complete the whole system.

## 1.1	General Specifications Overview

Processor:

w Processor Type		Intel  Pentium CPU including

P54C, P54CQS, P54CS,AMD K5

and Cyrix 6x86 CPU.

w External CPU clock		50/55/60/66 MHz

Chipset:

w Motherboard  chipset		Intel 82430VX

w Super I/O chipset		665/669 super I/O chipset

Cache Architecture:

w Internal Cache 	 	8KB data cache

8KB code cache

w External Cache		256/512KB Synchronous 					Pipelined Burst SRAM

Memory Subsystem:

w DRAM SIMM sockets		4 x 72 pin  4MB / 8MB / 16MB / 					32MB DRAM modules

w Max. Memory Size		128MB

w DRAM Type			Fast Page Mode or EDO DRAM

w Enhancement			Mix of Fast Page Mode or EDO 					DRAM supported

Input/Output Subsystem

w PCI bus slots		             2 x 32-bit PCI Bus slots (3 masters)

w ISA bus slots			3 x 16-bit ISA slots

w shared bus slots		1 x 32 bit PCI bus slot (master) OR 1 				 x 16-bit ISA slot

w I/O bus speed			Up to 33MHz (PCI bus)

Integrated IDE, Super I/O Subsystem

w IDE support			Chipset built-in PCI IDE support up 				to 4 IDE Drives

w On board I/O 			One Floppy Port supporting 2 floppy 				drives of 360KB / 720KB / 1.44MB/ 				2.88MB capacity.

Two serial ports (16550 Fast UART 				compatibles)

One parallel Port (Standard, ECP, 				EPP support)

PS/2 Mouse

w PS/2 Mouse			Supports PS/2 Mouse through a 1x4 				header

Power Management

w Green functions		Support various Power Management 				schemes

Sleep Switch for power saving

BIOS Subsystem

w BIOS Shadowing		Shadow RAM for System and Video 				BIOS

w BIOS Features		Built-in setup, Power-on self test, 				Drive table optimization, User-					definable drive types, Password 					protection, Shadowing options

Plug & Play / BIOS Update

w Plug & Play BIOS		Support Plug & Play for easy 					installation

w Flash EEPROM		Use Flash EEPROM (1M bits)  to 				allow easy BIOS update

USB Devices

w USB Devices   		Interfaced with both host and hub

control functions

2 programmable USB ports

System Support Functions

w System functions		7 DMA channels, 16 level interrupts, 				Programmable timers

w Support functions		Fast A20 gate and Fast Reset

w Clock				Enhanced real time clock/calendar 				with battery back-up

Other Features

w Power good			On board power good signal 					generation

w 3.3V supply			On board 3.3V supply to eliminate 				the need for special power supply for

3.3V component e.g. CPU, SRAM.

Maximum rating : 30 W.

w Switches			Reset, Keylock  switches, Sleep 				Switch.

w Size				8.5" (W) x 11" (L)

## 1.2	Central Processing Unit

The Pentium processor is a superscalar, pipelined CPU that provides next generation performance for the existing PC compatible software.

The processor is equipped with an 8K code cache and an 8K data cache . Each cache is organized in a 2-way set-associative architecture, offering higher hit rates. The data cache can be configured in write-back or write-through modes.

The internal numeric coprocessor  is redesigned to give three times the performance of the 80486 FPU. It is backward compatible with i486DX math coprocessor and complying to ANSI/IEEE standard 754-1985.

## 1.3	External Cache Subsystem

The external cache of RHINO 6VX is organized in a direct-mapped  configuration with sizes of  256KB, or 512KB in write-back mode using synchronous SRAM (pipelined burst SRAM).

There are two options to support 256KB SRAM :

(1)	256KB on board (2 pcs of 32Kx32 SRAM).

To support  a total of 512KB SRAM, we should have :

(1)	512KB on board (4 pcs of 32Kx32 SRAM).

(2)	256KB on board and 256KB via cache module.

In addition, the presence and size of synchronous SRAM used is auto-detected by BIOS.

## 1.4	DRAM Subsystem

The main memory in RHINO 6VX is organized as a 64-bit memory pool. Both fast-page mode and EDO DRAMs are supported.

EDO DRAM stands for Extended Data Out DRAM and is designed to improve the DRAM read performance. EDO DRAM holds the memory data valid until the next CAS# falling edge, unlike standard fast page mode DRAM which tri-states the memory data when CAS# is deasserted to precharge for the next cycle. As a result, the CAS# precharge can now overlap with the data valid time to allow CAS# to negate earlier while still satisfying the memory data valid window time.

In addition, mix of EDO/ Fast Page Mode DRAM could be used on RHINO 6VX. The presence of EDO/ Fast Page Mode DRAM is auto-detected by BIOS without any related jumper setting.

## 1.5	PCI Bus

The Peripheral Component Interconnect (PCI) local bus was specified to establish a high performance local bus standard. It is a 32-bit wide bus supporting burst transactions

The PCI local bus implemented in RHINO 6VX is fully compliant to v2.1 specification. Up to four PCI bus masters are supported.

1.6	Universal Serial Bus

The Universal Serial Bus (USB) is a cable bus that supports data exchange between a host computer and a wide range of simultaneously accessible peripherals.

## 1.7	Super I/O Subsystem

To facilitate system implementation, included in RHINO 6VX are two fast Enhanced IDE ports that can dramatically boost the system performance if fast  IDE drives are used.

Furthermore, various formats floppy drives are also supported through the floppy connector on board. The motherboard is also equipped with two serial ports (16550 Fast UART compatibles) and one parallel port that operates in standard, ECP or EPP mode.

In addition, RHINO 6VX is designed to support the PS/2 mouse using specialized keyboard controller and the use of interrupt IRQ 12. Nevertheless, user can disable the PS/2 mouse function by means of jumpers.

## 1.8 	Input/Output Subsystem

To allow greater system feasibility, RHINO 6VX has four ISA bus expansion connectors  and three PCI expansion connectors. One of the expansion slots is shared by connectors that will accommodate either an ISA or a PCI expansion, but not both at the same time. Therefore, up to six expansion slots can be populated on RHINO 6VX . Furthermore, all the PCI slots can accept PCI bus master cards.

## 1.9 	BIOS Subsystem

RHINO 6VX System BIOS is stored in Flash EEPROM (1 M bits) which allows easy upgrade through the utility found inside the diskette shipped with RHINO 6VX.

# CHAPTER 2

# INSTALLATION AND UPGRADE

## 2.1	CPU Installation

The CPU is composed of pins that can easily be bent during installation, causing permanent damage to the processor.  It is therefore very important that you make sure the pins are straight before installing the CPU onto the SPGA socket located on RHINO 6VX (refer to layout for exact location).  To properly align the CPU  with the socket, align pin 1 of the CPU ( with a notch at the corner) with pin 1 of the CPU socket as demonstrated below.

Figure  1   Socket 7 for Pentium CPU

## 2.2	Fast Page mode / EDO DRAM Installation

There are four SIMM sockets located on the RHINO 6VX motherboard, marked BANK 1 and BANK 2.  BANK 1 and BANK 2 are counted starting from right to left consecutively. Start to install the SIMM modules ( IN PAIRS) from either Bank 1 or Bank 2. Depending on how your memory is configured, you may not need to use all the memory banks. Either x32 or x36 of  72 pins SIMM can be installed.

Bank 2  Bank 1

1   1  1  1 11 1

Figure 2     SIMM  Sockets Location

Below is a memory configuration table for RHINO 6VX.

| BANK 2 |  | BANK 1 |  | Total |
| --- | --- | --- | --- | --- |
| -------- | -------- | 512Kx32 | 512Kx32 | 4MB |
| -------- | -------- | 1Mx32 | 1Mx32 | 8MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 16MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 32MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 64MB |
| 512Kx32 | 512Kx32 | -------- | -------- | 4MB |
| 1Mx32 | 1Mx32 | -------- | -------- | 8MB |
| 2Mx32 | 2Mx32 | -------- | -------- | 16MB |
| 4Mx32 | 4Mx32 | -------- | -------- | 32MB |
| 8Mx32 | 8Mx32 | -------- | -------- | 64MB |
| 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 8MB |
| 1Mx32 | 1Mx32 | 512Kx32 | 512Kx32 | 12MB |
| 512Kx32 | 512Kx32 | 1Mx32 | 1Mx32 | 12MB |
| 1Mx32 | 1Mx32 | 1Mx32 | 1Mx32 | 16MB |
| 2Mx32 | 2Mx32 | 512Kx32 | 512Kx32 | 20MB |
| 2Mx32 | 2Mx32 | 1Mx32 | 1Mx32 | 24MB |
| 512Kx32 | 512Kx32 | 2Mx32 | 2Mx32 | 20MB |
| 1Mx32 | 1Mx32 | 2Mx32 | 2Mx32 | 24MB |
| 2Mx32 | 2Mx32 | 2Mx32 | 2Mx32 | 32MB |
| 4Mx32 | 4Mx32 | 512Kx32 | 512Kx32 | 36MB |
| 4Mx32 | 4Mx32 | 1Mx32 | 1Mx32 | 40MB |
| 4Mx32 | 4Mx32 | 2Mx32 | 2Mx32 | 48MB |
| 512Kx32 | 512Kx32 | 4Mx32 | 4Mx32 | 36MB |
| 1Mx32 | 1Mx32 | 4Mx32 | 4Mx32 | 40MB |
| 2Mx32 | 2Mx32 | 4Mx32 | 4Mx32 | 48MB |
| 4Mx32 | 4Mx32 | 4Mx32 | 4Mx32 | 64MB |
| 8Mx32 | 8Mx32 | 512Kx32 | 512Kx32 | 68MB |
| 8Mx32 | 8Mx32 | 1Mx32 | 1Mx32 | 72MB |
| 8Mx32 | 8Mx32 | 2Mx32 | 2Mx32 | 80MB |
| 8Mx32 | 8Mx32 | 4Mx32 | 4Mx32 | 96MB |
| 512Kx32 | 512Kx32 | 8Mx32 | 8Mx32 | 68MB |
| 1Mx32 | 1Mx32 | 8Mx32 | 8Mx32 | 72MB |
| 2Mx32 | 2Mx32 | 8Mx32 | 8Mx32 | 80MB |
| 4Mx32 | 4Mx32 | 8Mx32 | 8Mx32 | 96MB |
| 8Mx32 | 8Mx32 | 8Mx32 | 8Mx32 | 128MB |

Table 1      Memory Configuration Table

## 2.3	Control of System Speed

System speed can be controlled by keyboard. To change the speed by keyboard, use the minus sign (-) and the plus sign (+).  Press <control> + <alt> + <“-”> for slow speed and  <control> + <alt> + <“+”> for fast speed.

## 2.4	Reset CMOS

If the setting of the system setup is done improperly, it may make the system malfunction. If this happens, turn off the power and set jumper JP7 to 2-3 to clear the internal CMOS status register. Wait at least 5 seconds to ensure that the CMOS content has been completely cleared.

Next, set the jumper JP7 back to 1-2 and turn on the power.  The BIOS will find the CMOS status register is  reset and will regard the setup information invalid, so it will prompt you to correct the information.

# APPENDIX-A

# CONNECTORS  PINOUT

## A.1	Reset Connector (P1)

| Pin | Signal Name |
| --- | --- |
| 1 | Reset |
| 2 | Ground |

## A.2	Turbo LED Connector (P2)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_150 |
| 2 | LED_Turbo- |

## A.3 	Speaker Connector (P3)

| Pin | Signal Name |
| --- | --- |
| 1 | Speaker Data_Out |
| 2 | N.C. |
| 3 | Ground |
| 4 | +5Vdc |

## A.4	Keylock Connector (P4)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | Mechanical Key |
| 3 | Ground |
| 4 | Keyboard Inhibit |
| 5 | Ground |

## A.5	HD LED Connector (P5)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_330 |
| 2 | HD_LED- |
| 3 | HD_LED- |
| 4 | Pull_Up_330 |

## A.6	External Battery Connector (P9)

| Pin | Signal Name |
| --- | --- |
| 1 | +3.6Vdc |
| 2 | N.C. |
| 3 | Ground |
| 4 | Ground |

## A.7	Sleep  Switch (P10)

| Pin | Signal Name |
| --- | --- |
| 1 | EPMI |
| 2 | Ground |

## A.8	USB Connector (P24)

| Pin | Signal Name | Pin | Signal Name |
| --- | --- | --- | --- |
| 6 | VCC | 1 | VCC |
| 7 | Port 1- | 2 | Port 0- |
| 8 | Port 1+ | 3 | Port 0+ |
| 9 | Ground | 4 | Ground |
| 10 | NC | 5 | NC |

## A.9	Power Connector (P18,P19)

| Pin | Signal Name |
| --- | --- |
| 1 | Power Good |
| 2 | +5Vdc |
| 3 | +12Vdc |
| 4 | -12V dc |
| 5 | Ground |
| 6 | Ground |
| 7 | Ground |
| 8 | Ground |
| 9 | -5Vdc |
| 10 | +5Vdc |
| 11 | +5Vdc |
| 12 | +5Vdc |

## A.10	PS/2 Mouse Connector (P22)

| Pin | Signal Name |
| --- | --- |
| 1 | +5V dc |
| 2 | GND |
| 3 | MDATA |
| 4 | MCLK |

## A.11	Keyboard Connector (KB1)

| Pin | Signal Name |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Not used |
| 4 | Ground |
| 5 | VCC |

# APPENDIX-B

# HARDWARE  SETTINGS

## B.1	System Component Map

| Jumper Connectors | Function |
| --- | --- |
| P1 | Reset |
| P2 | Turbo LED |
| P3 | Speaker |
| P4 | Keylock |
| P5 | IDE LED Connector |
| P9 | External Battery Connector |
| P10 | Sleep Connector |
| P11 | Primary IDE Connector |
| P12 | Secondary IDE Connector |
| P13 | Serial Port 2 |
| P14 | Floppy Connector |
| P15 | Printer Port Connector |
| P17 | Serial Port 1 |
| P18 | Power Connector |
| P19 | Power Connector |
| P22 | PS/2 Mouse Connector |
| P24 | USB Connector |
| KB1 | Keyboard Connector |

## B.2	Layout of RHINO 6VX Main Board

## B.3	Jumper Settings

All factory settings are marked by * in the following sections.

### B.3.1 	CPU related settings

CPU Voltage Core Selection

RHINO 6VX has on board voltage regulators support  Intel Pentium CPU such as P54C, P54CQS, P54CS, Cyrix 6x86 and AMD K5 CPU.  The voltage selection for Core voltage is done by JP52 as follows :

| JP52 | CPU Core voltage | CPU Type |
| --- | --- | --- |
| 3-4* | 3.3V | Intel P54C, P54CQS, P54CS. |
| 5-6 | 3.5V(VRE) | Cyrix 6x86, AMD K5. |

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

CPU Type

| JP4 | JP5 | JP8 | JP9 | JP10 | CPU Clock | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- |
| 1-2 2-3 1-2 | 1-2 1-2 1-2 | 2-3 2-3 2-3 | 2-3 2-3 2-3 | 1-2 1-2 1-2 | 50MHz | Intel P54C-75 Cyrix 6x86-P120+ (100MHz) AMD K5-PR75 |
| 2-3 | 1-2 | 1-2 | 1-2 | 1-2 | 55MHz | Cyrix 6x86-P133+ (110MHz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 1-2 2-3 | 1-2 1-2 2-3 2-3 1-2 1-2 1-2 1-2 | 1-2 1-2 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 60MHz | Intel P54C-90 Intel P54C-120 Intel P54C-150 Intel P54C-180 Cyrix 6x86-P150+ (120MHz) AMD K5-PR90 AMD K5-PR120 (90MHz) AMD K5-PR150 (120MHz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 2-3 | 66MHz | Intel P54C-100 Intel P54C-133 Intel P54C-166 Intel P54C-200 Cyrix 6x86-P166+ (133MHz) AMD K5-PR100 AMD K5-PR133 (100MHz) |

### B.3.2 	External cache (L2 cache) setting

Synchronous SRAM

256K Configuration :

1. 256K cache using on board 256K SRAM

| SRAM | Location | Type | Speed | Voltage |
| --- | --- | --- | --- | --- |
| Tag SRAM | U10 | 8Kx8 / 32Kx8 | 15ns | 5V I/O |
| Data SRAM | U41, U43 | 2 pcs 32Kx32 Pipelined Burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |

| JP16* | JP50* |
| --- | --- |
| OPEN | 2-3 |

512K Configuration :

1. 512K cache using On board 512K SRAM

(4pcs PB SRAM)

| SRAM | Location | Type | Speed | Voltage |
| --- | --- | --- | --- | --- |
| Tag SRAM | U10 | 32Kx8 | 15ns | 5V I/O |
| Data SRAM | U40, U41, U42, U43 | 4 pcs 32Kx32 Pipelined Burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |

| JP16 | JP50 |
| --- | --- |
| 2-3 | 1-2 |

2. 512K cache using on board 256K and 256K Cache

Module w/o Tag SRAM

| SRAM | Location | Type | Speed | Voltage |
| --- | --- | --- | --- | --- |
| Tag SRAM | U10 | 32Kx8 | 15ns | 5V I/O |
| Data SRAM (1st 256KB) | U41, U43 | 2 pcs of 32Kx32 pipelined burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |
| Data RAM (2nd 256KB) | DM1 | 256K Cache Module w/o Tag | 7ns (Clock to o/p valid) | 3.3V I/O |

| JP16 | JP50 |
| --- | --- |
| 2-3 | 1-2 |

### B.3.3	 Peripheral setup

PS/2 MOUSE support

|  | JP31 | JP32 | JP33 |
| --- | --- | --- | --- |
| Enabled | 1-2 * | 2-3 * | 2-3 * |
| Disabled | 2-3 | 1-2 | 1-2 |

### B.3.4 	Miscellaneous

Power Good Signal select

|  | JP27 |
| --- | --- |
| External Power Good | 1-2 * |
| On-board Power Good | 2-3 |

CMOS discharge

|  | JP7 |
| --- | --- |
| Preserve CMOS | 1-2 * |
| Clear CMOS | 2-3 |

Battery select

|  | JP6 |
| --- | --- |
| On-board Battery | 1-2 * |
| External Battery | 2-3 |

Reserved Jumpers

| JP15 | 1-2 |
| --- | --- |
| JP30 | 1-2 |