USER'S  MANUAL

RHINO 8

Pentium PCI Local Bus Motherboard

Copyright Notice

Copyright 1996 Ocean Office Automation Ltd.

All rights reserved.

Ocean Office Automation Ltd.

5th Floor, Kader Industrial Building,

22 Kai Cheung Road, Kowloon Bay,

Kowloon, Hong Kong.

All of the information contained in this manual is copyrighted and all rights reserved.  No part of this document, in whole or in part, may be reproduced or copied in any form without prior consent in writing from Ocean Office Automation Ltd..

___________________________________________________________________

Limitations of  Liability

While the information in this manual has been carefully reviewed and is believed to be accurate, Ocean Office Automation Ltd. (Ocean) assumes no responsibility in the event that any inaccuracies are found.  In no event shall Ocean be held liable for any loss or expenses whether directly or indirectly caused by the support materials provided with this product.  It is further acknowledged that Ocean is under no obligation to update the manual or to notify purchasers of any forthcoming updates.

___________________________________________________________________

Trademarks

Intel i486, Pentium and Intel are registered trademarks of Intel Corporation.

Octek is a registered trademark of  Ocean Office Automation Ltd.

OPTi is a registered trademark of OPTi Inc.

XT, AT, PS/2, OS/2, & IBM are registered trademarks of International Business Machines Corporation.

All other brand and product names mentioned in this manual are trademarks or copyrights of their respective holders.

REVISION 1.1  Sep, 1996

Table of Contents

Chapter 1: General Specification	1

Chapter 2:  Installation and Upgrade

2.1      CPU Installation	3

2.2      Fast Page mode / EDO DRAM Installation	3

2.3      Control of System Speed	4

2.4      Reset CMOS	4

2.5      System Component Map	5

Chapter 3: Connector Pinout

3.1      Layout of RHINO 8  Main Board	6

3.2      Connectors Pinout	7

3.2.1   Reset Connector (P1)	7

3.2.2   Turbo LED Connector (P2)	7

3.2.3   Speaker Connector (P4)	7

3.2.4   Keylock Connector (P5)	7

3.2.5   IDE LED Connector (P6)	8

3.2.6   External Battery Connector (P8)	8

3.2.7  Power Connector (P14)	8

3.2.8   PS/2 Mouse Connector (P16)	8

Chapter 4: Hardware Settings

4.1      Jumper Settings	9

4.1.1   CPU Setting	9

4.1.2   CPU Core Voltage	9

4.1.3   DRAM Type	9

4.1.4   PS/2 MOUSE support	10

4.1.5   COMS discharge	10

4.1.6   Battery select	10

## CHAPTER 1   GENERAL SPECIFICATION

Processor:

w Processor Type			Intel  Pentium CPU, AMD K5

CPU,  Cyrix 6x86 CPU and

future upgraded CPU

w External CPU clock		50/60/66 Mhz

Chipset:

w Motherboard  chipset		Opti Viper-M PCI/ISA Pentium 					motherboard chipset

w Super I/O chipset			Advanced super I/O chipset

Cache Architecture:

w Internal Cache 	 		8KB/16KB data cache

8KB/16KB code cache

w External Cache			On-board 256KB Sync. Pipeline

Burst SRAM

Memory Subsystem:

w DRAM SIMM sockets		4 x 72 pin  4MB / 8MB / 16MB / 					32MB DRAM modules

w Max. Memory Size		128MB

w DRAM Type			Fast Page Mode or EDO DRAM 					supported

Input/Output Subsystem

w PCI bus slots			2 x 32-bit PCI Bus slots (2 masters)

w ISA bus slots			3 x 16-bit  ISA slots

w Shared bus slots			1x 32 bit PCI bus slot (master ) or 1

x 16-bit ISA slot

w I/O bus speed			Up to 33MHz (PCI bus)

Integrated IDE, Super I/O Subsystem

w IDE support			Chipset built-in PCI IDE support up to 					4 IDE Drives

w On board I/O 			One Floppy Port supporting 2 floppy 					drives of 360K / 720K / 1.2M/ 1.44M/ 					2.88M capacity.

Two serial ports (16550  Fast UART                                         				compatibles)

One parallel Port (Standard, ECP,

EPP)

PS/2 Mouse

w PS/2 Mouse			Supports PS/2 Mouse through a 1x4 					header

Power Management

w Green functions			Support various Power Management 					schemes

BIOS Subsystem

w BIOS Shadowing			Shadow RAM for System and Video 					BIOS

w BIOS Features			Built-in setup, Power-on self test, 					Drive table optimization, User-					definable drive types, Password 					protection, Shadowing options

Plug & Play / BIOS Update

w Plug & Play BIOS		Support Plug & Play for easy 					installation

w Flash EEPROM			Use Flash EEPROM (1M bit)  to 					allow easy BIOS update

System Support Functions

w System functions			7 DMA channels, 16 level interrupts, 					Programmable timers

w Support functions		Fast A20 gate and Fast Reset

w Clock				Enhanced real time clock/calendar 					with battery back-up

Other Features

w 3.3V supply			On board 3.3V supply to eliminate 					the need for special power supply for

3.3V component e.g. CPU, SRAM.

w Switches			EPMI, Reset, Keylock  switches

w Size				8.5" (W) x 11" (L)

CHAPTER 2   INSTALL & UPGRADE

## 2.1   CPU Installation

The CPU is composed of pins that can easily be bent during installation, causing permanent damage to the processor.  It is therefore very important that you make sure the pins are straight before installing the CPU onto the SPGA socket located on RHINO 8 (refer to layout for exact location).  To properly align the CPU  with the socket, align pin 1 of the CPU ( with a notch at the corner) with pin 1 of the CPU socket as demonstrated below.

Figure  1   Socket 7 for Pentium CPU

## 2.2   Fast Page mode / EDO DRAM Installation

There are two memory banks located on the RHINO 8 motherboard, marked Bank1 & Bank2. They  are counted starting from right to left consecutively. Start to install the SIMM modules ( IN PAIRS) from the right hand side first. Depending on how your memory is configured, you may not need to use all the memory banks. Either X32 or X36 of  72 pins SIMM can be installed.

BANK2

BANK 2

BANK

1

BANK

1

Figure 2     SIMM  Sockets Location

To insert the modules into the banks follow these steps:

- Locate the notch (near pin 1) on the corner of the module.

- Hold the module so that the notch is at the bottom left corner.

- Insert the bottom edge of the module into the bank at an angle, then pull the module in the direction towards yourself so that it is locked into place by the latches located on the sides of the bank.  The latches should be locked tightly and the holes in the module should be aligned with the tabs on the bank.

1

72

®  Near KB

Connector

Figure 3      Proper Memory Module Installation

## 2.3    Control of System Speed

System speed can be controlled by keyboard. To change the speed by keyboard, use the minus sign (-) and the plus sign (+).  Press <control> + <alt> + <“-”> for slow speed and  <control> + <alt> + <“+”> for fast speed.

## 2.4    Reset CMOS

If the setting of the system setup is done improperly, it may make the system malfunction. If this happens, turn off the power and set jumper JP29 to 2-3 to clear the internal CMOS status register.  Next, set the jumper JP29 back to 1-2 and turn on the power.  The BIOS will find the CMOS status register is  reset and will regard the setup information invalid, so it will prompt you to correct the information.

## 2.5    System Component Map

| Jumper Connectors | Function |
| --- | --- |
| P1 | Reset |
| P2 | Turbo LED |
| P4 | Speaker |
| P5 | Keylock |
| P6 | IDE LED |
| P8 | External Battery |
| P9 | Primary IDE |
| P10 | Secondary IDE |
| P11 | Floppy Drive |
| P12 | Printer Port |
| P14 | Power Connector |
| P15 | Serial Port 1 |
| P16 | PS/2 Mouse |
| P17 | Serial Port 2 |
| KB1 | Keyboard Connector |

CHAPTER 3   CONNECTORS PINOUT

3.1    Layout of RHINO 8  Main Board

# 3.2   CONNECTORS  PINOUT

## 3.2.1	Reset Connector (P1)

| Pin | Signal Name |
| --- | --- |
| 1 | Reset |
| 2 | Ground |

## 3.2.2	Turbo LED Connector (P2)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_150 |
| 2 | LED_Turbo- |

## 3.2.3	Speaker Connector (P4)

| Pin | Signal Name |
| --- | --- |
| 1 | Speaker Data_Out |
| 2 | N.C. |
| 3 | Ground |
| 4 | +5Vdc |

## 3.2.4	Keylock Connector (P5)

| Pin | Signal Name |
| --- | --- |
| 1 | +5V |
| 2 | Mechanical Key |
| 3 | Ground |
| 4 | Keyboard Inhibit |
| 5 | Ground |

## 3.2.5	IDE LED Connector (P6)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | HD_LED- |

## 3.2.6	External Battery Connector (P8)

| Pin | Signal Name |
| --- | --- |
| 1 | +3.6Vdc |
| 2 | N.C. |
| 3 | Ground |
| 4 | Ground |

## 3.2.7	Power Connector (P14)

| Pin | Signal Name |
| --- | --- |
| 1 | Power Good |
| 2 | +5Vdc |
| 3 | +12Vdc |
| 4 | -12V dc |
| 5 | Ground |
| 6 | Ground |
| 7 | Ground |
| 8 | Ground |
| 9 | -5Vdc |
| 10 | +5Vdc |
| 11 | +5Vdc |
| 12 | +5Vdc |

## 3.2.8	PS/2 Mouse Connector (P16)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | Ground |
| 3 | MDATA |
| 4 | MClock |

## CHAPTER 4   HARDWARE SETTING

4.1     Jumper Setting

All factory settings are marked by * in the following sections.

### 4.1.1    CPU  Settings

| JP2 | JP7 | JP17 | JP18 | CPU Clock | CPU Type |
| --- | --- | --- | --- | --- | --- |
| 2-3 | 2-3 | 2-3 | 1-2 |  | Intel P54C-75 |
| 1-2 | 2-3 | 2-3 | 1-2 | 50 MHz | Cyrix 6x86-P120+(100MHz) |
| 2-3 | 2-3 | 2-3 | 1-2 |  | AMD- K5 - PR75 (75MHz) |
| 2-3 | 2-3 | 1-2 | 2-3 |  | Intel P54C-90 |
| 1-2 | 2-3 | 1-2 | 2-3 |  | Intel P54C-120 |
| 1-2 | 1-2 | 1-2 | 2-3 | 60 MHz | Intel P54C-150 |
| 2-3 | 1-2 | 1-2 | 2-3 |  | Intel P54C-180 |
| 1-2 | 2-3 | 1-2 | 2-3 |  | Cyrix 6x86-P150+(120MHz) |
| 2-3 | 2-3 | 1-2 | 2-3 |  | AMD-K5-PR90 (90MHz) |
| 2-3 | 2-3 | 1-2 | 1-2 |  | Intel P54C-100 |
| 1-2 | 2-3 | 1-2 | 1-2 |  | Intel P54C-133 |
| 1-2 | 1-2 | 1-2 | 1-2 | 66 MHz | Intel P54C-166 |
| 2-3 | 1-2 | 1-2 | 1-2 |  | Intel P54C-200 |
| 1-2 | 2-3 | 1-2 | 1-2 |  | Cyrix 6x86-P166+(133MHz) |
| 2-3 | 2-3 | 1-2 | 1-2 |  | AMD-K5-PR100 (100MHz) |

Note:  JP17 & JP18 are Clock frequency select while JP2 &  JP7 are Clock multiple.

4.1.2    CPU Core Voltage

| JP11 | CPU Core Voltage |
| --- | --- |
| 1-2 | 3.3V |
| 3-4 | 3.5V (VRE) |
| 5-6 | Reserved |
| 7-8 | Reserved |

4.1.3   DRAM Type

|  | JP25 |
| --- | --- |
| EDO DRAM(60ns or 70ns) | 1-2 |
| Fast page mode DRAM(60ns or 70ns) | 2-3 * |

4.1.4   PS/2 MOUSE support

|  | JP35 | JP38 | JP39 |
| --- | --- | --- | --- |
| Enabled | 2-3 | 1-2 | 2-3 |
| Disabled | 1-2 * | 2-3 * | 1-2 * |

4.1.5    CMOS discharge

|  | JP29 |
| --- | --- |
| Normal CMOS | 1-2 * |
| Clear CMOS | 2-3 |

4.1.6    Battery select

|  | JP21 |
| --- | --- |
| On-board Battery | 1-2 * |
| External Battery | 2-3 |