#### Add the USB Devices and Update Appendix-B, B.2 CPU related settings by changing CPU voltage core selection.

USB Devices

w USB Devices			Interfaced with both host and hub

control functions

2 programmable USB ports

USB Connector (P24)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| VCC | 6 | 1 | VCC |
| Port 1- | 7 | 2 | Port 0- |
| Port 1+ | 8 | 3 | Port 0+ |
| Ground | 9 | 4 | Ground |
| NC | 10 | 5 | NC |

#### B.2	Layout of RHINO 9 Main Board

## B.3	Jumper Settings

### All factory settings are marked by * in the following sections.

B.3.1	CPU related settings

CPU Voltage Core Selection

RHINO 9 has on board 2.8V and 3.5V (VRE) regulators to directly support  Intel Pentium P55C CPU. The P55C will specially need a 2.8V CPU Core voltage in order to function properly. Others Pentium CPU such as P54C, P54CQS, P54CS, Cyrix 6x86 CPU and AMD K5-PR75 - PR100 will need a Core voltage of 3.5V. The voltage selection for Core voltage is done by JP9, JP10, JP11, JP15, JP17 and JP18 as follows :

| CPU Core voltage | JP9 | JP10 | JP11 | JP15 | JP17 | JP18 | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.8V | 2-3 | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | Intel P55C CPU |
| 3.5V (VRE) | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 | Intel P54C, P54CQS or Intel P54CS,P54CTB or Cyrix 6x86 CPU or AMD K5-PR75 - PR100 |

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

CPU CLOCK

| JP2 | JP3 | JP8 | JP25 | JP28 | JP29 | CPU Clock | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2 2-3 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 2-3 2-3 2-3 | 2-3 2-3 2-3 | 50MHz | Intel P54C-75 Cyrix 6x86-P120+ (100Mhz) AMD K5-PR75 (75Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 60MHz | Intel P54C-90 Intel P54C-120 Intel P54C-150 Intel P54C-180 Cyrix 6x86-P150+ (120Mhz) AMD K5-PR90 (90Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 66MHz | Intel P54C-100 Intel P54C-133 Intel P54C-166 Intel P54C-200 Cyrix 6x86-P166+ (133Mhz) AMD K5-PR100 (100Mhz) |

CPU Fan voltage select

|  | JP1 |
| --- | --- |
| +5V | 1-2 * |
| +12V | 2-3 |